3GPP TSG-RAN WG2 Meeting #127bis	R2-2409016

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM measurement gaps/restrictions related enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Next, we list the agreements from the last RAN1 meeting on Rel-19 XR enhancements:

Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

In this paper we discuss potential impacts and implications to RAN2.

# 2	Discussion

## 2.1	Gap/restriction skipping timeline

The gap/restriction timeline for explicit indication by DCI to skip a particular gap/restriction has been heavily debated in RAN1. Different timelines were discussed, and UE processing time was one of them. However, RAN1 was eventually not able to reach an agreement and has tasked RAN4 to discuss and decide on such value(s). This in practice requires the introduction in the specifications of a minimum time offset between the last symbol of the PDCCH carrying the DCI and the start of the corresponding skipped gap/restriction occasion indicated by the DCI.

From MAC perspective, within a gap/restriction occasion, the MAC entity does not (with some exceptions during the RA procedure):

-	perform the transmission of HARQ feedback, SR, and CSI;

-	report SRS;

-	transmit on UL-SCH;

-	monitor the PDCCH;

-	receive on DL-SCH.

Therefore, to allow the UE to transmit and/or receive during a gap/restriction indicated as skipped via DCI, the minimum time offset described above should at least account for the UE preparation time. Especially PUSCH preparation requires processing at the UE. For example, UE PUSCH preparation time can be in the range between approximately 200 ms to 700 ms, depending on e.g. UE capability, SCS, and assuming no BWP switching (see TS 38.214, clause 6.4). If the minimum time offset (indicated with X in Figure 2.2-1) is smaller than the UE PUSCH preparation time, the UE may not be able to transmit on (at least some of) the UL CG resources within the duration of the gap/restriction (see 
Figure 2.2-1).

Observation 1: A UE is only required to transmit on UL CG resources overlapping with a skipped gap/restriction if the time between the last symbol of the PDCCH carrying the DCI format indicating to skip the gap/restriction and the first symbol of the UL CG resources overlapping with the gap/restriction is larger than or equal to the PUSCH preparation time as specified in TS 38.214, clause 6.4.

Proposal 1: The minimum time offset specified in RAN4 should at least be larger than the UE preparation time for DL feedback and UL transmission. More specifically, UL preparation procedure time for UL-SCH, HARQ feedback, SR, CSI and SRS should be considered.

Figure 2.1-1: PUSCH processing time consideration

## 2.2	Impacts of gap/restriction skipping on DRX operation

Another aspect that RAN2 needs to discuss is the interaction between measurement gap skipping and DRX operation, e.g. according to current procedure if the UE is not in active time, the UE would not be monitoring PDCCH even if a measurement gap is indicated as skipped. Inactivity timer might keep the UE awake, but usually it is not configured that long to cover offset plus the gap as otherwise it may impact the UE power consumption whenever a new transmission is scheduled.

Besides, if a measurement gap occasion overlaps with or is too close to the OnDuration, there might be no opportunity during which the UE monitors PDCCH to indicate gap skipping. As the measurement configuration needs to take SSB occasions into account while OnDuration mainly concerns traffic patterns and QoS requirements, it is not possible to always align them to make sure measurement occasions come at least offset after OnDuration.

Proposal 2: interaction of measurement gap skipping and DRX operation should be addressed to ensure DRX and measurement skipping work as intended.

## 2.3	Impacts of gap/restriction skipping on other Rel-19 enhancements

In the scheduling enhancement agenda items, it has been agreed to introduce additional priority for the LCHs with delay critical data for LCP enhancements and support multiple BS/RT report for DSR. The definition of delay-critical data may need to be revisited in line with the agreed LCP and DSR enhancements. For example, for delay-aware LCP, the solution agreed in RAN2 is to increase the priority of a logical channel when the LCH is with delay-critical data. In Rel-18, data is determined as delay-critical by comparing the remaining time till PDCP discard timer expiry to a configured threshold value. However, in proximity of a measurement gap or a time window with scheduling restrictions, the time available for scheduling the data may be significantly less that what indicated by the remaining time. Also in light of the RRM measurement gaps/scheduling restrictions related enhancements introduced in Rel-19 (where only some measurement gaps / windows with scheduling restrictions may be skipped), we propose that RAN2 discusses the potential impact that the overlap of the remaining time with a gap/restriction may have to the definition of delay-critical data. This may impact both delay-aware LCP and enhanced DSR triggering.

Figure 2.3-1: measurement gap skipping impact on remaining time

Proposal 3: RAN2 discusses the potential impact that the overlap of the remaining time with a gap/restriction may have to the definition of delay-critical data and in relation to delay-aware LCP and DSR triggering enhancements.

# 3	Conclusion

Measurement skipping implications to RAN2 is discussed in this contribution with the following proposals proposed:

Proposal 1: The minimum time offset specified in RAN4 should at least be larger than the UE preparation time for DL feedback and UL transmission. More specifically, UL preparation procedure time for UL-SCH, HARQ feedback, SR, CSI and SRS should be considered.

Proposal 2: interaction of measurement gap skipping and DRX operation should be addressed to ensure DRX and measurement skipping work as intended.

Proposal 3: RAN2 discusses the potential impact that the overlap of the remaining time with a gap/restriction may have to the definition of delay-critical data and in relation to delay-aware LCP and DSR triggering enhancements.