3GPP TSG-RAN WG2 Meeting #127bis	R2-2409017

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	LCP Enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The updated WID has included the following objectives for scheduling enhancements:

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In this contribution we discuss some of the FFSs related to delay-aware LCP enhancements based on the following agreements in RAN2 #126:

LCP enhancements – LCH prioritization - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

# 2	Discussion

In WG meeting #126, RAN2 agreed to use additional priority configured to LCHs in case of LCHs with delay-critical data. The following FFSs on LCH prioritization were identified:

- whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR)

On the first FFS, the agreement that “LCP prioritization within a logical channel will not be considered in RAN2 discussions” in practices rules out the possibility to only apply the LCH priority to delay-critical data within the LCH.

Proposal 1: the additional priority configured to LCHs in case of LCHs with delay-critical data is applied to the whole LCH.

Besides, it could be enough to decide whether to use the adjusted priority for a LCH only upon initiation of the LCP procedure based on whether there is any data that is with remaining time below the configure threshold for LCH priority adjustment.  To simplify the LCP procedure, no further priority change for any LCH during the LCP procedure, even if the data whose remaining time was below the configured threshold for some LCH has been all included, or there is some other LCH with new data whose remaining time becomes below threshold.

Proposal 2: which priority is to be used for a LCH is decided only upon initiation of the LCP procedure based on whether there is any data that is with remaining time below the configure threshold for LCH priority adjustment, without further change during the LCP procedure.

On the second FFS, DSR is likely triggered before an UL grant is issued and LCP is performed. Therefore, we think that there could be benefits if the remaining time threshold(s) used for delay-aware LCP is different (e.g., smaller) than the remaining time threshold used for triggering the DSR. It should also be noticed that a UE supporting delay-aware LCP may not necessarily support DSR, which would anyway require separate thresholds for DSR triggering and for delay-aware LCP.

Proposal 3: support configuration of separate remaining time threshold(s) for delay-aware LCP (i.e. no dependency on the one used for DSR triggering).

Related to configuration of the additional LCH priority for LCHs with delay-critical data, in our views, it could be beneficial to define multiple remaining time thresholds, and corresponding LCH priorities. It would assist the UE to manage the LCHs priorities as LCHs will be sorted based on their remaining time values.

Proposal 4: Support configuration of multiple remaining time thresholds and corresponding LCH priorities per LCH for delay-aware LCP.

# 3	Conclusion

In this contribution, we discussed the remaining issues on LCP enhancements for XR with the following proposals made:

Proposal 1: the additional priority configured to LCHs in case of LCHs with delay-critical data is applied to the whole LCH.

Proposal 2: which priority is to be used for a LCH is decided only upon initiation of the LCP procedure based on whether there is any data that is with remaining time below the configure threshold for LCH priority adjustment, without further change during the LCP procedure.

Proposal 3: support configuration of separate remaining time threshold(s) for delay-aware LCP (i.e. no dependency on the one used for DSR triggering).

Proposal 4: Support configuration of multiple remaining time thresholds and corresponding LCH priorities per LCH for delay-aware LCP.