3GPP TSG-RAN WG2 Meeting #127bis	R2-2409025

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.8.2

Source:	Samsung

Title:	Discussion on Downlink Coverage Enhancement

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NR NTN, SSB periodicity extension has been included as the objective of DL coverage enhancement in the latest WID [1].

Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

RAN1 has sent LS reply (R2-2407919 [2]) regarding RAN2’s questions on DL coverage enhancement for NTN.

RAN1 thanks RAN2 for their questions on the DL coverage enhancements and would like to provide the responses to the following questions. RAN1 will try to provide a further response at a later time. Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?. Answer to Q1: According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion. Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.. Answer to Q4: Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

In this contribution, we continue to share our views on the potential enhancements from RAN2 perspective.

# Discussion

According to the LS reply and also the latest WID, SSB enhancement is limited to SSB periodicity extension only. As we have discussed in the last meeting, SSB periodicity extension if supported leads to the extension of value range for relevant RRC configurations. We have confirmed as follows that the legacy NTN bar bit can be used to bar legacy UE from a cell with DL coverage enhancement, and there is an FFS for Rel-19 UE not supporting DL coverage enhancement.

If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

According to the latest WID [1], RAN1 will report on impact to backward compatibility, if any, for potential extension of the SSB periodicity. Based on the analysis of the impact to legacy UE (including Rel-19 UE not supporting such a feature), we need to discuss whether there is a need to bar legacy UE. In our view, if a DL coverage enhancement feature has large impacts to legacy operation, NW may need to bar such UEs from accessing a cell operating with the feature. In that case, the legacy NTN bar bit is used to bar UE that are not supporting such a feature from access this cell and a new bar bit is needed to bar UEs that are supporting the feature from access this cell.

Proposal 1: Discuss whether there is a need to bar legacy UE (including pre-Rel19 UE and Rel-19 UE not supporting SSB periodicity extension) from accessing a cell operating with SSB extension, based on RAN1 report on the impact to backward compatibility for potential extension of the SSB periodicity.

Based on the WID, Rel-19 DL coverage enhancement considers the scenario of beam hopping where only a limited number of satellite beams are illuminated at a certain time due to power sharing. We have sent a LS to RAN1 asking questions regarding this aspect.

RAN1 has replied that the beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology [2]. In our understanding, this implies that the beam footprint status in terms of "off", "common messages only" and "active traffic” may not be visible to UE. It is possible that beam hopping with different beam footprint status can already be supported transparently to UE. For instance, SSB beams in status “off” only happen in the duration between two consecutive SSB bursts (i.e., when SSBs are not transmitted), and beam management via TCI state framework (e.g., TCI state activation/deactivation by MAC CE and dynamic indication by DCI) can allows different “active traffic” beams to be used for different UEs at different time.

As NTN cell is larger than TN cell, all or part of beams of a cell can change status in beam hopping in a periodical manner instead of changing very dynamically. Thus, it would be beneficial to inform UE the beam pattern in terms of periodicity for UE power saving purpose. If all beams in a cell are in the same state at any time, Rel-18 NES cell DTX/DRX can be considered as a baseline. If beams in a cell can be in different states with different periodicities, multiple DTX/DRX patterns can be configured and one of them can be activated at a certain time.

Regardless of single or multiple DTX/DRX patterns, we think there should be no impact to idle/inactive UEs. Common signalling via SSB transmission and reception should not be impacted due to beam hopping. As RAN1 indicated, they will have further reply on this aspect.

Proposal 2: If beams in a cell are in the same state at any time, Rel-18 NES cell DTX/DRX is applicable for DL coverage enhancement.

Proposal 3: If beams in a cell can be in different states with different periodicities, consider to support multiple DTX/DRX patterns and activation by NW indication.

Proposal 4: Wait for more RAN1 reply on beam-level DTX/DTX for DL coverage enhancement.

In the last meeting, we have agreed that RAN2 can further consider SMTC impacts due to beam-hopping or larger SSB periodicity. In our view, the SMTC periodicity value range should be extended if SSB periodicity is larger than 160ms, but SSB periodicity extension does not impact the UE behaviour and procedure for SMTC as specified in TS 38.331 below.

Proposal 5: The SMTC periodicity value range is extended if SSB periodicity is larger than 160ms, SSB periodicity extension does not impact existing UE behaviour and procedure for SMTC.

For beam hopping, UE may initiate neighbour cell measurement based on neighbour cell’s beam status and apply the corresponding SMTC only when neighbour cell’s SSBs are available in beam hopping pattern. One approach is that NW takes into account the beam hopping of neighbour cells in SMTC.

Observation 1: NW can take into account the SSB beam hopping of neighbour cells in SMTC.

# Conclusion

In this contribution we discussed issues related to...

Proposal 1: Discuss whether there is a need to bar legacy UE (including pre-Rel19 UE and Rel-19 UE not supporting SSB periodicity extension) from accessing a cell operating with SSB extension, based on RAN1 report on the impact to backward compatibility for potential extension of the SSB periodicity.

Proposal 2: If beams in a cell are in the same state at any time, Rel-18 NES cell DTX/DRX is applicable for DL coverage enhancement.

Proposal 3: If beams in a cell can be in different states with different periodicities, consider to support multiple DTX/DRX patterns and activation by NW indication.

Proposal 4: Wait for more RAN1 reply on beam-level DTX/DTX for DL coverage enhancement.

Proposal 5: The SMTC periodicity value range is extended if SSB periodicity is larger than 160ms, SSB periodicity extension does not impact existing UE behaviour and procedure for SMTC.

Observation 1: NW can take into account the SSB beam hopping of neighbour cells in SMTC.

# Reference

RP-241667 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

R2-2407919 Reply LS on DL coverage enhancements