3GPP TSG-RAN WG2 Meeting #127bis	R2-2409026

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.8.4

Source:	Samsung

Title:	Discussion on MBS Broadcast Intended Area

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NTN, one objective is to support MBS broadcast service for smaller areas in an NTN cell. The detailed justification and objective in WID are provided as follows.

MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service. Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this contribution, we continue to share our views on the signaling of broadcast service area information and discuss service continuity with intended service area information.

# Discussion

## Signalling of intended service area

In the last two meetings, we have discussed signaling aspects and reached the following agreements.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

We have discussed the two signaling options for intended service area information in the last meeting: 1) provide in SIB20; 2) provide in MCCH. Our views on the pros and cons of the two options are as follows.

Option 1 is aligned with WID “specify SIB signaling”. It allows UE to skip acquiring MCCH if UE is not in any of the intended service areas. SIB may have overhead restriction.

Option 2 is cleaner in terms of providing all information in MBS broadcast configuration in MCCH. Overhead restriction is not an issue. UE may acquire MBS broadcast configuration in MCCH in vain if UE is not in any intended service areas of its interested services.

In our view, we would prefer to follow the WID to specify signaling in SIB as option 1. One aspect is that in the SIB every broadcast service session needs to be associated to a service area so that UE can be aware of the intended area of each service by acquiring SIB only and skip acquiring MCCH. We have agreed circles and polygons are used to describe the intended areas. This is needed only in the case that the intended service area of a service covers a part of the cell, not the whole cell. If a service is provided for the whole cell, it can be simply indicated by a flag without geographic area information.

Proposal 1: In SIB20 each broadcast service session is associated to an intended service area so that UE can be aware of the intended area of each session by acquiring the SIB only and can skip acquiring MCCH.

Proposal 2: Circles and polygons are used to describe the intended areas only in the case that the intended service area covers a part of one cell. If a service is provided for a whole cell, it can be simply indicated by a flag without geographic area.

Another aspect which has been mentioned for option 1 is the update of intended service area information will cause legacy UE to perform SIB modification procedure which is actually not needed for legacy UE as it does not read this new information. This impact to legacy UE is normal whenever we add new information to an existing SIB, which is not a specific issue for this case. And the geographic information is assumed to be semi-static which should not change frequently. If this is a strong concern, we can consider to introduce a new SIB.

Observation 1: The geographic area is assumed to be semi-static which does not cause frequent SIB update or large impact to legacy UE.

Proposal 3: The legacy SIB modification procedure is applied to update the intended service area information in SIB20.

RAN2 has agreed that both (quasi)-earth-fixed and earth-moving cells are supported for MBS broadcast service. We understand the intended service area represents a fixed geographic area on the ground for both cases. However, due to the fast movement of an earth moving cell, the cell may provide an MBS broadcast service to an intended area for a quite short duration, which will lead to frequent updates of MBS broadcast service and the associate intended area and, thus, UE frequent acquisitions of SIB or MCCH. To avoid this issue, some additional information (e.g., duration, etc.) can be provided for an MBS broadcast service and its intended area for earth-moving cells.

Proposal 4: Discuss for earth moving cell how to avoid the frequent update and acquisition of information on MBS broadcast service and the intended area.

## Service continuity

For MBS broadcast service, SIB21 contains information related to service continuity for MBS broadcast reception. As specified in stage-2, to allow the UE to start or continue receiving MBS service(s) when changing cells, the gNB may indicate in the MCCH the list of neighbour cells providing the same MBS broadcast service(s) as provided in the serving cell. This allows the UE, e.g., to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. To avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies, the UE is made aware of which frequency is providing which MBS broadcast services via PTM, through User Service Description, as defined in TS 26.517, or the combination of User Service Description (USD) and Frequency Selection Area Identities (FSAI) in SIB21.

For an MBS broadcast service, the intended service area can cover continuous area of the current serving cell and neighbor cells. To support service continuity across cells, the intended service area for neighbor cells can be provided. One approach is to support the mapping between intended service areas and neighbor cells in the MBS-NeighbourCellList in MBSBroadcastConfiguration. Another approach is to support the mapping between intended service areas and the frequency list in SIB21. Based on the intended service area associated to a frequency or a neighbor cell, UE can prioritize or deprioritize a frequency in cell reselection.

Proposal 5: Discuss service continuity with intended service area information, consider to support the signaling of the mapping between intended service area and frequency/neighbor cell and the corresponding UE behavior in cell reselection.

# Conclusion

In this contribution we discussed the following proposals.

Proposal 1: In SIB20 each broadcast service session is associated to an intended service area so that UE can be aware of the intended area of each session by acquiring the SIB only and can skip acquiring MCCH.

Proposal 2: Circles and polygons are used to describe the intended areas only in the case that the intended service area covers a part of one cell. If a service is provided for a whole cell, it can be simply indicated by a flag without geographic area.

Observation 1: The geographic area is assumed to be semi-static which does not cause frequent SIB update or large impact to legacy UE.

Proposal 3: The legacy SIB modification procedure is applied to update the intended service area information in SIB20.

Proposal 4: Discuss for earth moving cell how to avoid the frequent update and acquisition of information on MBS broadcast service and the intended area.

Proposal 5: Discuss service continuity with intended service area information, consider to support the signaling of the mapping between intended service area and frequency/neighbor cell and the corresponding UE behavior in cell reselection.

# Reference

RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

3GPP, TS 38.331 v18.0.0; Radio Resource Control (RRC); Protocol specification.