3GPP TSG-RAN WG2 Meeting #127bis	R2-2409029

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.2.2 (FS_Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on functionality aspects of ambient IoT

Document for:	Discussion and Decision

1.	Introduction

This document discusses functionality of ambient IoT (A-IoT). In this contribution, we discuss mainly segmentation, energy status reporting, message size reporting and TB size allocation. The related agreements in R2-127 meeting are as follows.

Agreements (Segmentation) - Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size. Agreements (Charging Impacts, Message Size/Status Indication) - RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages. FFS details on signaling - RAN2 will study the need and use of a simple message “size” reporting to the reader Agreements (Visibility of AIOT Information) 1 At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional 2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

2.	Discussion

Segmentation

Considering that an A-IoT is targeted for a very low complexity device, no segmentation and no reassembly in an A-IoT device is preferred. On the other hand, if the size of TB supported in physical layer is less than the size of message delivered from the upper layer, support of segmentation is required. However, the TB size and message size are not determined exactly yet in the responsible WGs. Therefore, RAN2 sent an LS to RAN1 regarding TB size and ask SA2 on the maximum and typical upper layer packet size in the previous meeting (R2-127 meeting).

When the responses from RAN1 and SA2 are going to be received, if the TB size can always accommodate the upper layer data delivered to layer 2, it is obvious that no segmentation and no reassembly in layer 2 is required. Otherwise, support of segmentation in layer 2 may be discussed. For the latter case, low complexity should be considered in segmentation function design. In the previous meeting, some companies explained about segmentation functions which do not require buffer for reduced complexity. In this sense, if segmentation is needed in layer 2, we also prefer a segmentation function which does not use additional buffer.

Proposal 1. If segmentation is required in layer 2 based on the responses from RAN1 and SA2, RAN2 discuss support of a segmentation function which does not require layer 2 buffer for reduced complexity.

Energy status reporting

During RAN2 discussion about A-IoT in the previous meeting, RAN2 decided to study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.

For successful A-IoT communication, the following condition needs to be presumed. It is that the reader has to make it sure that the target A-IoT devices are charged enough for completing the message transmissions belonging to the service request before triggering A-IoT devices. In other words, to meet the condition, the reader needs to provide A-IoT devices with enough time for energy harvesting before triggering the devices.

However, it is not easy to determine proper time duration for energy harvesting because the reader does not have the related information such as how much energy each device is charged with, how long each device needs to be charged fully, how much energy each device requires for a message transmission and etc. If the reader blindly determines the time duration with enough safe margin for A-IoT device’s energy harvesting, the A-IoT system will spend unnecessarily long time for energizing A-IoT devices and it will decrease efficiency of A-IoT system.

Observation 1. Without energy status reporting, A-IoT system may spend unnecessarily long time for energizing A-IoT devices to guarantee that target A-IoT devices are charged enough before triggering.

If energy status reporting is provided to the reader from A-IoT devices, the reader can exploit the reported information to allocate a time duration for device’s energy harvesting or to (re-)trigger the devices which doesn’t have energy for the follow up messages at the next access procedure. Then, the reader doesn’t need to choose unnecessarily long time for device’s energy harvesting because the reader can be aware of needs of additional energy harvesting and may give more time for energy harvesting to the devices based on energy status reporting. Energy status reporting is needed for avoiding degradation of A-IoT system efficiency by over-guaranteeing that target A-IoT devices are sufficiently charged.

Proposal 2. Support energy status report from a device to a reader in A-IoT.

For example, during a CFRA for a designated device, a device may send a simple energy status reporting which indicates that the device does not have energy for the follow up messages as shown in Figure 1. Then, the reader can allocate a time duration for device’s energy harvesting and the device performs energy harvesting during the duration. After that, the reader and the device may be able to perform data communication successfully.

Figure 1 An example for using a simple energy status report

Message size reporting and TB size indication

During RAN2 discussion about A-IoT in the previous meeting, RAN2 decided to study the need and use of a simple message size reporting to the reader.

A message size reporting from a device to the reader is a way to inform the size of a D2R message to reader. However, if the message size is pre-configured or it can be informed to the reader by the network, the message size reporting is not needed.

For A-IoT random accesses, the message size of each step can be pre-configured. For example, Msg1 of 3-step CBRA is a random number of 16bits and its size can be pre-configured as 16bits. Msg3 contains a device ID with a known size and its size also can be pre-configured considering the known size.

For the subsequent message transmissions, the message size can be informed to the reader by the network. The format and size of a message is expected to be pre-defined and known to the network because A-IoT communication seems to use simple and well-defined messages, considering that an A-IoT design aims for a device with very low complexity, and the representative use cases are inventory (e.g., automated warehousing), sensor (e.g., smart homes), and command (e.g., online modification of medical instruments status). For example, when a command (e.g., query for temperature) is sent from network, if the format and size of the corresponding response is already expected to be pre-defined (e.g., 8bit integer value for temperature), the network can inform the response message size to the reader in parallel with sending the command. Based on the above-mentioned analysis, if the message size can be pre-configured or informed to the reader by the network, message size reporting from a device to the reader is not needed.

Proposal 3. Assume the message size can be pre-configured or informed to the reader by the network, and message size reporting from a device to the reader is not needed.

Regarding D2R scheduling, it may be considered that the reader indicates a TB size, which can accommodate the D2R message, to the device when the reader triggers the D2R message transmission. However, assuming that the message size can be pre-configured or informed to the reader by the network, the TB size for D2R message transmission doesn’t need to be indicated to the A-IoT device by the reader. When a device receives a command sent from a reader, the corresponding response message is generated on the device. The size of the generated response message should be the same with the size that the reader knows. Then, the both the reader and the device know the response message size for the D2R transmission and they can be aware of the corresponding TB size which is able to accommodate the response message and the related headers without using explicit TB size indication from the reader to the device. If so, the explicit TB size indication from the reader to the device is redundant information for a D2R transmission and it can be skipped. The reader does not need to indicate the TB size for a D2R transmission to a device.

Proposal 4. The reader does not need to indicate the TB size for a D2R transmission to a device.

3.	Conclusion

This document discusses functionality of ambient IoT (A-IoT).

Proposal 1. If segmentation is required in layer 2 based on the responses from RAN1 and SA2, RAN2 discuss support of a segmentation function which does not require layer 2 buffer for reduced complexity.

Observation 1. Without energy status reporting, A-IoT system may spend unnecessarily long time for energizing A-IoT devices to guarantee that target A-IoT devices are charged enough before triggering.

Proposal 2. Support energy status report from a device to a reader in A-IoT.

Proposal 3. Assume the message size can be pre-configured or informed to the reader by the network, and message size reporting from a device to the reader is not needed.

Proposal 4. The reader does not need to indicate the TB size for a D2R transmission to a device.