3GPP TSG-RAN WG2 Meeting #127bis	R2-2409030

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.2.5 (FS_Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on topology 2 considerations of ambient IoT

Document for:	Discussion and Decision

1.	Introduction

This document discusses topology 2 considerations of ambient IoT (A-IoT). In this contribution, we discuss mainly topology 2 architecture between gNB and UE reader and resource control used in A-IoT air interface. The related agreements in R2-127 meeting are as follows.

Agreements 1 RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2. 2 A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device. 3 RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress. 4 RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied) 5 NW is in control of resources to be used by AIoT air interface. 6 Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

2.	Discussion

Topology 2 has the following connectivity topology.

- Topology 2: BS <--> intermediate node <--> Ambient IoT Device

To support topology 2, RAN3 considers a logical system architecture shown in Figure 1 for Topology 2.

Figure 1 Logical system architecture for topology 2

- Common reader function: A function that communicates with the A-IoT device by means of A-IoT radio.

- AIoT RAN node function: A function that contains e.g. the control of the A-IoT radio resources used towards the A-IoT device.

Since RAN2 agreed that A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2 in the previous meeting (R2-127 meeting), the common reader function seems the same with Topology 1.

Topology 2 architecture

Over NR-Uu interface, A-IoT related messages are transferred between common reader function in the UE reader and the correspond network functions in the network. SA2 proposed architecture options for delivering A-IoT messages over NR-Uu interface. During RAN2 discussion about A-IoT in the previous meeting (R2-127 meeting), RAN2 decided that for topology 2 architecture RAN2 will study the impact to RAN2 based on the SA2 identified architecture options and RAN2 does not consider new AS layer architecture options. The SA2 identified architecture options are categorized as follows according to the recent RAN3 discussion.

- RRC based solution: With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

- NAS based solution: With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

- UP based solution: With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

From the RAN2 point of view, RRC based solution will inevitably have impact to RRC for NR Uu interface and make changes in RRC spec. because new RRC message and IE are expected to be required to embed A-IoT upper layer messages in NR Uu RRC. On the other hand, NAS based solution and UP based solution may limit the expected impact to the upper layers only. Considering less impact to AS layer of NR Uu interface, NAS based solution and UP based solution are preferred.

Proposal 1. Prioritize NAS based solution and UP based solution for topology 2 architecture.

Resource allocation

During RAN2 discussion about A-IoT in the previous meeting (R2-127 meeting), RAN2 decided that NW is in control of resources to be used by A-IoT air interface. We would like to discuss about how to control resources to be used by A-IoT air interface and how to send the resource control information to the UE reader.

For comparison, we go through sidelink (SL) resource control in network scheduling mode. In SL network scheduling mode, each transmission over PSSCH can be controlled by the network and a new downlink control information (i.e., DCI format 3) is introduced to send the control information (i.e., SL scheduling information). On the other hand, in A-IoT, it seems inefficient to control each transmission over PRDCH and PDRCH in A-IoT air interface. A D2R transmission is triggered by the corresponding R2D message. A pair of a D2R transmission and an R2D transmission can be scheduled at once. When multiple subsequent message exchanges are expected in a procedure, it seems better to schedule the messages at once to avoid additionally triggering or paging the A-IoT device. Particularly, in case of an inventory procedure, multiple devices are triggered for responses and slotted ALOHA based access mechanism (i.e., CBRA) is used for accesses from the multiple devices. When the CBRA is triggered, how many access occasions are available for CBRA is signaled to the devices. Then, it seems fine that a single resource block for the CBRA is scheduled by the network. In detail, the resource block may correspond to an access round or a group of access rounds for the requested inventory procedure.

In addition to the above-mentioned analysis, we would like to emphasize that A-IoT paging triggers a service procedure and the related message transmissions happen in the A-IoT air interface for the service procedure. Therefore, it is preferred that, when network controls resources to be used by A-IoT air interface, the network configures or allocates enough resource for accommodating whole message transmissions triggered by an A-IoT paging message or multiple A-IoT paging messages.

Proposal 2. The network controls UE reader’s A-IoT air interface resources so that a resource can accommodate whole message transmissions triggered by at least an A-IoT paging message.

When the network controls A-IoT air interface resources per A-IoT paging, the control information can be sent by upper layer (e.g., NAS or user plane data) if required. It is also considered that A-IoT paging is initiated from the network for A-IoT service request. However, it may depend on topology 2 architecture. When RRC based solution is considered, the control information can be sent by RRC. Since it is proposed to prioritize NAS based solution and UP based solution for topology 2 architecture in this document, it is proposed that the control information is sent by upper layer.

Proposal 3. The resource control information for UE reader’s A-IoT air interface is sent to the UE reader by upper layer (e.g., NAS or user plane data).

3.	Conclusion

This document discusses topology 2 considerations of ambient IoT (A-IoT). We discuss topology 2 architecture between gNB and UE reader and resource control used in A-IoT air interface.

Proposal 1. Prioritize NAS based solution and UP based solution for topology 2 architecture.

Proposal 2. The network controls UE reader’s A-IoT air interface resources so that a resource can accommodate whole message transmissions triggered by at least an A-IoT paging message.

Proposal 3. The resource control information for UE reader’s A-IoT air interface is sent to the UE reader by upper layer (e.g., NAS or user plane data).