3GPP TSG-RAN WG2 Meeting #127bis	R2-2409038

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	Functionality based LCM

Document for:	Discussion and Decision

1. Introduction

RAN2 has made following agreements and signalling procedure on applicable functionality reporting for beam management UE-sided model:

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

- (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

For further detailed signaling procedure for applicable functionality reporting, RAN2 needs RAN1 reply.  Other than detailed procedure/configuration related to RAN1 decision, RAN2 needs to discuss RAN2 sided FFSs. This contribution aims to address issued regarding applicable functionality reporting and LCM.

2. Discussion

RRC message for applicable functionality reporting

Whether the RRCReconfigurationComplete message can be used for reporting applicable functionality is still under discussion. If steps 3 to 5 only happen once during the initial configuration, sending UAI in step 4 might not be a big burden for the UE or the network. But if step 5 can trigger the UAI reporting again, steps 3 to 5 might repeat several times. In that case, sending UAI repeatedly would add more signaling, making it more of a strain on both the UE and the network. Additionally, reporting functionality promptly after RRC connection could help AIML start working faster, which is another advantage. Therefore, it is proposed that the RRCReconfigurationComplete message, in addition to UAI, be allowed for reporting applicable functionality.

Proposal 1. RRCReconfigurationComplete message, in addition to UAI, is allowed for reporting applicable functionality

LCM

During the RAN2 meeting, we have reached agreement regarding functionality management.

Agreement on RAN2 #125bis For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

For ‘network decision, network-initiated’, the network can make LCM related decisions based on monitoring data. Regarding monitoring data, there are two aspects being discussed in RAN1: data for performance metric calculation (Option 1 of Type 1) and calculated performance metric (Option 2 of Type 1). In the case of Option 1, it is unclear whether there is any RAN2 impact on the measurement result reporting if L1-RSRP is transmitted via L1 reporting. In the case of Option 2, it is unclear what the calculated performance metric is. Depending on what the calculated performance metric is and how it is transmitted, the RAN2 impact may be determined. Therefore, the discussion regarding ‘network decision, network-initiated’ can be started after receiving RAN1 input.

Agreement on RAN1 #117 For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement on RAN1 #118 For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Additionally, RAN1 has left Type 2 performance monitoring as FFS. That is, UE initiated LCM, such as ‘UE autonomous, decision reported to the network’, 'network-decision, UE-initiated' is still unclear in RAN1. Therefore, RAN2 should wait for the discussion on ‘UE autonomous, decision reported to the network’, 'network-decision, UE-initiated'.

Proposal 2: To wait monitoring related discussion until the followings are clear from RAN1

- For ‘network decision, network-initiated’:

  - Option 1: whether monitoring data, such as L1-RSRP, is transmitted via L1 reporting

  - Option 2: what the calculated performance metric is and how it is transmitted

- For ‘UE autonomous, decision reported to the network’, 'network-decision, UE-initiated', whether Type 2 performance monitoring is considered in RAN1

Functionality continuity

For beam management, inference is performed only on beams within a single cell. In the case of inference, those configurations are significantly influenced by the current cell's CSI settings. Therefore, when a cell change occurs, the applicability exchange steps must be restarted based on the new cell's settings. To this end, the applicable functionality reporting procedure under discussion could apply during handover. In Step 3, the HO settings for the target cell could be included, and in Step 4, the applicability for the target cell could be exchanged, allowing the functionality to be quickly activated in the new cell. As a result, the functionality can continue operating after the HO without significant interruption. Therefore, to ensure proper operation during a handover, functionality should be deactivated during the HO process

With this in mind, since the applicability for the target cell can be exchanged during the HO process, there is no need for the serving cell to receive applicable functionality reports for neighboring cells.

Proposal 3: To deactivate functionality and remove inference configuration during handover

Proposal 4. Do not consider applicable functionality reporting for a neighbor cell

3. Conclusion

Proposal 1. RRCReconfigurationComplete message, in addition to UAI, is allowed for reporting applicable functionality

Proposal 2: To wait monitoring related discussion until the followings are clear from RAN1

- For ‘network decision, network-initiated’:

  - Option 1: whether monitoring data, such as L1-RSRP, is transmitted via L1 reporting

  - Option 2: what the calculated performance metric is and how it is transmitted

- For ‘UE autonomous, decision reported to the network’, 'network-decision, UE-initiated', whether Type 2 performance monitoring is considered in RAN1

Proposal 3: To deactivate functionality and remove inference configuration during handover

Proposal 4. Do not consider applicable functionality reporting for a neighbor cell