3GPP TSG-RAN2 Meeting #127-bis	R2-2409059

Hefei, China, 14 - 28 October 2024	Revision of R2-2407397

Agenda Item:	8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS and RRM measurements

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS and RRM measurements are discussed further.

For the related RAN1/RAN2/RAN4 agreements the reader is referred to the Annex.

# Discussion

## RAN2 and RAN4 work split

RAN4 agreed to study different use cases for offloading and relaxation:

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

RAN4 to further discuss case #2 to #4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

And RAN4 assumes that RAN2 will discuss offloading/relaxation criteria:

Agreement:

- Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

We agree that RAN2 should discuss the offloading/relaxation criteria and that RAN4 should discuss the relaxed measurement requirements. During Rel-16 relaxed RRM measurements in idle/inactive a similar work split was envisioned. However the relaxation criteria, i.e. the possible configuration options and the possible combinations of fulfilment of low mobility and/or not-at-cell-edge were captured in both 38.304 and 38.133. Furthermore the way this was captured was different in 38.304 and 38.133 which made it difficult to compare them:

This duplication has led to many LS exchanges between RAN2 and RAN4, and still possible discrepancies between RAN2 and RAN4 specification are still being discussed [1].

A potential mismatch between RAN2 and RAN4 w.r.t. LP-WUS can already be identified, i.e. RAN2 agreed to use the Rel-16 relaxation criteria as a baseline, i.e. “not at cell edge” and “low mobility” and use entry/exit conditions for MR serving cell measurement offloading/relaxation:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

This does not clearly match the use cases (1, 2, 3, and 4) identified by RAN4, for which RAN4 will define measurement requirements. In case the Rel-16 relaxation criteria are re-used then no new work is needed in RAN4 concerning the relaxed measurement requirements:

Inform RAN4 that RAN2 uses the Rel-16 relaxation criteria as a baseline for the MR neighbour cell measurement relaxation criteria and entry/exit conditions for MR serving cell measurement offloading/relaxation.

It should be discussed how to avoid duplication of the relaxation criteria in RAN2 and RAN4 specification. One option is that only the “not at cell edge” and “low mobility” criterion are specified in RAN2 specification. And that the different use cases, that depend on what is configured and which criteria are fulfilled is captured in RAN4 specification:

RAN2 captures the relaxation criteria (e.g. “not at cell edge” and “low mobility”) while RAN4 captures the different uses that can be identified based on the configuration and which criteria are fulfilled.

## MR serving cell measurements offloading and relaxation

RAN2 agreed that from RAN2 perspective MR serving cell measurements is only supported when the UE supports WUR. Obviously MR serving cell measurements can only be offloaded when the UE supports WUR:

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

Furthermore RAN2 made agreements on the entry/exit conditions for MR serving cell measurement offloading:

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

In our view there is no need to discuss other use cases, except “fully” offloading scenarios:

RAN2 only discusses fully offloading use cases and use cases where the MR and LR temporarily perform serving cell measurements simultaneously (e.g. when performing MR and LR measurements to determine whether the entry condition is met). RAN2 does not discuss use cases where the MR is performs relaxed serving cell measurements while the LR performs non-relaxed/continuous serving cell measurements.

An example when MR and LR may have to perform serving cell measurements simultaneously is when MR measurements are used to retune the LR timing/frequency e.g. when the LR quality is bad [2]. These cases can be discussed on a case by case basis, and they do not need to impact the entry/exit conditions.

RAN2 agreed for the entry/exit condition for LP-WUS monitoring:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

There is little power saving when the entry condition for serving cell offloading is fulfilled, but the entry condition for LP-WUS monitoring is not fulfilled. Furthermore we do not see a good motivation why the entry/exit condition for serving cell offloading and LP-WUS monitoring should be different, because it concerns the same type of physical signal:

The same entry/exit condition for serving cell measurement offloading and LP-WUS monitoring can be configured.

Details can be further discussed during stage 3 when the ASN.1 structures are discussed.

## MR neighbour cell measurement relaxation

RAN2 agreed to use the Rel-16 relaxation criteria as a baseline:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

Neighbour cell measurement relaxation (i.e. intra-/inter-/higher priority frequency measurement relaxation) is supported in NR for normal broadband UEs and (e)RedCap UEs. In both cases the basic principle is the following:

- Mobility and/or good coverage criteria can be configured in SIB2.

- When mobility has not been detected for a period of TSearch and/or when the UE is in good coverage:

  - UE may perform relaxed RRM measurements for intra-/inter-frequencies as specified in 38.133.

  - The relaxation level depends on whether both mobility and good coverage criterion need to be fulfilled simultaneously or not.

- When highPriorityMeasRelax is not configured then the UE is required to perform higher priority frequency measurements every 60 seconds as in legacy. Otherwise the higher priority frequency measurements can be relaxed when no mobility has been detected and the UE is in good coverage.

Higher priority frequency measurements ensure that the UE is always on the highest priority frequency, even when the UE is above the inter-frequency measurement threshold on the current frequency. These higher priority frequency measurements are performed in the background, i.e. the UE is only required to measure an inter-/iRAT frequency every 60 seconds.

The neighbour cell measurement relaxation depends on the relaxedMeasurement configuration in SIB2 and whether the relaxation criteria are met. In the best case, i.e. when the UE is stationary and in good coverage, the UE is not required to perform intra-/inter-frequency measurements for 1 hour. In such case the UE can also relax higher priority frequency measurements up to 1 hour. In other cases the UE may relax intra-/inter-frequency measurements to every 3rd DRX cycle.

Using the legacy measurements thresholds (SIntraSearch, SNonIntraSearch) and Rel-16 threshold (Not-at-cell-edge) different areas can be identified with different measurement requirements, that also depend on the UE mobility (lowMobility):

The LP-WUS coverage is independent from the neighbour cell measurements areas above, i.e. LP-WUS can cover the complete cell, or only a small portion in the cell centre.

When the UE supporting LP-WUS is outside LP-WUS coverage or LP-WUS is not configured then the UE uses MR serving cell measurements with the Rel-16 criteria, similar as UEs that do not support LP-WUS:

When WUR is not configured, the UE does not support WUR, or the LR entry condition is not fulfilled, then the MR serving cell measurements are used to determine whether the neighbour cell measurements can be relaxed using the Rel-16 relaxed neighbour cell measurement criteria, i.e. legacy and UE supporting WUR behave the same.

RAN2 only needs to discuss how Rel-16 neighbour cell relaxation requirements are met when the entry condition is fulfilled and the LR is performing serving cell measurements.

The OFDM-based serving cell measurements using PSS/SSS are quite similar in latency and performance compared to the MR serving cell measurements, if not identical. In this case the LR can continue to do serving cell measurements using the Rel-16 criteria:

For OFDM-based WUR when the LR performs serving cell measurements the LR determines, based on the Rel-16 neighbour cell relaxation criteria, when the MR needs to be activated to perform neighbour cell measurements.

FFS for OOK-based WUR whether separate Rel-16 neighbour cell relaxation criteria (values) need to be configured.

The trigger for MR neighbour cell measurements is not part of the LR exit condition, i.e. the LR is allowed to continue serving cell measurements when the MR is performing neighbour cell measurements.

Below a picture of the LR entry/exit condition and triggers to activate the MR for paging/neighbour cell measurements:

There is perhaps limited power saving gain when the MR only needs to perform neighbour cell each DRX cycle, i.e. LR can continue to perform serving cell measurements. The serving cell measurements come for free when the MR performs intra-frequency neighbour cell measurements. But when the neighbour cell measurements only need to be performed every 3rd DRX cycle, then it may be beneficial to have the LR perform continuous serving cell measurements.

Furthermore, when the Rel-16 measurement requirements are further relaxed, this also means that the relaxation criteria will become stricter, i.e. the measurement requirements cannot be relaxed without a change in the relaxation criteria. The area and scenarios where the UE is allowed to even further relax measurements will become smaller and fewer, i.e. the impact is significantly reduced. Furthermore a correlation can be expected between LR exit (LR link is bad and can no longer perform serving cell measurements) and criteria for neighbour cell relaxation are no longer fulfilled, i.e. there is only a certain/small area where further relaxation can be applied. The power saving gain for a feature for which the power consumption is already low is in most case insignificant.

Neighbour cell relaxation beyond the Rel-16 criteria is not discussed further unless it can be proven that it can provide significant power saving gains.

An example how Sintrasearch and Snonintrasearch are configured is given below:

The UE is configured to start intra-frequency measurements before inter-frequency measurements, i.e. typically the (lower) frequencies provide full coverage and intra-frequency cell re-selection is preferred. Only when there are coverage issues, or when the UE is on a higher priority small cell then the mobility is performed using inter-frequency cell reselection.

The Sintrasearch can be configured quite conservatively, i.e. basically requiring the UE to measure intra-frequency neighbouring cell in the complete cell. This is typically motivated in urban environments where cell coverage does not follow a perfect circle but is rather irregular, and the UE may be detect a better cell suddenly around the corner. Example of coverage in urban environment:

Furthermore, the coverage is beam dependent which also does not follow the perfect “flash-light” coverage. Therefore it is motivated to configure the Sintrasearch conservatively in certain use cases. Furthermore it is not obvious that intra-frequency measurements can be relaxed when the UE “suddenly” can detect a better intra-frequency neighbour around the corner, i.e. relaxation may be limited to when the UE is stationary:

In some use cases the intra-frequency measurements requirements can be relaxed using the Rel-16 relaxation criteria. But in some use cases where coverage is highly irregular this may not be possible.

For this reason it is beneficial to discuss neighbour cell measurements performed by OFDM-based LR, i.e. the LR can perform continuous intra-frequency measurements in the cell, but only wake-up the MR to trigger intra-frequency cell reselection when needed:

RAN2 to discuss the need for intra-frequency neighbour cell measurements performed by OFDM-based LR.

## LP-SS measurement accuracy and latency

In legacy the UE has to take the average of 2 RSRP/RSRQ measurement samples made during consecutive DRX cycles:

Thus the DRX cycle determines the latency in which the UE detects when the serving cell exceeds or drops below a threshold.

In case of OOK-based WUR and LP-SS the measurement latency is independent of the DRX cycle. The assumption is that the UE is required measure 5 samples of LP-SS measurements that are spaced 320 ms apart:

This means that if the DRX cycle is 320 ms, then the LR measurements are 2x slower compared to the MR measurements.

The MR and OFDM-based WUR measurement delay equals the DRX cycle, while the OOK-based WUR measurement delay is 1.28 seconds (with 230 ms LP-SS periodicity), i.e. the delay is independent of the DRX cycle.

We are not sure if there is a problem with the LP-SS measurement latency that needs to be solved, i.e. in many cases a DRX cycles of 1.28 seconds is configured, in which case the MR and LR measurement latency are the same.

RAN4 has been discussing how to capture the LP-SS measurement accuracy requirements, e.g. as dedicated accuracy requirements, potentially including test cases, or as a cell reselection margin (see legacy 3dB margin below) to the core requirements:

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

In legacy the UE is required to be able to determine that an intra-frequency neighbour cell is 3 dB better within Tevaluate,NR_Intra in FR1. For LP-WUS a larger margin would need to be defined. A larger cell reselection margin implies that cell reselection is delayed:

RAN2 to discuss the impact of a larger cell reselection margin when LP-SS measurements are used.

## LP-SS measurement quantity in multibeam operation

In case of multi-beam operation the measurement quantity for LP-SS serving cell measurements needs to be discussed further, e.g. whether the UE uses the highest beam measurement quantity or the average over a number of beams.

In legacy, i.e. when the MR performs RSRP/RSRQ measurements, then in case of cell selection the measurement quantity is up to UE implementation. But in case of cell reselection the UE has to follow the configuration provided in nrofSS-BlocksToAverage and absThreshSS-BlocksConsolidation in SIB1 (see 38.304):

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation.

For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows:

-	if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA):

-	derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11].

-	else:

-	derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

LP-SS serving cell measurements are used to determine when the MR has to start neighbour cell measurements and potentially perform cell reselection.

In our view the cell reselection decision should be based on new/recent MR measurements, including new/recent MR serving cell measurements. However RAN4 is discussing the possibility to combine MR and LR serving cell measurements.

Thus it makes sense to apply the same measurement quantity for MR and LR serving cell measurements. When the UE determines whether LP-SS serving cell measurement has dropped below the measurement threshold based on the highest serving cell beam and the MR has to start neighbour cell measurements, then the cell reselection decision based on MR measurements should also use the highest neighbour cell beam:

The LP-SS measurement quantity in multibeam operation is the same as the measurement quantity used for cell reselection.

## UE mobility and speed

For the simulations RAN4 assumed for the UE speed:

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UE speed |  | FR1: 30 km/h |  |

The WID does not explicitly mention mobility requirements. But it is assumed that WUR is not limited to low mobility scenarios like NB-IoT, but that WUR is supported for a normal broadband UE. Some UEs however may move with very high speed e.g. UEs on a High Speed Train (HST). Unless explicitly confirmed by RAN4, RAN2 should assume that WUR is not supported for UEs moving with very high speed like HST.

When the UE is configured with both high speed measurements and RRM measurement relaxation then it is up to UE implementation to apply the high speed measurement or relaxed RRM measurement, when the UE is stationary [3]:

When UE is configured with both high speed measurements and RRM measurement relaxation as specified in TS 38.331 [12], it is up to UE implementation whether to apply the FR1 high speed RRM requirements or the relaxed RRM requirements when the low mobility related criterion is configured and fulfilled as specified in TS 38.133 [13].

A similar principle can be applied when the UE is configured with both WUR and high speed measurements:

The LR entry/exit condition is independent of the UE speed/mobility.

When low mobility criterion is configured and fulfilled it is up to UE implementation whether to apply WUR or HST measurement requirements when both are configured.

## LR serving cell measurement relaxation and LR sleep state

The LP-SS monitoring occasions are decoupled from the DRX, i.e. paging monitoring occasions. RAN4 assumes that the UE has to take 5 LP-SS measurements samples to obtain an RSRP/RSRQ measurement quantity, while for SSB only 2 measurement samples are required spaced a DRX cycle apart. Furthermore the SSB measurement occasions are close to the DRX cycle when the UE has to monitor paging while the LP-SS measurement occasions are more distributed:

This means that an OOK-based LR has to wake more frequently compared to an OFDM-based LR, i.e. and OFDM-based LR has more time to sleep. An OFDM-based LR only needs to wake up every DRX cycle.

During the study phase it was assumed that the time to switch LR on/off (TLR, ramp-up) is FFS and the transition energy is TLR, ramp-up *(PON-POFF)/2, see TR 38.869 [4]. Whether the LR goes to sleep in between LP-SS MOs and POs can and should be left to UE implementation:

It is left to UE implementation whether the LR enters sleep state between measurement occasions for serving cell measurements and Paging Occasions (POs).

When the LR is performing serving cell measurements the power consumption is very low, and OOK-based WUR UE has to measure LP-SS every 320 ms to meet the accuracy/latency requirement:

Relaxation of LR serving cell measurement requirements is not discussed further.

# Summary

RAN2 is kindly asked to discuss WUR and RRM measurements:

Observation 1	The MR and OFDM-based WUR measurement delay equals the DRX cycle, while the OOK-based WUR measurement delay is 1.28 seconds (with 230 ms LP-SS periodicity), i.e. the delay is independent of the DRX cycle.

Proposal 1	Inform RAN4 that RAN2 uses the Rel-16 relaxation criteria as a baseline for the MR neighbour cell measurement relaxation criteria and entry/exit conditions for MR serving cell measurement offloading/relaxation.

Proposal 2	RAN2 captures the relaxation criteria (e.g. “not at cell edge” and “low mobility”) while RAN4 captures the different uses that can be identified based on the configuration and which criteria are fulfilled.

Proposal 3	RAN2 only discusses fully offloading use cases and use cases where the MR and LR temporarily perform serving cell measurements simultaneously (e.g. when performing MR and LR measurements to determine whether the entry condition is met). RAN2 does not discuss use cases where the MR is performs relaxed serving cell measurements while the LR performs non-relaxed/continuous serving cell measurements.

Proposal 4	The same entry/exit condition for serving cell measurement offloading and LP-WUS monitoring can be configured.

Proposal 5	When WUR is not configured, the UE does not support WUR, or the LR entry condition is not fulfilled, then the MR serving cell measurements are used to determine whether the neighbour cell measurements can be relaxed using the Rel-16 relaxed neighbour cell measurement criteria, i.e. legacy and UE supporting WUR behave the same.

Proposal 6	RAN2 only needs to discuss how Rel-16 neighbour cell relaxation requirements are met when the entry condition is fulfilled and the LR is performing serving cell measurements.

Proposal 7	For OFDM-based WUR when the LR performs serving cell measurements the LR determines, based on the Rel-16 neighbour cell relaxation criteria, when the MR needs to be activated to perform neighbour cell measurements.

Proposal 8	FFS for OOK-based WUR whether separate Rel-16 neighbour cell relaxation criteria (values) need to be configured.

Proposal 9	The trigger for MR neighbour cell measurements is not part of the LR exit condition, i.e. the LR is allowed to continue serving cell measurements when the MR is performing neighbour cell measurements.

Proposal 10	Neighbour cell relaxation beyond the Rel-16 criteria is not discussed further unless it can be proven that it can provide significant power saving gains.

Proposal 11	In some use cases the intra-frequency measurements requirements can be relaxed using the Rel-16 relaxation criteria. But in some use cases where coverage is highly irregular this may not be possible.

Proposal 12	RAN2 to discuss the need for intra-frequency neighbour cell measurements performed by OFDM-based LR.

Proposal 13	RAN2 to discuss the impact of a larger cell reselection margin when LP-SS measurements are used.

Proposal 14	The LP-SS measurement quantity in multibeam operation is the same as the measurement quantity used for cell reselection.

Proposal 15	The LR entry/exit condition is independent of the UE speed/mobility.

Proposal 16	When low mobility criterion is configured and fulfilled it is up to UE implementation whether to apply WUR or HST measurement requirements when both are configured.

Proposal 17	It is left to UE implementation whether the LR enters sleep state between measurement occasions for serving cell measurements and Paging Occasions (POs).

Proposal 18	Relaxation of LR serving cell measurement requirements is not discussed further.

# References

- R2-2407341, Correction to Relaxed measurement, LG, CR 38.304, Rel-16, RAN2#127

- R4-2412505, Discussion on LP-WUS RRM requirement, Ericsson, DISC, RAN4#112

- R2-2404670, Clarification on the combination of HST and RRM measurement relaxation, CR 38.300, Apple, Rel-16, RAN2#126

- TR 38.869, Study on low-power wake up signal and receiver for NR, RAN1, v18.0.0, Rel-18

# Annex: Agreements

## RAN1 agreements (selection)

Agreement

For LP-SS design from RAN1 perspective, consider at least the following as the design target:

- For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s.

  - FFS: X

  - Note: Y is chosen for evaluating LP-SS design.

  - Network overhead and network power consumption are to be considered

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Agreement

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

## RAN2 agreements (selection)

- ?? On RRM relaxation of UE MR for serving cell measurements: RAN2 assumes RRM measurement relaxation on serving cell via MR if the configured criteria is fulfilled is possible, final decision is up to RAN4.

- ?? FFS in this case whether LR is also used for RRM measurement on serving cell.

- ?? Detailed methods of measurement relaxation and offloading are up to RAN4.

Serving cell measurements related

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

Neighboring cell measurements related

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

LP-WUS/Paging monitoring related

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

General aspects

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

Serving cell measurment offloading (i.e., there is no serving cell measurement by MR)

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

## RAN4 agreements (selection)

Agreement:

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

  - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

  - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- Other related requirements are FFS

Agreement: Agreement on the target SNR for RAN4 simulation:

- The outcome of RAN1’s study in Rel-19 WI on SNR/SINR target is used as the starting point for RAN4 LP-WUR requirement study.

- Aspects not considered in RAN1 (if any) can be further discussed in RAN4.

Agreement:

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states:

- RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples.

- RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement.

For information only:

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states:

Options for further discussion (the options are only for information purpose)

- Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case.

- Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part.

- Other options are not precluded.

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

RAN4 to further discuss case #2 to #4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

Agreement:

- As starting point for RAN4 simulation purpose, uses 320 ms as the LP-SS periodicity in IDLE/Inactive mode, and other values based on RAN1/2 discussion are not precluded.

  - Not consider the MR DRX configuration in the simulation.

Agreement:

First start the simulation for FR1, and FFS for FR2 pending on the RF progress.

Recommendations:

Companies are encouraged to provide simulation assumptions based on framework in the appendix.

Agreement: Simulation assumptions (for information purpose only)

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| System bandwidth | 20/100 MHz; |
| DRX | No |
| Measurement period (in number of measurement samples) | OOK based: [5, other number of samples may also be studied upon a need] OFDM based: [5] |
| LP-SS/SSB burst periodicity | [320 ms] OFDM based: [320 ms] |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UE speed |  | FR1: 30 km/h |  |

Agreement:

- Support case #3

  - For serving cell measurement, further discuss:

    - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR

    - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR

    - Option 3: It is up to UE implementation whether to combine the measurement across the two radios.

    - Other options are not precluded.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

- Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

Recommendations:

Suggest to discuss this issue on the separately issue listed as the following:

- On requirements for evaluating entry/exit criteria for LP-WUR monitoring

- On requirements for evaluating entry/exit criteria for RRM relaxation and serving cell measurement offloading