3GPP TSG-RAN WG2 #127bis	R2-2409064

Hefei, China, Oct 14th – 19th 2024

Source:                    	DENSO CORPORATION

Title:  	Discussion on Store & Forward operation

Document for:        	Discussion and decision

Agenda Item:         	8.9.2	Support of Store & Forward

- Introduction

In Rel-19, the objectives for Store & Forward operation of IoT-NTN are described in the WID of Rel-19 IoT-NTN [1] as follows:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Based on the chair notes for last meeting RAN2#127[2], the following agreements are made for S&F operation:

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

This paper attempts to investigate about consideration on S&F operation based on the above objectives and agreements.

- Discussion

2.1 Remaining time of current operation mode

During the last meeting #127, it was discussed that the remaining time of current operation mode can be informed via System Information broadcast, and it was not concluded. We support the proposal so that UE can adjust its behavior
e.g.) not to start communication which takes long time just before the operation mode is switched. Such UE behavior would be beneficial to reduce unnecessary resource consumption due to the communication delay or retransmission at the operation mode switch. Furthermore, UE may be able to prioritize to communicate with a cell which has longer remaining time of “normal mode” if the remaining time of neighbor satellite’s current operation mode is also informed somehow, like the neighbor cell config list of SIB19 for NR NTN.

Observation 1: Broadcasting the remaining time of the satellite’s current operation mode would be beneficial to reduce unnecessary resource consumption.

Proposal 1: RAN2 to introduce broadcasting the remaining time of current operation mode.

Proposal 2: S&F related information for neighbor satellites (current operation mode, remaining time) could also be informed to UE.

2.2 Existing RRC connection at the mode switch

It has been already agreed that legacy UE (not supporting S&F mode) should be barred on “S&F mode”. Based on the assumption, RRC connection of legacy UE which is established during “normal mode” should be released when the satellite is going to “S&F mode” from “normal mode”.

Proposal 3: RRC connection of legacy UE should be released when the satellite is going to “S&F mode” from “normal mode”.

On the other hand, in case the UE supports both normal and S&F mode, it is not clear whether the connection should be released. It seems to be simpler to release all connection regardless of UE capability. However, in case of the mode switch going to “normal mode” from “S&F mode”, UE would not have any disadvantage with keeping RRC connection. Therefore, we would like to propose to discuss the case.

Proposal 4: RAN2 to discuss whether the UE which supports both “normal mode” and “S&F mode” should be released or not when the satellite switches the S&F operation mode.

2.3 UE requirement indication

It could be expected that UE’s UL data may have period of validity even though S&F UE should be delay-tolerant, since the delay on S&F operation could be significantly longer than the delay expected on the legacy delay-tolerant communication. Based on this assumption, storing & forwarding the UL data would be wasteful after the actual delay exceeds UE’s tolerable delay. Therefore, we would like to propose to introduce a mechanism to indicate UE’s requirement for UL data transmission (e.g., level of delay-tolerance, data expiration time) from UE to satellite, so that satellite can discard the data for storing & forwarding when the actual delay exceeds the requirement.

Proposal 5: RAN2 to consider whether and how UE can indicate its requirement for the UL data transmission (e.g., level of delay-tolerance, data expiration time) to satellite

2.4 EDT support on S&F mode

Aside from S&F indication, introducing Early Data Transmission (EDT) is discussed on Uplink Capacity Enhancement. We think EDT is beneficial to reduce the number of signaling and the total delay even for S&F operation. Therefore, RAN2 may discuss about considerations for using EDT on S&F operation, however, we would like to clarify if EDT is applicable for S&F operation before the discussion.

Proposal 6: RAN2 to clarify if EDT is applicable for S&F operation

- Summary and proposal

This paper discussed about consideration on S&F operation and proposed the following:

Observation 1: Broadcasting the remaining time of the satellite’s current operation mode would be beneficial to reduce unnecessary resource consumption.

Proposal 1: RAN2 to introduce broadcasting the remaining time of current operation mode.

Proposal 2: S&F related information for neighbor satellites (current operation mode, remaining time) could also be informed to UE.

Proposal 3: RRC connection of legacy UE should be released when the satellite is going to “S&F mode” from “normal mode”.

Proposal 4: RAN2 to discuss whether the UE which supports both “normal mode” and “S&F mode” should be released or not when the satellite switches the S&F operation mode.

Proposal 5: RAN2 to consider whether and how UE can indicate its requirement for the UL data transmission (e.g., level of delay-tolerance, data expiration time) to satellite

Proposal 6: RAN2 to clarify if EDT is applicable for S&F operation

- References

[1] RP-234007, “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

[2] R2-2407573, “Report from Break-out session on NR-NTN and IoT-NTN”