3GPP TSG-RAN WG2 Meeting #127bis	R2-2409070

# Hefei, China, 14 – 18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38321 | CR | 1961 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Timing Advance field in the Timing Advance Report MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2024-10-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current specification, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value, and the symbol duration is based on the subcarriers spacing the UE is currently configured with. However, the UE may be configured with multiple BWP with multiple SCS, so it’s unclear how to interpret the symbol duration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the symbol duration is based on the subcarriers spacing of the active BWP. Impact analysis Impacted functionality: NR ATG Inter-operability: If the UE is implemented according to this CR but the network is not, the network may be unclear how to interpret symbol duration. If the network is implemented according to this CR but the UE is not, the UE may be unclear how to interpret symbol duration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s unclear how to interpret the symbol duration when the UE is configured with multiple BWP with multiple SCS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 6.1.3	MAC Control Elements (CEs)

Unchanged Text is omitted

#### 6.1.3.56	Timing Advance Report MAC CE

The Timing Advance Report MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.56-1):

-	R: Reserved bit, set to 0;

-	Timing Advance: Except for ATG in FR1, the Timing Advance field indicates the least integer number of slots, using subcarrier spacing of 15 kHz, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1).

For ATG in FR1, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarriers spacing of the active BWP. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used.

The length of the field is 14 bits.

Figure 6.1.3.56-1: Timing Advance Report MAC CE

End of changes