3GPP TSG-RAN WG2 Meeting #127bis	R2-2409076

Hefei, China, 14 – 18 October 2024

Agenda item:	8.4.4

Source:	Nokia

Title:	LP-WUS in RRC_CONNECTED

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

For LP-WUS in RRC_CONNECTED, RAN1 made some progress in RAN1#118:

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18. - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration). - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration. - LP-WUS triggers the start of a timer during which UE monitors PDCCH. - FFS the timer is existing timer and/or new timer. - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected. - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL. - No impact on RRM/RLM/BFD measurement requirements is assumed. - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1). - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up. - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not. - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE. Note: Above can be revisited considering RAN2 decisions.

In RAN2#127, RAN2 also discussed LP-WUS for RRC_CONNECTED and agreed on the followings:

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

In this contribution, we continue the discussion.

# 2	Discussions

## 2.1	Option 1-2

### Issue 1. Which Timer is used

In RAN2#127, it has been agreed that the UE starts one timer after LP-WUS triggers the UE to perform PDCCH monitoring. Two options are on the table:

-	Option A. Introduce a new timer

-	Option B. Reuse drx-InactivityTimer

In DRX, the drx-InactivityTimer is configured by considering the new transmission scheduling, with a value range from 0 ms to 2560 ms. This range might seem sufficient for PDCCH monitoring after LP-WUS detection.

However, in the context of LP-WUS, the UE might only need to monitor the PDCCH for a shorter period – possibly just tens of milliseconds. This is because LP-WUS detection does not always guarantee that a new transmission will be scheduled for that UE. For instance, LP-WUS can be signalled to a group of UEs, and the UE may not be the intended recipient.

On the other hand, the UE might need to monitor the PDCCH for longer duration. This is because there can be a gap, at least larger than the offset, between the LP-WUS MO (Monitoring Occasions) and the actual PDCCH occasion. The length of this gap depends on how the MOs are configured and whether/when the traffic occurs.

Consequently, flexible approach would be preferred to balance the various monitoring duration in LP-WUS and a new timer appears to be more appropriate.

Proposal 1. In option 1-2, a new LP-WUS On-duration timer is introduced, which starts an offset after detection of LP-WUS.

### Issue 2. UE behaviour upon PDCCH detection after LP-WUS

If PDCCH is detected for the UE after LP-WUS detection, the UE should prepare for further scheduling, meaning that the PDCCH monitoring should be continued.

Since PDCCH monitoring is primarily governed by drx-InactivityTimer, it would make sense to start drx-InactivityTimer upon the reception of the PDCCH. In the legacy DRX mechanism, when drx-InactivityTimer expires, the UE starts to use either short or long DRX cycle based on configuration. This raises the question of when the UE resumes LP-WUS monitoring if it starts drx-InactivityTimer upon PDCCH detection. One option is that the UE switches back to LP-WUS monitoring if no data activity is detected such as when no PDCCH is received for several DRX cycles or when Long DRX cycle begins.

However, for more efficient power savings, it might be preferable for the UE to immediately resume LP-WUS monitoring without waiting for a few more DRX cycles. To enable such immediate transition to LP-WUS, starting the drx-InactivityTimer upon PDCCH detection might require the UE to distinguish whether the drx-InactivityTimer was initially triggered by PDCCH detection following LP-WUS or not. In other words, when drx-InactivityTimer expires,

-	if drx-InactivityTimer was started by PDCCH reception following LP-WUS, the UE resumes LP-WUS monitoring instead of transitioning to a short or long DRX cycle.

-	if drx-InactivityTimer was started by PDCCH reception without LP-WUS, the UE proceeds with either short or long DRX cycle as configured.

This added complexity could complicate both specification and implementation. To avoid these complications, it might be more straightforward to introduce a new timer instead of reusing drx-InactivityTimer, allowing clearer behavior and better alignment with power saving goals.

Proposal 2. In option 1-2, a new LP-WUS inactivity timer is introduced, which

- 	starts upon PDCCH detection following LP-WUS detection, and restarts upon PDCCH scheduling a new transmission in UL/DL while the LP-WUS inactivity timer is running.

-	triggers LP-WUS monitoring upon expiry of the timer

## 2.2	Option 1-1

For option 1-1, RAN2 has not progressed much but RAN1 assumes that, in option 1-1, the UE monitors LP-WUS before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. It still needs to be discussed further, e.g., the linkage between the impacted ON-DURATION and LP-WUS detection. For instance, we don’t know yet whether the LP-WUS indicates PDCCH monitoring at every ON-DURATION or for several number of ON-DURATIONs.

Proposal 3: For option 1-1, RAN2 waits until RAN1 makes further progress on option 1-1.

## 2.3	Option 1-3

Option 1-3 is understood that the UE monitors LP-WUS inside C-DRX active time. As concluded from the study [TR 38.869], the power saving gain is expected by reducing unnecessary PDCCH monitoring within C-DRX active time. However, there may be limited use of this option 1-3 considering the length of On duration [R2-2404380].

In our view, we already have some mechanism to end or reduce unnecessary PDCCH monitoring during active time, e.g., sending DRX command MAC CE or PDCCH skipping. Thus, it would be desirable to focus more on first two options, i.e., option 1-1 and option 1-2.

Proposal 4: Option 1-3 is excluded in RAN2.

## 2.4	gNB awareness

RAN1 has agreed that LP-WUS monitoring by UE is known to gNB and it is FFS whether implicit/explicit indication from UE is necessary. It should be noted that the coverage of LP-WUS can be different than the NR coverage and devices may equipped with different type of LR receiver with different performance. For these reasons NW is not always aware whether the UE is monitoring the LP-WUS or not. If the UE is not monitoring LP-WUS then NW should be aware of this, because otherwise NW resources are wasted for LP-WUS, which is not monitored by the UE.

Proposal 5: RAN2 to discuss explicit UE indication on LP-WUS monitoring status.

## 2.5	Any other impacts

### Issue 1. SPS/CG

For RRC_CONNECTED, RAN1 agreed that LP-WUS indicates to wake-up PDCCH monitoring. It was further agreed that RLM/BFD/CSI as well as RRM measurements are performed by the MR. However, it is unclear whether the MR keeps the SPS and CG resources activated while monitoring the LP-WUS.

If SPS and CG are assumed deactivated while monitoring LP-WUS, the gNB needs to send the LP-WUS to wake up the MR and then activates the SPS/CG via DCI, i.e., the activation command. This would introduce latency and power consumption because the UE waits for receiving the activation command after starting the PDCCH monitoring. Moreover, the gNB may intend to rely on SPS/CG scheduling instead of dynamic scheduling in the MR, in which cases further PDCCH monitoring after SPS/CG activation would be needlessly consuming power.

Thus, RAN2 needs to discuss how to reduce unnecessary PDCCH monitoring when using SPS/CG. For instance, we can consider that the LP-WUS indicates SPS/CG activation.

Proposal 6: RAN2 to discuss the LP-WUS impact on SPS and CG Type 2.

### Issue 2. UL synchronization and CSI measurement/report

During LP-WUS monitoring, it would be desirable for the UE remain in an UL synchronous state for possible data transfer, which can occur at any time. Otherwise, RA procedure would be initiated when uplink transmission is required, which could introduce latency.

In RRC_CONNECTED, timeAlignmentTimer is used for maintaining the uplink synchronization. If the timer expires, the UE flushes the HARQ buffers and releases resources such as the PUCCH resources. Therefore, RAN2 needs to discuss how to manage timeAlignmentTimer during LP-WUS monitoring.

Additionally, the periodic CSI report is one method that the gNB uses to estimate the UL timing. This report not only plays an important role in scheduling decision at the network side but also assists in UL synchronization. During LP-WUS monitoring, however, more sporadic CSI report may be sufficient to prepare for potential scheduling events while conserving power.

It’s important to note that CSI report can operate in conjunction with a CSI mask, which restricts CSI reporting to ON-DURATION period. Since the ON-DURATION depends on DRX cycle in use, RAN2 also needs to discuss how the DRX cycle is managed during LP-WUS monitoring.

Proposal 7: RAN2 to discuss how to manage the uplink synchronization status and DRX cycle while the UE monitors the LP-WUS in RRC_CONNECTED.

# 3	Conclusion

This document discusses LP-WUS in RRC_CONNECTED, and has made the following proposals:

Proposal 1. In option 1-2, a new LP-WUS On-duration timer is introduced, which starts an offset after detection of LP-WUS.

Proposal 2. In option 1-2, a new LP-WUS inactivity timer is introduced, which

- 	starts upon PDCCH detection following LP-WUS detection, and restarts upon PDCCH scheduling a new transmission in UL/DL while the LP-WUS inactivity timer is running.

-	starts LP-WUS monitoring upon expiry of the timer

Proposal 3: For option 1-1, RAN2 waits until RAN1 makes further progress on option 1-1.

Proposal 4: Option 1-3 is excluded in RAN2.

Proposal 5: RAN2 to discuss explicit UE indication on LP-WUS monitoring status.

Proposal 6: RAN2 to discuss the LP-WUS impact on SPS and CG Type 2.

Proposal 7: RAN2 to discuss how to manage the uplink synchronization status and DRX cycle while the UE monitors the LP-WUS in RRC_CONNECTED.