3GPP TSG-RAN WG2 Meeting #127bis	R2-2409077

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.6

Source:	Nokia, Nokia Shanghai Bell

Title:	XR rate control

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN#105 included UL congestion signaling in the Rel-19 XR Phase 3 objectives as follows [RP-241771]:

Specify uplink congestion signaling - Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

# 2	Discussion

## 2.1	Congestion estimation

In RAN2#120, SA2 asked if it is feasible for RAN to estimate congestion status per QoS flow/DRB in DL/UL, and without UE impacts in UL [R2-2211138]. Assuming that the QoS flows requiring similar QoS treatment will belong to the same DRB, RAN2 concluded [R2-2213226] that:

1.	It is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in downlink and uplink directions.

2.	It is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in uplink without UE impacts.

Based on the past discussion, for XR rate control, we expect that there will be no new mechanism required at the UE side, e.g., assistance information or trigger for XR rate control. Then, it should be up to the gNB how to detect the uplink congestion and when to send the rate control signalling. Similar was assumed for PSI-based discard for UL congestion as well.

Observation 1: RAN2 concluded in RAN2#120 that RAN can estimate the congestion information per QoS flow/DRB in UL without UE impacts.

Proposal 1: For XR uplink rate control, RAN2 does not specify how RAN estimates uplink congestions and when RAN transmits the rate control signalling.

## 2.2	Applicability of the XR Rate Control

From networking perspective each modality may generate one or more application layer traffic stream(s) which need to be transported within the network with specific QoS requirements. The different QoS flows of the XR applications may be mapped to different DRBs and in RAN, a single DRB may contain QoS flows from different applications when their QoS requirements are similar.

As XR applications are latency sensitive, latency indication, specifically, RAN congestion information, is a piece of information that would benefit XR application rate control performance and thus improve overall QoS and user experience. However, not all QoS flows of an XR application have the capability to dynamically adjust their codec bit rate and encoding parameters. For example, a video flow may have an adaptive codec which can be adjusted based on the congestion in the network, but same may not be true for the haptic feedback. Therefore, it becomes important to identify which QoS flow(s) and DRB(s) have the possibility to dynamically adjust their bit rate. On the high level, two options can be explored further in RAN2:

-	Option 1: UE based approach, for example via RRC signalling, UE can inform gNB which QoS flow(s)/DRB(s) are with the capability of adjusting their bitrate when provided with congestion information.

-	Option 2: CN based approach, for example gNB getting the information from CN entity (e.g. SMF) of which QoS flow(s) are with the capability of adjusting their bitrate when provided with congestion information.

Both options are feasible, while considering potential UE impact, Option 2 is slightly preferred.

Observation 2: XR rate control signalling is applicable only to the QoS flows or DRBs with QoS flows which have the possibility of adaptive codec/bitrate.

Proposal 2: For XR rate control, RAN2 discuss different options how the gNB identifies which QoS flows are subject to adaptive bitrate when provided with congestion information.

## 2.2	Potential impact in MAC

The primary goal of XR rate control is to adjust the bit rate to ensure optimal quality for transmission of XR application data. In this context, the WID includes an objective to specify XR rate control signalling over the downlink per QoS flow or per DRB in MAC layer.

In TS 38.321, a Recommended Bit Rate procedure is already specified, which offers a framework for controlling the bit rates based on e.g., radio link quality. In the Recommended Bit Rate procedure,

-	The UE receives the Recommended bit rate MAC CE per logical channel from the gNB;

-	The UE may request the gNB to provide the Recommended Bit Rate for a certain logical channel;

-	The bit rate command MAC CE is of 2 octets long, including 6 bits of LCID field, 5 bits of Bit Rate field, 1 bit of bit rate multiplier, and 1 bit of UL/DL field.

The Recommended Bit Rate procedure provides information similar to what is intended for XR rate control. Thus, it can be considered as a baseline when designing the XR rate control. However, there may be something to be considered for XR rate control such as how to indicate a specific QoS flow in the XR rate control MAC CE or whether the same granularity of bit rate defined in the Recommended bit rate MAC CE is reused.

Proposal 3: RAN2 discuss the range of QoS flow IDs and bit rates to be supported in XR rate control by taking the existing Recommended bit rate MAC CE as a baseline.

# 3	Conclusion

In this contribution, we discussed the XR rate control, newly added as an objective in RAN#105. Our observations and proposals are:

Observation 1: RAN2 concluded in RAN2#120 that RAN can estimate the congestion information per QoS flow/DRB in UL without UE impacts.

Proposal 1: For XR uplink rate control, RAN2 does not specify how RAN estimates uplink congestions and when RAN transmits the rate control signalling.

Observation 2: XR rate control signalling is applicable only to the QoS flows or DRBs with QoS flows which have the possibility of adaptive codec/bitrate.

Proposal 2: For XR rate control, RAN2 discuss different options how the gNB identifies which QoS flows are subject to adaptive bitrate when provided with congestion information.

Proposal 3: RAN2 discuss the range of QoS flow IDs and bit rates to be supported in XR rate control by taking the existing Recommended bit rate MAC CE as a baseline.