3GPP TSG-RAN WG2 Meeting #127bis	R2-2409087

Hefei, China, 14 – 18 October 2024

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SIB1 for Idle/Inactive mode UEs

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

The release 19 work item on network energy saving (RP-242354) includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we will discuss the issues related to the on-demand SIB1 for idle/inactive mode UEs. And in our companion contribution, we will discuss the issues on-demand SSB SCell operation and adaptation of common signal/channel transmissions in [1] and [2], respectively.

In RAN2#127, the following agreements were made:

Agreements on OD-SIB1

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

In this paper we discuss further details of the OD-SIB1 for case 2 agreed in the RAN#105 plenary meeting.

# 2	Baseline operation for case 2

- UE obtains the WUS configuration from cell A,

  - PCI+Frequency (at least) used to associate configuration to a cell.

  - Cell A may provide configuration for multiple cells.

  - Multiple Cell A(s) may provide WUS configuration for same cell.

  - At least configuration is provided by cell A SIB(s). in addition to cell A SIB (s), RRCRelease may provide WUS configuration. This makes sense to us as there is likely scenarios that cell starts to use OD-SIB1 and releases UES to IDLE/INACTIVE – It seems quite good idea not to mandate reselection to Cell A and then back to NES Cell

- Once UE (re)selects to cell for which UE has WUS configuration and NW indicates it is utilizing OD-SIB1 procedure - UE may request OD-SIB1. Legacy UE should bar this kind of cell but OD-SIB1 supporting UE should not.

  - Request is assumed to be preamble transmission

    - We don’t combine this with direct request for moving to RRC_CONNECTED i.e. UE will first acquire SIB1 and after that resume/request to be moved to RRC_CONNECTED

  - RAN2 assumes UE will receive RAR

  - Assuming NW detects OD-SIB1 request it will provide SIB1 to UE. It has not been discussed in RAN2 whether SIB1 provision is:

    - via OD-SIB1 specific transmission AND/OR

    - NW resumes regular SIB1 transmission

- If SIB1 reception is successful

  - UE may camp on the cell will apply regular operations – paging reception, SI change modification monitoring etc…

  - It was left for further consideration how UE determines validity of SIB1

  - RAN2 also agreed that UE needs reacquire SIB1 in case of SIB change notification (legacy behaviour).

- If not successful

  - As baseline upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. Without modification to specification if UE has not been able to acquire SIB1 it will bar the cell for up to 300 seconds

As we can see RAN2 has established baseline operation for case 2 but of course there are still various aspects that need to be handled.

# 2	Access restriction for legacy UEs and NES UE behavior

RAN2#127 agreed about the following:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Legacy UEs should not access to the NES cell supporting OD-SIB1. However, the different options have different impacts:

Regarding option1: barring legacy UEs by setting the barring bit in MIB does not work for all legacy UEs. For example, Redcap, Re1 18 NES UE and NTN UEs are barred using barring information carried in SIB1. Thus, for these UEs, they need to acquire SIB1 to obtain the barring information. But a cell with OD-SIB1 operation is not transmitting SIB1 unless it receives an UL WUS, thus these legacy UEs cannot be barred by setting the barring bit in MIB.

Observation 1: Option 1 does not allow barring for all legacy UEs. At least RedCap, Rel18 NES and NTN UE cannot be barred using option 1.

Proposal 1: RAN2 to preclude option 1 from the list of options to handle legacy UEs.

Regarding option 2: this option covers the initial cell selection as well as cell re-selection. We provide a detailed discussion about the selection of ssb-SubcarrierOffset (K_SSB) value for NES cell with OD-SIB1 operation:

- K_SSB > 23 for FR1 and K_SSB > 11 for FR2

For these values, the cell is referred as non-Cell defining SSB. However, 38.300 specifies the following for cell selection and cell reselection for NCD-SSB:

Cell selection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4):

Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4).

This mean legacy UE will not select or re-select a NES with OD-SIB1 operation if K_SSB > 23 for FR1 and K_SSB > 11 for FR2. In addition, based on 38.304, if legacy UE is unable to acquire SIB1, legacy UE may treat the cell as barred for 300s.

Observation 2: Option 2 can cover initial cell-selection and cell re-selection.

Observation 3: For a NES cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs do not select or re-select the NES cell.

Observation 4: For a NES cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs may treat the cell as barred for 300s due to not able to acquire SIB1.

In the following, we provide analysis which set of K_SSB values can be used to avoid any impact to legacy UE:

- For 23 < K_SSB < 30 for FR1 and 11 < K_SSB < 14

Based on 38.213 clause 13, UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH. UE determines the GSCN offset using the associated GSCN offset to K_SSB provided by Table 13-16 for FR1 and Table 13-17 for FR2.

Thus, for these values of K_SSB, legacy UE keeps monitoring and searching for a second SS/PBCH block having a CORESET. This will consume UE power for unnecessary operation as NES cell is not transmitting any SS/PBCH block with pdcchSIB1config pointing to a CORESET.

Observation 5: For a NES cell with 23 < K_SSB < 30 for FR1 and 11 < K_SSB < 14 for FR2, unnecessary operation is applied by legacy to search for a second SS/PBCH block pointing to valid Type0-PDCCH.

- For K_SSB = 30 for FR1 and K_SSB = 14 for FR2

Based on 38.213 clause 13, legacy UE determines that the 8 bits associated to pdcchSIB1config are reserved. SS/PBCH block does not have an associated Type0-PDCCH. Based on these values of K_SSB, legacy UE cannot determine any offset to a second SS/PBCH block associated to a valid Type0-PDCCH. Therefore, legacy UE does not apply unnecessary procedure to search for a second SS/PBCH block having a CORESET for an associated Type0-PDCCH. Legacy UE identifies that cell is with SIB1-less operation and based on 38.304, legacy UE may treat the cell as barred for 300s as UE unable to acquire SIB1.

For NES UEs, these values of K_SSB can be used to identify the cell as a cell operating with OD-SIB1 operation.

In addition, the 8 reserved bits for pdcchConfigSIB1(see Table 13-16 for FR1, and Table 13-16 for FR2, 38.213 v17.1.0) associated to these values of K_SSB may be used to provide indication, e.g. about GSCN, of Cell A. This can help UE to switch quickly to Cell A to obtain WUS configuration.

Observation 6: For a NES cell with K_SSB = 30 for FR1 and K_SSB = 14 for FR2, legacy UE can determine that NES cell is with SIB1-less operation and UE may treat directly the cell as barred for 300s without spending unnecessary operation to search or to monitors for another SS/PBCH block with valid Type0-PDCCH.

Proposal 2: A NES cell with K_SSB limited to 30 for FR1 and 14 for FR2 may use the 8 reserved bits for pdcchConfigSIB1 to provide indication about Cell A.

- For K_SSB = 31 for FR1 and K_SSB = 15 for FR2

Based on 38.213, clause 13, these values are used to indicate SIB1-less operation for legacy UEs. Same interpretation can be applied for NES UEs.

Observation 7: For a NES cell with K_SSB = 31 for FR1 and K_SSB = 15 for FR2, legacy UE and NES UEs can determine that NES cell is with SIB1-less operation. Legacy UE and NES UEs may treat directly the cell as barred for 300s.

The following diagram summarizes K_SSB value selection on NES cell identification and UE barring aspects.

Regarding option 3, this option is applicable only for re-selection and cannot cover initial cell selection. Thus, this would be complimentary to options handling initial cell selection. One of the motivations for this kind of option is to limit legacy UE impacts to minimum as NW can configure layers/cells using OD-SIB1to be not in the candidate for reselections. This way legacy UE does not need to reselect to cell and find out that it does not provide SIB1 and thus legacy UE would need to bar the cell.

Observation 8: Option 3 helps to minimize legacy UE reselection impacts from OD-SIB1 cells

UE is assumed camping on CellA and receiving via SIB, e.g. SIB3, the legacy intra/interFreqExcludedCellList that prevents legacy UEs to re-select NES cells (OD-SIB1 cells) as these cells are added to this legacy list. To avoid preventing NES UEs to re-select NES cells, one would need to allow NES UE to consider reselections to OD-SIB1 cells

Observation 9: Option 3 can be applied only to cell re-selection. Option 3 cannot cover initial cell-selection.

In our view option 3 would be beneficial but just relying on “excluded cell list” may be bit cumbersome in scenarios where whole layer of cells is using OD-SIB1 – then it would be easier to consider having NES specific reselection priorities i.e. for legacy UEs one would not even provide reselection priority for OD-SIB1 layer. But we also see that there are scenarios where subset of cell or in the border area it is beneficial to allow reselection to some cells of the layer. Thus, we propose:

Proposal 3: NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

# 3	RAR response handling

RAN1 consider dedicated PRACH resources for SIB1 request. Dedicated PRACH resources are not referring to UE-specific PRACH resources but rather PRACH resources dedicated to on-demand SIB1 request.

For PRACH resources for UL WUS, RAN2 should consider on-demand SI procedure as starting point. For on-demand SI, the network may configure the UE with dedicated RA resources in SI-RequestConfig within SI-Scheduling Info IE in SIB1. For on-demand SIB1, the dedicated RA resources could be part of the WUS configuration. Any of time, frequency and/or PRACH preamble resources may be used as dedicated PRACH resources to allow full flexibility to the network.

If common PRACH preamble is used for all UEs requesting on-demand SIB1 operation, this can allow a RAR response that can be interpreted/decoded by all UEs. Thus, a UE intended to transmit UL WUS for requesting SIB1, can monitor if any RAR response is sent by network to check if on-demand SIB1 is already requested by other UEs. However, if dedicated ROs, time/frequency resources are considered as dedicated PRACH resources and it is up to UE to select a preamble, RAR response is a dedicated response that can be decoded only by UE requesting on-demand SIB1.

Observation 10: Designing dedicated PRACH resources for SIB1 request assuming a common preamble for requesting SIB1 used by all UEs. It allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

It might be difficult though with current RAR monitoring based on RA-RNTI of the PRACH occasions as the UE would need to monitor all the potential RA-RNTIs of PRACH for OD-SI before WUS transmission. Common RA-RNTI would be required to enable this.

Observation 11: To enable UEs monitoring on-demand SIB1 without transmitting UL WUS, may require configuring a common RNTI.

Proposal 4: RAN2 to study further the procedure allowing UE to monitor on-demand SIB1 transmission requested by other UEs before UE to transmit UL WUS for on-demand SIB1.

# 3	WUS configuration and validity

In RAN2#126 we had following discussion:

Proposal 9b (modified): After the UE camps on NES cell, UE can acquire WUS configuration from the NES cell.

- Postponed.

[Session chair]: Understand the scenario is the UE reselected from cell A to NES cell. While UE stays in the NES cell, the UE may need to re-acquire SIB1. Then the question is how the UE gets WUS configuration? E.g. performs cell reselection to cell A and reacquires WUS configuration or reacquire from NES cell itself? [Rakuten]: If NW tries to transmit SIB1 always to guarantee 3 hours invalidity, how NW can have energy saving? [Nokia]: Understand NES cell does not transmit SIB1, then how its SIB1 can be transmitted to cover this concern?

As described in 2.1.1 in our view while camping on NES cell operating on OD-SIB1 mode UE gets indications whether SIB1 is updated and UE would then request OD-SIB1 from the cell based on the configuration it has. With existing agreements we can only provide WUS configuration via SIB of cell A (and possible in RRCRelease) thus UE would need to somehow know whether the WUS configuration it has is still valid.

Same issue lies in the case UE acquires WUS configuration from cell A and reselecting immediately or later to NES cell – how does UE know the WUS configuration is still valid? We could consider following options:

- Time limitation for validity after which UE needs reacquire WUS configuration in order to use OD-SIB1 procedure. But with this there is always chance that NW needs to update WUS configuration while validity time is still running.

- We have some sort of value tag on WUS configuration and UE can check validity from the NES cell using OD-SIB1 mode. This would require transmission of value tag somewhere else than SIB1 (as that is not transmitted on the cell). Having this in MIB is challenging and likely it is quite difficult to have any new signal associated to MIB with space for value tag.

Observation 12: It seems quite difficult to avoid UE never using outdated WUS configuration unless we can have some value tag added in MIB/new signal.

So question is that how severe the problem is if UE uses old WUS configuration. If it is small then likely no extra solutions are needed but e.g. similar time limitation as for SIB (3 hours) we use for WUS configuration. But if the problem is severe then it might be that OD-SIB1 procedure is quite difficult to realize. In case UE uses old WUS configuration it will fail to receive SIB1 and will bar the cell and obtain new configuration from cell A. Based on this from technical perspective it would be best if one could have method for UE to check whether the used WUS configuration is valid or not.

Proposal 5: RAN2 to consider a value tag in WUS configuration and a value tag in MIB. RAN1 needs to be consulted if this is feasible from their point of view.

In RAN2#127, the following agreement was made:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

To provide clarity about this agreement, we would like to reformulate the agreement as following:

Proposal 6: Once Rel-19 NES UE camps on cell providing UL WUS configuration(s) the UE expects to receive UL WUS configuration updates of any WUS configuration provided by the cell (e.g. WUS configuration for the cell UE where UE is camped and/or UL WUS configuration about other NES cells) e.g., via legacy SI modification procedures.

# 5	Conclusions:

Observation 1: Option 1 does not allow barring for all legacy UEs. At least RedCap, Rel18 NES and NTN UE cannot be barred using option 1.

Observation 2: Option 2 can cover initial cell-selection and cell re-selection.

Observation 3: For a NES cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs do not select or re-select the NES cell.

Observation 4: For a NES cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs may treat the cell as barred for 300s due to not able to acquire SIB1.

Observation 5: For a NES cell with 23 < K_SSB < 30 for FR1 and 11 < K_SSB < 14 for FR2, unnecessary operation is applied by legacy to search for a second SS/PBCH block pointing to valid Type0-PDCCH.

Observation 6: For a NES cell with K_SSB = 30 for FR1 and K_SSB = 14 for FR2, legacy UE can determine that NES cell is with SIB1-less operation and UE may treat directly the cell as barred for 300s without spending unnecessary operation to search or to monitors for another SS/PBCH block with valid Type0-PDCCH.

Observation 7: For a NES cell with K_SSB = 31 for FR1 and K_SSB = 15 for FR2, legacy UE and NES UEs can determine that NES cell is with SIB1-less operation. Legacy UE and NES UEs may treat directly the cell as barred for 300s.

Observation 8: Option 3 helps to minimize legacy UE reselection impacts from OD-SIB1 cells

Observation 9: Option 3 can be applied only to cell re-selection. Option 3 cannot cover initial cell-selection.

Observation 10: Designing dedicated PRACH resources for SIB1 request assuming a common preamble for requesting SIB1 used by all UEs. It allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

Observation 11: To enable UEs monitoring on-demand SIB1 without transmitting UL WUS, may require configuring a common RNTI.

Observation 12: It seems quite difficult to avoid UE never using outdated WUS configuration unless we can have some value tag added in MIB/new signal.

Proposal 1: RAN2 to preclude option 1 from the list of options to handle legacy UEs.

Proposal 2: A NES cell with K_SSB limited to 30 for FR1 and 14 for FR2 may use the 8 reserved bits for pdcchConfigSIB1 to provide indication about Cell A.

Proposal 3: NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

Proposal 4: RAN2 to study further the procedure allowing UE to monitor on-demand SIB1 transmission requested by other UEs before UE to transmit UL WUS for on-demand SIB1.

Proposal 5: RAN2 to consider a value tag in WUS configuration and a value tag in MIB. RAN1 needs to be consulted if this is feasible from their point of view.

Proposal 6: Once Rel-19 NES UE camps on cell providing UL WUS configuration(s) the UE expects to receive UL WUS configuration updates of any WUS configuration provided by the cell (e.g. WUS configuration for the cell UE where UE is camped and/or UL WUS configuration about other NES cells) e.g., via legacy SI modification procedures.