3GPP TSG-RAN-WG2 Meeting #127bis	R2-2409095

Hefei, China, Oct 14th – 18th, 2024

Agenda item	:		8.3.3

Source	: 	Sharp

Title	: 	Discussion on Measurement Event Predictions

Document for	:	Discussion and Decision

# Introduction

In RAN2#125bis the following agreements were made for AI/ML-based measurement event predictions.

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed.

- Clarifications on what is being as input should be provided with results.

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made.

Further, in RAN2#127, the following agreements were made with respect to direct and indirect prediction models for AI/ML mobility.

Agreements

-	Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using

-	Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models.

-	Output for direct: probability of RLF within an window

-	Companies should report the prediction time window they have used in the simulations

In this contribution, preliminary views on AI/ML-based measurement event predictions are discussed.

# Discussion

In the SID [1], it is considered that the measurement predictions consider both UE-sided models and network-sided models. As agreed in the previous meetings, several aspects including cell-level, beam-level temporal measurement predictions have already been considered in the study. As shown in Figure 1, RRM measurements in general are utilized as inputs to the configured AI/ML model to produce predictions or inferences for future RRM measurements or predict mobility events (e.g. A3).

Figure 1: Measurement event prediction based on past measurements, and other parameters

Irrespective of the type of measurements, since the measurements are performed at the UE, the following aspects need to be considered. It can be expected that the UE performing the measurement would have access to the measurement data earlier than the network, which needs to wait until the UE reports the current measurements. Further, since reporting more measurements than existing procedures results in additional signaling, it can be expected that the UE typically has more measurement data relevant to itself than the network. Irrespective of the measurement reporting configurations, the UE may also independently perform measurements if required, without reporting the results to the network. Given these factors, it may be beneficial to first focus the study on UE-sided model. Although UE may have lesser compute/energy resources than the network, UE-sided model can potentially provide benefits associated with reduced signaling and possibly also reduce latency associated with inferences of AI/ML mobility models. Therefore, it is proposed that the UE-sided models are prioritized in the initial stages of the study.

Observation 1: The UE performing measurements has access to the measurement data earlier than the network, since the network only has access to measurement results after the UE reports.

Observation 2: Since the UE does not report all measurements to the network, it can be expected that the UE has access to more measurements than the network.

Proposal 1: RAN2 to prioritize UE-sided model for RRM measurement event prediction.

Since it is agreed in the SID [1] that any decisions on handovers are made by the network, in case UE-sided AI/ML mobility models are used, it would be essential that the UE communicates some information on the model predictions to the network. Since predicted measurements inherently differ from legacy measurement results, it might require further considerations during the study on whether the existing measurement reporting frameworks are sufficient for UE-sided AI/ML models, even though the predicted results may be in the same format (RSRP) as the legacy measurements. This is required to enable the network to differentiate clearly between actual measurements and predicted measurements. Further, the measurement events (e.g. A3) corresponding to actual measurements and predicted measurement events (e.g. predicted A3 event) based on predicted measurement results would also require similar differentiation when being reported by the UE.

Observation 3: Measurement report predictions and measurement event predictions are inherently different from legacy (or actual) measurement results, even though the two entities might represent the same quantities, e.g. actual RSRP and predicted RSRP or actual A3 event and predicted A3 event.

Proposal 2: RAN2 to discuss the applicability of the legacy measurement reporting procedure to predicted measurement results and predicted events in the case of UE-sided AI/ML Mobility models.

# Conclusion

In this contribution, we have the following observations:

Observation 1: The UE performing measurements has access to the measurement data earlier than the network, since the network only has access to measurement results after the UE reports them.

Observation 2: Since the UE does not report all measurements to the network, it can be expected that the UE has access to more measurements than the network.

Observation 3: Measurement report predictions and measurement event predictions are inherently different from legacy (or actual) measurement results, even though the two entities might represent the same quantities, e.g. actual RSRP and predicted RSRP or actual A3 event and predicted A3 event.

In this contribution, we have the following proposals:

Proposal 1: RAN2 to prioritize UE-sided model for RRM measurement event prediction.

Proposal 2: RAN2 to discuss the applicability of the legacy measurement reporting procedure to predicted measurement results and predicted events in the case of UE-sided AI/ML Mobility models.

# Reference

[1] RP-240082 Revised SID on AIML for mobility in NR.