3GPP TSG-RAN WG2 Meeting #127bis	R2-2409097

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:	8.6.4

Source:	Samsung

Title:	Support of Conditional Intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

Based on the revised WID for Rel-19 Mobility WI [1], the objective of the support of conditional intra-CU LTM was confirmed as below.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, we will further address how to support the conditional intra-CU LTM in detail.

Discussion

Introduction

Issue 1: Execution condition for subsequent conditional Intra-CU LTM

Rel-18 LTM needs the explicit signalling (i.e. LTM cell switch MAC CE) to trigger the cell switch based on the measurement report by either L3 or L1 manner. In Rel-19, the event triggered measurement reporting would be introduced but it also cannot guarantee the signalling weakness of cell switch message i.e. UE might fail to receive the LTM cell switch signalling because of weak channel qualities. In order to enhance the reliability of the handover, the conditional LTM which UE autonomous triggering to execute the cell switch based on the Layer 1 measurement results is needed. From our understanding, conditional LTM could be applied for MCG and SCG but as a starting point only MCG CLTM is introduced in Rel-19, see the objective in [1]. So, in the following we only focus on the MCG CLTM for intra-CU.

According to the WID [1], support of the subsequent conditional LTM for intra-CU is the scope of Rel-19 so we want to first consider how to support subsequent conditional LTM for intra-CU. Each source cell and LTM candidate cells need to prepare the multiple execution conditions considering when each candidate cell becomes the source cell. In CHO, one target cell is associated with one or two condition(s) using the measId which provides the execution condition. RRC structure of CHO and LTM could be good reference and the CLTM structure could include the association between the L1 measurement resource configuration and the execution conditions.

From our understanding, the first question to support subsequent CLTM is which node generates the execution condition for the subsequent CTLM. For the legacy CHO, we think the source CU generates the execution conditions based on the measurement configuration, for the subsequent CPAC, the source SN generates the initial execution condition and the candidate SN generates the following execution conditions. This is because the execution condition is L3 based measurement but LTM is based on the beam measurement (L1). Therefore, we think the entity to decide the execution condition for the conditional LTM should be the DU (i.e. source cell and candidate cells). To support the subsequent conditional intra-CU LTM, each candidate cell needs to generate the execution conditions when the candidate cell becomes the source cell and provide it to the source CU in LTM candidate cell configuration.

Proposal 1: Execution conditions of the subsequent conditional LTM are determined by DU.

      - Source DU generates the initial execution condition

      - Candidate DU generates the following execution conditions when the candidate cell becomes the source cell after CLTM is success.

Proposal 2: Each DU (i.e. source cell and candidate cells) provides the execution conditions and associated L1 measurement resources to the source CU by F1 interface.

We think the legacy LTM-CSI-ResourceConfig IE could be reused (or slightly updated) for the Rel-19 conditional LTM resource configuration. It means the new IE for the conditional LTM CSI measurement resource configuration is not needed. Then how to provide the execution condition and the associated L1 measurement resources are needed. We think ltm-CSI-ReportConfig or same level of configuration (i.e. in the source cell configuration) could be used for providing this configuration (execution condition and the associated L1 measurement resources). For the explicit target cell configuration, the legacy LTM candidate cell configuration could be referred in the conditional LTM configuration which provides the associated information and execution conditions.

Proposal 3: LTM measurement resource configuration for the conditional LTM is provided in LTM-CSI-ResourceConfig-r18.

Proposal 4: Execution conditions with the associated L1 measurement resources and index of the candidate cell configuration to be applied are provided in ltm-CSI-ReportConfig or same level of configuration (i.e. in the source cell configuration).

For the execution condition for the conditional LTM, we think the events for the LTM what RAN2 agreed in the previous meeting could be reused for CLTM purpose.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In this case, we think similar as CHO, only following three events could be introduced for CLTM. For these event the offsets, hysteresis and timeToTrigger could be configured to avoid the frequent trigger of CLTM.

      - CondEvent LTM3: Beam of candidate cell becomes amount of offset better than beam of PCell/PSCell;

      - CondEvent LTM4: Beam of candidate cell becomes better than absolute threshold;

      - CondEvent LTM5: Beam of PCell/PSCell cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Proposal 5: CondEvent LTM3, CondEvent LTM3 and CondEvent LTM5 are defined for conditional LTM and these events could provide the offsets, hysteresis and timeToTrigger.

      - CondEvent LTM3: Beam of candidate cell becomes amount of offset better than beam of PCell/PSCell;

      - CondEvent LTM4: Beam of candidate cell becomes better than absolute threshold;

      - CondEvent LTM5: Beam of PCell/PSCell cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Issue 2:  Support of RACH-less CLTM

In the revised WID [1], there are no description to support the RACH-less CLTM for intra- CU case. We want to highlight what aspects could be added if the RACH-less CLTM is introduced (i.e. possible options). Basic concept is that UE try to RACH-less LTM if UE have valid TA for the target cell which meets the execution condition.

      - How to acquire the TA for RACH-less LTM

        - Option 1: early TA PDCCH order with RAR

          - Need to discuss which cell provides RAR i.e. from candidate cell or from source SpCell

          - UE transmit the RACH preamble for early TA calculation

          - TA should be provided to the UE

          - UE could start the TA timer when UE receives the RAR including valid TA

        - Option 2: UE based TA

          - Similar operation in Rel-18 could be applied

          - Need to be confirmed by RAN4

      - How to configure the UL grant for the first UL transmission after RACH-less C-LTM

        - Configured Grant (CG) resources per candidate cell

      - How to reduce the interruption time between RACH preamble Tx and RAR reception for the candidate cell if RAR is received from the candidate cell (no issue if the source SpCell provides the RAR)

        - New RAR receive window

        - UE capability for simultaneous RACH preamble TX  and RAR Rx to/from the candidate cells

      - Which beam is used for RACH-less CLTM

        - The beam triggers the CLTM execution condition

Proposal 6: RAN2 discuss if the RACH-less conditional LTM is supported in Rel-19.

Issue 3:  Security aspects

Since the scope of Rel-19 CLTM is limited to intra-CU, no security key change issues are foreseen. It means that the master key change is not supported and this restriction could be added in the specification similar as Rel-18 LTM.

Proposal 7: For intra-CU conditional LTM, master key change is not supported.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: Execution conditions of the subsequent conditional LTM are determined by DU.

      - Source DU generates the initial execution condition

      - Candidate DU generates the following execution conditions when the candidate cell becomes the source cell after CLTM is success.

Proposal 2: Each DU (i.e. source cell and candidate cells) provides the execution conditions and associated L1 measurement resources to the source CU by F1 interface.

Proposal 3: LTM measurement resource configuration for the conditional LTM is provided in LTM-CSI-ResourceConfig-r18.

Proposal 4: Execution conditions with the associated L1 measurement resources and index of the candidate cell configuration to be applied are provided in ltm-CSI-ReportConfig or same level of configuration (i.e. in the source cell configuration).

Proposal 5: CondEvent LTM3, CondEvent LTM3 and CondEvent LTM5 are defined for conditional LTM and these events could provide the offsets, hysteresis and timeToTrigger.

      - CondEvent LTM3: Beam of candidate cell becomes amount of offset better than beam of PCell/PSCell;

      - CondEvent LTM4: Beam of candidate cell becomes better than absolute threshold;

      - CondEvent LTM5: Beam of PCell/PSCell cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Proposal 6: RAN2 discuss if the RACH-less conditional LTM is supported in Rel-19.

Proposal 7: For intra-CU conditional LTM, master key change is not supported.

References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.