3GPP TSG RAN WG2 #127bis	 R2-2409099

# Hefei, China, October 14th – 18th, 2024

Agenda item:	7.25.2

Source:	Samsung

Title:	Discussion on the applicable BWP for certain RRC parameters for DCI format 0_3/1_3

Document for:	Discussion and decision

- Introduction

This contribution discusses the correction proposed in the CR [1] for TS 38.331 v18.3.0 regarding the applicable BWP for certain fields in DCI formats 0_3/1_3.

- Applicable BWP in case of invalid value of BWP field

For a DCI format 0_3/1_3 associated with a set of cells MC-DCI-SetOfCells, higher layer configuration corresponding to a number of DCI fields are in the form of respective joint tables that are provided within the configuration of MC-DCI-SetOfCells. Such fields and corresponding RRC parameters include: {rate matching pattern indication field (RateMatchDCI-1-3), TCI field (TCI-DCI-1-3), ZP-CSI trigger field (ZP-CSI-DCI-1-3), SRS offset field (SRS-OffsetCombo), SRS request field (SRS-RequestCombo)}.

Since the configured values/indexes within the joint tables for these DCI fields are independent of an applicable BWP, RAN1 discussed how the UE interprets the field values/indexes provided by a Type-1B field for different BWPs, and agreed to the following [2,3]:

Agreement (RAN1#114bis) - Single joint table is configured per set of cells for each of Type-1B fields other than TDRA (i.e., rateMatchListDCI-1-3, zp-CSI-RSListDCI-1-3, tci-ListDCI-1-3, srs-RequestListDCI-1-3, srs-OffsetListDCI-1-3, srs-RequestListDCI-0-3, srs-OffsetListDCI-0-3). - Entries for each CC are interpreted based on the new/target BWPs per cell that is indicated by the BWP indicator field of DCI 0_3/1_3. - Single joint table is configured per set of cells for TDRA (i.e., TDRA-FieldIndexListDCI-1-3, TDRA-FieldIndexListDCI-0-3). - Entries of the joint table for TDRA (i.e., TDRA-FieldIndexDCI-1-3) are configured for each BWP of each CC. - Columns of the indicated entry corresponding to the new/target BWPs per cell that is indicated by the BWP indicator field of DCI 0_3/1_3 are applied. - The maximum size of TDRA-FieldIndexListDCI-1-3 is 32. - The maximum size of TDRA-FieldIndexListDCI-0-3 is 64.

The above agreement was captured in TS 38.331, for example as follows for the rate matching pattern indication field.

Excerpt from [TS 38.331 v18.3.0] RateMatchDCI-1-3 Configure each row of the joint rate matching indication table for DL scheduling via DCI format 1_3, where bitmap for a cell points to a corresponding rate matching indication applicable for DCI format 1-1 (i.e., MSB and LSB of bitmap refer rateMatchPatternGroup1 and rateMatchPatternGroup2 for a cell, respectively), the order of rate matching indication bitmap in each row refers the order of cells in ScheduledCell-ListDCI-1-3, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP (i.e., first bitmap is for the first cell in ScheduledCell-ListDCI-1-X, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP and so on), the number of entries in a row of rateMatchDCI-1-3 should be the same as the number of cells, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP, included in ScheduledCell-ListDCI-1-3, and entries for co-scheduled cells in a row of rateMatchDCI-1-3 are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3.

RAN1 identified, however, that the BWP indicator field can indicate a codepoint that is valid for some cells in the set of cells, while invalid for some other cells. Therefore, the following related agreement was made in RAN1#115 that clarifies the application of the current active BWP in such event.

Agreement (RAN1#115) - BWP indicator in a DCI format 0_3/1_3 applies only to the scheduled cell(s) with valid FDRA value(s). - For a cell scheduled by DCI format 0_3/1_3 with valid FDRA value, if the BWP indicator indicates a code point that does not correspond to a configured BWP for the cell, the UE does not perform dynamic BWP switching based on the BWP indicator and transmits/receives data on the current active BWP of the cell.

The description in TS 38.331 v18.3.0 does not capture the above agreement from RAN1#115 for an applicable BWP in case of invalid value for the BWP codepoint. Therefore, this missing case needs to be captured in TS 38.331.

Proposal 1: For RRC parameter X = {RateMatchDCI-1-3, TCI-DCI-1-3, ZP-CSI-DCI-1-3, SRS-OffsetCombo, SRS-RequestCombo)}

- Clarify that, entries for co-scheduled cells in a row of the RRC parameter X are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3 when the BWP indicator field indicates a code point that corresponds to a respective configured BWP for the respective cell, and based on the respective current active BWPs otherwise.

- Adopt the corresponding TP as proposed in the CR [1];

- Applicable BWP for TCI field

The following UE procedure for TCI state indication was endorsed in RAN1#117 [4]. A TCI field of a DCI format 1_3 can indicate TCI states both for co-scheduled cells and non-scheduled cells of a set of cells scheduledCellListDCI-1-3. For example, the set of cells scheduledCellListDCI-1-3 includes cells {1, 2, 3, 4}, and a DCI format 1_3 co-schedules cells {1, 2}. The TCI field of the DCI format 1_3, however, indicates TCI state for both the scheduled cells {1, 2} and the non-scheduled cells {3,4}. In addition, the indicated TCI states apply to other cells outside the set of cells {1, 2, 3, 4} (thereby clearly not scheduled and) in a same CC list by simultaneousU-TCI-UpdateList1/2/3/4-r17.

Excerpt from TS 38.214 v18.4.0 based on the endorsed CR in [4] When tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, a UE configured with dl-OrJointTCI-StateList with activated TCI-State or ul-TCI-StateList with activated TCI-UL-State receives DCI format 1_1/1_2/1_3 providing indicated TCI-State(s) and/or TCI-UL-State(s) for a CC or all CCs in the same CC list configured by simultaneousU-TCI-UpdateList1-r17, simultaneousU-TCI-UpdateList2-r17, simultaneousU-TCI-UpdateList3-r17, simultaneousU-TCI-UpdateList4-r17. The DCI format 1_3 provides indicated TCI state(s) and/or TCI-UL-State(s) for the CC(s) in a scheduledCellListDCI-1-3 if the UE is scheduled by the DCI format 1_3 to receive PDSCH at least on one serving cell in the scheduledCellListDCI-1-3. The DCI format 1_1/1_2 can be with or without …

An applicable BWP for the indicated TCI state was captured in TS 38.331 based on RAN1 input in [2, 3] before the CR in [4] was agreed.

Excerpt from [TS 38.331 v18.3.0] TCI-DCI-1-3 Configure each row of the joint TCI table for DL scheduling via DCI format 1_3, where index for a cell points to a corresponding TCI applicable for DCI format 1-1, and the order of TCI index in each row refers the order of cells in scheduledCellListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3 that configured with tci-StatesToAddModList and so on), the number of entries in a row of TCI-DCI-1-3 should be the same as the number of cells that configured with tci-StatesToAddModList on at least one DL BWP, included in scheduledCellListDCI-1-3, and entries for co-scheduled cells in a row of TCI-DCI-1-3 are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3.

Based on the above excerpt from TS 38.331, the UE applies the indicated BWP for the TCI states corresponding to the co-scheduled cells. However, the description in TS 38.331 does not capture an applicable BWP for the non-scheduled cells while RAN1 has an agreement on a similar issue for PDSCH/PUSCH transmission.

Agreement (RAN1#115) - BWP indicator in a DCI format 0_3/1_3 applies only to the scheduled cell(s) with valid FDRA value(s). - For a cell scheduled by DCI format 0_3/1_3 with valid FDRA value, if the BWP indicator indicates a code point that does not correspond to a configured BWP for the cell, the UE does not perform dynamic BWP switching based on the BWP indicator and transmits/receives data on the current active BWP of the cell.

Similar behavior can be considered for TCI state indication for non-scheduled cells, i.e., entries for non-scheduled cells in a row of TCI-DCI-1-3 are interpreted based on the respective current active BWPs of respective cells that are not scheduled by a DCI format 1_3. Therefore, this missing case needs to be captured in TS 38.331.

Proposal 2: For TCI state indication by a DCI format 1_3:

- entries for non-scheduled cells in a row of TCI-DCI-1-3 are interpreted based on the respective current active BWPs;

- Adopt the corresponding TP as proposed in the draft CR [1].

- Conclusions

This contribution discusses the corrections proposed in the draft CR [1] for TS 38.331 v18.3.0 regarding an applicable BWP for certain fields in a DCI format 0_3/1_3.

Proposal 1: For RRC parameter X = {RateMatchDCI-1-3, TCI-DCI-1-3, ZP-CSI-DCI-1-3, SRS-OffsetCombo, SRS-RequestCombo)}

- Clarify that, entries for co-scheduled cells in a row of the RRC parameter X are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3 when the BWP indicator field indicates a code point that corresponds to a respective configured BWP for the respective cell, and based on the respective current active BWPs otherwise.

- Adopt the corresponding TP as proposed in the CR [1];

Proposal 2: For TCI state indication by a DCI format 1_3:

- Entries for non-scheduled cells in a row of TCI-DCI-1-3 are interpreted based on the respective current active BWPs;

- Adopted the following TP for TS 38.331 as proposed in the CR [1];

References:

- R2-2409100 Correction on the applicable BWP for certain RRC parameters for DCI format 0_3/1_3, 38.331 CR 5077, RAN2#127bis, Samsung

- R1-2308626 Summary of discussion on higher layer signalling for MC-Enh, RAN1#114, Moderator (NTT DOCOMO, INC.)

- R1- 2308676 Summary of Email discussion on LS for Rel-18 higher layer parameters, RAN1#114, Moderator (Ericsson)

- R1-2405734 Correction on indicated unified TCI states for multi-cell scheduling, RAN1#117, Moderator (Lenovo), Samsung, Nokia, Spreadtrum, NTT DOCOMO, INC., Cybercore