3GPP TSG-RAN WG2 Meeting #127bis	R2-2409101

Hefei, China, Oct 14th – 18th, 2024

Source:				ETRI

Title:					Discussion on DSR enhancements

Agenda Item:		8.7.4.2	DSR enhancements

Document for:		Discussion and Decision

1. Introduction

During the study of DSR enhancements by the RAN2#127 meeting, agreements on DSR enhancements have been made as the followings:

Enhanced DSR contents

- Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

- For enhanced DSR:

- There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

- FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

- FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

DSR enhancements – multiple thresholds and grouping of data

- Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG.

- FFS whether DSR triggering is impacted

- FFS whether PDU set importance needs to be included

DSR enhancements

- RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance. FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

In this contribution, we provide discussions on the DSR enhancements to support scheduling enhancements for delay-critical data. The DSR enhancements MAC CE is extended to report multiple pairs of remaining time and buffer size for a LCG.

2. Discussion

Multiple reporting thresholds

Delay status reporting using legacy DSR MAC CE is triggered by a single time threshold, remainingTimeThreshold, when the smallest remaining value of delay-critical data is less than the time threshold. If there is no pending DSR MAC CE, a new DSR MAC CE is transmitted. A time threshold is configured for each LCG and data is classified as delay-critical data based on this threshold.

Proposal 1) RAN2 confirms that delay-critical data is classified based on a single time threshold, remainingTimeThreshold, per LCG when the smallest remaining value of delay-critical data is less than the time threshold.

It’s important that UE reports buffer status information to a scheduler to receive UL grants. The delay status in the buffer is more valuable when it is used to prioritize resource allocation for delay-critical data transmission. As time passes, the remaining delay time of the data decreases and it requires high priority for resource allocation. By utilizing information on the latency distribution of data in the buffer during scheduling, it is possible to anticipate resource requirements over time and provide UL grants. This could reduce the number of DSR transmissions and signalling overhead, thereby improving radio resource utilization.

Observation 1) The utilization of data delay time distribution in the scheduler is expected to yield improvements in utilizing radio resource efficiently.

The information about the delay time distribution of both delay-critical and non-delay-critical data is valuable for the scheduler. Time-critical data changes its classification over time and non-delay critical data changes to delay-critical data. When designing multiple reporting time thresholds, there is no inherent limitation to restricting report content to delay-critical data only. Multiple reporting time thresholds are configured by the RRC message for each LCG to define remaining time ranges between time thresholds, and the delay remaining times of data within these ranges are reported.

Proposal 2) Multiple reporting time thresholds are configured by the RRC message for each LCG to define remaining time ranges between time thresholds, and the delay remaining times of data within these ranges are reported.

Proposal 3) The number of reporting time thresholds is configured by the RRC message for each LCG.

The delay-critical data is transmitted earlier than non-delay critical data transmission. This priority operation does not guarantee the sequential transmission and PDCP or RLC SDU order may be changed, resulting in a retransmission/recovery procedure. This is expected to have a specification impact because it affects functions such as data reassembly and re-ordering in the reception of PDCP or RLC. It is reasonable that data in the buffer is transmitted sequentially. If non-delay critical data is ahead of delay-critical data in the buffer, the non-delay critical data transmitted earlier when UL grant is available. It results in mismatch between resource allocation procedure and transmission procedure

Proposal 4) RAN2 discusses whether to include non-delay critical data in the buffer size field.

Updating the DSR MAC CE

Legacy DSR MAC CE consists of delay information presence, BT, and single pair of remaining time and buffer size per LCG. These fields reused in enhanced DSR MAC CE as many as possible. The legacy DSR MAC CE is extended to include multiple pairs of remaining time and buffer size using the reserved field, R. In the position of 1-bit R field a new N field is defined as 1-bit indicator to indicate the start of a reporting pair for new LCG. Otherwise, the N fields indicate a reporting pair belonging to the previous LCG.

Observation 2) The legacy DSR MAC CE includes a reserved R field that can be utilized to introduce new indicator.

The figure below illustrates a modified DSR MAC CE structure that accommodates multiple reporting pairs within each Logical Channel Group (LCG) by introducing an additional N field. Each reporting pair is uniquely identified by its index K. BTm_K, Remaining Time m_K, and Buffer Size m_K are included in the Kth reporting pair. The N field serves as an indicator of whether a new LCG is being introduced. A value of 1 signifies a new LCG, while 0 denotes a continuation of the previous LCG. While the depicted example contains K reporting pairs per LCG, the flexibility of the N field allows for varying the number of reports on a per-LCG basis.

Proposal 5) The legacy DSR MAC CE is extended to include multiple reporting pairs of remaining time and buffer size per LCG.

Proposal 6) The different number of reporting pair of remaining time and buffer size is allowed per LCG.

An example of enhanced DSR MAC CE

3. Conclusion

In the discussion we made the following observations:

Observation 1) The utilization of data delay time distribution in the scheduler is expected to yield improvements in utilizing radio resource efficiently.

Observation 2) The legacy DSR MAC CE includes a reserved R field that can be utilized to introduce new indicator.

We propose the followings:

Proposal 1) RAN2 confirms that delay-critical data is classified based on a single time threshold, remainingTimeThreshold, when the smallest remaining value of delay-critical data is less than the time threshold.

Proposal 2) Multiple reporting time thresholds are configured by the RRC message for each LCG to define remaining time ranges between time thresholds, and the delay remaining times of data within these ranges are reported.

Proposal 3) The number of reporting time thresholds is configured by the RRC message for each LCG.

Proposal 4) RAN2 discusses whether to include non-delay critical data in the buffer size field.

Proposal 5) The legacy DSR MAC CE is extended to include multiple reporting pairs of remaining time and buffer size per LCG.

Proposal 6) The different number of reporting pair of remaining time and buffer size is allowed per LCG.

4. References

RAN2#127 meeting report

R2-2406202, “Report of 3GPP TSG RAN WG2 meeting #126,” ETSI MCC

R2-2404102, “Report of 3GPP TSG RAN WG2 meeting #125bis,” ETSI MCC