3GPP TSG-RAN WG2 #127bis	R2-2409103

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	LCM for UE-side models for beam management

Document for:	Discussion

# 1	Introduction

The previous RAN2#127 meeting made progress on the topic of LCM for the Beam Management use case, in particular on the procedures for applicability and inference configuration. However, many LCM aspects still need to be discussed or clarified.

In this contribution we share our views on the following LCM aspects for UE-sided models:

- Triggering of data collection for training and resource configuration for training

- Performance monitoring

- Applicability reporting

- AI/ML functionality activation/deactivation.

# 2	Discussion

## 2.1	Training

RAN2 has mainly focussed on the impact in 3GPP specification of the AI/ML inference so far, e.g. when/how to provide the UE with the inference configuration, the provisioning of the NW-side additional conditions, the applicability reporting, etc. Related to the training phase of the LCM, RAN2 has discussed at length the topic of UE-side data transferring (during Rel-18 SI, and Rel-19 WI); however how/when the UE can initiate a training session at RAN level, what configuration the UE needs for training, and in general how training impacts RAN2 protocols was just barely discussed so far in RAN2. Since training is a fundamental LCM functionality that can impact RAN2, we propose that RAN2 should start discussing it.

RAN2 has discussed at length the topic of UE-side data transferring (during Rel-18 SI, and Rel-19 WI); however, the impact in RAN2 protocols of UE-side model training procedures (e.g. how/when the UE can initiate a training session at RAN level and what configuration the UE needs for training) was barely discussed in RAN2.

First, we believe there should be a mechanism that allows the UE to inform the gNB about the need to start (or stop) data collection for model training. This step is important because the UE itself, or the UE-side training entity, may decide that the UE-side AI/ML model needs to be trained and the UE may need assistance from the network to properly carry out data collection for training. For example, the gNB may need to temporarily provide extra reference signals (besides the ones configured for conventional radio measurements), so that the UE can collect the data which is helpful in building a robust AI/ML model. The applicability reporting, e.g. in UAI, could be used to convey this information. For example, the UE could indicate as part of the applicability functionality reporting that the AI/ML functionality is not applicable and training is needed. Furthermore, the network can use RRCReconfiguration (e.g. OtherConfig if the UE request is agreed to be transmitted via UAI), or even SIB signalling to indicate whether the UE is allowed to send this training request.

The gNB indicates to the UE whether the UE is allowed to request resources for data collection for UE-side model training (e.g. via RRCReconfiguration or SIB signalling).

If allowed, the UE can request training resources (e.g. via UAI or via applicability functionality report) to start the data collection for UE-side model training, or to stop it.

Second, there should be signalling enabling the configuration of radio resources for training, that should coexist with the legacy radio measurement configuration (used for conventional “real time” measurement reports). For example, the gNB may need to provide a specific measurement configuration (CSI-RS/SSBs) for the UE to perform data collection for training and for the related reporting, including the Set A/B, and the associated NW-side additional conditions. For example, it should be possible to configure the UE to not report measurement results associated to resources in which the UE is performing data collection for UE-side model training, given that such measurements are not intended to be used by the gNB for conventional “real time” decisions, such as scheduling, beam/mobility management, etc.

RAN2 to discuss how to configure radio measurement resources (including Set A/B, NW-side additional conditions, etc.) and radio measurement reports for data collection for UE-side training.

## 2.2	Performance monitoring

For the performance monitoring phase, the UE may report performance results of functionality monitoring to the NW, e.g., accuracy, or other performance metrics. In our view, which specific layer (layer-1/2/3) should be adopted to report this information may depend on the type and amount of information to be reported, and the time when this information should be reported. Since all these issues seem to be more in the RAN1 domain and the impact in RAN2 protocols might be limited, we suggest letting RAN1 drive this topic.

RAN2 waits for RAN1 progress related to performance monitoring reporting of UE-side functionality.

## 2.3	Applicability reporting

In RAN2#127 it was agreed to use the procedures below for functionality applicability reporting in the case of beam management with UE-sided model, as shown in Figure 1. In this section we provide our view on further issues to be discussed.

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

Figure 1: Signalling procedure for functionality applicability reporting.

Overall, in our opinion there could be two approaches for ensuring the consistency between training and inference, which determine the content of Step 3 and Step 4.

- Approach 1: Step 3 contains NW-side additional conditions

- Approach 1a: Additionally, in Step 3 the UE is requested to report the applicability status of one or more functionalities. Then, in Step 4 the UE confirms the applicability of the functionalities for which the applicability status is requested. The UE can decide the applicability by verifying whether the NW-side additional conditions for inference – in the form of associated IDs included in Step 3 – are aligned with the associated IDs used/acquired previously during data collection for training. Then, in Step 5, the gNB can provide an inference configuration matching the NW-side additional conditions included in Step 3.

- Approach 1b: Additionally, Step 3 contains one or more inference configuration(s) (including the NW-side additional conditions) for one or more functionalities, and the UE is requested to confirm the applicability of the received configuration(s) for those functionalities. Then, in Step 4 the UE indicates the one or more of the inference configuration(s) that is/are applicable. If there is at least an inference configuration signalled in Step 3 that is applicable, then the UE can acknowledge that it supports the inference according to such inference configuration and it can apply such inference configuration straight away; otherwise the UE operates according to a fallback configuration indicated in the inference configuration. In this case, no Step 5 is needed.

- Approach 2: Step 3 does not contain NW-side additional conditions

- Consistency between training and inference can be ensured via performance monitoring and the UE does not need associated ID(s). Also, the UE can signal in Step 4 the recommended/preferred associated ID(s) to make the AI/ML functionality applicable, e.g. based on the training data set used for generating the AI/ML functionality. Then in Step 5, the gNB can provide an inference configuration matching the recommendation/preference of the UE in Step 4.
Step 3 can still contain one or more inference configurations, and the UE could do a similar evaluation as in approach 1b, without the need to receive an inference configuration in the Step 5.

As discussed for Approach 2, Step 3 does not always need to contain NW-side additional conditions. Therefore, the indication of NW-side additional condition (i.e. associated ID) from the network to the UE in Step 3 is an optional feature and can be done via RRC signaling.

The indication of NW-side additional condition (i.e. associated ID) from the network to the UE in Step 3 is optional.

As discussed for Approach 1 and Approach 2, it should be possible for the network to provide an inference configuration already in Step 3. For example, the gNB can provide the UE with an AI/ML-related resource configuration (e.g. CSI measurement configuration with Set A and/or Set B) for model inference purposes. This is possible, because the network may have previously stored applicability information from this UE, or it may have collected applicability information from other similar UEs. In such cases, providing an inference configuration in Step 3 has the benefit of signalling overhead reduction, since Step 5 may be skipped, and it allows a quick configuration/activation of the AI/ML functionality, without the need to go through two loops of RRC reconfigurations (i.e. one RRC Reconfiguration to configure the UE to report the applicability reporting, and one to configure the UE for the AI/ML inference), which is very desirable, e.g. during the mobility. Additionally, the network may be interested in specific inference configurations to optimize the reference signal transmission overhead (e.g. using a common Set A across multiple UEs to minimize the number of Set A reference signal transmissions). Also in this scenario the network can provide the inference configuration in Step 3. Furthermore, Step 3 could provide a performance monitoring configuration.

The network can provide inference and/or performance monitoring configuration(s) in Step 3.

Importantly, a fallback configuration needs to be ensured at the UE side as early as possible, for the case where no inference configuration is found applicable, to avoid confusion in the UE behaviour and suboptimal performances. Thus, Step 3 could also include a fallback configuration that the UE should apply.

The network can provide a fallback configuration in Step 3.

Given these possible cases for Step 3, the content of Step 3 can be similar to that of Step 5, regarding the configuration for inference/performance monitoring/fallback. Of course, not all these configurations are necessarily needed in both Steps 3 and 5, e.g. a fallback configuration is required in Step 5 only if it was not provided in Step 3, or if it is updated compared to Step 3. Nonetheless, Step 3 and Step 5 should allow sending the same type of configuration information.

The content of the inference/performance monitoring/fallback configurations can be the same in Step 3 and Step 5.

Regardless of when the inference configuration is sent from the NW to the UE (in Step 3 or Step 5), the UE should behave in the same way and take the same action. That means the UE should in any case check the functionality applicability when receiving an inference configuration and report to the NW if it is not applicable, irrespective of whether the inference configuration is received in Step 3 or Step 5. This is because it is still possible that an inference configuration sent in Step 5 is also not applicable, e.g. due to changing UE conditions between transmission of Step 4, i.e. transmission of applicability report, and reception of Step 5, i.e. reception of inference configuration. Thus, it is important that the UE always handles the inference configuration the same way, no matter if that is received in Step 3 or Step 5.

The UE behaves and replies in the same way when receiving an inference configuration from the NW, regardless of whether the inference configuration is sent in Step 3 or Step 5.

We note that the agreed diagram in Figure 1 does not contain the RRCReconfigurationComplete messages that are sent by the UE as responses to Step 3 and Step 5. To avoid potential confusion in the ongoing RAN2 discussions and since these RRCReconfigurationComplete messages are mandatorily sent by the UE regardless of whether they are going to be enhanced or not, we propose to add Step 3b and Step 5b to the diagram, to explicitly mark these messages. This is illustrated in Figure 2.

Figure 2 Proposed updates for signalling procedure diagram.

Add Step 3b and Step 5b to the signalling procedure for functionality applicability reporting, to mark the RRCReconfigurationComplete messages, as shown in Figure 2.

As discussed above, the content of Step 4 depends on the content of Step 3. For instance, for Approach 1a or 2 (i.e. Step 3 does not contain inference configuration, and it may or may not contain the NW-side additional conditions), it should be possible that the UE replies with more detailed information about the functionality applicability and its preferred configuration. It is thus meaningful to signal such information in the UAI, which already provides a framework to signal detailed UE preferences. 
However, for Approach 1b (Step 3 contains NW-side additional conditions and also one or more inference configurations), it is sufficient that the UE replies acknowledging whether the inference configuration is applicable or which one of them is applicable (in case more inference configurations are provided). It is thus suitable for this latter approach to carry this acknowledgement in the RRCReconfigurationComplete message (Step 3b).

Both the UAI and RRCReconfigurationComplete support applicable functionality reporting in Step 4:

  - If Step 3 does not contain an inference configuration, the UE reports applicable functionality in Step 4 via UAI.

  - If Step 3 contains inference configuration(s), the UE reports applicable functionality in Step 4 via RRCReconfigurationComplete, by indicating whether the inference configuration is applicable or which one of them is applicable.

## 2.4	Functionality activation/deactivation

RAN2#126 agreed the following regarding functionality activation/deactivation after inference configuration:

Agreements - RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved. FFS what Deactivation refers to: examples discussed: 1) fallback to legacy 2) switching, etc.

RAN2#127 has further agreed the following:

Agreements on procedures - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality.

Regarding the FFS on Step 3, in our view, the UE can activate an AI/ML inference configuration already in Step 3, if Step 3 provides at least one applicable inference configuration for the AI/ML functionality. Hence, the UE can apply such inference configuration and consider the AI/ML functionality also active already after Step 3, i.e. no need to wait for the inference configuration in Step 5 for activating the AI/ML functionality.

The UE activates an AI/ML functionality upon reception of an AI/ML inference configuration received either in Step 3 or Step 5, if determined to be applicable .

Regarding the FFS on additional L1/L2 signaling for activation/deactivation of inference functionality, RAN2 has already sent a question about this to RAN1 in the LS in [1]. Thus, RAN2 should wait for RAN1’s reply before discussing further related signaling details. In the meantime, RAN2 can assume that whenever the AI/ML functionality is deactivated by the NW, the UE can apply a fallback configuration, if already configured at the UE.

Upon AI/ML functionality deactivation, the UE applies a fallback configuration.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 has discussed at length the topic of UE-side data transferring (during Rel-18 SI, and Rel-19 WI); however, the impact in RAN2 protocols of UE-side model training procedures (e.g. how/when the UE can initiate a training session at RAN level and what configuration the UE needs for training) was barely discussed in RAN2.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The gNB indicates to the UE whether the UE is allowed to request resources for data collection for UE-side model training (e.g. via RRCReconfiguration or SIB signalling).

Proposal 2	If allowed, the UE can request training resources (e.g. via UAI or via applicability functionality report) to start the data collection for UE-side model training, or to stop it.

Proposal 3	RAN2 to discuss how to configure radio measurement resources (including Set A/B, NW-side additional conditions, etc.) and radio measurement reports for data collection for UE-side training.

Proposal 4	RAN2 waits for RAN1 progress related to performance monitoring reporting of UE-side functionality.

Proposal 5	The indication of NW-side additional condition (i.e. associated ID) from the network to the UE in Step 3 is optional.

Proposal 6	The network can provide inference and/or performance monitoring configuration(s) in Step 3.

Proposal 7	The network can provide a fallback configuration in Step 3.

Proposal 8	The content of the inference/performance monitoring/fallback configurations can be the same in Step 3 and Step 5.

Proposal 9	The UE behaves and replies in the same way when receiving an inference configuration from the NW, regardless of whether the inference configuration is sent in Step 3 or Step 5.

Proposal 10	Add Step 3b and Step 5b to the signalling procedure for functionality applicability reporting, to mark the RRCReconfigurationComplete messages, as shown in Figure 2.

Proposal 11	Both the UAI and RRCReconfigurationComplete support applicable functionality reporting in Step 4:

a.	If Step 3 does not contain an inference configuration, the UE reports applicable functionality in Step 4 via UAI.

b.	If Step 3 contains inference configuration(s), the UE reports applicable functionality in Step 4 via RRCReconfigurationComplete, by indicating whether the inference configuration is applicable or which one of them is applicable.

Proposal 12	The UE activates an AI/ML functionality upon reception of an AI/ML inference configuration received either in Step 3 or Step 5, if determined to be applicable .

Proposal 13	Upon AI/ML functionality deactivation, the UE applies a fallback configuration.

# References

R2-2407848, “LS on applicable functionality reporting for beam management UE-sided model”, 3GPP TSG RAN WG2#127, Maastricht, Netherlands, Aug 19th – 23rd, 2024.