3GPP TSG RAN WG2 Meeting #127bis	 R2-2409105

Hefei, China, Oct. 14th – 18th, 2024

Agenda Item:	8.6.3

Source: 	ITL

Title: 	Discussion on event triggered L1 measurement reporting

Document for:	Discussion and decision

Introduction

In the previous RAN2 meetings [1-2], agreements related to measurement enhancements are as follows:

Agreements on measurement enhancements for LTM: - Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met. Agreements on L1 MR event evaluation - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering.

In this contribution, we discussed the event triggered L1 measurement reporting for LTM.

Discussion

Associating event triggered report configuration with LTM measurement resource configuration

In RAN2#125bis, RAN2 agreed that measurement event configuration is associated with L1 measurement resource configuration as following:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

In L3 measurement, the measurement ID (MeasId) is associated with both the resource configuration ID and report configuration ID where the event is defined. With this association, one resource configuration can be associated with one or more report configuration and vice versa.

We think that one or more event triggered report configuration need to be associated with an LTM measurement resource configuration to allow for the appropriate configuration of various events, and that one event triggered report configuration can be applied to multiple LTM measurement resource configuration. Therefore, as in L3 measurement, it is clear to associate one or more event triggered report configuration with an LTM measurement resource configuration and vice versa.

Proposal 1: One or more event triggered report configuration can be associated with LTM measurement resource configuration and vice versa

Two options can be considered to associate event triggered report configuration with LTM measurement resource configuration as follows:

- Option 1: As in L3 measurement, introduce LTM measurement ID (LTM-MeasId) to associate event triggered report configuration with LTM measurement resource configuration.

- Option 2: Introduce LTM measurement resource configuration ID list in the event triggered report configuration

For Option 1, introduce an LTM Measurement ID, such as L3 measurement, and associate it with the event triggered report configuration ID and LTM measurement resource configuration ID. For this purpose, event triggered report configuration ID should also be introduced.

For Option 2, one or more LTM measurement resource configuration ID(s) can be configured in event triggered report configuration.

Since Option 1 has an ID per configuration (report and resource), we think Option 1 is more straightforward than Option 2.

Proposal 2: RAN2 is kindly asked to discuss the following options to associate event triggered report configuration with LTM measurement resource configuration

- Option 1: As in L3 measurement, introduce LTM measurement ID (LTM-MeasId) to associate event triggered report configuration with LTM measurement resource configuration.

- Option 2: Introduce LTM measurement resource configuration ID list in the event triggered report configuration

LTM Measurement reporting MAC CE

In RAN2#127, RAN2 has agreed that event-triggered L1 measurement results are reported by the UE to the network via MAC CE. Therefore, RAN2 needs to discuss what information should be included in the LTM Measurement reporting MAC CE.

Information of MAC CE

In Rel-18 LTM, CSI resource configuration for LTM candidate cells (LTM-CSI-ResourceConfig) is identified by an ID, SSB index(s) and associated candidate cell ID are provided in LTM CSI resource configuration. The LTM CSI report configuration (LTM-CSI-ReportConfig) is associated with the CSI resource configuration ID.

Also, in RAN2#127, RAN2 has agreed to reuse the Rel-18 signaling structure of LTM CSI resource and report configuration for inter-CU LTM. In our understanding, it means that an event triggered report configuration can be associated with one or more candidate cell(s) and CSI resource(s) for intra/inter-CU LTM.

Therefore, we think that if an event condition is met, there may be one or more candidate cells and RS resources in the associated LTM measurement resource configuration, and information to distinguish them should be included in the LTM Measurement reporting MAC CE.

- Candidate cell ID

- RS resource index (SSB index or CSI-RS ID)

For the network to determine LTM execution, the beam RSRP information should also be included in the LTM Measurement reporting MAC CE.

- RSRP of beam(s) that satisfy the condition

Since RAN2 does not decide how to associate event triggered report configuration with LTM measurement resource configuration, additional information may need to be considered later, for example, Event triggered report configuration ID or LTM Measurement ID.

- (FFS) Event triggered report configuration ID

- (FFS) LTM Measurement ID

Therefore, we think the following information can be considered in the LTM measurement reporting MAC CE.

Proposal 3: LTM measurement reporting MAC CE includes following information:

- Candidate cell ID

- RS resource index (SSB index or CSI-RS ID)

- RSRP of beam(s) that satisfy the condition

- (FFS) Event triggered report configuration ID

- (FFS) LTM Measurement ID

Multiple measurement reporting for one triggered event

In RAN2#127, there is an issue whether the event triggered measurement reporting can be reported multiple times.

We understand that the purpose of introducing multiple measurement reporting is to enhance reliability and mitigate channel fluctuation.

As mentioned above, since RAN2 has agreed MAC CE based LTM measurement reporting, we think there are no reliability concerns due to HARQ operation. Channel fluctuation can be resolved by applying the parameters of hysteresis, TTT, and offset in L1 Event evaluation.

Therefore, since multiple measurement reporting can cause signaling overhead and be redundant, we prefer not to introduce multiple measurement reporting for one triggered event.

Proposal 4: Not to introduce multiple measurement reporting for one triggered event

Conclusion

In this contribution, we discussed event triggered L1 measurement reporting for LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: One or more event triggered report configuration can be associated with LTM measurement resource configuration and vice versa

Proposal 2: RAN2 is kindly asked to discuss the following options to associate event triggered report configuration with LTM measurement resource configuration

- Option 1: As in L3 measurement, introduce LTM measurement ID (LTM-MeasId) to associate event triggered report configuration with LTM measurement resource configuration.

- Option 2: Introduce LTM measurement resource configuration ID list in the event triggered report configuration

Proposal 3: LTM measurement reporting MAC CE includes following information:

- Candidate cell ID

- RS resource index (SSB index or CSI-RS ID)

- RSRP of beam(s) that satisfy the condition

- (FFS) Event triggered report configuration ID

- (FFS) LTM Measurement ID

Proposal 4: Not to introduce multiple measurement reporting for one triggered event

Reference

RAN2#126 meeting report

RAM2#127 meeting report