3GPP TSG RAN WG2 Meeting #127bis	 R2-2409106

Hefei, China, Oct. 14th – 18th, 2024

Agenda Item:	8.6.4

Source: 	ITL

Title: 	Discussion on Conditional LTM

Document for:	Discussion and decision

Introduction

In revised WID of Mobility [1], the objectives related to conditional LTM have been revised as follows:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, we discussed the conditional LTM for Intra-CU LTM.

Discussion

For conditional LTM, NW can configure conditions for candidate cell(s), and if UE determines that the conditions are met, perform LTM to the candidate cell.

The first thing to consider is that there is no LTM Cell switch command MAC CE. Therefore, we think the discussion point is how the UE receives information in the LTM Cell switch command MAC CE for conditional LTM.

Observation 1: There is no LTM Cell switch command MAC CE in conditional LTM procedure

Figure 1. LTM Cell switch command MAC CE

In Figure 1, the LTM Cell switch command MAC CE contains information for the UE to perform RACH-less/RACH-based LTM.

- Target config ID: Indicates candidate cell ID

- Timing advance command: Indicates whether the TA is valid or not (based on this, UE performs either RACH-based LTM or RACH-less LTM)

- TCI state ID: Indicates and activates the TCI state for LTM target cell

- Random access related information (preamble index, SSB index, PRACH mask index, C, S/U, Repetition number): Indicates RACH related information for CBRA and CFRA

In our understanding, we think that RACH-less LTM should be supported for conditional LTM to enable faster cell switching to the target cell.

Proposal 1: Conditional LTM supports both RACH-less and RACH-based procedure

Therefore, we show our views on what information should be used and how to receive for UE based on supporting RACH-less and RACH-based conditional LTM.

Timing advance command (TAC)

In R18 LTM, if the TAC indicated in LTM cell switch command MAC CE is valid (not FFF), UE can perform RACH-less based LTM.

Therefore, before performing conditional LTM, we think that the UE should have TAC information about candidate cell(s) to decide whether to perform RACH-less based LTM or RACH-based LTM.

Proposal 2: Support early TA acquisition for conditional LTM

To acquire the TA of candidate cell(s) before performing conditional LTM, two options can be considered as follows:

- Option 1: RAR (Random access response)

- Option 2: Introduce candidate cell TAC MAC CE

For Option 1, it was discussed for early sync in R18 LTM and also for inter-CU LTM. At that time, since RAR-based TA acquisition for candidate cell(s) may interrupt the date reception and transmission, it was not introduced. However, in conditional LTM, since there is no LTM Cell switch command MAC CE, it can be considered as one way to obtain the TA of candidate cell(s).

For Option 2, since the TA of candidate cell(s) belonging to other gNB-DU(s) are transferred to source gNB-DU via F1 interface, source gNB-DU can transmit the TACs of candidate cell(s) to the UE via MAC CE. So, we propose that candidate cell TAC MAC CE be considered as one option.

Proposal 3: RAN2 is kindly asked to discuss the following options to acquire TAC of candidate cell(s) for conditional LTM

- Option 1: RAR (Random access response)

- Option 2: Introduce candidate cell TAC MAC CE

RACH-less Conditional LTM

Although R18 LTM supports both DG-based and CG-based RACH-less LTM, it is challenging to support DG-based RACH-less LTM for conditional LTM unless UE notifies the network of whether conditional LTM has occurred and information of satisfied beam(s). We think that additional signalling should be avoided after conditional LTM has occurred.

Proposal 4: Support only CG-based RACH-less conditional LTM

In R18 LTM, TCI state ID is used to decide the validity of CG occasion. In other words, if one of the associated SSB(s) in the CG occasion matches the indicated SSB in the TCI state ID, UE considers the CG occasion as valid and indicates the SSB to the PHY.

Therefore, we propose to discuss how UE checks the validity of a CG occasion for CG-based RACH-less conditional LTM. In our understanding, there are several options can be considered as follows:

- Option 1: Indicate early TCI state ID

- Option 2: Re-use the satisfied RS in the triggered conditional LTM event

- Option 3: Check the RSRP of associated SSB(s)

For Option 1, the network configures the TCI state ID of candidate cell(s) based on the measurement report from the UE. In this case, additional procedures may be required to confirm the validity of the TCI state IDs of the indicated candidate cell(s). Moreover, the UE should perform the reporting of LTM measurement results to the network and the checking of conditional LTM in parallel.

For Option 2, the UE reuses the RS that was evaluated and met the conditions during the triggered conditional LTM event. For example, If the beam satisfied by the conditional LTM event is SSB X, the UE considers the CG occasion associated with SSB X as valid.

For Option 3, similar to CG-SDT, if at least one SSB configured for CG with SS-RSRP above the threshold, the CG occasion is considered as valid.

We think Option 3 has less spec impact and is straightforward, and Option 1 and Option 2 have the advantage of skipping the procedure of checking RSRP.

Proposal 5: RAN2 is kindly asked to discuss the following options to check the CG validity

- Option 1: Indicate early TCI state ID

- Option 2: Re-use the satisfied RS in the triggered conditional LTM event

- Option 3: Check the RSRP of associated SSB(s)

Conclusion

In this contribution, we discussed the conditional LTM for Intra-CU LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: Conditional LTM supports both RACH-less and RACH-based procedure

Proposal 2: Support early TA acquisition for conditional LTM

Proposal 3: RAN2 is kindly asked to discuss the following options to acquire TAC of candidate cell(s) for conditional LTM

- Option 1: RAR (Random access response)

- Option 2: Introduce candidate cell TAC MAC CE

Proposal 4: Support only CG-based RACH-less conditional LTM

Proposal 5: RAN2 is kindly asked to discuss the following options to check the CG validity

- Option 1: Indicate early TCI state ID

- Option 2: Re-use the satisfied RS in the triggered conditional LTM event

- Option 3: Check the RSRP of associated SSB(s)

Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4