3GPP TSG RAN WG2 #127bis		R2-2409110

Hefei, China, Oct 14th  – 18th , 2024

Source:	CEWiT

Title:	Discussion on random access for Ambient IoT

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

In RAN#102 meeting, a new SI for Ambient IoT in NR has been approved [1]. The following RAN2-led objectives have been identified from the SID.

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In RAN2#125bis meeting [2], RAN2 made following agreements on random access procedure.

Agreement

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

In RAN2#126 [3] meeting, RAN2 made following agreements on random access procedure.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

-   The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

-   “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement

- From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In RAN2#127 [4] meeting, , RAN2 made following agreements on random access procedure.

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

Agreements

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

In this contribution, we discuss some additional aspects related to random access procedure, including cases for RACH failure, determination of resources and selection of 2-step or 3-step RACH.

- Random access

2.1 Determination of resources for random access

The RAN2 has agreed that slotted-ALOHA is the baseline for A-IoT random access procedure. For A-IoT random access, the resources for the MSG1 transmission can be communicated in two possible ways i.e. either through a paging message or via a separate R2D trigger message that is sent after the paging. This ensures that the A-IoT device is aware of the resources available for transmitting its initial message.

For Msg3’s transmission resource configuration, the resources can be communicated in two possible ways i.e. the reader can indicate the resources in MSG2 based on the received MSG1 which provides a more efficient resource allocation process or via paging message.

Proposal 1: Support to study the transmission resources configuartion for MSG1 and MSG3. The configuration can be indicated in:

- Paging message.

- Separate R2D trigger message after paging transmission.

- MSG2 for transmission of MSG3.

2.2 RACH failure during Random Access Procedure

The RACH procedure is initiated by a reader to identify a device trying to connect to the network. Therefore, the trigger/paging is broadcast by the reader and is received by multiple devices.

In the random access procedure there can be multiple stages where RACH failure can happen, those cases are defined below:

Case1: Device transmitted MSG1, but reader did not receive MSG1

Case 2: Device transmitted MSG1 and reader responded back with MSG2 to device but device did not receive MSG2

Case 3: Device transmitted MSG3 but reader did not receive MSG3

For the scenarios outlined, the behavior of A-IoT devices can vary depending on whether messages are successfully received or transmitted during the RACH process. The RACH process is crucial for establishing a connection between devices and the network. If the A-IoT device successfully completes the RACH procedure, it can proceed with further communication or data transmission.

However, if the device determines that the random access procedure has failed, different behaviors are triggered, which need to be studied. Unlike in RFID-like access procedures, where there is typically no handling for failed access attempts, A-IoT devices require mechanisms to handle such failures. For instance, the device may initiate a back-off timer and retry the access procedure after a specified time interval, or it may switch to another frequency or channel to attempt re-access.

Proposal 2: RAN 2 to study the behaviour of the A-IoT for following cases during RACH process:

- Case1: Device transmitted MSG1, but reader did not receive MSG1

- Case 2: Device transmitted MSG1 and reader responded back with MSG2 to device but device did not receive MSG2

- Case 3: Device transmitted MSG3 but reader did not receive MSG3

2.3 Additional aspects for RACH

The random access procedures is of two types such as 2 step RACH and 3 step RACH. For A-IoT devices, we need to analyze the 3 step and 2 step RACH based on their capability. The 3 step RACH procedure for the A-IoT devices involves more uplink transmission than 2 step RACH. Therefore, the 3 step RACH requires more power consumption than a 2 step RACH. Keeping this in consideration, the device with 1 µW power consumption is more suitable for the 2 step RACH due to its low power constraint. Based on the device capability the 2 step and 3 step RACH procedures can be chosen. Due to a large number of messages that are involved in 3 step RACH procedure than 2 step RACH, that need be exchanged between the reader and the device, the time taken to complete this 3 step RACH will be more, which result in more delay. Therefore, due to larger delay, the device will consume more power. And, in this case the device with few hundreds µW device may not be able to hold for the whole inventory cycle and become unavailable due to lack of energy storage. Thus the 2 step RACH and 3 step RACH procedures needs to be analyze properly for selecting any one for the A-IoT devices.

# Observation 1: Following aspects needs to be considered:

- Device capability

- Device unavailability due to larger delay during random access process

# Proposal 3: Down selection of the 2 step RACH and 3 step RACH should consider observation 1.

- Conclusion

In this contribution, we provide our views on random access for A-IoT. The proposals are summarized as follows.

Proposal 1: Support to study the transmission resources configuartion for MSG1 and MSG3. The configuration can be indicated in:

- Paging message.

- Separate R2D trigger message after paging transmission.

- MSG2 for transmission of MSG3.

Proposal 2: RAN 2 to study the behaviour of the A-IoT for following cases during RACH process:

- Case1: Device transmitted MSG1, but reader did not receive MSG1

- Case 2: Device transmitted MSG1 and reader responded back with MSG2 to device but device did not receive MSG2

- Case 3: Device transmitted MSG3 but reader did not receive MSG3

# Observation 1: Following aspects needs to be considered:

- Device capability

- Device unavailability due to larger delay during random access process

# Proposal 3: Down selection of the 2 step RACH and 3 step RACH should consider observation 1.

# References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair).

[2] RAN2#125-bis Chair Notes.

[3] RAN2#126 Chair Notes.

[4] RAN2#127 Chair Notes.