3GPP TSG-RAN WG2 Meeting #127bis		                               R2-2409112

Hefei, China, 14-18 October, 2024

Agenda Item:		8.7.4.2

Source:			MediaTek Inc.

Title:				Remaining issues for DSR enhancement

Document:			Discussion and decision.

### 1	Introduction

This document discusses the DSR enhancement according to the revised objective in R19 XR WID [1].

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In SI phase, there are still some FFS points should be further discussed:

- FFS whether DSR triggering is impacted

- FFS whether PDU set importance needs to be included

We will discuss above FFS points one by one and other remaining issues.

### 2	Discussion

DSR triggering

In current MAC spec, we already have DSR triggering condition in clause 5.4.9. We don’t see any impact if multiple pairs DSR reporting is introduced. Triggering and reporting can be independent to each other.

-- Clause 5.4.9 --

RRC controls the DSR procedure by configuring the following parameter:

- remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG.

-- --

Proposal 1: DSR triggering condition remains the same as in Rel-18.

PDU set importance in DSR

PSI is introduced in Rel-18 XR, its purpose is to further distinguish the different level of importance within PDU set, although, PSI has 16 level difference importance level. It is unclear how to use the PSI information other than discard mechanism when timer expires in PDCP layer. Before RAN2 agree to introduce importance information into DSR, companies need to show the benefit of DSR enhancement with PSI information.

Proposal 2: RAN2 need to show the benefit before introducing importance information into DSR, up to now, it is unclear how PSI in DSR brings any performance gain.

Need of thresholds configuration constraints

UL scheduling is up to gNB implementation, in order to let different infra vendors configure different granularity as they want, the remaining time reporting steps should be configurable. The configuration can be delivered via RRC message.

Proposal 3: RAN2 to introduce new parameter(s) in NR RRC spec to configure the reporting granularity of remaining time.

Delay-critical data definition

We already have delay-critical data definition and according to our proposal 1, we don’t any impact to the definition of delay-critical data.

-- TS 38.323 --

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

-- --

Proposal 4: The definition of delay-critical data remains the same as in Rel-18.

Inclusion of non-delay critical data

For non-delay critical data, it can be handled by BSR. gNB can figure out the amount of non-delay critical data via BSR and DSR (For example, buffer size in BSR minus buffer size in DSR). Therefore, we don’t see the need to include non-delay critical data in DSR.

Proposal 5: There is no need to report non-delay critical data in DSR.

### 3	Conclusion

This document promulgated the following proposals:

Proposal 1: DSR triggering condition remains the same as in Rel-18.

Proposal 2: RAN2 need to show the benefit before introducing importance information into DSR, up to now, it is unclear how PSI in DSR brings any performance gain.

Proposal 3: RAN2 to introduce new parameter(s) in NR RRC spec to configure the reporting granularity of remaining time.

Proposal 4: The definition of delay-critical data remains the same as in Rel-18.

Proposal 5: There is no need to report non-delay critical data in DSR.

### 4	Reference

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).