3GPP TSG RAN WG2 Meeting #127bis	R2-2409137

Hefei, China, 14 - 18 October 2024

Agenda Item:	7.4.2

Source:	Huawei, HiSilicon

Title:	LTM UE capabilities for inter-frequency L1 measurements

Document for: Discussion and Decision

# 1	Introduction

At RAN2#127, RAN2 added R1/R2/R4 UE capabilities for LTM in R2-2407599 (38.331) and R2-2407600 (38.306).

In the discussions, the following point was not concluded:

[Huawei]: One additional bit per UE would be ok and it distinguishes two cases, i) UE supports inter-F measurements only for candidate cells in the BC that inter-F measurement capability is reported and ii) UE supports inter-F measurements for any UE’s supported bands. [ZTE]: With one bit per UE capability, we need to discuss what is default meaning. Understand default should be ii) UE supports inter-F measurements for any supported bands. [Nokia]: W/o additional bit, what will happen?

- More time for discussion seems needed. We can revisit it either CB session or next meeting.

In CB session:

[Huawei]: Additional bit is per UE. It will be applicable on the BC that is indicated where L1 inter-F measurement is supported. [MediaTek]: It is not clear how per UE bit works. Also not clear where to add the bit. [Session chair]: Suggest to revisit it next meeting (with TP).

- Will be revisited next meeting (with TP).

This document comes back to this point.

# 2	Discussion

## 2.1	Granularity of L1 inter-frequency measurements

The following UE capability was introduced:

| interFreqL1-MeasConfig-r18 (R1 45-1a) Indicates whether UE supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s). This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqCellsConfig-r18 indicates the maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqCellsPerReport-r18 indicates maximum number of candidate cells in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqBeamsPerCellReports-r18 indicates maximum number of candidate beams per candidate cell in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqBeamsReports-r18 indicates maximum number of candidate cells beams in total across all cells in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

When the UE has serving cells in a BC for which it indicates support of interFreqL1-MeasConfig-r18, the UE can be configured with inter-frequency L1 measurements of SSBs of candidate cells.

This does not indicate whether the candidate cells are in the same band like serving cells, or in bands of the BC.

interFreqL1-MeasConfig-r18 is a direct pre-requisite for two other UE capabilities:

| interFreqSSB-L1-MeasWithoutGaps-r18 (R4 39-2) Indicates whether UE supports SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps (without interruption on serving cell(s)) for LTM. UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ltm-InterFreqMeasGap-r18 (R4 39-2a) Indicates whether the UE supports SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM. A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. | UE | No | No | No |  |

According to RAN4 description in R4-2410747:

-	if R4 39-2 is not supported, the UE does not support inter-frequency L1-RSRP measurements without gaps;

-	if R4 39-2a is not supported, the UE does not support inter-frequency L1-RSRP measurements with measurement gaps.

Therefore, if the UE supports neither of those, it does not support inter-frequency L1-RSRP measurements of SSB of candidate cell, so it does not indicate support of interFreqL1-MeasConfig-r18.

R4 39-2a also does not say the SSBs of which bands the UE can measure.

There are other UE capabilities for L1 inter-frequency measurements:

| multiCellL1-measRTD-greaterThan-CP-r18 (R4 39-1) Indicates the capability of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. A UE supporting this feature shall also indicate support of either intraFreqL1-MeasConfig-r18, interFreqSSB-L1-MeasWithoutGaps-r18 or ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| maxFreqLayersL1-Meas-r18 (R4 39-3-1) Indicates the number of frequency layers for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqLayersWithoutGaps-r18 indicates the maximum number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 and/or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxIntraInterFreqLayersWithGaps-r18 indicates the maximum number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| maxNeighCellsPerFreqLayerL1-Meas-r18 (R4 39-3-2) Indicates the number of neighbouring cells per frequency layer for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxNeighCellsPerFreqLayersWithoutGaps-r18 indicates the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxNeighCellsPerFreqLayersWithGaps-r18 indicates the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| supportedMaxCellsWithoutGapsL1-Meas-r18 (R4 39-3-3) Indicates the max number of total cells of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |
| supportedMaxSSB-WithinSlotL1-Meas-r18 (R4 39-3-4) Indicates the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |
| maxSSB-PerFreqLayerL1-Meas-r18 (R4 39-3-5) Indicates the maximum number of SSB resources for L1-RSRP measurement per frequency layer UE can measure. This capability signalling comprises of the following parameters: - supportedMaxSSB-PerFreqLayersWithoutGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxSSB-PerFreqLayersWithGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| supportedMaxSSB-L1-Meas-r18 (R4 39-3-6) Indicates the max number of total SSB resources of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |

These UE capabilities also do not give any restriction on the frequency band of the inter-frequency L1 measurements.

Still, these are SSBs of LTM candidate cells, so they must be in bands in which the UE supports LTM, as indicated by the following UE capabilities:

| ltm-MCG-IntraFreq-r18 Indicates whether the UE supports intra-frequency LTM for MCG with RACH as defined in TS 38.331 [9] and TS 38.321 [8] without NR-DC configured. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. UE supporting this feature shall also indicate support for ltm-BeamIndicationJointTCI-r18 or ltm-BeamIndicationSeparateTCI-r18. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ltm-SCG-IntraFreq-r18 Indicates whether the UE supports intra-frequency LTM for SCG with RACH as defined in TS 38.331 [9] and TS 38.321 [8]. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. UE supporting this feature shall also indicate support for ltm-BeamIndicationJointTCI-r18 or ltm-BeamIndicationSeparateTCI-r18. | Band | No | N/A | N/A |

According to the green highlighted sentence, when the UE supports MCG (respectively SCG) LTM, it supports it in all bands of a duplex mode and FR that it supports for "normal" (i.e., non-LTM) operation.

Therefore, if the UE indicates support of ltm-InterFreqMeasGap-r18, the UE supports inter-frequency L1 measurements of SSBs in any supported band (for normal operation) of a (FR, duplex mode) in which it supports MCG (or SCG) LTM.

Observation 1: If the UE supports interFreqL1-MeasConfig-r18 (R1 45-1a), either:

-	the UE supports ltm-InterFreqMeasGap-r18 (R4 39-2a)  it can measure SSB in any supported band for normal operation; or

-	the UE does not support ltm-InterFreqMeasGap-r18 (R4 39-2a) but it supports interFreqSSB-L1-MeasWithoutGaps-r18)  it can measure SSB in the DL active BWP.

If the SSBs of candidate cells are all in the serving BC, the UE can take advantage of its CA capability to measure them. If they are not in the serving BC, this is not possible so extra resources must be used. This could imply that the maximum number of frequency layers, of cells/SSBs in a frequency layer, of total number of cells/SSBs, on which the UE can do inter-frequency L1 measurements can be lower, resulting is less flexibility for LTM measurements.

While the UE could only support measurements in the DL active BWP, this is very restrictive, while, with gaps, the UE could easily do measurements in the whole BC.

Proposal 1: The UE that indicates support of interFreqL1-MeasConfig-r18 (R1 45-1a) can indicate that it can do L1 inter-frequency measurements of SSB in the serving BC, by indicating a new per UE capability interFreqL1-OnlyInBC-r18.

This could be indicated with an extra bit defined as:

| interFreqL1-OnlyInBC-r18 When included, indicates that, for each BC in which the UE indicates support of interFreqL1-MeasConfig-r18, the UE only supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) in the BC i.e., the UE does not support L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) out of the BC. | UE | No | No | No |
| --- | --- | --- | --- | --- |

For example:

-	in BC1 = (band A, band B, band C), the UE indicates support of interFreqL1-MeasConfig-r18

-	in BC2 = (band B, band C, band D), the UE indicates support of interFreqL1-MeasConfig-r18

-	the UE indicates interFreqL1-OnlyInBC-r18

If the UE currently has serving cells:

-	in band A and band B: the network can only configure inter-frequency L1 measurements in bands A, B and C, according to capabilities reported in BC1.

-	in band B and band C: the network can either configure inter-frequency L1 measurements in bands A, B and C, according to capabilities reported in BC1, or in bands B, C and D, according to capabilities reported in BC2.

## 2.2	Impact on other UE capabilities

The components of interFreqL1-MeasConfig-r18 are:

-	the maximum number of RRC configured candidates for L1 measurements (intra- and inter-frequency);

-	the maximum number of cells in one report;

-	the maximum number of beams per cell in one report;

-	the maximum number of beams across all cells in one report.

The above definitions do not depend whether the candidate cells of the L1 SSB measurements are in the serving BC or not. These candidate cells could all belong to the serving BC, or all not belong to the serving BC, or only a subset of them could belong the serving BC, this does not affect these components. There components are also applicable irrespective of whether gaps are used or not.

If the UE indicates interFreqL1-OnlyInBC-r18, the UE cannot perform inter-frequency L1 measurements for candidate cells that do not belong to the serving BC, this may exclude doing L1 measurements for certain candidate cells but the way components set maximum values for the configured L1 measurements is the same.

Observation 2: The components of interFreqL1-MeasConfig-r18 have the same meaning and effect, regardless whether the UE indicates interFreqL1-OnlyInBC-r18 or not.

We discussed above the other UE capabilities for L1 inter-frequency measurements:

| ltm-InterFreqMeasGap-r18 (R4 39-2a) Indicates whether the UE supports SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM. A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. | UE | No | No | No |
| --- | --- | --- | --- | --- |

As discussed above, this capability does not say anything about the frequency location of SSBs that the UE can measure according to it, this is derived from other UE capabilities.

Observation 3: ltm-InterFreqMeasGap-r18 only says that the UE can perform measurements with gaps, what the UE can measure is derived from other capabilities, so it is not impacted by interFreqL1-OnlyInBC-r18.

The other associated UE capability is:

| interFreqSSB-L1-MeasWithoutGaps-r18 (R4 39-2) Indicates whether UE supports SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps (without interruption on serving cell(s)) for LTM. UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

If there would be a candidate cell in another band, overlapping in frequency with the band of a serving cell, the SSB of a candidate cell could be in the active DL BWP of a serving cell while the candidate cell does not belong to the band of the serving cell and also not to the serving BC.

If such a case could exist, this capability should be clarified as:

| interFreqSSB-L1-MeasWithoutGaps-r18 (R4 39-2) Indicates whether UE supports SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps (without interruption on serving cell(s)) for LTM. If the UE indicates interFreqL1-OnlyInBC-r18, this only applies to SSBs of candidate cells in the BC. UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 2a: Discuss whether a candidate cell could belong to a band that is not the band of a serving cell but is overlapping in frequency with the band of a serving cell, and its SSB could be in the DL active BWP.

Proposal 2b: If this is possible, clarify that if the UE indicates interFreqL1-OnlyInBC-r18, interFreqSSB-L1-MeasWithoutGaps-r18 only applies to SSBs of candidate cells in the BC.

Besides, as mentioned above, the UE which indicate support of interFreqL1-MeasConfig-r18 (R1 45-1a) need to support at least one of interFreqSSB-L1-MeasWithoutGaps-r18 in the BC and ltm-InterFreqMeasGap-r18 but this is not captured.

Proposal 3: Discuss whether to capture that the UE that indicates support of interFreqL1-MeasConfig-r18 shall indicate support of interFreqSSB-L1-MeasWithoutGaps-r18 in the same BC or support of ltm-InterFreqMeasGap-r18 or both.

# 3	Conclusion

We made the following proposals:

Proposal 1: The UE that indicates support of interFreqL1-MeasConfig-r18 (R1 45-1a) can indicate that it can do L1 inter-frequency measurements of SSB in the serving BC, by indicating a new per UE capability interFreqL1-OnlyInBC-r18. defined e.g., as

| interFreqL1-OnlyInBC-r18 When included, indicates that, for each BC in which the UE indicates support of interFreqL1-MeasConfig-r18, the UE only supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) in the BC i.e., the UE does not support L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) out of the BC. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Proposal 2a: Discuss whether a candidate cell could belong to a band that is not the band of a serving cell but is overlapping in frequency with the band of a serving cell, and its SSB could be in the DL active BWP.

Proposal 2b: If this is possible, clarify that if the UE indicates interFreqL1-OnlyInBC-r18, interFreqSSB-L1-MeasWithoutGaps-r18 only applies to SSBs of candidate cells in the BC e.g., as:

| interFreqSSB-L1-MeasWithoutGaps-r18 (R4 39-2) Indicates whether UE supports SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps (without interruption on serving cell(s)) for LTM. If the UE indicates interFreqL1-OnlyInBC-r18, this only applies to SSBs of candidate cells in the BC. UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 3: Discuss whether to capture that the UE that indicates support of interFreqL1-MeasConfig-r18 shall indicate support of interFreqSSB-L1-MeasWithoutGaps-r18 in the same BC or support of ltm-InterFreqMeasGap-r18 or both.

| interFreqL1-MeasConfig-r18 (R1 45-1a) Indicates whether UE supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s). This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqCellsConfig-r18 indicates the maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqCellsPerReport-r18 indicates maximum number of candidate cells in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqBeamsPerCellReports-r18 indicates maximum number of candidate beams per candidate cell in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqBeamsReports-r18 indicates maximum number of candidate cells beams in total across all cells in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18. UE supporting this feature shall also indicate support of interFreqSSB-L1-MeasWithoutGaps-r18 in the same BC or support of ltm-InterFreqMeasGap-r18 or both | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |