3GPP TSG RAN WG2 Meeting #127bis												              R2-2409146

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.5.4

Title:						Adaptation of Common Signals and Channels for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The latest Release 19 WID for Network Energy Saving (NES) was approved in RAN#104 [1]. One of the objectives for specification is as follows:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution focuses on relevant aspects concerning the adaptation of common signals and channels.

# SSB Periodicity adaptation: Coverage vs Capacity Cells

- In the previous meeting, discussions on the scope of SSB periodicity adaptation on different cells was mainly from the UE’s perspective, i.e., depending on whether it is in connected or in idle/inactive mode. The discussion focus was on which extent the SSB adaptation should be applicable for the UE’s PCell or SCell and a given RRC state.

- In our view, before delving into details of the UE perspective a consensus need to be reached how the network is supposed to use SSB adaptation. In particular, we need to decide not only how the SSB periodicity can be adapted but on which cells the technique is applicable.  The main distinction is between a coverage layer, responsible to guarantee coverage and mobility, and capacity cells where NES adaptations do not pose challenges because the coverage layer assures coverage and mobility.  Essentially, we propose to limit the applicability of this technique to the capacity cells while maintaining the legacy SSB periodicity on the coverage layer as depicted in Figure 1. Such an approach can greatly aid backward compatibility for legacy UEs.

Figure 1: SSB periodicity adaptation restricted to the capacity cells while legacy SSB pattern is maintained on coverage cells.

- More specifically, the UEs searching for CD-SSBs will always succeed on the coverage layer (with legacy SSBs). Thus, legacy UE access is guaranteed, and Initial Cell Search keeps working because there is a coverage layer. The capacity layer is where energy savings and adaptations apply. On the capacity layer, diverse aspects of performance can be regulated via periodicity adaptation.

- Observation 1: First addressing SSB periodicity adaptation from the network’s perspective rather than the UE’s perspective can streamline discussion and help to clarify core questions on Initial Cell Search and legacy UE support.

- Proposal 1: SSB periodicity adaptation is limited to capacity cells while maintaining legacy SSB periodicity on the coverage layer.

- Note that legacy UEs can also access the capacity layer, as an SCell (with PCell on coverage layer) or even directly, via cell re-selection. Our companion RAN1 contribution [2] describes in detail how periodicity adaptation can be reconciled with backward compatibility, so that it is not necessary to define cells exclusively for Rel-19 UEs.

# Adaptation of Paging Occasions

In cellular networks, paging occasions are typically distributed across time to ensure distributing the paging load over time resulting in timely delivery of paging messages to UEs. However, spreading paging occasions throughout time may lead to suboptimal energy utilization in the network. Compact paging occasions, concentrated within specific time slots, can potentially yield energy savings by allowing gNB to enter deeper sleep modes during idle periods, particularly if SSB and SIB-1 are also adapted. A more compact pattern may also be defined without affecting the paging capacity by maintaining the same capacity as the sparse pattern.

However, if legacy UEs are present, they will use the legacy paging pattern. Therefore, the energy saving potential resulting from the adaptation of the paging occasions cannot be fulfilled with legacy UEs. For this reason, operators should have the possibility to bar legacy UEs from cells applying paging adaptation.

- Observation 2: The energy saving potential resulting from adapting the paging occasions cannot be fulfilled with legacy UEs.

- Proposal 2: RAN2 to investigate the solutions to optionally bar the legacy UEs from cells applying the paging occasion adaptation for NES.

# Conclusions

In this contribution, we discussed some aspects of Common signals adaptation for NES. The following observations and proposals have been made:

- Observation 1: First addressing SSB periodicity adaptation from the network’s perspective rather than the UE’s perspective can streamline discussion and help to clarify core questions on Initial Cell Search and legacy UE support.

- Observation 2: The energy saving potential resulting from adapting the paging occasions cannot be fulfilled with legacy UEs.

- Proposal 1: SSB periodicity adaptation is limited to capacity cells while maintaining legacy SSB periodicity on the coverage layer.

- Proposal 2: RAN2 to investigate the solutions to optionally bar the legacy UEs from cells applying the paging occasion adaptation for NES.

# References

- RP-241650, “Revised WID: Enhancements of network energy savings for NR ”, Ericsson, Apple (Moderators), 3GPP TSG RAN Meeting #104, Shanghai, China, June 2024.

- R1-2407100, “Adaptation of Common Signals and Channels for NES”, Fraunhofer IIS, Fraunhofer HHI, 3GPP TSG RAN WG1 #118, Maastricht, Netherlands, August 2024