3GPP TSG-RAN2 Meeting #127bis	R2- 2409149

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.7.4.1 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on LCP enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to define additional Logical channel priority handling based on the delay/deadline information.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This document continues discussion on further details of LCP enhancement for XR.

2.	Discussion

Aspects on scheduling enhancement for delay-critical data for XR

In Rel-19 XR, it is agreed to specify LCP enhancement with the additional LCP using delay/deadline information, in order to ensure the transmission of data with low remaining time and to achieve the purpose of DSR. Regarding some details of delay-aware LCP, followings are agreed in RAN2#126[2]:

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In RAN2#126, it is agreed to avoid the dynamic indication to apply the delay-aware LCP, in order to minimize the RAN1 impacts. However, if the delay-critical LCP is always applied based on configuration, additional UE complexity is expected without any specific benefits, since the UE needs to check whether there is delay-critical data in each logical channel whenever the UE receives uplink grant. In general, current LCP procedure with PBR has enough efficiency for scheduling, since the data with stringent QoS requirement would have higher logical channel priority. Therefore, in normal situation (e.g., when the remaining time of data is large enough), existing LCP procedure is sufficient to meet the QoS requirement on average, so it is not needed to consider any enhancement on LCP procedure, e.g., considering the remaining time or importance.

On the other hand, in some cases, the network may not be able to allocate the sufficient UL grant instantly, causing the delay-critical data and triggering DSR procedure. In this case, delay-aware LCP is needed to avoid unnecessary discard operation due to the transmission of data with large remaining time in the highest priority logical channel, even though the network allocates uplink grants for delay-critical data after receiving DSR. Therefore, given that the DSR is transmitted in order to request the uplink grant for delay-critical data, the prioritization of data with short remaining time should be performed only if it is needed, i.e., after DSR is transmitted. After the transmission of DSR, i.e., to request uplink grant for urgent data, the delay-aware LCP should be applied until the corresponding delay-critical data is transmitted.

Proposal 1. The LCP enhancement to prioritize delay-critical data should be applied after DSR is transmitted.

Regarding how to apply the additional priority for LCHs with delay-critical data, it was remained as FFS on whether the additional priority is applied for whole LCH or only for the delay-critical data within the LCH as follows:

- Option 1: the additional priority is applied for delay-critical data within the LCH

- Option 2: the additional priority is applied for all data within the LCH

For Option 1, it would be better to prioritize delay-critical data over other data, when there are more than one LCHs with delay-critical data. However, if there is delay-critical data and non-delay critical data for one LCH, the LCP operation would be more complicated, since the same LCH should be processed with two priorities for each delay-aware LCP operation; the LCH is firstly applied with the adjusted LCP for the delay critical data and then the LCH is applied for the legacy LCP for the remaining data. Given that the scheduling enhancement is to prioritize the delay-critical data with minimized LCP complexity, it would be better to prioritize all the data in the LCH with delay-critical data, i.e. Option 2.

Proposal 2. For adjustment the priority of LCH with delay-critical data, the adjusted priority is applied to all data in the same LCH.

Meanwhile, according to the current LCP procedure, only PBR of each LCH is included in the first round. That is, if the LCH1 has delay critical data and LCH2 has not delay-critical data, even though LCH1 has higher priority over LCH2 due to additional LCP for delay-critical data in LCH1, the MAC entity would assembly the data from LCH1 only up to its PBR, then the data in LCH2 will be included before including the remaining data in LCH1. In this case, if the amount of buffered delay-critical data in LCH1 is larger than the PBR, non-delay data in LCH2 will be included prior to the delay-critical data in LCH1.

Therefore, in order to ensure that all the delay-critical data in a logical channel can be included prior to non-delay critical data in other logical channels, it should be possible to set the PBR of LCH with delay-critical data to infinity.

Proposal 3. For adjustment of LCP of LCH with delay-critical data, the PBR should be set to infinity.

During the RAN2#125bis meeting [3], some companies suggested to additionally consider the PDU set importance for delay-aware LCP for XR. It is true that low importance data should not be prioritized in congestion scenario, since the transmission of low importance data is not essential although it has less remaining time due to reduced PDCP discard timer value. However, since the main purpose of delay-aware LCP is to avoid the unnecessary performance degradation due to discard operation of the data with short remaining time, the PDU set importance does not need to be carefully considered for delay-aware LCP in general (e.g., other than congestion scenario). Given that delay-critical data is already defined in Rel-18 XR and it does not include the low importance data in congestion scenario, it would be sufficient to prioritize delay-critical data for scheduling enhancement. Then, there is no need to additionally consider the PDU set importance.

Proposal 4. Reuse the definition of delay-critical data specified in Rel-18 XR for the delay-aware LCP.

Regarding an issue on whether the separated delay threshold is needed for delay-aware LCP, if the separated delay threshold for delay-aware LCP is defined, additional definition for delay-critical data for delay-aware LCP is needed, e.g., in order to exclude low importance data in congestion scenario as explained in proposal 3. However, there is no critical reason to define the separated delay threshold to identify delay-critical data for delay-aware LCP. Specifically,

- If the delay threshold for delay-aware LCP is larger than the delay threshold for DSR trigger, the principle of existing LCP procedure would not be applied even though the corresponding data is not urgent, which unnecessarily increase the UE complexity of LCP procedure.

- If the delay threshold for delay-aware LCP is less than delay threshold for DSR trigger, the DSR would be triggered again even though the delay-critical data for LCP is transmitted based on the delay-aware LCP.

Rather, if the new delay threshold and delay-critical data is defined for delay-aware LCP, the additional UE complexity is expected since the UE should identify additional types of delay-critical data, i.e., delay-critical data for DSR and delay-critical data for delay-aware LCP. In this sense, it would be simpler to reuse the existing definition used in DSR, i.e., delay threshold for triggering DSR, in order to avoid the additional work and the UE complexity to determine two types of delay-critical data.

Proposal 5. Reuse the delay threshold of triggering DSR for delay-aware LCP.

In RAN2#127 meeting [4], it was remained as FFS on whether the additional priority for LCH with delay-critical data can be applied to intra-UE prioritization as well.

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In NR Rel-16, the intra-UE prioritization mechanism is introduced to support tighter latency and reliability. In the Rel-16 intra-UE prioritization, when the two UL grants are overlapped, the priority of each UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, i.e., logical channel priority is only considered to determine which UL grant is prioritized.

However, the Rel-16 intra-UE prioritization does not consider the remaining time of data, and it may have trouble to satisfy strict delay requirement for XR service because one UL grant which is used for data from the highest priority of logical channel may be always prioritized over any other UL grant. For example, when two UL grants are given for XR service and one of UL grants has the highest priority logical channel with the data having long remaining time till the PDCP discard timer expiry but another UL grant has the relatively lower priority with the data having short remaining time (i.e., delay-critical data), the Rel-16 intra-UE prioritization prioritizes the UL grant having data from the highest priority logical channel. However, from XR service perspective, another UL grant for delay-critical data should be prioritized in this condition because it is not urgent to transmit the data with long remaining time in the highest priority logical channel and it can be transmitted later due to sufficient remaining time. Thus, to support tight XR latency requirement, the uplink grant for the delay-critical data should be prioritized in intra-UE prioritization.

Therefore, given that delay-aware LCP is defined in order to ensure the transmission of delay-critical data, the additional LCH priority should also be applied to the intra-UE prioritization as well, in order to ensure the transmission of delay-critical data when the two UL grants are overlapped.

Proposal 6. The additional priority for LCH with delay critical data should also be applied on intra-UE prioritization.

3.	Conclusion

In this contribution, it is discussed for the LCP enhancement for XR. This document includes following proposals.

Proposal 1. The LCP enhancement to prioritize delay-critical data should be applied after DSR is transmitted.

Proposal 2. For adjustment the priority of LCH with delay-critical data, the adjusted priority is applied to all data in the same LCH.

Proposal 3. For adjustment of LCP of LCH with delay-critical data, the PBR should be set to infinity.

Proposal 4. Reuse the definition of delay-critical data specified in Rel-18 XR for the delay-aware LCP.

Proposal 5. Reuse the delay threshold of triggering DSR for delay-aware LCP.

Proposal 6. The additional priority for LCH with delay critical data should also be applied on intra-UE prioritization.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft Report of 3GPP TSG RAN WG2 meeting #126 Fukuoka, Japan, 20 - 24 May, 2024

[3] Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China,15 - 19 April, 2024

[4] Draft Report of 3GPP TSG RAN WG2 meeting #127 Maastricht, Netherlands, 19 - 23 August, 2024