3GPP TSG-RAN2 Meeting #127bis	R2- 2409150

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.7.4.2 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on DSR enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to specify the DSR enhancements with multiple pairs of remaining time and buffer size for an LCG.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This document continues discussion on further details of DSR enhancement for XR.

2.	Discussion

In RAN2#127 meeting [2], it is agreed that multiple remaining time threshold can be configured for DSR reporting, while DSR triggering operation is remained as in Rel-18 XR, i.e., only one threshold is used for triggering the DSR.

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In RAN2#127 meeting, there were some discussions on relation between delay threshold to trigger the DSR and the delay thresholds for DSR reporting. Specifically, following cases can be considered when there are multiple delay thresholds for DSR reporting:

- Case 1: DSR triggering threshold could be largest value of the reporting threshold, in order to provide the finer granularity of delay status information when the triggering time point of DSR and the transmission time point of DSR are different.

- Case 2: DSR triggering threshold could be smallest value of the reporting threshold, in order to report the data volume which can possibly trigger the additional DSR.

In our view, it would be better to allow network flexibility to configure delay thresholds for both cases, given that DSR triggering threshold and DSR reporting thresholds would be separately configured by network. Therefore, there is no critical need to specify any constraints between DSR triggering and reporting thresholds.

Proposal 1. No need to specify constraints between DSR triggering threshold and reporting thresholds.

According to the current PDCP/RLC specification, delay-critical PDCP SDU and delay-critical RLC SDU is defined to calculate the data volume to report the DSR. For example, in PDCP layer, delay-critical PDCP data volume is calculated for DSR reporting, based on the delay threshold for DSR trigger/reporting (i.e., remainingTimeThreshold).

For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume: - the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. If a PDCP SDU becomes a delay-critical PDCP SDU, and if the corresponding PDCP Data PDU has already been submitted to lower layers, the delay-critical indication for the PDCP Data PDU is provided to lower layers.

In our view, enhanced DSR MAC CE is defined to provide additional information on delay status on top of existing DSR MAC CE, i.e., the delay-critical data volume and the minimum remaining time till PDCP discard timer expiry for an LCG. Therefore, for enhanced DSR MAC CE, remainingTimeThreshold and additional delay thresholds for delay-status reporting would be configured. For discussion purpose, following terminologies are used:

- Reporting Time threshold: additional delay thresholds for delay-status reporting

- Delay-reporting data: PDCP/RLC data whose remaining time till PDCP discardTimer expiry is less than Reporting Time threshold.

Here, existing remainingTimeThreshold is same as the DSR triggering time threshold as in Rel-18, since there is no need to separately configure the DSR triggering time threshold.

Then, if one or more Reporting Time threshold is configured in addition to remainingTimeThreshold, the amount of delay-reporting data is additionally reported with the amount of delay-critical data in enhanced DSR MAC CE. For example, it is possible that remainingTimeThreshold = [20ms], while Reporting Time threshold is configured as [10ms, 30ms, 40ms]. Then in enhanced DSR MAC CE, delay-critical data volume (i.e., data volume whose remaining time till PDCP discardTimer is less than 20ms) and multiple sets of delay-reporting data volume (e.g., data volume whose remaining time PDCP discardTimer is less than 10ms/30ms/40ms) can be reported together. Here, delay-reporting data volume may include non-delay critical data, so non-delay critical data can be reported in enhanced DSR MAC CE.

In this sense, even though the multiple reporting time thresholds are configured for DSR reporting, it is not needed to change the definition of delay-critical data, i.e., PDCP/RLC data whose remaining time till PDCP discardTimer expiry is less than remainingTimeThreshold.

Proposal 2. The definition of delay-critical data is not changed, i.e., PDCP/RLC data whose remaining time till PDCP discardTimer expiry is less than remainingTimeThreshold.

Proposal 3. In enhanced DSR MAC CE, multiple sets of delay-reporting data volume are additionally reported, in addition to delay-critical data volume and remaining time defined in Rel-18.

3.	Conclusion

In this contribution, it is discussed for the DSR enhancement for XR. This document includes following proposals.

Proposal 1. No need to specify constraints between DSR triggering threshold and reporting thresholds.

Proposal 2. The definition of delay-critical data is not changed, i.e., PDCP/RLC data whose remaining time till PDCP discardTimer expiry is less than remainingTimeThreshold.

Proposal 3. In enhanced DSR MAC CE, multiple sets of delay-reporting data volume are additionally reported, in addition to delay-critical data volume and remaining time defined in Rel-18.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft Report of 3GPP TSG RAN WG2 meeting #127 Maastricht, Netherlands, 19 - 23 August, 2024