3GPP TSG-RAN2 Meeting #127bis	R2- 2409151

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.7.3 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on measurement gap enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR [1], it is agreed to define an enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

This document shows initial views on MG enhancement for XR in RAN2 aspects.

2.	Discussion

In RAN1 meetings [2-3], measurement gap skipping operation triggered/enabled by network signalling is discussed. Based on long discussion among multiple alternatives to enable the Tx/Rx procedure during measurement gap/scheduling restrictions, it is agreed to define explicit indication by DCI to skip a particular gap/restriction as a working assumption:

Agreement Consider at least solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. - FFS: Other types of solutions. - Whether or not/how to account for any UE assistance information/indication in addition to other information available at the network Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

In order to apply the network-initiated solution with DCI indication to skip the measurement gap in MAC layer, a cross-layer indication is needed to ignore a specific measurement gap/scheduling restriction. For example, if MAC layer receives a notification of skipping a specific measurement gap within the PDCCH indication, the UE may perform UL transmission/DL reception during the indicated measurement gap.

Proposal 1. For network-initiated MG skipping operation, measurement gap can be ignored in MAC layer based on the cross-layer indication from PHY layer.

Meanwhile, in previous RAN1 meetings, it was discussed whether the UE assistance information for MG skipping is needed. However, in RAN1#117 [4], it was concluded with no consensus on whether the UAI is needed for MG enhancement:

Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility.

In our view, it is true that some information would help the network to decide the MG skipping, given that the network may not know the buffer/delay status and channel condition at UE in timely manner. However, the providing this information using the UAI framework is not appropriate since the UAI is not transmitted timely, while those information (i.e., channel condition, buffer/delay status) changes dynamically. Therefore, given that it was already discussed in RAN1 but there was no consensus whether additional UAI is needed, there is no need to duplicate the discussion to define the additional UAI for MG enhancement in RAN2 as well.

Proposal 2. From RAN2 perspective, UE assistance information for MG enhancement is not needed.

In our understanding, the measurement gap/restriction skipping indication initiated by the network is to enable the data transmission/reception, when the network is already aware of UL/DL traffic information and corresponding delay requirements. Based on the network awareness of UL/DL traffic information, the performance degradation of XR traffic due to measurement gap/scheduling restriction can be avoided, e.g., when the periodicity of XR data is not aligned with the MG periodicity. In other words, when the network is aware of the delay information of XR data for UL/DL traffic, the network may decide to indicate MG skipping using scheduling DCI, in order to ensure the transmission of the UL/DL traffic before exceeding PDB/PSDB requirement of XR traffic.

Meanwhile, when the delay-critical data is arrived in the UE buffer, i.e., when the remaining time until the PDCP discard timer expiry is less than the threshold, DSR is triggered in order to indicate the amount of delay-critical data. However, even though DSR is triggered, the transmission of DSR MAC CE may be delayed due to the measurement gap, causing the discard operation of the corresponding the delay-critical data, due to lack of time of transmitting DSR and scheduling the uplink grant for the correspond delay-critical data.

In this case, MG skipping triggered by the network signalling cannot ensure the transmission of DSR MAC CE, since it is not possible for the network to indicate MG skipping for DSR MAC CE, as the network does not know whether the DSR is triggered or not at UE side. Therefore, UE-initiated MG skipping operation should be defined in order to ensure the transmission of DSR MAC CE, when there is delay-critical data exists in the UE side.

Proposal 3. Define UE-based solution to skip measurement gap/scheduling restriction, in order to ensure the transmission of DSR MAC CE.

3.	Conclusion

In this contribution, it is discussed for the MG enhancement for XR. This document includes following proposals.

Proposal 1. For network-initiated MG skipping operation, measurement gap can be ignored in MAC layer based on the cross-layer indication from PHY layer.

Proposal 2. From RAN2 perspective, UE assistance information for MG enhancement is not needed.

Proposal 3. Define UE-based solution to skip measurement gap/scheduling restriction, in order to ensure the transmission of DSR MAC CE.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft Report of 3GPP TSG RAN WG1 #116

[3] Draft Report of 3GPP TSG RAN WG1 #118

[4] Draft Report of 3GPP TSG RAN WG1 #117