3GPP TSG-RAN2 Meeting #127bis	R2- 2409152

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Discussion on Random Access procedure for SBFD

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 SBFD[1], it is agreed to support the Random Access procedure using the SBFD symbols.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - …

In RAN2#127 meeting[2], followings are agreed on RA procedure in SBFD:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

This document further discusses RAN2 aspects on RA procedure for SBFD.

2.	Discussion

Scenarios for enable RA procedure using SBFD symbol

In RAN2#127 meeting [2], it is agreed that RA procedure using SBFD symbols can be applied to all cases as a working assumption for UEs in any RRC state, considering that RA procedure using SBFD symbols is beneficial for latency reduction, collision handling, and coverage enhancement of RA procedure:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

For CBRA cases, in MAC layer, common RA procedure is applied for all triggering events. In addition, in previous RAN1 and RAN2 meetings, it is agreed to define common CBRA procedures for UEs in RRC_CONNECTED state and UEs in RRC_IDLE/INACTIVE state. Specifically,

- In RAN1#118 meeting [3], it is agreed to extend the previous agreements on SBFD RA procedure to UEs in RRC_IDLE/INACTIVE mode

- In RAN2#127 meeting, it is agreed to define common SBFD CBRA framework independent of RRC state

Therefore, there is no need to exclude any triggering events to support CBRA procedures using SBFD symbol.

For CFRA case for the UEs in RRC_CONNECTED state, the network may configure dedicated RA preambles on SBFD RO(s), the network already knows whether the UE is SBFD-aware UE. Therefore, CFRA cases can be easily supported as well considering benefits of supporting RA procedure using SBFD symbol, e.g., latency reduction and coverage enhancement of RA procedure.

However, for Msg1-based SI request, the benefits of supporting RA procedure using SBFD symbol may not be essential. Specifically, it is not super urgent to request the SI message, so latency reduction for RA procedure on Msg1-based SI request is not needed. In addition, there is no collision issue on Msg1-based SI request, since any UE can perform Msg1-based SI request, and the network does not need to identify the UE performing the Msg1-based SI request. Last but not least, for coverage issue, Msg1-based SI request with Msg1 repetition is already supported in order to ensure the coverage of RA procedure for Msg1-based SI request. Thus, additional feature for coverage enhancement is not needed for Msg1-based SI request.

Rather, if Msg1-based SI request is supported using SBFD symbol, further discussion and large spec impact is expected, causing additional workload for SBFD WI. In current specification, Msg1-based SI request is already defined for various cases, including NUL/SUL, Redcap, Positioning SI, and Msg1-based SI request with Msg1 repetition. In addition, there are combinational cases for Msg1-based SI request, for example, Msg1-based SI request with Msg1 repetition can be performed for RedCap UE on Positioning SI (i.e., Msg1 repetition + RedCap + Positioning SI). In this sense, there are already 12 separated cases to determine and initiate Msg1-based SI request in current RRC specification.

Therefore, if the Msg1-based SI request is supported using SBFD symbol, at least following aspects should be discussed:

- whether the Msg1-based SI request configuration (e.g., si-RequestConfigSBFD) should be separately defined in SIB;

- whether SBFD operation can be supported with other features (e.g., for (e)RedCap UE, SUL, Msg1 repetition, etc), in order to decide whether the separated procedure should be specified for all of the aforementioned 12 cases; and

- which condition should be checked first for Msg1-based SI request, between SBFD and other features. For example, if Msg1-based SI request using SBFD symbol and Msg1-based SI request with Msg1 repetition is supported as well,

  - Case 1: Msg1 repetition condition is checked first. In other words, if Msg1-based SI request with Msg1 repetition is configured and the corresponding condition is met, UE performs Msg1-based SI request with Msg1 repetition even though Msg1-based SI request using SBFD symbol is configured.

  - Case 2: SBFD condition is checked first. In other words, if Msg1-based SI request using SBFD symbol is configured and the corresponding condition is met, UE performs Msg1-based SI request using SBFD symbol even though Msg1-based SI request with Msg1 repetition is configured.

Therefore, given that Msg1-based SI request using SBFD symbol causes lots of further discussion with limited benefits, it should be discussed after RAN2 finishes discussion on SBFD RA procedure for other cases.

Proposal 1. RAN2 agrees to support RA procedure using SBFD symbol for all the existing RACH trigger events, except for Msg1-based SI request.

Signaling of RACH configuration with SBFD symbol

In RAN1#117 [4] and RAN1#118 meetings [3], it is agreed to support Option 1 with Alt 1-1 and Option 2 for the configuration of RA resource in SBFD symbol:

[RAN1#117 agreement] Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. Conclusion For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs. [RAN1#118 agreement] Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

In our understanding, based on RAN1 agreements, RA resource in SBFD symbol can be configured by followings:

- Option 1 with Alt 1-1: reuse the existing RA resource configuration for legacy UE, and define additional rule to interpret valid ROs within the UL subband of the SBFD symbol, i.e.,

  - For legacy UE, only the ROs configured in UL slot (and/or UL usable PRB configured within the flexible slot, e.g., RO2 in the Figure 1) are valid. The ROs configured in the SBFD symbol is not valid.

  - For SBFD-aware UE, both ROs configured within the UL slots (e.g., RO2) and ROs configured within the UL subband in SBFD symbol (e.g., RO1) are valid.

Figure 1. An example of RACH configuration Option 1 with Alt 1-1

- Option 2: define additional RACH configurations (hereinafter, referred to as ‘SBFD RACH configuration’) within the UL subband in SBFD symbol (e.g., RO1), which is other than legacy RACH configuration. Note that SBFD RACH configuration only includes SBFD ROs.

Figure 2. An example of RACH configuration Option 2

For RACH configuration Option 1 with Alt 1-1, there is no additional parameter for SBFD RO. In other words, SBFD-aware UE determines SBFD RO based on the additional rule in current RACH configuration. Therefore, no additional signaling is needed in RAN2 perspective, since it will reuse the existing RA configuration. Note that Option 1 with Alt 1-2 (i.e., using the existing RACH configuration with newly introduced parameter(s)) is not supported in RAN1 for now.

For RACH configuration Option 2, additional RACH configuration for SBFD RO (i.e., ‘SBFD RACH configuration’) should be defined in order to configure SBFD RO and corresponding RA parameters. Meanwhile, according to the current RA procedure, following methods were defined in order to configure separated ROs for various purpose:

- Method 1: define a new IE for the separated RACH configuration, e.g., RACH-ConfigCommonIAB defined for IAB

- Method 2: define a feature for RACH partitioning, and use RACH configuration within AdditionalRACH-Config, e.g., RO partitioning configured for SDT, Slicing, Msg1 repetition, etc.

Similarly, one of the above methods can be reused to define SBFD RACH configuration. In our understanding, Method 2 is slightly preferred since RACH partitioning framework is already introduced to define common RA procedure for RA resource separation on various purposes. In addition, the additional step is not needed to select between SBFD RACH configuration and legacy RACH configuration, since it can be defined within the RACH partition selection.

Proposal 2. For the details of configuring the SBFD RACH configuration for RACH configuration Option 2, following option can be considered:

- Option A: define the new signaling of RACH configuration, e.g., RACH-ConfigCommonSBFD IE.

- Option B: define additional RACH partitioning feature for SBFD RO, and reuse the existing AdditionalRACH-Config IE for SBFD RACH configuration.

RA procedure using the RA resource using SBFD symbol

For RA resource selection for initial PRACH transmission, there are two options to be considered for RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 [3]:

Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations. - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs. Agreement For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs.

- Option A1: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations

- Option A2: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs.

For RACH configuration Option 1 with Alt 1-1, given that the SBFD-aware UE and legacy UE will use the same RACH configuration for RA procedure, there would be no critical impact on current MAC procedure. In other words, the MAC procedure on selecting RACH partition, Preamble Group, and/or SSB selection will be happened using the same RA parameters for SBFD-aware UE and legacy UE. It is true that the selection of RO may be different for SBFD-aware UE and legacy UE due to different rule to identify valid ROs, but the RO validation rule is currently defined in RAN1 spec. In RA procedure defined in MAC specification, the UE selects an RO among the valid ROs defined in TS 38.213. In this sense, the MAC procedure for RO selection would be same for SBFD-aware UEs and legacy UEs, i.e., select an RO among the valid ROs for selected SSB.

Proposal 3. For RACH configuration Option 1 with Alt 1-1 (use one single RACH configuration with possible enhancement), RAN2 assumes that the same MAC procedure can be applied for both SBFD-aware UE and legacy UE.

Similarly, for SBFD-aware UE, there would be no need to define any selection rule between SBFD ROs and legacy ROs, since the UE behaviour would be same when the UE uses SBFD ROs or legacy ROs within the same RACH configuration. In other words, given that RA procedure in MAC layer does not differentiate SBFD RO and legacy RO in the same RACH configuration, prioritization rule to select between the SBFD RO and legacy RO is not needed as well.

Proposal 4. For RACH configuration Option 1 with Alt 1-1, it’s up to UE implementation to select one type between legacy-ROs and additional-ROs (i.e., no prioritization rule between SBFD RO and legacy RO).

For RACH configuration Option 2, in our understanding, it is supported in order to configure separated RA parameters (e.g., prach-ConfigurationIndex, number of SSBs per an RO, RACH partition configuration, etc.) between SBFD ROs and legacy ROs.

Meanwhile, according to MAC procedure, RA preamble transmission is performed as following steps:

Step 1) UL carrier selection, BWP operation

Step 2) RACH partitioning procedure

  - Based on the selected RACH partition, MAC entity identifies RRC parameters based on the selected RACH partition (e.g., RACH-ConfigGeneric including prach-ConfigurationIndex, …)

Step 3) RA type selection, between 2-step RA and 4-step RA

  - Based on the selected RA type, MAC variables are initialized based on the RRC parameters configured in RACH configuration for the selected RACH partition and RA type

Step 4) SSB selection, RA preamble selection, RO selection

In other words, according to the current RA procedure, RRC parameters for this RA procedure are identified right after the Step 2, i.e., selection of the RACH partition. Therefore, given that the RA parameters in SBFD RACH configuration and legacy RACH configuration are different, selection between SBFD RACH configuration and legacy RACH configuration should be also determined during or after the Step 2, in order to initialize the RRC parameters based on the corresponding RACH configuration. Otherwise, it would be ambiguous that which RA parameters should be applied in the following procedure, e.g., for the selection of RA type, SSB, and RA preamble group, etc., between the RA parameters from SBFD RACH configuration and legacy RACH configuration.

Proposal 5. For RACH configuration Option 2, whether the SBFD-aware UE performs RA procedure using the legacy RACH configuration or additional RACH configuration should be determined before the RA type selection.

In this sense, it cannot be determined by UE implementation (i.e., Option A2) for the selection between the SBFD RO configured in SBFD RACH configuration and legacy RO configured in legacy RACH configuration. That is, some predefined rule/conditions to select between the SBFD RO and legacy RO (i.e., Option A1) is needed, since the corresponding RA parameters for the following procedure should be determined, e.g., for RA type selection, SSB selection, and RO determination. In our view, for SBFD-aware UE, SBFD RACH configuration should be prioritized over legacy RACH configuration, since the legacy UE would not use the SBFD RACH configuration. In other words, for SBFD-aware UE,

- if the SBFD RACH configuration is configured, RA is performed using SBFD RACH configuration

  - Some additional conditions to use SBFD RACH configuration can be defined, e.g., RSRP threshold.

- Otherwise, RA is performed using legacy RACH configuration

Proposal 6. For RACH configuration Option 2, SBFD-aware UE performs RA procedure using the SBFD RACH configuration, if it is configured. Additional conditions to use SBFD RACH configuration (e.g., RSRP threshold) can be defined, based on RAN1 discussion.

Similarly, for re-attempt in one random access procedure, following options can be considered based on RAN1#118 discussion [3]:

Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

- Option B1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure

- Option B2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure.

- Option B3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

For RACH configuration Option 1 with Alt 1-1, in our view, it is not needed to define any separated procedure for SBFD RO, as discussed in Proposal 2 and Proposal 3. In other words, there is no need to identify whether an RO is SBFD RO or legacy RO in MAC layer, since the UE selects an RO among the valid ROs defined in TS 38.213, even though the validation rule is different for SBFD ROs and legacy ROs. Therefore, for retransmission of RA preamble, UE may independently select between legacy RO and SBFD RO, i.e., select an RO among the valid ROs including SBFD ROs and legacy ROs (i.e., Option B3).

Proposal 7. For PRACH transmission re-attempt in one random access procedure on RACH configuration Option 1 with Alt 1-1, SBFD-aware UE independently selects an RO between legacy RO and SBFD RO for each re-attempt.

On the other hand, for RACH configuration Option 2, it is not reasonable to independently select between legacy RO and SBFD RO for each attempt (i.e., Option B3), since the MAC layer should perform RA initialization again whenever the UE changes RO, i.e., when the UE uses SBFD RO for previous RA attempt and changes to use legacy RO in the next RA attempt and vice versa.

For Option B2, it could be beneficial to define fallback procedure from the SBFD RO to legacy RO, e.g., based on the number of RA attempt. However, if the fallback procedure from SBFD RO to legacy RO is defined, there could be lots of discussions considering various feature combinations defined in existing RACH partitioning framework discussed in Rel-17/18.

In Rel-17 RACH partitioning discussion, there were lots of discussion on fallback procedure from one RACH partition to another RACH partition, considering its benefits (e.g., from slice-specific RACH to common RACH, etc). However, it was decided to disallow reselecting the RACH partition during RACH procedure in order to simplify the overall RA procedure considering various feature combination cases. In other words, once the feature combination is determined, the same feature combination is used until the RA failure (i.e., no fallback to select another feature combination). For example, for Msg3 repetition in Rel-17, fallback procedure from the legacy RA to Msg3 repetition was not supported even though it is beneficial for coverage enhancement of Msg3 transmission.

For Rel-18 discussion, same principle is remained, so fallback procedure is not allowed from one feature combination to another feature combination (e.g., from common RACH to Msg1 repetition RACH). The only fallback procedure is allowed to increase the Msg1 repetition number (e.g., from Msg1 repetition number 2 to Msg1 repetition number 4), but it is only allowed within the same feature combination, in order to minimize the complexity to modify the general RACH partitioning framework defined in Rel-17.

Therefore, if the fallback from one type of RO to another type of RO (e.g., from SBFD RO to legacy RO) is allowed, follow-up discussion would be caused in order to define re-initialization of RA parameter and to modify the existing RACH partitioning framework. In addition, given that fallback from low repetition number to high repetition number is already defined in Rel-18, it would be very complicated if two types of fallback procedures are performed in combinational manner.

In this sense, for simplicity, it is proposed to use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure, i.e., Option B1 for RACH configuration Option 2.

Proposal 8. For PRACH transmission re-attempt in one random access procedure on RACH configuration Option 2, use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission.

Early indication for SBFD aware UE

In RAN2#127 [2], it was remained as FFS on whether and how to support the early indication for SBFD-aware UE:

FFS whether/how early indication is used during a SBFD RA procedure.

Meanwhile, in current specification, early indication (e.g., for (e)RedCap UE) can be performed as follows:

- Msg1-based early indication, using the separated RA preamble or RACH occasion

- Msg3-based early indication, using the separated LCID value for CCCH message. Note that it is only applied for UEs in RRC_IDLE/INACTIVE state (e.g., for RRC setup/resume procedure)

In our view, early indication for SBFD aware UE seems beneficial since the network may allocate uplink grant from Msg3/Msg5 transmission using the UL subband within SBFD symbols. Otherwise, even though the UE is SBFD-aware UE, the network cannot allocate the uplink grant using the UL subband within SBFD symbols, before the UE transfers its UE capability information with SBFD indication.

Between the two types of early indication (i.e., Msg1-based early indication and Msg3-based early indication), Msg3-based early indication seems better, since it can be used in any cases, while Msg1-based early indication can be used only if the network configured separated RA resource for SBFD-aware UE. In addition, Msg3-based early indication is easily implemented, since the only works for Msg3-based early indication is to define additional LCIDs for SBFD-aware UE.

On the other hand, the benefit of Msg1-based early indication over Msg3-based early indication is limited, i.e., SBFD symbol can be used for Msg3 transmission, compared to further issues due to additional RA resource partitioning.

Therefore, it is proposed to specify Msg3-based early indication by defining additional LCIDs for SBFD-aware UE.

Proposal 9. Support Msg3-based early indication for SBFD-aware UE.

For Msg1-based early indication, the main discussion point is whether the preamble partitioning for SBFD indication is supported, since the RO separation between SBFD RO and legacy RO is already available, e.g., for RACH configuration Option 2. If the network configures SBFD RO using RACH configuration Option 2, the network knows whether the UE is SBFD-aware UE, when the network receives RA preamble configured in SBFD RACH configuration.

In our view, the preamble partitioning for Msg1-based early indication can be used in limited case, e.g., when RO is configured only in non-SBFD symbols but the network intends to allocate the uplink grant using SBFD symbol for Msg3 transmission. As discussed above, given that only one type of early indication is enough and lots of features for preamble partitioning (e.g., SDT/Slicing/(e)RedCap/Msg1 repetition/…) are already defined, it is not desirable to define additional feature with preamble partitioning, given that RA resource is limited.

Proposal 10. Do not additionally define the Msg1-based early indication for SBFD-aware UE, i.e., no need to define preamble partitioning for SBFD-aware UE.

3.	Conclusion

In this contribution, it is discussed about RAN2 aspects on Random Access operation for SBFD. This document includes following proposals.

Proposal 1. RAN2 agrees to support RA procedure using SBFD symbol for all the existing RACH trigger events, except for Msg1-based SI request.

Proposal 2. For the details of configuring the SBFD RACH configuration for RACH configuration Option 2, following option can be considered:

- Option A: define the new signaling of RACH configuration, e.g., RACH-ConfigCommonSBFD IE.

- Option B: define additional RACH partitioning feature for SBFD RO, and reuse the existing AdditionalRACH-Config IE for SBFD RACH configuration.

Proposal 3. For RACH configuration Option 1 with Alt 1-1 (use one single RACH configuration with possible enhancement), RAN2 assumes that the same MAC procedure can be applied for both SBFD-aware UE and legacy UE.

Proposal 4. For RACH configuration Option 1 with Alt 1-1, it’s up to UE implementation to select one type between legacy-ROs and additional-ROs (i.e., no prioritization rule between SBFD RO and legacy RO).

Proposal 5. For RACH configuration Option 2, whether the SBFD-aware UE performs RA procedure using the legacy RACH configuration or additional RACH configuration should be determined before the RA type selection.

Proposal 6. For RACH configuration Option 2, SBFD-aware UE performs RA procedure using the SBFD RACH configuration, if it is configured. Additional conditions to use SBFD RACH configuration (e.g., RSRP threshold) can be defined, based on RAN1 discussion.

Proposal 7. For PRACH transmission re-attempt in one random access procedure on RACH configuration Option 1 with Alt 1-1, SBFD-aware UE independently selects an RO between legacy RO and SBFD RO for each re-attempt.

Proposal 8. For PRACH transmission re-attempt in one random access procedure on RACH configuration Option 2, use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission.

Proposal 9. Support Msg3-based early indication for SBFD-aware UE.

Proposal 10. Do not additionally define the Msg1-based early indication for SBFD-aware UE, i.e., no need to define preamble partitioning for SBFD-aware UE.

4.	Reference

[1] RP‑241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] Draft Report of 3GPP TSG RAN WG2 meeting #127 Maastricht, Netherlands, 19 - 23 August, 2024

[3] Report of 3GPP TSG RAN WG1 #118

[4] Report of 3GPP TSG RAN WG1 #117