3GPP TSG-RAN WG2 Meeting #127bis		                               R2-2409153

Hefei, China, 14-18 October, 2024

Agenda Item:		8.7.5

Source:			MediaTek Inc.

Title:				RLC AM enhancements with small packet delay budget.

Document:			Discussion and decision.

### 1	Introduction

This document discusses RLC AM enhancement according to the fourth objective in R19 XR WID [1].

-	Specify the following user plane enhancements [RAN2]

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

### 2	Discussion

## 2.1	Avoiding unnecessary RLC AM retransmissions

In previous several meetings, we reach some agreements related to how to avoid unnecessary RLC AM retransmissions, up to now, 3 approaches are on the tables for further discussion.

- Tx initiated approach

- Rx initiated approach

- Combined approach

There are only there key issues need to be discussed to avoid unnecessary RLC AM retransmissions.

- The first issue is how to stop retransmit obsolete SDUs.

- The second issue is how to address the consequence when these obsoletes SDUs are not retransmitted anymore.

- The last issue is how to move Tx window when the obsolete SDUs are not abandoned or not transmitted anymore

The first issue is quite easy to address in Tx initiated approach and combined approach, Because the transmitter side can know the PDB of the packets, once its PDB equal to zero, that means the SDU is already out of date. Therefore, no need to retransmit anymore. For Rx initiated approach, receiver has more actions to do. First, receiver needs to determine the SDU is out of date or not. Up to now, timer seems to be one way to achieve that. In PDCP layer, there is t-reordering timer. In RLC layer, there is t-Reassembly timer. The timer-based mechanism only works to prevent unnecessary RLC AM retransmissions when the timer value is set to a smaller value than PDB. And usually, the value of t-reordering and t-Reassembly is larger than the PDB (e.g., 30ms) of XR packets. Therefore, Rx initiated approach can not solve the first issue properly.

Observation 1: Only Tx initiated approach and combined approach perfectly avoid unnecessary RLC AM retransmissions.

Proposal 1: RAN2 don’t consider Rx initiated approach as a candidate solution to avoid unnecessary RLC AM retransmissions.

The second issue is the hole or gap (created by not transmitting obsolete SDUs) at the receiver side. For Tx initiated approach, we already had some agreement to address this issue: “The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs” and “Rx side updates state variables according to the information from Tx side”. For combined approach, there is similar mechanism like t-Reassembly timer, when timer expires, receiver side to remove the hole or gap. But there are some uncertainty about timer-based mechanism. If the timer value is too short. It is possible that the hole or gap still under HARQ retransmission is skipped. If the timer value is too long, the window might stall for a while.

|  | Tx initiated approach | Combined approach |
| --- | --- | --- |
| Pros | Can perfectly solve the unnecessary RLC AM retransmissions problem | Timer mechanism is simple |
| Cons | A new RLC control PDU or other mechanism is needed. | Might not perfectly solve the unnecessary RLC AM retransmissions problem |

Proposal 2: For the sake of performance, RAN2 to adopt Tx initiated approach to avoid unnecessary RLC AM retransmissions.

One last issue is that how Tx window moves or maintain after obsolete SDUs are not transmitted or abandoned. There are three mechanisms can achieve this purpose.

- The first one is when “The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs”, transmitting side move the Tx window at the same time.

- The second one is when Receiving side receives the information from transmitting side, receiving side consider the SDUs included in those information is successfully received. The rest is to follow legacy procedure to process the status report as legacy, send the status report back to transmitter side to move the Tx window.

- The last one is timer-based approach, when timer expires, receiver side consider the SDUs in the hole or gap as successfully received and use the legacy status report to move the Tx window.

All above 3 mechanisms should works, consider performance and agreement from last meeting “Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first.” The second mechanism is better than the other two mechanism.

Proposal 3: RAN2 to adopt below mechanism to move Tx window:

- Step 1: The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs via a RLC control PDU, FFS on the content of RLC control PDU.

- Step 2: When Rx side receive the control PDU, it considers the SDUs indicated from Tx side is successfully received and updates state variables accordingly.

- Step 3: Rx side send status report as legacy way to move Tx window.

## 2.2	Timely retransmissions for XR traffic in RLC layer

In last meeting (RAN2 #126), we discuss three mechanisms to achieve timely retransmissions, among these three mechanisms, we can further categorize them into two types. One is retransmission without feedback, the other one is retransmission with feedback.

We use one example to show why we prefer the retransmission should based on feedback from receiver side. Assume each RLC SDU size is 1500 bytes and SN is 18 bits. Transmitter side already send SDU 1~7 to receiver side. Assume SDU 1, 3, 5, 7 are correctly received. The STATUS PDU is at most 31bytes. With the feedback, transmitter side only need to retransmit SDU 0, 2, 4, 6. (4*1500 = 6000 bytes). However, if there is no feedback, transmitter side needs to retransmit SDU 1~7 again, which cost (7*1500 = 10500bytes).

Capacity consumption of retransmission with feedback in this example is (6000 + 31) bytes which is less than retransmission without feedback (15000 bytes). It is clearly that retransmission based on feedback has less negative impact on capacity.

| SDU SN | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 (ACK_SN) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Correct received or not | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (~3bytes) |

Observation 2: Retransmission according to feedback has less negative impact on capacity.

Proposal 4: RAN2 to consider the following two options on timely retransmission:

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

### 3	Conclusion

This document promulgated the following observation and proposals:

Avoiding unnecessary RLC AM retransmissions

Observation 1: Only Tx initiated approach and combined approach perfectly avoid unnecessary RLC AM retransmissions.

Proposal 1: RAN2 don’t consider Rx initiated approach as a candidate solution to avoid unnecessary RLC AM retransmissions.

Proposal 2: For the sake of performance, RAN2 to adopt Tx initiated approach to avoid unnecessary RLC AM retransmissions.

Proposal 3: RAN2 to adopt below mechanism to move Tx window:

- Step 1: The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs via a RLC control PDU, FFS on the content of RLC control PDU.

- Step 2: When Rx side receive the control PDU, it considers the SDUs indicated from Tx side is successfully received and updates state variables accordingly.

- Step 3: Rx side send status report as legacy way to move Tx window.

Timely retransmissions for XR traffic in RLC layer

Observation 2: Retransmission according to feedback has less negative impact on capacity.

Proposal 4: RAN2 to consider the following two options on timely retransmission:

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

### 4	Reference

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).

[2]	R2-2405706, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei).

[3]       R2-2409115, Discussion on RLC re-transmission related enhancements, OPPO, China Telecom, Sharp, Samsung, HONOR, CMCC, KDDI, Apple, Intel, vivo, MediaTek Inc, ZTE.