3GPP TSG RAN WG2 #127-bis		                                                       R2-2409157

Hefei, China, 14th – 18th October 2024

Agenda item:	8.4.1

Source: 	Vodafone

Title: 	On LR and MR operating frequencies (related to LS in R1-2407559)

Document for:	Discussion

- Introduction

An open issue relating to the operation of the Low-power Wake-up Radio (LR) and Main Radio (MR) in different frequency bands was recently discussed in the RAN1#118 meeting [1]:

Question 5-1: Please share you views on whether to support MR and LR operating on different band/carrier and explain why.

As a consequence, an LS (R1-2407559) was sent to RAN2/RAN4 about this issue:

# 1 Overall description In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. # 2 Actions To RAN2 and RAN4: ACTION: RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

In this contribution we present our views on this issue.

- Discussion

## 2.1 Background

As mentioned in previous meetings, it is expected that LR will only support a limited number of frequency bands when compared to MR, as adding more bands will increase overall complexity and costs and in turn battery consumption of the LR. Given this expectation, in order to not limit the applicability of the LP-WUS/WUR framework it is necessary to support the operation of the LR and MR on different frequency bands.

In our view it should be possible by UE implementation to allow the two receivers (LR and MR) to operate in different frequencies thus allowing, for example, the LR monitoring LP-WUS in the 700 MHz (this might also be 800 or 900 MHz or any other band, but preferably in the sub 1 GHz range as this increases the coverage of the LR) but “waking up” the MR in 3.5 GHz, as represented in Figure 1.

Observation 1: LR is expected to support a limited number of bands when compared to MR. It is possible by UE implementation to have LR monitoring a different band than MR but still trigger the wake-up procedures in idle/inactive.

Figure 1 – Scenario with MR in coverage of 3.5 GHz and LR supporting lower frequency bands.

One possible drawback if the LR supports lower bands (e.g. 700 MHz) or indeed any different band, in scenarios where the UE MR was using 3.5 GHz is that LR measurements of the 700 MHz cell might not be reliable for MR returning to service in 3.5 GHz.

One alternative being proposed in [2] is to define a delta factor to counteract the effect of the measurements in different frequencies and thus allow the support of LR and MR in different frequencies:

  - Xiaomi in [2] proposes to introduce a delta factor to reflect the difference between RRM measurements when operating in different carriers:

LR and MR operating on different bands Another issue is offloading RRM measurement to LR when LP-SS and NR channels are located in different carriers/bands. Since LP-SS and SSB are located far away in frequency, the channel experienced by LP-SS and SSB may not be very similar, for example, when the measurement results by LR is still acceptable, the channel quality for NR may degraded to the level that paging can not be successfully received. But since the MR is deactivated, UE is not aware of it. In this case, when UE receives LP WUS and wake up MR, UE has to do cell reselection procedures first and then try to monitor PO. So the latency can be quite long, and UE may even miss the paging. One solution is that gNB configures a delta factor to reflect the difference exists between RRM measurements of LR and MR when they are in different bands. Proposal 22: Support offloading RRM measurement to LR when LP-SS and NR channels are located in different carriers/bands.

Since our intention is to allow the operation of LR and MR in different carriers/bands, in consequence we would be interested that the offloading of RRM measurements could be achieved. However, it is unclear how the delta factor in [2] is defined and moreover we see that such definition would require RAN4 expertise.

Proposal 1: Support offloading RRM measurement to LR when LP-SS and NR channels are located in different carriers/bands if deemed feasible by RAN4.

Another possibility related to this issue was proposed in [3] would be to change the priority of bands where the UE is able to camp when using the MR, to provide the highest priority to bands that the LR operates in:

  - Huawei in [3] proposes the support of LR and MR in the same band and assign higher priority to the LP-WUS band:

To avoid too much complicated design for LP-WUS/LP-WUR, it is better to keep that the MR and LR of a UE operate in the same band/carrier. For IDLE/INACTIVE mode, the cell re-selection is performed by UE, and UE has the flexibility to choose a band/carrier to camp on. Therefore, UE’s MR is able to re-select to a cell deployed with LP-WUS. However, the RRM measurement and cell re-selection may be controlled somehow by gNB via the priority of frequency. To solve this issue, some simple enhancements may be needed, e.g., UE can report the capability to network which band/carrier it supports for monitoring LP-WUS. For simplicity for the network, gNB can take a band/frequency/carrier with LP-WUS deployment as higher priority based on the capability report from majority of UEs (which can be obtained from CN).

By following this direction, it would severely impact the user experience as traffic would become overload coverage layer (such layers are also normally layers with a limited bandwidth) in the case the LR only supports 700 MHz band or any other coverage layer band, which is expected due to lower frequencies allowing for a higher LR coverage range.

By providing a higher priority on bands supported by the LR, it is expected that the UEs would always camp, receive MIB, SIB1, receive paging and perform random access in 700MHz, thus overloading bandwidth limited bands (e.g., low bands like 700 have normally only 10 MHz of bandwidth, and, often have further capacity reduction from the use of 4G/5G Dynamic Spectrum Sharing).

Observation 2: Assigning higher priority to the band/frequency/carrier with LP-WUS deployment can significantly impact the user experience and overload  operator network on the lower bands

## 2.2 Support for MR and LR operating in different frequencies

The LP-Wake Up Signal is expected to be sent to the LR a short time before the mobile’s paging occasion (PO) for the MR.

When the LR and MR are using the same frequency, this is straightforward. However, since different carriers may have different paging occasion configurations, some extra design work is needed when the LR and MR are on different bands. This is pointed out in [4]:

- Apple in [4]:

On the other hand, if we want to support LR and MR operating on different carriers/bands, additional specification effort is needed: - (…) - LO configurations may also need to be modified. E.g., if an offset w.r.t. the PO is used, whether we should consider the PO on the MR camping carrier or the PO on the LR carrier.

Since there might be multiple carriers in a single base station, multiple offsets (e.g., the offset between LP-WUS transmitted in 700 MHz and Paging occasions in 700MHz, 1800 MHz, 2.1 GHz, and 3.5GHz) would need to be defined.  This can result in many more LP-WUS transmissions on the LR band and reduce the capacity of that important coverage band as highlighted in the Figure 2 below.

Figure 2 – Multiple offsets for the LO configurations.

Observation 3: Defining multiple offsets from the Low-Power Wake-up Signal Occasions to the Paging Occasions for each MR band, may lead to more LP-WUS transmissions on the LR band and reduce the capacity in case LP-WUS is transmitted in coverage layer bands.

To improve the system and UE performance when LR and MR are on different bands, it is proposed that the network sends the LP-Wake Up Signal on the LR band, and the MR receives the paging on LR band, and (only if the UE is paged), the UE reselects to the MR band with a higher priority (e.g. the one it was camping on before) and perform RACH procedure there.

Proposal 2: To avoid overloading LR band, support that the MR of a UE is able to receive paging messages on the LR band and reselects to another band to perform random access.

Procedure wise our proposal (see Figure 3 below for a simplified schematic view) is the following:

- UE in idle/inactive mode, equipped with MR and LR operating in different frequencies, will switch on a deep sleep mode on MR storing the information for RACH on the cell/cells it is camping on, and other system information.

- LR performs monitoring of LP-WUS on LR band.

- The UE should wake in the PO associated with LR band, the LR will proceed to wake up the MR in this band.

- MR will perform synchronization to the LR band and receive the paging message on this band (700 MHz in our example).

- If the paging message is addressed to the UE, the MR would then re-synchronize to the band it was camping on (3.5 GHz in our example) which can be the same as the LR band or different.

- Upon synchronization and based on the stored configuration received from the previous access, the UE will perform RACH to the band it was camping on.

Figure 3 – Proposal for the MR to receive paging message in LR band and access the previously accessed MR band.

Before going into deep sleep mode, the UE stores the PCI of the MR cell it was camping on and the (e.g., SIB 1) information needed to respond to the cell in the band it was camping on (e.g., the parameters needed to send the PRACH.

It is also required to store information (e.g.ARFCN) for  the LR to know the band it needs to monitor LP-WUS.

Proposal 3: Before going to deep sleep mode, the UE shall store information about the LR band in order to monitor paging on this band. The UE shall also keep information on the cell it was camping on to perform random access upon being paged.

After being paged on the LR band, the UE moves to the band it was camping on and performs some rapid measurements (e.g., it measures 3 successive SSBs, taking 60-80ms) to have confidence that it is in coverage of an MR cell and then responds to the paging on the MR cell. If the MR cell has a different PCI, then the UE acquires SIB 1 before accessing the cell (40-160ms delay). If there is insufficient (or no) coverage on the band the MR was camping on, then the mobiles may respond to paging on the LR band.

The re-selection parameters are controlled by the operator, and the re-selection criteria may take into account a coverage gap different between the carrier/frequency where the MR has received paging and the carrier/frequency to reselect.

While using the LR, the UE performs serving cell measurements on the LR carrier, at least until the LR signal strength goes below a threshold. Below this threshold, the UE can use the MR on either the original MR band, or, on the LR band. Neighbour cell measurements could be made on either LR (depending on receiver implementation) or MR band. After cell reselection, relevant System Information needs to be obtained by the MR on the new gNB’s priority band.

It is assumed that the reception of the paging message and re-selection to another carrier will not impact the latency of the initial access procedure much more that the already expected delay for the UE MR to go from deep sleep mode to active, and the offset between the Low-power wake up signal occasions on one frequency 700/800/900 MHz and paging occasions on other frequency 3.5GHz may be designed taking into consideration the extra delay in this procedure. Note that the above delays are small compared to typical DRX values of 1.28 or 2.56s (and smaller than the AMF retransmission timers that are likely to be 2 and 3 seconds, respectively) and the entity causing the page (e.g., remote person making the phone call) has no idea of the timing of the paging occasion.

Observation 4: No major impact on the latency is expected due to the MR receiving paging in a carrier and performing random access in another carrier.

- Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: LR is expected to support a limited number of bands when compared to MR. It is possible by UE implementation to have LR monitoring a different band than MR but still trigger the wake-up procedures in idle/inactive.

Observation 2: Assigning higher priority to the band/frequency/carrier with LP-WUS deployment can significantly impact the user traffic capacity of network operators when deployed in the coverage layer frequency bands.

Observation 3: Defining multiple offsets from the Low-Power Wake-up Signal Occasions to the Paging Occasions for each MR band, may lead to more LP-WUS transmissions on the LR band and reduce the capacity in case LP-WUS is transmitted in coverage layer bands.

Observation 4: No major impact on the latency is expected due to the MR receiving paging in a carrier and performing random access in another carrier.

Proposal 1: Support offloading RRM measurement to LR when LP-SS and NR channels are located in different carriers/bands if deemed feasible by RAN4.

Proposal 2: To avoid overloading LR band, support that the MR of a UE is able to receive paging messages on the LR band and reselects to another band to perform random access.

Proposal 3: Before going to deep sleep mode, the UE shall store information about the LR band in order to monitor paging on this band. The UE shall also keep information on the cell it was camping on to perform random access upon being paged.

# References

- R1-2407507, Summary #5 on LP-WUS operation in IDLE/INACTIVE mode, Moderator (Apple), 3GPP RAN#118 meeting, 19 – 23 August 2024.

- R1-2406296, Discussion on LP-WUS operation in Idle/Inactive modes, Xiaomi, 3GPP RAN#118 meeting, 19 – 23 August 2024.

- R1-2405868, Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode, Huawei, HiSilicon, 3GPP RAN#118 meeting, 19 – 23 August 2024.

- R1-2406851, LP-WUS operation in IDLE/INACTIVE modes, Apple, 3GPP RAN#118 meeting, 19 – 23 August 2024.