3GPP TSG-RAN WG2 #127bis	R2-2409165

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.1.5

Source:	Ericsson

Title:	On model transfer/delivery

Document for:	Discussion

# 1	Introduction

The topic of model transfer/delivery was studied by RAN2 during the Rel.18 SI. However, during the Rel.19 WI, so far only RAN1 worked on that, as a study objective of the Rel.19 WI.
In particular, during the last RAN1 meeting, RAN1 recommended extending the study of the model transfer/ delivery, because based on RAN1 understanding the study is not completed.

In this paper, we provide our view on model transfer alternatives discussed in RAN2 during Rel.18, and its dependency on the RAN1 discussions and progress.

# 2	Model transfer feasibility and necessity

During the Rel-18 Study Item, RAN1 introduces different options for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. The different options are listed below:

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

In the current Rel-19 discussions, RAN1 concluded in RAN2#116-bis on deprioritizing case z2 (specifically for one-sided UE model), z3, and in RAN1#116 on deprioritizing case z5.

RAN1 has deprioritized case z2, z3, z5 as options for the model transfer/delivery.

For case y, the model is delivered to the UE from outside the MNO (e.g. Over-The-Top (OTT) server). Hence, no 3GPP specification impact is expected.

RAN1 has not concluded on the necessity or feasibility of specifying a model transfer procedure from NW to UE involving 3GPP network during the study phase of the Rel-19 WI. In other words, RAN1 has not concluded yet whether the model transfer will be via case y (i.e. no spec impact) or via case z1 or z4. 
Additionally, we note that from RAN1 discussion, it appears that the model transfer is mainly discussed in the context of two-sided use case, as an option to eliminate the inter-vendor collaboration complexity for the two-sided use case.

RAN1 has not concluded on the necessity or feasibility of model transfer for one-sided use cases.

No concluding agreements in RAN1 on feasibility or need for model transfer involving 3gpp network (case z1/z4).

In general, we observe that RAN1 has discussed the model transfer cases since long time, and they are in a better position than RAN2 to assess the feasibility/necessity of the various cases. For this reason, we propose letting RAN1 continuing the discussion and concluding the feasibility analysis.

The discussion on the feasibility/necessity of the model transfer/delivery cases is up to RAN1.

# 3	RAN2 options for model transfer/delivery

Related to options for AI/ML model transfer/delivery, the following solutions were considered in RAN2 during the study item:

-	Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

-	Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

-	Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

-	Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

-	Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

-	Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

## 3.1 Considerations for UE-sided model (single-sided)

In our view, for the case of single-sided UE-side model, the topic of model transfer/delivery has some clear dependency with the topic of data transfer for the UE-side collected data that RAN2 studied during the Rel.19 WI. This is because the entity collecting the data might be, or might not be, also in charge of transferring the model to the UE.

In particular, in the endorsed CR to TR 38.843 [1], RAN2 introduced the terminology “first termination entity” to identify the network entity that first receives the collected data from the UE, and in all the options there might be a final termination entity of the collected data, e.g. OTT server. Hence, in all the options captured in [1], it should be possible for the first termination entity to forward the collected data to the final termination entity, e.g. the OTT server, which will be in charge of training the UE-side model and finally deliver it to the UE. 
Given this analysis, in our view solution 4a above, i.e. “OTT transfers/delivers the AIML model to the UE” is always doable, irrespective of the options in [1] used for data collection, i.e. irrespective of whether 3GPP will  finally select as the UE-side data collection option, option 1a,1b, 2, or 3. Solution 4a also maps to RAN1 case y in which model delivery is transparent to 3GPP. This option seems to be the most straightforward. It does not have any 3GPP impact, i.e. model information is autonomously transferred as user plane data from the OTT server to the UE.

- RAN2 to conclude that Solution 4a is feasible and can be used for model delivery from OTT server to the UE without specification impact, and irrespective of the solution adopted for the UE-side data collection (i.e. 1a, 1b, 2, 3 in [1]).

For the discussion on model transfer, it is important to differentiate the roles of the various network nodes (e.g., gNB, core network, or OAM) that could be involved. Specifically, there is a need to distinguish between: (1) nodes that are responsible for the transferring of the UE-side models and also for the UE-side model training and data collection; and (2) nodes that are solely responsible for collecting the data for the UE-side model training, but without participating in the training process or in the model transfer. 
Let’s consider OAM/CN involvement in model transfer (solution 4b/2a/2b) as an example. This option maps well to the data collection option 3 or 2 in [1], i.e. the UE gathers training data and sends it to OAM/CN, which is the first termination entity in this case. Now, two potential scenarios arise:

- If the data, including training information and other UE properties (e.g. hardware/software capabilities), is fully visible to OAM/CN, then the OAM/CN could potentially handle the UE-side model training, and hence also handle the UE-side model transfer.

- The OAM/CN forwards the collected data to the server for UE-side model training/OTT server which trains the model and sends it directly to the UE (solution 4a).

Scenario 2, as we already said, is feasible and can be achieved via solution 4a. 
On the other hand, Scenario 1 poses some feasibility concerns. For a node to train a model, it must have full visibility of the data collected from the UE, e.g. knowledge of HW/SW capabilities (e.g. parameter quantization, pre-processing steps, etc..). This means that the OAM/CN would need full access to the UE’s collected data, and also to the inherent device HW/SW characteristics. The additional burden to train, store, and maintain different models to accommodate for different UE HW/SW capabilities should not be underestimated. Otherwise, without such knowledge, models will not be tailored to the UE hardware and the AIML model performance will be sacrificed. Whether this is somehow feasible or not depends on RAN1 progress, and cannot be assessed by RAN2.

Whether the entities involved in the UE-side data collection (first termination entity in [1]) can also be in charge of UE-side model training and UE-side model transfer depends on whether full visibility of the collected data (including UE proprietary information) can be achieved, and also on the capability of the first termination entity to handle training/storing/maintenance of the UE-specific models.

In the meantime, RAN2 can discuss the above solutions for the UE-side model transfer/delivery in relationship to the options for UE-side data collection identified in [1]. To this end, we first note that the gNB is not identified as a termination entity for the data collection in any of the options in [1], hence the gNB cannot initiate the model transfer/delivery. For this reason, the solutions 1a, and 1b for model transfer/delivery are not applicable, at least for the one-sided UE-side model.

The gNB was not identified in [1] as a data collection entity.

Study of solutions 1a and 1b for model transfer/delivery are deprioritized for the case of one-sided UE-side model.

Similarly, solution 3a and 3b are relevant for the positioning, but the LMF is not explicitly considered as a termination entity for the data collection. RAN1 has also not agreed on the need to support model transfer via 3GPP network for the positioning use case, and such solutions seem to be more valid for use Case 2a, i.e. “UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning” which is of 2nd priority according to the WID. Additionally, the LMF is a CN entity, hence solution 3a/3b could be considered as part of solution 2a and 2b. Hence, RAN2 should not discuss solution 3a and 3b further.

Study of solutions 3a and 3b for model transfer/delivery are deprioritized for the case of one-sided UE-side model.

Still looking at the various options identified by RAN2 for the UE-side data collection in [1], we note that for the option 1b, i.e. “ the UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO)”, there is no corresponding model transfer/delivery solution mentioned in TR 38.843. Hence, we suggest adding to TR a new solution for the model transfer/delivery that originates from the model server for data collection for UE-side model training (inside the MNO) which is somewhat similar to solution 4a, but with the difference that it is not transparent with respect to 3GPP.

The server for data collection for UE-side model training (inside the MNO) was identified as a data collection entity in [1].

Add to TR 38.843 the following solution for the AIML model transfer/delivery: “The server for data collection for UE-side model training (inside the MNO) can transfer/deliver AI/ML model(s) to UE (non-transparent to 3GPP)”.

Given the above discussion, it appears that apart from solution 4a (which does not have any spec impact, and that should be considered always feasible), for all the other solutions, the feasibility very much depends on the conclusion on the UE-side data collection. Hence, it is therefore recommended that RAN2 waits for progress on the feasibility of UE data collection options, before discussing the feasibility of remaining model transfer options, i.e. 2a, 2b, and 4b.

RAN2 to clarify whether the entities involved in model transfer are also involved in the training of the models or solely responsible for delivering the model.

It is recommended that RAN2 waits for progress on the feasibility of data collection (e.g. option 1b, 2 and 3 in [1]), before discussing feasibility of model transfer solutions 2a/2b and 4b.

## 3.2 Consideration for two-side model

As previous said, from RAN1 discussion, it appears that the model transfer is mainly discussed in the context of two-sided use case, as an option to eliminate the inter-vendor collaboration complexity for the two-sided use case. To resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, RAN1 is still discussing the following options:

As shown in the table above, there are other options being considered that are not dependent on model transfer involving 3GPP network. Yet, there is still no concluding agreements in RAN1 on the feasibility of any of the options and which of those options will be standardized if two-sided use case is agreed for normative work.

Option 1: Fully standardized reference model (structure + parameters)

In this option, the network is not involved in training the model. However, depending on the UE implementation, models may not always be available on the device. Even fully standardized models might be downloaded on an as-needed basis. Therefore, RAN2 needs to evaluate the various model transfer options for storing and transferring the standardized model in this scenario.

Option 3a/4/5:

The procedure for exchanging parameters, datasets, or models between the network side and the UE side is currently under discussion in RAN1. Once RAN1 agrees on the method for exchanging (over the air, offline, etc..) RAN2 can study the implications of RAN1 on the L2/L3 signalling, if any.

The UE side receives the parameters or dataset and performs offline processing to generate the actual model to be run on the device. However, once the UE side (e.g., OTT server) trains the model, the model needs to be transferred to the UE device. At this stage, the applicability of RAN2 solutions for model transfer must be examined. It is evident that training the UE-side model can only be carried out by the UE side. Therefore, RAN2 should consider two scenarios in this case:

- Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP)

- Solution 2a/2b or 4b: in which the CN or OAM is involved only in the transfer from the OTT server to the UE device i.e. to ensure network controllability

Option 3b: Standardized reference model structure + Parameter exchange between NW-side and UE-side. Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

According to RAN1, the method of exchanging is over the air-interface via model transfer/delivery Case z4. The relevance of any RAN2 model transfer solution for this option will depend on RAN1's decision regarding which network-side entity will be responsible for training (e.g., gNB, OAM, or CN). RAN1 has not yet addressed the data collection framework for two-sided CSI compression.

Anyhow, it seems premature for RAN2 at this stage to start discussing aspects related to two-sided models. It is suggested that RAN2 focus for the time being only on the one-sided case, and once RAN1 has progressed more or concluded the discussion on two-sided models, then RAN2 can start analysing implications in RAN2 protocols (if any)

RAN2 to wait more progress on RAN1, before examining the relevance of the various model transfer/delivery solutions in relation to the inter-vendor collaboration options addressed in RAN1.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN1 has deprioritized case z2, z3, z5 as options for the model transfer/delivery.

Observation 2	RAN1 has not concluded on the necessity or feasibility of model transfer for one-sided use cases.

Observation 3	No concluding agreements in RAN1 on feasibility or need for model transfer involving 3gpp network (case z1/z4).

Observation 4	The gNB was not identified in [1] as a data collection entity.

Observation 5	The server for data collection for UE-side model training (inside the MNO) was identified as a data collection entity in [1].

Based on the discussion in the previous sections we propose the following:

Proposal 1	The discussion on the feasibility/necessity of the model transfer/delivery cases is up to RAN1.

Proposal 1	RAN2 to conclude that Solution 4a is feasible and can be used for model delivery from OTT server to the UE without specification impact, and irrespective of the solution adopted for the UE-side data collection (i.e. 1a, 1b, 2, 3 in [1]).

Proposal 2	Whether the entities involved in the UE-side data collection (first termination entity in [1]) can also be in charge of UE-side model training and UE-side model transfer depends on whether full visibility of the collected data (including UE proprietary information) can be achieved, and also on the capability of the first termination entity to handle training/storing/maintenance of the UE-specific models.

Proposal 3	Study of solutions 1a and 1b for model transfer/delivery are deprioritized for the case of one-sided UE-side model.

Proposal 4	Study of solutions 3a and 3b for model transfer/delivery are deprioritized for the case of one-sided UE-side model.

Proposal 5	Add to TR 38.843 the following solution for the AIML model transfer/delivery: “The server for data collection for UE-side model training (inside the MNO) can transfer/deliver AI/ML model(s) to UE (non-transparent to 3GPP)”.

Proposal 6	RAN2 to clarify whether the entities involved in model transfer are also involved in the training of the models or solely responsible for delivering the model.

Proposal 7	It is recommended that RAN2 waits for progress on the feasibility of data collection (e.g. option 1b, 2 and 3 in [1]), before discussing feasibility of model transfer solutions 2a/2b and 4b.

Proposal 8	RAN2 to wait more progress on RAN1, before examining the relevance of the various model transfer/delivery solutions in relation to the inter-vendor collaboration options addressed in RAN1.

# 5	References

R2-2407807, CR on RAN2 inputs to TR 38.843, RAN2#127