3GPP TSG-RAN WG2 #127-bis	R2-2409168

Hefei, China, 14th – 18th October 2024

Source:	NTT DOCOMO, INC.

Title:	Triggering condition of PHR for UL-only TRP

Document for:	Discussion and decision

Agenda Item:	8.12.2

- Introduction

Based on following objective in the WID, RAN1 started discussion on introduction of “UL-only cell” before RAN2 TU starts.

WID Objective [1] - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

This contribution provides our view on a potential RAN2 impact related to UL-only cell, i.e., the triggering condition of PHR.

- Discussion

In Rel-19 MIMO, the following scenario, where the UE has connections to one DL TRP and one or more UL TRP(s), is considered.

Figure 1: Asymmetric DL sTRP/UL mTRP scenarios. [2]

In this asymmetric DL sTRP/UL mTRP scenario, the UR cannot receive DL reference signals from its UL TRP(s). Consequently, RAN1 agreed to introduce solutions specific to the scenario.

One of the solutions is related to PHR MAC CE from the UE.

According to Section 7.1.1 in TS 38.213 [3], the transmission power for PUSCH is calculated as following.

TS 38.213, 7.1.1 [3] PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dBm]

Please note that the UE needs to input the estimated downlink pathloss, PL, to calculate the PUSCH Tx power. The UE typically measures the downlink pathloss using a reference signal from the TRP to which the UE transmits the PUSCH. However, as mentioned above, the UE cannot receive reference signal from UL TRP, while the UE receives RS from DL TRP. Therefore, RAN1 agreed to introduce pathloss offset configured per TCI state. The UE can regard (PL – PL offset) as a pathloss corresponding to the UL only TRP, and the UE can report PH using the quasi-pathloss value.

Now the problem is the triggering condition of PHR. According to TS 38.321 [4], the PHR is triggered in the following condition.

TS 38.321, 5.4.6 [4] A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

In other words, PHR is triggered when the pathloss values change by a certain amount of gap. As referred above the pathloss value is estimated using RS from DL TRP, but on the other hand the actual UL Tx power is calculated considering per-TCI state PL offset.

Therefore, RAN2 can discuss whether to change the triggering condition of PHR reporting to consider the per-TCI state PL offset value, e.g., change to when the (PL – PL offset) value has changed more than phr-Tx-PowerFactorChange dB”.

Figure 2. Comparison on whether to consider PL offset in PHR triggering condition.

Our preference is to change the triggering condition of PHR reporting to consider the per-TCI state PL offset value. This is because the actual UL Tx power is changed when the PL offset is changed. For instance, as depicted in Figure 2, the network can change/activate TCI states by MAC CE or DCI. If the UE is in the asymmetric DL sTRP/UL mTRP scenario, the PL offset value is also changed when the TCI state is changed. Then the UE adjusts/changes UL Tx power based on the new PL offset value. In our understanding, the UE should report PHR when its UL Tx power is changed more than a certain threshold, therefore we think the triggering condition of PHR should also consider the PL offset value.

Proposal.	Change the triggering condition of PHR to consider pathloss offset configured per TCI state.

- Summary and proposal

Proposal.	Change the triggering condition of PHR to consider pathloss offset configured per TCI state.

- References

[1]	RP-242394, “WID revision: NR MIMO Phase 5,” Samsung, RAN#105.

[2]	R1-2406928, “Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios,” NTT DOCOMO, INC.

[3]	TS 38.213, “NR; Physical layer procedures for control,” V18.4.0.

[4]	TS 38.321, “NR: Medium Access Control (MAC) protocol specification,” V18.3.0.