3GPP TSG-RAN WG2 Meeting #127-bis	R2-2409174

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.7.6

Source:		Futurewei

Title:		Discussions on XR UL rate control

Document for:		Discussion and Decision

# Introduction

In RAN#105, a new objective for Rel-19 XR WI was agreed as the following [1]:

-	Specify uplink congestion signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In this contribution, we provide our views on how to achieve this objective.

# Discussions

The purpose of the MAC layer rate control signaling is for the gNB to indicate to the UE a recommended bit rate that can be sustained over the UL at the moment. The recommended bit rate information enables the video codec at the UE (i.e., the video source) to adjust its video resolution accordingly so that the average bit rate of the video stream that it produces can roughly match with the recommended bit rate. This MAC layer signaling may be realized by either enhancing the existing Recommended bit rate MAC CE or introducing a new MAC CE similar to the Recommended bit rate MAC CE.

The existing Recommended bit rate MAC CE has several codepoints in the Bit Rate field reserved, as shown in the Annex. Hence, one simple enhancement is using the codepoints 57-62 to extend the data rate, while leaving codepoint 63 reserved. For example, codepoint 57 for 8500 kbps, codepoint 58 for 9000 kbps, …, and codepoint 62 for 11000 kbps. An indication of up to 11 Mbps should be good enough for UL AR video baseline data rate, which is 10 Mbps as in TR 38.838. But this enhancement alone is not good enough to cover the optional data rate for UL AR video traffic, which is 20 Mbps as in TR 38.838.

Although there are two more reserved bits in the MAC CE, they are separated from the Bit Rate field by a X field and are in a position that may be awkward for using them as the MSBs for extending the Bit Rate field.

Fortunately, there is the X field, which indicates whether a bit rate multiplier, as specified by dedicated RRC signaling, is to be multiplied with the data rate derived from the Bit Rate field and the lookup table or not for the final data rate. Unfortunately, the current bitRateMultiplier parameter is defined with a few values in Rel-16, which are generally unsuitable for XR traffic, as shown below:

One way to fix this is that we introduce a Rel-19 version of the bit rate multiplier parameter in Rel-19 RRC spec. For example, bitRateMultiplier-r19 can be {2, 3, 4, …, 15, spare, spare} or simply {2, 4, 8, 16}. Then, by applying a factor of 2 on the data rate derived from codepoint 62 (i.e., by configuring bitRateMultiplier-r19 to indicate a factor of 2, X field being set to 1 and the Bit rate field being set to 62 in the enhanced Recommended bit rate MAC CE), it would cover up to 22 Mbps, which is more than the optional data rate for UL AR video data rate as in TR38.838, and by applying a factor of 15 or 16 on the data rate derived from codepoint 62, it would cover more than 150 Mbps, the average data rate of 8k video, which video resolution is not currently used by any UL AR applications and should be sufficient for any emerging UL XR video applications in a foreseeable future.

The above proposed enhancements would enable a simple solution with a small effort required from RAN2 to meet the new objective on XR UL rate control, which is agreed in RAN plenary.

Proposal. RAN2 to achieve the new objective on XR UL rate control by the following:

- reusing the existing the Recommended bit rate MAC CE and further defining codepoints 57-62 (e.g., codepoint 57 for 8500 kbps, codepoint 58 for 9000 kbps, …, and codepoint 62 for 11000 kbps) in the Bit Rate field in the Recommended bit rate MAC CE in the MAC spec; and

- introducing bitRateMultiplier-r19 in the RRC spec and down-select one from the following value sets: {2, 3, 4, …, 15, spare, spare} or {2, 4, 8, 16}.

# Conclusions

We propose the following:

Proposal. RAN2 to achieve the new objective on XR UL rate control by the following:

- reusing the existing the Recommended bit rate MAC CE and further defining codepoints 57-62 (e.g., codepoint 57 for 8500 kbps, codepoint 58 for 9000 kbps, …, and codepoint 62 for 11000 kbps) in the Bit Rate field in the Recommended bit rate MAC CE in the MAC spec; and

- introducing bitRateMultiplier-r19 in the RRC spec and down-select one from the following value sets: {2, 3, 4, …, 15, spare, spare} or {2, 4, 8, 16}.

# References

RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).

3GPP TS 38.321 v18.2.0.

- Annex. Recommended bit rate MAC CE related text extracted from TS 38.321 [2]