3GPP TSG-RAN WG2 #127bis 	R2-2409181

Hefei, China, 14th – 18th Oct. 2024

Agenda Item:	8.9.3

Source:	Ericsson

Title:	UL capacity enhancements for IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was updated [1], however, the UL capacity objective and justification is the same as before.

The main objective related to support of capacity enhancements for uplink are:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

And the justification for this objective is:

Need for Uplink capacity enhancement: NB-IoT NTN is already being deployed live at this very moment. In these early and upcoming deployments, it is clearly emerging that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g. low cost devices, wearables, etc). Multiplexing of UEs by usage of orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH should therefore be studied and if beneficial being specified. Therefore, in order to unlock the additional UL capacity potential, there is a need to identify methods to de-couple the UL from the DL as much as possible. Apart from enhancements above, reducing the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction could be beneficial, therefore should be studied and if beneficial, specified.

In this contribution we look further into this objective.

# 2	Discussion

## 2.1	Baseline and terminology

During RAN2#126, a significant portion of the discussion was centred around the decision of whether the new feature should adopt EDT or PUR as its baseline. Several companies argued that this is merely a matter of nomenclature.

However, as EDT and PUR are distinctly different given their respective original purposes. Specifically, PUR is designed for periodic traffic and requires dedicated configuration. Moreover, PUR is associated with a suite of predefined procedures (e.g., Layer 1 Acknowledgement) and UE behaviours (e.g., PUR-responseWindowTimer) that may not be applicable in the NTN context.

In the WID, there is no mentioning of PUR while the justification and objective mention improving EDT. Therefore, RAN2 should take EDT as the baseline for enhancements.

The baseline for uplink capacity enhancements is EDT.

Further, the WID mention EDT msg3 transmission without msg1/msg2. To avoid further confusion and debates over terminology, we suggest that the feature be designated as Preamble-less Early Data Transmission (preamble less EDT) or RACH-less EDT.

The Rel-19 uplink capacity enhancement is referred to as preamble-less EDT or RACH-less EDT.

## 2.2	Multiplexing of UEs and orthogonal cover codes

The first part of the objective which only targets NB-IoT NTN, aims at enabling multiplexing multiple UEs on NPUSCH format 1 and NPRACH. This hints at A) improved NPUSCH multiplexing and B) improved NPRACH collisions.

For improved NPRACH collisions to make sense, 1) UEs must be told in broadcasting or dedicated signalling that they can use OCCs, 2) eNB must be able to detect OCCs on the NPRACH, 3) eNB must be able to address RAR to different OCCs (for example, RAR messages can include the OCC besides the RA-RNTI or the RA-RNTI itself tells which OCC it is for), 4) legacy UEs must not be severely affected. The combined changes seem to result in large spec changes, even though NPRACH collisions is not a limiting issue of RA capacity today.

Improved NPUSCH multiplexing means the eNB may be able to schedule more users on the same NPUSCH resources. The spec impacts will be changes to the DCI to indicate what OCC to use.

There is one combination of NPRACH and NPUSCH where OCCs may be beneficial, and that is if 2-step RA is defined for NB-IoT… This will require extensive RAN1 and RAN2 specification work.

OCCs for NPRACH improves preamble collisions but will have large specification impact, and legacy random access capacity is not limited by preamble collisions

OCCs for NPUSCH Format 1 has potential to improve UL capacity and depending on the number of simultaneous UEs to be supported, seems to have reasonable specification impact

2-step random access would benefit from OCCs on NPRACH and NPUSCH but will have very large specification impact and will probably require its own work item

## 2.3	Reduce necessary UL and DL signalling to complete EDT

This is the second sub-objective of the UL capacity enhancements.

It was clear during the discussions at RAN2#127 that this objective and the justification leave out important information of the background, namely limits of current deployments where the DL NPDCCH capacity can severely increase the delay and limit the achievable UL capacity of 4-step random access.

The UL capacity enhancement objective needs to consider the signaling load on NPDCCH and the potential delays that a limited NPDCCH capacity can lead to.

The NPDCCH has a very limited capacity in NB-IoT and when considering the signalling that 4-step RA will need on NDPCCH to schedule RAR, it is clear 4-step RA cannot achieve similar capacity as the Diversity Slotted Aloha (DSA) and Contention Resolution DSA (CRDSA) schemes.

Therefore, we look further into the details of DSA and CRDSA.

### 2.3.1	Practical considerations for DSA and CRDSA

In DSA and CRDSA, the arriving packet at a UE need to start by waiting until the next frame

- Average delay for one packet is increased by one half frame

- Possible to have a sliding frame, more complex and CRDSA will need relative indications to the slot where a replica is transmitted instead of absolute within the frame

DSA is simple to specify and simple for the eNB to decode:

- The same UE ID in the EDT message can identify replicas within a frame

CRDSA is simple to specify and complex for the eNB to decode:

- eNB need to save and process a full frame at a time (and save data for future if using sliding frame)

- eNB need to implement interference cancellation

- interference cancellation may not be possible unless the propagation channel is the same in a correct decoded slot as in the slot where a replica was received

### 2.3.2	Comparison DSA and CRDSA

We assume the same models as described in R2-2405202 [3]. That is, a slotted system with Poisson arrivals, Slotted Aloha (SA) or DSA and CRDSA with 2 or 3 replicas and a frame size N of 100.

Non-colliding packets are assumed to always be correctly received.

We assume all collisions leads to loss of all colliding packets (that is no power capture [4])

- Unless all other replicas are cancelled in the case of CRDSA

For CRDSA, we assume that a replica that is correctly received in one slot can be cancelled in the other slots where the same replica was transmitted.

This is a quite strong assumption, as it is doubtful whether it is possible to make a correct interference cancellation unless the propagation channel is the same in the slot where replica is correctly received and the slot where the replica is to be cancelled.

Feasibility of CRDSA need to be checked with RAN1 and RAN4 regarding the performance of replica cancelling.

Further, we ignore overhead from the CRDSA scheme (that is the header info on which other slots in the window where the UE transmitted the same packet, this overhead would be 7 bits with 2 replicas and 14 bits with 3 replicas).

For CRDSA, we investigate some assumptions on the replica cancellation:
1) Ideal: any replica that is correctly received are cancelled on the other slots where the same rplicas was transmitted and this is repeated until all packets are correctly received, or there are only unresolvable collisions remaining (for example two packets that uses the exact same slots for the replicas)
2) I1: the eNB process the frame once and cancel all correctly received replicas, then it process the slots where there was a cancelled replica and count all correctly received packets
3) I2: the eNB process the frame once and cancel all correctly received replicas, then it process the frame a second time and cancel all correctly received replicas, then it process the slots where there was a cancelled replica and count all correctly received packets
4) max2c: collisions of more than 2 replicas are assumed to never be correctly received.

The I2 and I1 are meant to model potential processing limitations in the eNB (like Niter in [5]), and max2c is meant to model imperfect interference cancellation.

In Figure 1, we have the throughput for SA, DSA and ideal CRDSA. The ideal CRDSA has much higher throughput than Slotted-Aloha or DSA. For very low and very high load, DSA and CRDSA has the same throughput for the same number of replicas.

Figure 1: Throughput versus load

Figure 2: Throughput versus load, non-ideal CRDSA

With less ideal interference cancellation in Figure 2, the CRDSA throughput tends towards the DSA scheme with the same number of replicas.

In figure 3 we have the packet loss ratio (the probability of an arriving packet failing to be correctly received in one frame).

Figure 3: Packet loss ratio versus load

Figure 4: Packet loss ratio versus load for non-ideal CRDSA with 3 replicas

Figure 5: Packet loss ratio versus load for non-ideal CRDSA with 2 replicas

In Figure 4 and 5 we have the non-ideal CRDSA. We can see that with less ideal CRDSA, the PLR tends towards the PLR of the DSA scheme with the same number of replicas.

Because of the unknown feasibility of CRDSA, and that with realistic assumptions on the CRDSA interference cancellation the throughput of CRDSA tends towards the DSA with same number of replicas, we propose to not consider CRDSA further.

Diversity Slotted Aloha is considered beneficial and will be supported

RAN2 will not further consider CRDSA.

## 2.4	Uplink resource allocation

Preamble-less EDT transmission without Msg1/Msg2 is somewhat similar to NR’s 2-step random access. In this procedure, the cell specific PRACH and PUSCH resource parameters for the transmission of MsgA are configured by MsgA-ConfigCommon included in SIB1, and the network can provide separate PRACH and PUSCH configurations for a specific feature such as small data transmission (SDT, which is similar to EDT in LTE) in FeatureCombinationPreambles. For preamble-less EDT, cell specific common PUSCH resources for Msg3 transmission can be configured via system information. Signalling details such as which SIB to use can be further studied.

System Information is used to provide preamble-less EDT cell specific PUSCH resources for Msg3 transmission. FFS on signalling details.

## 2.5	Configuration of uplink resources

LTE-M and NB-IoT devices support coverage enhancement (CE) to compensate the simplifications in their radio interfaces and improve connectivity in challenging coverage conditions. The most significant CE technique is the support of repetitions.

In EDT, PRACH parameters are configured per CE level in edt-PRACH-ParametersListCE. The UE selects the CE level for the preamble transmission (Msg1) based on network configuration. In addition, transport block size (TBS) and number of repetitions for Msg3 transmission on PUSCH depend on the PRACH CE level.

In legacy EDT, the number of repetitions and the largest TBS for Msg3 PUSCH depend on the CE level.

In legacy EDT, RSRP thresholds are configured for the UE to select the CE level.

For preamble-less EDT, the support of CE is also necessary. Similar to legacy, we think the network should provide cell specific common PUSCH resources for Msg3 EDT transmission per CE level. For each CE level, PUSCH resources can be associated with a certain number of repetitions, largest allowed TBS and a RSRP selection threshold.

Preamble-less EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level.

Preamble-less EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold and largest TBS for Msg3 transmission.

## 2.6	UL time alignment for Msg3 transmission

For uplink timing handling, RAN2 already agreed that the UE needs to verify/update the uplink synchronization (e.g., get GNSS fix, precompensate TA) just before sending Msg3. In our view, this is sufficient to ensure the required uplink synchronization for Msg3 transmission and there is no need to use TAT. Note that the UE may not receive any Timing Advance Command (in the current serving cell) before sending Msg3 thus it has no trigger to start TAT and therefore it is meaningless to introduce TAT for Msg3 transmission without Msg1 and Msg2.

The UE will verify uplink synchronization before sending Msg3.

The UE may not receive any Timing Advance Command before sending Msg3.

Preamble-less EDT does not require a running TAT for Msg3 transmission.

Further, RAN2 had a reply LS from RAN1 [6] and RAN4 [7] and for some cases Ran1 will look further, and for the others it works fine if the UE uses NTA=0 for Msg3 transmission without Msg1/Msg2.

## 2.7	Contention mechanism: collision detection and backoff

Msg3 transmission may fail due to collision or bad reception quality at the network side. In NR 2-step random access, when the transmission of MsgA fails, the UE may have another MsgA transmission attempt with a random backoff and an increased MsgA transmission power (i.e., power ramping). The UE would switch to 4-step random access if the number of MsgA transmissions has reached msgA-TransMax, provided that preambleTransMax is greater than msgA-TransMax. A similar mechanism can be introduced for contention based Msg3 EDT transmission to handle Msg3 transmission failures.

After a failure in Msg3 transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power.

After a number of failed Msg 3 transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

## 2.8	UE identity for Msg4 transmission

Another open issue for preamble-less EDT is which identity (e.g., RNTI) should be used to schedule Msg4 transmission. In NR’s 2-step random access, MsgB-RNTI is derived based on the time/frequency position of the MsgA PRACH occasion. This principle can naturally be applied to preamble-less EDT, i.e., the RNTI used to schedule Msg4 transmission is derived based on the used Msg3 PUSCH time-frequency resource. The RNTI may be associated with either one or multiple Msg3 PUSCH time-frequency resources, which is up to network implementation.

The RNTI used to schedule Msg4 transmission is derived based on the PUSCH time-frequency resource(s) used for contention based Msg3 EDT transmission.

## 2.9	Efficient delivery of Msg4

The second WI sub-objective of this topic is the efficient delivery (reduced overhead) of Msg4. In RAN2#126 meeting, [2] showed how the overhead reduction cannot come from optimizations in the RRCEarlyDataComplete ASN.1 structure. Thus, RAN2 should focus on different alternatives such as the transmission of a shared Msg4.

In 2-step random access, MsgB can consist of one or multiple successRAR and/or fallbackRAR sub-PDUs. Each successRAR consists of UE Contention Resolution Identity, the PDSCH-to-HARQ feedback timing indicator for MSGB HARQ feedback and Timing Advance Command, etc. Hence, a MsgB can be used to respond to multiple UEs which have transmitted MsgA, i.e., a shared MsgB.

NR’s 2-step random access already supports multiplexing multiple UE Contention Resolution Identities in MsgB.

In our view, a similar design can be applied to Msg4, i.e., Msg4 can consist of multiple contention resolution identities for different UEs associated with the PDSCH-to-HARQ feedback timing indicator for Msg4 HARQ feedback, where the contention resolution Identity could be e.g., the TMSI included in RRCEarlyDataRequest in Msg3, or just the first 48 bits of Msg3 (similar to a regular 4-step random access or a successRAR sub-PDU in MsgB). This avoids the transmission of separate Msg4 for each individual UE whose Msg3 transmission is decoded successfully.

Introduce multiplexing of several UE Contention Resolution Identities in Msg4.

## 2.2	Repetitions

RAN2 agreed to continue discussing Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for preamble-less EDT transmissions. As described above, repetition is a crucial feature for LTE-M and NB-IoT. If introducing DSA and CRDSA for preamble-less EDT transmissions, we shall make sure they can work together with repetition. In our view, RAN2 shall first discuss the feasibility and complexity of integrating DSA and CRDSA with repetition for 2 step Msg3-EDT transmission. DSA and CRDSA shall only be introduced if they can work together with repetition.

For DSA and CRDSA, RAN2 shall first discuss whether it is feasible to integrate them with repetition.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	OCCs for NPRACH improves preamble collisions but will have large specification impact, and legacy random access capacity is not limited by preamble collisions

Observation 2	OCCs for NPUSCH Format 1 has potential to improve UL capacity and depending on the number of simultaneous UEs to be supported, seems to have reasonable specification impact

Observation 3	2-step random access would benefit from OCCs on NPRACH and NPUSCH but will have very large specification impact and will probably require its own work item

Observation 4	The UL capacity enhancement objective needs to consider the signaling load on NPDCCH and the potential delays that a limited NPDCCH capacity can lead to.

Observation 5	Feasibility of CRDSA need to be checked with RAN1 and RAN4 regarding the performance of replica cancelling.

Observation 6	In legacy EDT, the number of repetitions and the largest TBS for Msg3 PUSCH depend on the CE level.

Observation 7	In legacy EDT, RSRP thresholds are configured for the UE to select the CE level.

Observation 8	The UE will verify uplink synchronization before sending Msg3.

Observation 9	The UE may not receive any Timing Advance Command before sending Msg3.

Observation 10	NR’s 2-step random access already supports multiplexing multiple UE Contention Resolution Identities in MsgB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The baseline for uplink capacity enhancements is EDT.

Proposal 2	The Rel-19 uplink capacity enhancement is referred to as preamble-less EDT or RACH-less EDT.

Proposal 3	Diversity Slotted Aloha is considered beneficial and will be supported

Proposal 4	RAN2 will not further consider CRDSA.

Proposal 5	System Information is used to provide preamble-less EDT cell specific PUSCH resources for Msg3 transmission. FFS on signalling details.

Proposal 6	Preamble-less EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level.

Proposal 7	Preamble-less EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold and largest TBS for Msg3 transmission.

Proposal 8	Preamble-less EDT does not require a running TAT for Msg3 transmission.

Proposal 9	After a failure in Msg3 transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power.

Proposal 10	After a number of failed Msg 3 transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

Proposal 11	The RNTI used to schedule Msg4 transmission is derived based on the PUSCH time-frequency resource(s) used for contention based Msg3 EDT transmission.

Proposal 12	Introduce multiplexing of several UE Contention Resolution Identities in Msg4.

Proposal 13	For DSA and CRDSA, RAN2 shall first discuss whether it is feasible to integrate them with repetition.

# 4	References

RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc., RAN#105, Sept. 2024

R2-2405201, “Efficient delivery of RRCEarlyDataComplete message”, TOYOTA Info Technology Center, RAN2#126, May, 2024.

R2-2405202, “Discussion on EDT enhancements”, ESA, RAN2#126 Fukuoka, May 2024.

J. Moberg, M. Löfgren, and R S Karlsson, “Throughput of the WCDMA random access channel”, in IST Mobile Communication Summit, vol. 10. Oct. 2000.

R2-R2-2406640, “Discussion on EDT enhancements”, Qualcomm, RAN2#127, Aug. 2024.

R1- 2407548, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, From: RAN1 To: RAN2; RAN4, RAN1#118 Aug. 2024.

R4-2414114, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, From: RAN4, To: RAN2, Cc: RAN1, RAN4#112, Aug. 2024.

# Appendix	Models for evaluating UL capacity enhancements

## A	CRDSA evaluation

- Given a fixed number of users 1, 2, 3,… , N, …, 3*N users that do pl-EDT transmission in the frame N, find the packet loss ratio (PLR) and save the number for each fixed number of users

  - simplest is to do a short Monte Carlo simulation to get the spread in selection of slots to transmit and collisions etc.

  - model collisions etc.

  - PLR is the number of packets that cannot be correctly decoded within the frame N (all replicas of the packet is colliding with at least one other packet that cannot be correctly decoded)

- Calculate for a Poisson load lambda, what is probability of getting 1, 2, …, 3*N users in the frame N

- Sum the product of the probability of 1, 2, 3, 4, … users with the PLR results from A., this gives the average of packet loss ratio for that lambda

- Repeat from B. with a different load

- Possibly repeat from A. for a different frame N size or different time between pl-EDT occasions etc.

# Appendix B	LS from RAN1 and RAN4

RAN2 had a reply LS from RAN1 [6]:

1. Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B 2. Actions: To RAN2: ACTION: RAN1 respectfully asks RAN2 to take the response into account. To RAN4: ACTION: RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid.

And this reply from RAN4 [7]:

# 1. Overall Description: RAN4 would like to thank RAN2 for the LS R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. RAN2 asked for RAN4 following questions: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? RAN4’s feedback: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP. # 2. Actions: RAN4 respectfully asks RAN2 to take the above information into account. # 3. Date of Next TSG-WG4 Meetings: TSG-WG4#112bis from 2024-10-14 to 2024-10-18 Hefei, China TSG-WG4#113 from 2024-11-18 to 2024-11-22 Orlando, US # A. Annex A: UE transmit timing for Category M1 for Satellite Access (TS36.133) 7.20A.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received. Table 7.20A.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 0.18; 97*TS. Note 1: TS is the basic timing unit defined in TS 36.211. # A. Annex B: UE transmit timing for NB-IoT for Satellite Access (TS36.133) 7.24A.2 Requirements The UE initial transmission timing error shall be less than or equal to Te_NTN_M1 where the timing error limit value Te_NTN_M1 is specified in Table 7.24A.2-1. This requirement applies when it is the first transmission in a DRX cycle, eDRX_CONN cycle, or the first transmission in a repetition period (R>1) for PUCCH, PUSCH, and SRS, or the first transmission after an uplink transmission gap in a repetition period (R>1) for PUCCH or PUSCH, or it is the PRACH transmission, or it is the transmission on PUR, or the first transmission in each segment of PUSCH/PUCCH for eMTC in a repetition period. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving cell minus. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving cell. NTA_Ref for PRACH is defined as 0. (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA_Ref for other channels is not changed until the next timing advance is received. Table 7.24A.2-1: Te_NTN_M1 Timing Error Limit CE Mode; Te_NTN_M1_. A; 41*TS. B; 65*Ts. Note 1: TS is the basic timing unit defined in TS 36.211. Note 2: This requirement applies regardless of the downlink carrier bandwidth..

# Appendix C	RAN2 agreements

## B.1	RAN2#125bis

Agreements:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements

- Both C-plane and U-plane solutions are within scope of uplink capacity enhancements.

- Only CIoT EPS is within scope of uplink capacity enhancements

## B.2	RAN2#126

1 RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) 3. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

## B.2	RAN2#127

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.