3GPP TSG-RAN WG2 Meeting # 127bis	R2-2409185

# Hefei, China, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1899 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IoT NTN UE capabilities correction for GNSS and HARQ enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#127, RAN2 clarified that while the ntn-HarqEnhScenarioSupport-r18 and ntn-GNSS-EnhScenarioSupport-r18 indications are independent from ntn-ScenarioSupport-r17, they should be set consistently. The following agreement was reached: - “RAN2 understands that R18 IoT NTN features other than GNSS/HARQ enhancements refer only to ntn-ScenarioSupport-17.” However, this is not clearly understood from the specification and requires an explicit mention in ntn-ScenarioSupport-17. In RAN2#127, RAN2 attempted to clarify the relationship between ntn-ScenatioSupport-17 and ntn-HarqEnhScenarioSupport-r18 or ntn-GNSS-EnhScenarioSupport-r18, reaching the following agreement: - “RAN2 understands that GNSS/HARQ enhancements scenario support need to be set consistently with ntn-ScenatioSupport-17 (discuss how to add this clarification in the field description during the CR review)” The current specification text appears to mandate that if GNSS/HARQ features are only supported in one type of orbit, either NGSO or GSO, the UE shall set ntn-ScenatioSupport-17 consistently and the rest of UE capabilities shall only be supported for that specific orbit. This was not the original RAN2 intention and precludes the possibility that ntn-ScenarioSupport-17 is not present (both NGSO and GSO are supported), and ntn-HarqEnhScenarioSupport-r18 or ntn-GNSS-EnhScenarioSupport-r18 are supported in only one type of orbit, either GSO or NGSO. Finally, RAN2 has received an LS from RAN1 (R1-2407388) with updated UE feature groups that affect the definition ntn-Autonomous-GNSS-Fix-r18 when NGSO scenario is supported for GNSS HARQ enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the description of ntn-ScenarioSupport-r17 so that it explicitly indicated that the field does not apply to GNSS enhancement and HARQ enhancement. Clarify the consistency example in ntn-HarqEnhScenarioSupport-r18 and ntn-GNSS-EnhScenarioSupport-r18 so that ntn-ScenarioSupport-17 can be absent (both NGSO and GSO are supported), while ntn-HarqEnhScenarioSupport-r18 or ntn-GNSS-EnhScenarioSupport-r18 may be set to either GSO or NGSO. Align specification text with RAN1 updated feature list to include the conditional support of ntn-Triggered-GNSS-Fix-r18 when ntn-Autonomous-GNSS-Fix-r18 is supported in NGSO. Impact analysis Impacted functionality: IoT NTN, GNSS and HARQ enhancements Inter-operability: If the netwok is implemented according to this CR but the UE is not, for the last change, GNSS autonomous fix capability and its conditional relation to GNSS triggered fix upon NGSO may be misconfigured. For the other two changes, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network may wrongly understand the UE’s supported scenarios of Rel-18 features and the conditional dependency of GNSS enhancement capabilities, leading to wrong UE capability configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.38.5, 4.3.38.30, 4.3.38.32, 4.3.38.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 4.3.38.5	ntn-ScenarioSupport-r17

This field indicates whether the UE supports NTN features in GSO or NGSO scenario. The UE indicating support of ntn-ScenarioSupport-r17 shall also indicate support of ntn-Connectivity-EPC-r17. If a UE does not include this field but includes ntn-Connectivity-EPC-r17, the UE supports the NTN features for both GSO and NGSO scenarios. This field does not apply to the features UL and DL HARQ process enhancements and GNSS measurement and UL transmission extension enhancements which have a separate scenario capability, ntn-HarqEnhScenarioSupport-r18 and ntn-GNSS-EnhScenarioSupport-r18, respectively.

END OF CHANGE

START OF CHANGE

#### 4.3.38.30	ntn-HarqEnhScenarioSupport-r18

This field indicates whether the UL and DL HARQ process enhancements that are indicated as supported are applicable in GSO or NGSO scenarios for UE indicating support of GSO and NGSO scenarios. If this field is not included, the UL and DL HARQ process enhancements that are indicated as supported are applicable in both GSO and NGSO scenarios. The UL and DL HARQ process enhancements that are indicated as supported are mandatory for GSO scenario. This field is only applicable if the UE supports at least one of ntn-RRC-HarqDisableSingleTB-r18, ntn-OverriddenHarqDisableSingleTB-r18, ntn-DCI-HarqDisableSingleTB-r18, ntn-RRC-HarqDisableMultiTB-r18, ntn-OverriddenHarqDisableMultiTB-r18, ntn-DCI-HarqDisableMultiTB-r18, ntn-RRC-HarqDisableSingleTB-CE-ModeA-r18, ntn-RRC-HarqDisableSingleTB-CE-ModeB-r18, ntn-OverriddenHarqDisableSingleTB-CE-ModeB-r18, ntn-DCI-HarqDisableSingleTB-CE-ModeB-r18, ntn-RRC-HarqDisableMultiTB-CE-ModeA-r18, ntn-RRC-HarqDisableMultiTB-CE-ModeB-r18, ntn-OverriddenHarqDisableMultiTB-CE-ModeB-r18, ntn-DCI-HarqDisableMultiTB-CE-ModeB-r18, ntn-UplinkHarq-ModeB-SingleTB-r18 and ntn-UplinkHarq-ModeB-MultiTB-r18. If ntn-ScenarioSupport-r17 is included, this field is set in consistency with ntn-ScenarioSupport-r17 (i.e., this field is set to GSO if the ntn-ScenarioSupport-r17 is absent or indicates GSO).

END OF CHANGE

START OF CHANGE

#### 4.3.38.32	ntn-Autonomous-GNSS-Fix-r18

This field indicates whether the UE supports autonomous GNSS position fix in RRC_CONNECTED as specified in TS 36.331 [5]. A UE supporting this feature shall also indicate the support of ntn-Connectivity-EPC-r17. A UE supporting ce-ModeA-r13 and this feature in NGSO scenario shall also indicate the support of ntn-Triggered-GNSS-Fix-r18. If the UE indicates this capability, the UE shall support the following enhancements:

-	UE reports GNSS position fix time duration for measurement in RRCConnectionSetupComplete (-NB), RRCConnectionResumeComplete (-NB), and RRCConnectionReestablishmentComplete (-NB) and RRCConnectionReconfigurationComplete messages;

-	UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger;

-	UE reports the remaining GNSS validity duration with MAC CE in RRC_CONNECTED.

END OF CHANGE

START OF CHANGE

#### 4.3.38.34	ntn-GNSS-EnhScenarioSupport-r18

This field indicates whether the GNSS measurement and UL transmission extension enhancements in RRC_CONNECTED that are indicated as supported are applicable in GSO or NGSO scenario for UE indicating support of GSO and NGSO scenarios. If this field is not included, the GNSS measurement and UL transmission extension enhancements in RRC_CONNECTED that are indicated as supported are applicable in both GSO and NGSO scenario. The GNSS measurement and UL transmission extension enhancements that are indicated as supported are mandatory for GSO scenario. This field is only applicable if the UE supports at least one of ntn-Triggered-GNSS-Fix-r18, ntn-Autonomous-GNSS-Fix-r18 and ntn-UplinkTxExtension-r18. If ntn-ScenarioSupport-r17 is included, this field is set in consistency with ntn-ScenarioSupport-r17 (i.e., this field is set to GSO if the ntn-ScenarioSupport-r17 is absent or indicates GSO).

END OF CHANGE