3GPP TSG-RAN WG2 Meeting # 127bis	R2-2409187

# Hefei, China, 14 - 18 October 2024

Agenda Item:	7.7.2

Source:	Ericsson

Title:	Remaining open issues

Document for:	Discussion, Decision

# 1	Introduction

The aim of this contribution is to address two topics, namely NTN measurements in TN cells and SMTCs in satellite switch with re-sync, that were marked earlier as possible come backs.

# 2	Discussion

## 2.1	NTN measurements in TN cells

In RAN2#127, extensive discussions were held regarding UE capabilities for NR NTN enhancements. The discourse encountered an impasse when addressing mobility capabilities between TN and NTN cells. Previously, RAN2 had agreed to support idle and connected mode mobility between TN and NTN by enabling the provision of satellite assistance information through broadcast (e.g., SIB19) and dedicated signaling (e.g., Measurement Object) in TN cells. However, because RAN4 has not yet defined requirements for this scenario, some parties opposed the introduction of a capability that informs the network about the support for these measurements. Ultimately, an incomplete agreement was reached as a compromise:

[Introduce] a new R18 signalled UE capability to receive SIB19 in TN cell in connected mode is introduced (without any relation to RRM requirements) (can come back to further clarify the capability description in the future)

Unfortunately, this new capability is inadequate for the network to understand the expected UE behaviour. For example, the network may broadcast SIB19 in a TN cell and configure a UE to measure an NTN neighbour cell using a Measurement Object which relies on the satellite assistance information provided via broadcast signalling, i.e. SIB19. Under this incomplete capability, the network remains unaware of whether the UE can perform such measurements, potentially leading to unsuccessful configurations and mobility procedures (e.g., blind handover). Therefore, we propose that RAN2 revisits this topic to clarify whether mobility procedures in idle/inactive and connected modes are fully supported between TN and NTN from RAN2's perspective.

RAN2 to clarify whether a UE can measure a NTN cell while camping or being connected to a TN cell.

## 2.2	SMTC update in satellite switch with re-sync

The topic of SMTCs in satellite switch with re-sync has been extensively discussed in previous meetings, yet no definitive conclusions have been reached. According to Rel-17 behaviour in RRC_CONNECTED, it is the network's responsibility to adjust the UE’s SMTCs based on propagation delay difference (PDD) reports. This adjustment may occur frequently. Figure 1 illustrates the variation of propagation delay between a UE and a satellite as a function of time. Considering the maximum SMTC size of 5ms, the SMTC offset may need to be adjusted multiple times during the satellite's service coverage period.

Figure 1. Variation of propagation delay due to satellite movement.

Upon a satellite switch with re-sync, the propagation delay difference between the new satellite and the neighbouring satellites changes significantly (i.e., exceeding the threshold), which will lead to simultaneous PDD reports from all the configured UEs. Consequently, the network will need to send an RRCReconfiguration message to each UE in RRC_CONNECTED in the cell to reconfigure the SMTCs upon satellite switch.

UEs configured with PDD report may (simultaneously) generate and send a PDD report upon satellite switch with resynchronization.

The outcomes of the offline discussions during RAN2#127 [1] highlighted the necessity of updating SMTCs whenever a "shifted SSB location" is used. This is the scenario when the target satellite employs a certain SSB time offset to avoid collision with SSBs from the source satellite. The report [1] indicates that "the SMTC would be off even in the same orbit (A, C), assuming those cells perform the same change in a synchronized manner." Therefore, it suggests that, in RRC_CONNECTED, a UE performing a satellite switch with re-sync would benefit from applying the SSB-timeOffset to the neighbour cell's SMTC. We recognize that there may be significant impacts on SMTCs during a soft satellite switch with re-sync, and we urge RAN2 to revisit this topic and reach a definitive conclusion.

RAN2 to revisit the impact on neighbour cell SMTCs upon satellite switch with resync.

Upon satellite switch with re-sync, the UE should apply the ssb-TimeOffset to the configured SMTCs for neighbouring satellites before receiving updated SMTCs from the network.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	UEs configured with PDD report may (simultaneously) generate and send a PDD report upon satellite switch with resynchronization.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to clarify whether a UE can measure a NTN cell while camping or being connected to a TN cell.

Proposal 2	RAN2 to revisit the impact on neighbour cell SMTCs upon satellite switch with resync.

Proposal 3	Upon satellite switch with re-sync, the UE should apply the ssb-TimeOffset to the configured SMTCs for neighbouring satellites before receiving updated SMTCs from the network.

# References

R2-2405680