3GPP TSG RAN WG2#127bis	R2-2409188

Hefei, China, 14th - 18th October 2024

Agenda item:	         8.3.2

Source:	CEWiT

Title:	Simulation results for RRM Measurement Prediction

Document for:	     Discussion and Decision

# Introduction

In the SID on AI/ML Mobility , it was agreed to study the simulation parameters and AI/ML methods for different sub use cases. Additionally, in the RAN2#127[2] meeting, more agreements were reached on defining the AI/ML terminologies, frameworks, use cases, evaluation methodologies, KPI’s etc.

Agreements - companies are encouraged to considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell, but only low to high is expected. - For the agreed frequencies for inter-frequence case, only one UE speed is considered for inter-frequency prediction in simulation, e.g., 30km/h. Companies can consider other speeds for other frequencies if they chose to simulate them. - For temporal domain case B prediction the input is historical measurement values and the output is the values at the subsequent time instances that measurement is skipped, i.e., the prediction is always after the measurement and is at future time instance(s).

In this contribution we present our initial simulation results for RRM Measurement prediction in temporal domain for the remaining case and our views on the evaluation methodologies and KPI’s.

# 2. Discussion

In this document, we will present our results for the temporal domain, covering Case B  for Sub Use Case 2.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |

## 2.1 Evaluation Methodology

Case 4 - FR1 to FR1 intra-frequency temporal domain case B

Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

In Case B, measurement result in future of one cell e.g., cell A is predicted based on historical ones of the

same cell A.

In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with sampling period(s) (with sliding L1/L3 filtering option)  after measurement result(s) in previous prediction window is/are skipped.

Figure 1: Temporal domain case B [1]

2.2 Evaluation assumptions

In our simulations we followed the simulation agreements that were agreed in the RAN2 previous meetings. The detailed simulation assumptions are given in the Appendix. According to the simulation assumptions, the gNB is equipped with one panel which has 128 antennas The gNB antenna array supports 1 transmitting beams,10 users in 2 tier model having 57 gNB. Measurement period as 10s and sample period 40ms.

The dataset was generated using system level simulations based on the simulation parameters given in the appendix. Out of the around  80640 datasets generated using SLS, 80% of data points are used for training, 10% for validation and 10% for testing for each sector. Since the RSRP values can be quite varied, we normalize the RSRP values before giving it as input to the AI/ML model.

We modeled the ML model as neural network with three fully connected LSTM layer and one dense layer in output. The activation function used is linear. The DNN was trained for a maximum of 150 epochs with a batch size of 8. The loss function used was MSE(mean square error) and an Adam optimizer was used to minimize the loss.

In the subsequent discussion simulated results are given.

2.3 Simulation results

The data is preprocessed with an observation window of 3 instances and a prediction window of one instance, the CDF plot reflects how well the model predicts the RSRP differences based on those 3 observations.

The input to the model is a multi-dimensional tensor of shape (#Users, #Measured_Timesinstances, #TX_beams) and the output of the model is (#Predicted_Timeinstances, #TX_beams)

Fig : CDF of average L3 RSRP difference

The x-axis represents the average RSRP difference in dB

Observations :

1. 80% of RSRP difference falls under 1.5dB for the temporal domain, covering Case B  for Sub Use Case 2

# 3. Conclusion

1.  80% of RSRP difference falls under 1.5dB for the temporal domain, covering Case B  for Sub Use Case 2.

# References

- [1] R2-2407849 Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO)

- [2]3GPP 127 ChairNotes

# Appendix

| Parameters | Values |
| --- | --- |
| Frequency Range | FR1 @ 4 GHz - SCS: 30 kHz |
| Deployment | 500m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 20MHz |
| UE Speed | 30km/h |
| UE distribution | 100% outdoors |
| BS Antenna Configuration | [One panel: ( Mg, Ng, M, N, P,) = (1,1,8, 8, 2), (dV, dH) = (0.8, 0.5)λ as baseline] Azimuth angles (degrees) = [0] Zenith angles (degrees) = [102] |
| BS Tx Power | 44 dBm |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 5dB |
| UE receiver Noise Figure | 9 dB |
| Inter site distance | 500m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | bouncing circle model |
| Sampling period | 40ms |