3GPP RAN WG2 Meeting #127bis 	R2-2409191

# Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.9.4

Source:	InterDigital, Inc.

Title:	Support of PWS.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we look at some of the practicalities.

# Service and regulatory requirements

According to [4], the term PWS refers to a number of different systems, namely PWS, ETWS, CMAS, EU-ALERT, and KPAS deployment which depend on operator decision or national regulations.

In LTE RRC specification, two types of PWS are defined. These are ETWS (Japan), and CMAS, which covers CMAS (US), EU-Alert (Europe) and KPAS (Korea).

The WID uses the generic term “PWS” therefore it would be good to clarify whether this covers ETWS, CMAS, or both because the RRC impact is different depending on which systems are to be supported.

Proposal 1: Clarify whether the scope of the WID applies to ETWS, CMAS, or both.

When NB-IoT was first introduced in Rel-13, the issue of whether to support ETWS and CMAS was discussed. At the time, it was noted that the service requirements for PWS systems were not compatible with the device requirements for NB-IoT.

For example, according to [3] and [4] the requirement for ETWS is as follows:

Duration of the delivery time for PLMN operators is the time from the receipt of the Warning Notification by the PLMN operator, i.e. the edge of the 3GPP network, to the time that the Warning Notification is successfully delivered to the UEs. Provisioning of delivery of Primary and Secondary Notification may be required: - Primary Notification shall be delivered within 4 seconds to the UE in the Notification Area even under congestion situation. - Secondary Notification is delivered to the users in the Notification Area even under congestion situation. Note 1: UEs that are out of coverage or switched off are not considered in the requirements. Note 2: Secondary Notification may not always be generated as it depends on the Warning Notification Provider’s policy. Note 3: Primary Notification may not always be generated (i.e. the warning may start with a Secondary Notification).

In contrast, according to [2]:

When extended DRX (eDRX) is used in idle mode, the following are applicable: - The DRX cycle is extended up to and beyond 10.24s in idle mode, with a maximum value of 2621.44 seconds (43.69 minutes); For NB-IoT, the maximum value of the DRX cycle is 10485.76 seconds (2.91 hours);

Clearly, the requirements are incompatible. NB-IoT devices and standards are designed to support up to 10 years battery life, hence the very long supported DRX cycles, while PWS requirements are designed to notify users within a very short time of emergency situations.

In addition, for Rel-13 NB-IoT many of the features supported in LTE were not specified if they were not essential, in order to minimize (in theory as much as possible) the device complexity. At the time of introducing NB-IoT, the primary use case was for machine type devices such as smart meters and therefore there was no strong requirement for supporting PWS due to the complexity and the incompatible service and device requirements.

Observation: PWS was not specified for NB-IoT in Rel-13 due to complexity, incompatible service and device requirements, and lack of use-case.

Having noted the above, we acknowledge that NB-IoT use-cases have expanded and evolved, and this is apparent due to many of the enhancements introduced from Rel-14 onwards. In particular, support of IoT-NTN provides an opportunity to enable more complex types of device such as a smartphone or wearable, which would normally be using a terrestrial network (E.g. non-MTC LTE), to gain access to improved coverage in rural area using satellite coverage. For this type of device, the battery life requirement is less of an issue, and DRX cycles can be set to relatively short values, in the order of milliseconds in line with regular LTE.

PWS is supported for LTE, including eMTC devices. As a result, the following is captured in [2]:

- ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. For EAB, if the UE supports SIB14, when in extended DRX, it acquires SIB14 before establishing the RRC connection;

It should be acknowledged that the same will apply for NB-IoT – i.e. the ETWS, CMAS, PWS requirement may not be met when the UE is in eDRX. This essentially limits UEs supporting PWS in a network configuring PWS from using longer DRX cycles, limiting the power saving benefits of longer DRX cycles.

Proposal 2: Clarify that for NB-IoT the ETWS, CMAS, PWS requirement may not be met when the UE is in eDRX.

Additionally, this is noted in the SA1 specification [4]

Note: A bandwidth reduced low complexity UE or a UE supporting eDRX may not support all requirements for PWS, including ETWS, CMAS, EU-Alert and KPAS

The SA1 specification might need to be updated to clarify that this additionally applies to NB-IoT.

In addition, there is a possibility that RAN3 and SA2 specifications are impacted, potentially in more significant ways than just noting the requirement, and clearly they are in the best position to analyse the impact, if any.

Proposal 3: Send an LS to SA1, SA2, RAN3 to inform them that PWS is being specified for NB-IoT and to update their specifications, if needed.

# Overview of existing LTE functionality

The RRC specification [6] contains most of the functionality needed to support ETWS and CMAS. However, there is also some minor impact in the idle mode specification [5], because section 4.4 lists unsupported functionality which should be updated to remove ETWS, CMAS, PWS.

The following shows the main impact in RRC:

System information procedures:

5.2.1.3	System information validity and notification of changes

5.2.1.4	Indication of ETWS notification

5.2.1.5	Indication of CMAS notification

5.2.2.4	System information acquisition by the UE

- These sections section refer to LTE specific SIBs and therefore needs to be updated to apply to NB-IoT new SIBs.

Paging procedures:

5.3.2	Paging

5.3.2.1	General

- this section excludes NB-IoT from receiving ETWS and CMAS notifications in paging so needs to be updated.

5.3.2.3	Reception of the Paging message by the UE

- These sections section refer to LTE specific SIBs and therefore needs to be updated to apply to NB-IoT new SIBs.

Proposal 4: Procedures related to system information acquisition and paging need to be updated to be applicable to NB-IoT.

System  Information definitions

–	SystemInformationBlockType10

This contains ETWS primary notification and is missing from NB-IoT

–	SystemInformationBlockType11

This contains ETWS primary notification and is missing from NB-IoT

–	SystemInformationBlockType11

This contains CMAS indication and is missing from NB-IoT

Proposal 5: SIB10-NB, SIB11-NB need to be added to NB-IoT to support ETWS.

Proposal 6: SIB12-NB needs to be added to NB-IoT to support CMAS.

Paging:

–	Paging-NB

The Paging-NB message is used for the notification of one or more UEs and needs to be updated to include ETWS notifications and CMAS notifications.

Proposal 7: Paging-NB needs to be updated to add support for ETWS and CMAS notifications.

MAC Config:

ce-ETWS-CMAS-RxInConn

Indicates UE shall monitor for ETWS/CMAS notification on control channels associated with the shared data channel in RRC_CONNECTED as specified in TS 36.213 [23], clause 7.1. It needs to be discussed whether this is needed for NB-IoT. We assume that ETWS and CMAS need to be supported in RRC_CONNECTED for NB-IoT, however this may need some time for checking and disucssion. Note that this may also imply RAN1 impact.

Proposal 8: FFS whether MAC configuration need to be updated to indicate if UE shall monitor for ETWS/CMAS notification on control channels associated with the shared data channel in RRC_CONNECTED. RAN1 to be informed if so.

Finally, as it may be the case that, in addition to SA WG impacts, there is impact to other RAN WGs (e.g. RAN1, RAN3) an updated WI may be necessary to include those RAN working groups, if impact is found. We do not have any proposal for now but this should be kept in mind as the impact may not be limited to RAN2.

# Conclusion

In this contribution we discuss support of PWS and make the following proposals for discussion:

Proposal 1: Clarify whether the scope of the WID applies to ETWS, CMAS, or both.

Observation: PWS was not specified for NB-IoT in Rel-13 due to complexity, incompatible service and device requirements, and lack of use-case.

Proposal 2: Clarify that for NB-IoT the ETWS, CMAS, PWS requirement may not be met when the UE is in eDRX.

Proposal 3: Send an LS to SA1, SA2, RAN3 to inform them that PWS is being specified for NB-IoT and to update their specifications, if needed.

Proposal 4: Procedures related to system information acquisition and paging need to be updated to be applicable to NB-IoT.

Proposal 5: SIB10-NB, SIB11-NB need to be added to NB-IoT to support ETWS.

Proposal 6: SIB12-NB needs to be added to NB-IoT to support CMAS.

Proposal 7: Paging-NB needs to be updated to add support for ETWS and CMAS notifications.

Proposal 8: FFS whether MAC configuration need to be updated to indicate if UE shall monitor for ETWS/CMAS notification on control channels associated with the shared data channel in RRC_CONNECTED. RAN1 to be informed if so.

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”

- 22168-900, “Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1”

- 22268-i30,  “Public Warning System (PWS) requirements (Release 18)”

- 36304-i20, “User Equipment (UE) procedures in idle mode”

- 36331-i31, “Radio Resource Control (RRC);”