3GPP TSG-RAN WG2 Meeting #127bis	R2-2409211
Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.1

Source: 		Vice Chairman (Samsung)

Title: 			Report from session on V2X/SL, R18/19 MOB, and R19 NES

Document for:	Approval

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

## Approved outgoing LSs

R2-2409376	Reply LS on Early TA acquisition for the inter-DU scenario	LS out	To:RAN3	Cc:RAN1

R2-2409359	Reply LS on intra-SN SCPAC in MN format	LS out		To:RAN3

R2-2409360	LS on UE capabilities for inter-frequency L1 measurements for LTM	LS out		To:RAN1, RAN4

## List and Status of Offline/Email Discussions

[AT127bis][111][MOB] (Vodafone)

Scope: To prepare response LS to RAN3.

Intended outcome: Response LS in R2-2409373. Email approval.

Deadline: 19:00pm Thursday

Session chair cannot find out related email. Will be double checked in main session.

R2-2409373	L3 measurement based LTM support over F1	LS out		To:RAN3

SHORT EMAIL DISC:

[POST127bis][113][MOB] (Ericsson)

Scope: To prepare merged RRC CR (including all agreements made this meeting).

Intended outcome: RRC CR in R2-2409353 (or R2-2409375 if new tdoc number is needed) to be agreed in principle

Deadline: Short email discussion

[POST127bis][114][MOB] (MediaTek)

Scope: To prepare LS on the capability ltm-FastProcessingConfig-r18 to RAN4.

Intended outcome: LS in R2-2409377 to be approved.

Deadline: Short email discussion

[POST127bis][115][MOB] (ZTE)

Scope: To prepare LS to RAN3 to inform RAN2 agreements that impact on RAN3.

Intended outcome: LS in R2-2409378 to be approved.

Deadline: Short email discussion.

COMPLETED AT OFFLINE DISC:

[AT127bis][101][V2X/SL] (LG)

Scope: To prepare detailed wordings for Re-TX loop.

Intended outcome: MAC CR in R2-2409351 to be agreed in principle. LG will inform the offline discussion schedule later.

Deadline: Comeback in Thursday CB session. => Completed.

[AT127bis][102][MOB] (Google)

Scope: To prepare response LS to RAN3 (including the discussion on detailed wordings)

Intended outcome: Response LS in R2-2409352 to be approved. Email approval.

Deadline: Until 10:00am Thursday. => Completed.

[AT127bis][103][MOB] (Ericsson)

Scope: 1) To discuss and capture P1 in R2-2407994, P3 in R2-2408751, P1 and P4 in R2-2408819 in RRC CR rapporteur’s miscellaneous CR. 2) To discuss P2 and P3 in R2-2407994, P1 in R2-2408126, P2 in R2-2408751, P4 in R2-2408945, P4 in R2-2408258.

Intended outcome: 1) RRC CR in R2-2409353 to be in principle agreed. Email approval. 2) Discussion summary in R2-2409354. Comeback in Thursday CB session. Ericsson will inform offline schedule later.

Deadline: 1) Thursday 10:00am, 2) Thursday CB session => Completed. Note CR will be further checked via short email discussion.

[AT127bis][104][MOB] (Nokia)

Scope: To discuss need of corrections on P3 in R2-2408945, R2-2408452, R2-2408632, and capture agreeable corrections in 38.300 miscellaneous CR.

Intended outcome: 38.300 CR in R2-2409355 to be in principle agreed. Discussion summary in R2-2409356 if needed. Email approval.

Deadline: Thursday 10:00am. => Completed.

[AT127bis][105][MOB] (ZTE)

Scope: To discuss correction on R2-2408522 (including need of correction).

Intended outcome: 37.340 CR in R2-2409357 to be in principle agreed. Discussion summary in R2-2409358 if needed. Email approval.

Deadline: Thursday 10:00am. => Completed.

[AT127bis][106][MOB] (ZTE)

Scope: To prepare the response LS to RAN3.

Intended outcome: Response LS in R2-2409359 to be approved. Email approval.

Deadline: Thursday 10:00am. => Completed.

[AT127bis][107][MOB] (Huawei)

Scope: To prepare the LS to RAN4.

Intended outcome: Response LS in R2-2409360 to be approved. Email approval.

Deadline: Thursday 10:00am. => Completed.

[AT127bis][108][MOB] (Huawei)

Scope: 1) To discuss and capture R2-2409138, R2-2408297, R2-2408784, and 1st change in R2-2408755 in MAC CR rapporteur’s miscellaneous CR. 2) To discuss 2nd change in R2-2408755 and R2-2408817.

Intended outcome: 1) MAC CR in R2-2409361 to be in principle agreed. Email approval. 2) Discussion summary in R2-2409362. Comeback in Thursday CB session.

Deadline: 1) Thursday 10:00am, 2) Thursday CB session => Completed.

[AT127bis][109][V2X/SL] (ZTE)

Scope: To prepare the corresponding MAC CR.

Intended outcome: MAC CRs in R2-2409363/R2-2409364/R2-2409365 to be agreed in principle. Email approval.

Deadline: 10:00am Thursday => Completed.

[AT127bis][110][NES] (OPPO)

Scope: To calculate signalling overheads between option-a and option-b. To check companies’ opinion if P1 in R2-2407974 is ok or if option-a should be considered instead.

Intended outcome: Discussion summary in R2-2409372. OPPO will announce the offline discussion schedule via email.

Deadline: Comebacks in Thursday CB session => Completed.

[AT127bis][112][MOB] (ZTE)

Scope: To discuss P1-P8 in R2-2408524, ZTE, P4 in R2-2408596, Apple, P1 in R2-2408758, Ericsson, and P2 in R2-2408596, Apple. And to make agreeable proposals.

Intended outcome: Discussion summary in R2-2409374. Offline schedule will be announced.

Deadline: Comeback in Friday CB session. => Completed.

## 4.2	V2X and Sidelink corrections Rel-15 and earlier

REL-15 and Earlier WIs related to V2x and Sidelink are in scope but not listed explicitly (long list).

This Agenda Item is treated in the V2X and Sidelink Breakout session

Tdoc Limitation: 1 tdocs

## 5.2	NR V2X

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Aug 20; WID: RP-200129).

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please first contact / coordinate with CR rapporteur company for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

Tdoc Limitation: 1 tdocs

R2-2408541	Discussion on intra-UE prioritization	ZTE Corporation, Nokia	discussion	Rel-16	5G_V2X_NRSL-Core

Proposal1: Regarding SR triggered by TA report/DSR/Positioning/ BFR, if PUCCH for the SR overlaps with SL-SCH/SL-PRS transmission, RAN2 is suggested to discuss following options to handle the missing intra-UE prioritization procedure for SR triggered by MAC CEs:

Option1: No MAC specification change is needed:

Option1a: PHY layer prioritization can handle this, as highlighted in green in Table II.

Option1b: Following Uu MAC prioritization rule: de-prioritize SR triggered by above MAC CEs, as highlighted in blue in Table II.

Option2: Leave it to UE implementation, and capture it as a NOTE in current MAC specification, as clarified in Annex 4.1.

Option3: Change normative text of MAC specification to clarify how to handle the SR triggered by above MAC CEs.

A note that prioritization between SR triggered by other Uu MAC CEs (except BSR MAC CE) and SL SCH is left to UE implementation will be added. Detailed wordings can be further discussed in the CR preparation.

[AT127bis][109][V2X/SL] (ZTE)

Scope: To prepare the corresponding MAC CR.

Intended outcome: MAC CRs in R2-2409363/R2-2409364/R2-2409365 to be agreed in principle. Email approval.

Deadline: 10:00am Thursday => Completed.

[OPPO, Apple]: Prefer option1. [ASUSTek]: Prefer option3. [Qualcomm]: SR can be sent via PUCCH or PUSCH, and thinks it is already covered by PHY in Rel-16. [Lenovo]: Agree with Qualcomm. Do not see any real need to change. Assume the UE typically prioritizes SR. [Samsung]: In Rel-16, not all MAC CE cases were discussed however it is related to commercialization (to judge which other features are prioritized), then we don’t need to take any action for Rel-16 for now. [Ericsson]: With option1, it seems companies understand it will anyway leave it to UE implementation. Then it is better to go with option2. [ZTE]: If we don’t do anything, it means all SRs triggered by other MAC CEs will be deprioritized according to the current MAC. [OPPO]: Assuming we have a note, do we really need to restrict specific MAC CEs as indicated in proposal 1? [ZTE]: Not really, we can have a general note. [OPPO]: Make sure the note is applicable for other MAC CE cases (except BSR MAC CE). [Ericsson]: Should it be considered as NBC change? [ZTE]: It is only for UE internal behavior and it’s a note to leave it to UE implementation. Do not consider it as NBC change.

R2-2409363	Correction on intra-UE prioritization	ZTE Corporation	CR	Rel-16	38.321	16.17.0	1974	-	F	5G_V2X_NRSL-Core

R2-2409364	Correction on intra-UE prioritization	ZTE Corporation	CR	Rel-17	38.321	17.10.0	1975	-	A	5G_V2X_NRSL-Core

R2-2409365	Correction on intra-UE prioritization	ZTE Corporation	CR	Rel-18	38.321	18.3.0	1976	-	A	5G_V2X_NRSL-Core

Agreed in principle.

R2-2408585	Corrections on UE behaviour upon reception of RRCReconfigCompleteSL	Apple	CR	Rel-16	38.331	16.18.0	5019	-	F	5G_V2X_NRSL-Core

R2-2408586	Corrections on UE behaviour upon reception of RRCReconfigCompleteSL	Apple	CR	Rel-17	38.331	17.10.0	5020	-	F	5G_V2X_NRSL-Core, NR_SL_relay-Core

R2-2408587	Corrections on UE behaviour upon reception of RRCReconfigCompleteSL	Apple	CR	Rel-18	38.331	18.3.0	5021	-	F	5G_V2X_NRSL-Core, NR_SL_relay-Core, NR_SL_relay_enh-Core, NR_SL_enh2

Noted.

[OPPO]: “consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied” is directly applied to TX UE. Seems the proposed correction is somewhat different than what the current specification intends to. [Vivo]: Share the view with OPPO. [CATT]: With the current spec, we didn’t have to put all detailed parameters (including RAN1 related parameters). With this change, we may need to introduce more changes for the UE behaviours to other parameters.

R2-2408913	RRC correction on sl-X-Overhead field description of SL-ResourcePool	Philips International B.V.	CR	Rel-16	38.331	16.18.0	5058	-	F	5G_V2X_NRSL-Core

R2-2408914	RRC correction on sl-X-Overhead field description of SL-ResourcePool	Philips International B.V.	CR	Rel-17	38.331	17.10.0	5059	-	A	5G_V2X_NRSL-Core

R2-2408915	RRC correction on sl-X-Overhead field description of SL-ResourcePool	Philips International B.V.	CR	Rel-18	38.331	18.3.0	5060	-	A	5G_V2X_NRSL-Core

Coverage page will be updated to have inter-operability analysis between UEs and to remove “NE-DC and NR-DC” from impacted 5G architecture options.

Agreed in principle with the changes above, in R2-2409366, R2-2409367 and R2-2409368.

[Ericsson]: For inter-operability, analysis between UEs is missed. [ZTE]: We need to remove “NE-DC and NR-DC” from impacted 5G architecture options in the cover page.

## 6.6	NR Sidelink enhancements

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

Tdoc Limitation: 1 tdoc

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please first contact / coordinate with CR rapporteur company for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

R2-2408361	Misc RRC corrections for SL enhancements	Huawei, HiSilicon	CR	Rel-17	38.331	17.10.0	5001	-	F	NR_SL_enh-Core

R2-2408362	Misc RRC corrections for SL enhancements	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5002	-	A	NR_SL_enh-Core

Agreed in principle.

R2-2408539	Correction on unit of SL DRX timer	ZTE Corporation	CR	Rel-17	38.321	17.10.0	1940	-	F	NR_SL_enh-Core

R2-2408540	Correction on unit of SL DRX timer	ZTE Corporation	CR	Rel-18	38.321	18.3.0	1941	-	A	NR_SL_enh-Core

The intention is agreed (to change the slot level description to subframe level).

Corresponding CRs will be submitted next meeting.

[LG]: Slot level offset was actually decided, however in RRC, it is specified in 1ms unit. [ASUSTek]: Ok with the intention in general, however for the wording, it would be more accurate with “the subframe of the slot where the DRX cycle starts”. [Apple]: Seems parameter does not exist. Actually offset is combined with DRX cycle. [Huawei]: Confirms Apple’s understanding, it followed Uu signalling way. [Samsung]: The correct parameter name would be “sidelink DRX-StartOffset” [Apple]: No, it is not a parameter name. [ZTE]: Suggest to agree with the original intention for this meeting, and if companies want to correct IE name, we can consider further update it for the next meeting. [Session chair]: Suggest to agree the original intention for now. Assume ZTE will provide a corresponding CR with the consideration of the other companies’ comments next meeting.

R2-2408639	Discussion on MAC correction for IUC's resource selection	LG, OPPO, Ericsson	discussion	NR_SL_enh-Core

=> Withdrawn

R2-2408642	Discussion on MAC correction for IUC's resource selection	LG, OPPO, Ericsson	discussion	NR_SL_enh-Core

Note approach is agreed.

Detailed wordings for the note will be revisited next meeting.

[Apple, Qualcomm, Huawei, Samsung]: Prefers a note-based solution. [Huawei]: Traditionally we specify TX and RX UE behaviour in separate. If we accept normative text-based solution, it may impact other sections also. [LG, OPPO]: Prefer a normative text for a clear UE behaviour. [OPPO]: Without a normative text, the UE meets two conditions, then brings duplicated resource selection procedures. Ok with a note, however we may need more description to avoid it in the note. [Ericsson]: Ok with a note with better description. [OPPO]: Suggest to agree with the note-based approach and for the detailed wordings, we can update the CR next meeting.

R2-2408680	Correction to MAC on random resource selection for sidelink	Ericsson	CR	Rel-17	38.321	17.10.0	1944	-	F	NR_SL_enh-Core

R2-2408681	Correction to MAC on random resource selection for sidelink	Ericsson	CR	Rel-18	38.321	18.3.0	1945	-	A	NR_SL_enh-Core

“:” should replace the “.”

Agreed in principle with the change above, in R2-2409369 and R2-2409370.

[Apple]: “:” should replace the “.”

## 7.4	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

Time budget: 0 TU)

Tdoc Limitation: 2 tdocs.

### 7.4.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2407915	LS on missing RRC parameters for early UL sync PRACH transmission for LTM (R1-2407447; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_enh2-Core	To:RAN2

Noted.

[Session chair]: Does RRC already capture them? [Fujitsu]: It was already captured in RRC.

R2-2407924	LS on Early TA acquisition for the inter-DU scenario (R3-244793; contact: Google)	RAN3	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2	Cc:RAN1

Noted.

R2-2409382   Reply LS on L3 measurement based LTM support over F1 (R3-245710; contact: Vodafone)

[Apple, Lenovo]: Prefer having at least UL pre-sync. [Nokia]: RAN4 defines what is required for early synchronization and it allows L3 measurements at least for FR1. So it should be supported. [MediaTek]: Without early synchronization, there is no real difference compared to CHO. Early synchronization should be supported. [Ericsson]: For early synchronization purpose, a CU still just indicates to the DU to perform early synchronization. No need to exchange of measured result between CU and DU. [Samsung]: Want to understand what happened in RAN3. Does RAN3 want that a CU indicates L3 measured result to a DU, then the DU decides whether to perform early synchronization.

RAN2 understand early synchronization is also applied to L3 measurement based LTM.

RAN2 also understand for L3 measurement based LTM, that the CU decides early synchronization according to the received L3 measured results, and indicates that decision with the corresponding beam information to a DU, then the DU will trigger early synchronization.

[AT127bis][111][MOB] (Vodafone)

Scope: To prepare response LS to RAN3.

Intended outcome: Response LS in R2-2409373. Email approval.

Deadline: 19:00pm Thursday

Session chair cannot find out related email. Will be double checked in main session.

### 7.4.2	Control plane corrections

Including stage 2 and control plane (e.g. RRC) corrections.  Minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note RRC CR rapporteur’s summary and suggestion (based on the submitted contributions) may be provided. Agreed changes may be merged into a single or multiple CRs containing similar issues.

1. On RAN3 LS (R2-2407924)

R2-2408432	Discussion for Replying LS on inter-DU Early TA acquisition and Preamble Resources for PDCCH order in LTM	Google	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: For inter-DU LTM, shared preamble resources as agreed upon by RAN2 can be formed by combining SSB index, PRACH mask index, and preamble index.

Will respond that preamble index list is sufficient.

[Ericsson]: Thinks it’s not possible PRACH resource is shared among multiple UEs. Because source DU cannot understand the container that early UL sync information is included. [Samsung]: Although it is OCTET string container, the source DU can understand it. It has been already allowed in many cases. [Huawei]: RAN2 agreed with sharing the preamble. Don’t think we need further improvements. [Google]: What RAN2 agreed was that for the same preamble, it can be shared among multiple UEs in different PRACH occasions. [Vivo, LG]: Agree with Google. [Nokia, Huawei, Ericsson]: Think preamble index would be sufficient, however the second question is more important to respond to RAN3, and think SSB positions in burst would be required. [Xiaomi]: Additional question is whether it should be preamble index or preamble index list? Why do we need preamble index list for a given UE? [Vivo]: With preamble index list, the source DU can assign the different preambles to the different UEs. We can respond the preamble index list is ok but it’s good to add original RAN2 intention also in the response LS. [Ericsson]: Do not see any need for further explanation.

Proposal 2: The 'SSB Positions In Burst' parameter is essential for the source gNB-DU to calculate the RA-RNTI, which in turn allows for associating a TA value with a UE when RACH occasion differentiation is required.

Agreed. Also we will indicate it should not be included in RRC container, e.g. via separate IE outside of container between DU and CU.

[Samsung]: To be more accurate, it should be included in early UL sync configuration. [Ericsson]: It should not be included in RRC container, otherwise we need NBC change.

Proposal 3: RAN2 to reply RAN3 considering the draft reply LS in R2-2408433.

[AT127bis][102][MOB] (Google)

Scope: To prepare response LS to RAN3 (including the discussion on detailed wordings)

Intended outcome: Response LS in R2-2409352 to be approved. Email approval.

Deadline: Until 10:00am Thursday. => Completed.

R2-2409352	Reply LS on Early TA acquisition for the inter-DU scenario	Google	LS out	Rel-18	NR_Mob_enh2-Core	To:RAN3	Cc:RAN1

Source should be changed to RAN2.

Approved in R2-2409376 with the change above.

R2-2408433	[Draft] Reply LS on Early TA acquisition for the inter-DU scenario	Google	LS out	Rel-18	NR_Mob_enh2-Core	To:RAN3	Cc:RAN1

Nokia (P1: R2-2408945)

Samsung (P4: R2-2408436)

Proposal 4: The candidate gNB-DU needs provide SFN, where the preamble is transmitted, along with the RA-RNTI and the TA value for a specific UE.

Noted.

[Google]: NW implementation can handle this issue, e.g. NW may not assign the same preamble to multiple UEs within the enough time period. [Ericsson]: Agree with Google.

2. Corrections that 1) “agreed” and 2) “not pursued” by RRC CR rapporteur

[AT127bis][103][MOB] (Ericsson)

Scope: 1) To discuss and capture P1 in R2-2407994, P3 in R2-2408751, P1 and P4 in R2-2408819 in RRC CR rapporteur’s miscellaneous CR. 2) To discuss P2 and P3 in R2-2407994, P1 in R2-2408126, P2 in R2-2408751, P4 in R2-2408945, P4 in R2-2408258.

Intended outcome: 1) RRC CR in R2-2409353 to be in principle agreed. Email approval. 2) Discussion summary in R2-2409354. Comeback in Thursday CB session. Ericsson will inform offline schedule later.

Deadline: 1) Thursday 10:00am, 2) Thursday CB session. => Completed. Note CR will be further checked via short email discussion.

R2-2409354	Summary of [AT127bis][103][MOB]	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1	UE release/clear all current common radio configurations except for PCell upon subsequent CPAC execution.

Proposal 2	Confirm that For the SCPAC scenario where initial serving SN/PSCell is not part of SCPAC , the Network should ensure that the servingCellSetID is set to a unique value different than the security-cell-set-ID of the candidates.

Proposal 3	Instead of using “MN format” it is spelled out what this does mean in the RRC specification.

Proposal 4	Clarify in the Annex B2 that the RRCReconfiguration shall not be sent unprotected if it includes the LTM-Config.

Proposal 5	There is no need to clarify the handling of the srb-Identity in 5.3.5.13.8 (Proposal 2 of R2-2407994).

Proposal 6	The field description of securityCellSetId is moved from ConditionalReconfiguration field descriptions to CondReconfigToAddMod field descriptions.

Proposal 7	The current restriction for the s-measure for the LTM measurements is deleted and is captured in the chairman’s notes a RAN2 understanding that UE is not required to perform LTM measurements in case the serving cell measurements is above the s-measure threshold.

Proposal 8	The size of sk-CounterConfigToReleaseList-r18 and sk-CounterConfigToAddModList-r18 is not changed.

Proposal 9	Current procedural text already limits the attempts of LTM fast recovery to one. No specification impact

Proposal 10 (modified)	RAN2 to send an LS to RAN4 in order to clarify the meaning of the capability ltm-FastProcessingConfig-r18, i.e. should the UE know the number of serving cells included in candidate configurations in advance for fast RRC processing, or it doesn’t matter ?

All proposals above are agreed.

[POST127bis][113][MOB] (Ericsson)

Scope: To prepare merged RRC CR (including all agreements made this meeting).

Intended outcome: RRC CR in R2-2409353 (or R2-2409375 if new tdoc number is needed) to be agreed in principle

Deadline: Short email discussion

[POST127bis][114][MOB] (MediaTek)

Scope: To prepare LS on the capability ltm-FastProcessingConfig-r18 to RAN4.

Intended outcome: LS in R2-2409377 to be approved.

Deadline: Short email discussion

3. Stage 2 corrections

[AT127bis][104][MOB] (Nokia)

Scope: To discuss need of corrections on P3 in R2-2408945, R2-2408452, R2-2408632, and capture agreeable corrections in 38.300 miscellaneous CR.

Intended outcome: 38.300 CR in R2-2409355 to be in principle agreed. Discussion summary in R2-2409356 if needed. Email approval.

Deadline: Thursday 10:00am. => Completed.

R2-2409356	Report from [AT127b][104][MOB] (Nokia)	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Stage 2 changes in R2-2408945 are postponed to next meeting (i.e. more time is needed to decide how to clarify the measurement reporting requirement for steps 4a and 4b in Figure 9.2.3.5.2-1).

Proposal 2: The change proposed in R2-2408452 for section 9.2.3.5.1 of TS 38.300 is not pursued at RAN2#127bis.

Proposal 3: Changes in R2-2408452 suggested for section 9.2.3.5.2 are agreed with the following modification: procedural text for step 5 under Figure 9.2.3.5.2-1 clarifies it is either L1 or L3 measurement report that can be used for LTM (to avoid the suggestion the UE cannot perform both types of measurements).

Proposal 4: Similar changes are pursued also in section 9.2.3.5.1 General and 3.2 Definitions of TS 38.300 to reflect L3 measurement report can be also used for LTM.

Proposal 5: The change suggested in R2-2408632 is not agreed.

All proposals above are agreed.

R2-2409355	Misc state 2 corrections for LTM mobility	Vodafone, Ericsson, Nokia, ZTE	CR	Rel-18	38.300	18.3.0	0911	1	F	NR_Mob_enh2-Core

Agreed in principle.

[AT127bis][105][MOB] (ZTE)

Scope: To discuss correction on R2-2408522 (including need of correction).

Intended outcome: 37.340 CR in R2-2409357 to be in principle agreed. Discussion summary in R2-2409358 if needed. Email approval.

Deadline: Thursday 10:00am. => Completed.

R2-2409358	Report of [AT127bis][105][MOB] (ZTE)	ZTE	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: To adopt the 1st change in R2-2408522 with the following modifications:

-	Update the definition of subsequent CPAC as follows:

Subsequent Conditional PSCell Addition or Change (subsequent CPAC): a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA which is not implicitly released by the UE upon (conditional or non-conditional) PSCell addition, (conditional or non-conditional) PSCell change, (conditional or non-conditional) PCell change or SCG release.

-	Only add “(conditional or non-conditional)” for the definition text of subsequent CPAC, but no change for other texts.

Proposal 2: To adopt the 2nd change in R2-2408522.

Proposal 3: The 3rd change in R2-2408522 is not pursued at RAN2#127bis meeting.

All proposals above are agreed.

R2-2409357	Corrections for mobility enhancements in stage-2	ZTE Corporation, Ericsson	CR	Rel-18	37.340	18.3.0	0404	1	F	NR_Mob_enh2-Core

Agreed in principle.

4. Change 2 in R2-2408819 (Ericsson)

A further issue to address is that the configured grant for LTM gives the opportunity to the network to configure an SSB mapping so UE can select one of those SSB to use the special configured grant for LTM. However, in LTM the TCI state is a mandatory field in the LTM cell switch MAC CE and thus there is no case for the UE on when an SSB needs to be selected. According to this, it should be clarified that the field that configures the SSB mapping shall not be configured in case of LTM.

Not pursued.

[Huawei]: It is needed for NW to have more flexibility, e.g. to map CG occasions according to SSB indexes. [Samsung]: It was agreed during WI and the current spec is aligned with RAN2 agreement.

5. P1 in R2-2407993 (CATT)

Proposal 1: On how to identify the power control parameters after LTM cell switch, RAN2 down selects between the following two solutions:

-	Solution 1: clarify in the field description of ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State that it refers to the uplink-PowerControlToAddModList configured in the SpCell of the candidate configuration;

-	Solution 2: introduce uplink-PowerControlToAddModList-r18 and uplink-PowerControlToReleaseList-r18 in LTM-TCI-Info to align RAN1 LS, the ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State refer to the power control parameters in LTM-TCI-Info;

TPs for Solution 1 and Solution 2 are provided in Annex 1 and Annex 2 respectively.

Solution 1 (clarify in the field description of ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State that it refers to the uplink-PowerControlToAddModList configured in the SpCell of the candidate configuration) is agreed. RRC CR rapporteur will capture it.

[Huawei]: Understand solution1 would be simpler. [MediaTek, CATT]: It’s for power control after LTM cell switch, thus the UE already decoded candidate configuration.

6. P1, P2, P6, P7 in R2-2408436 (Samsung)

Proposal 1: RAN2 to confirm that the intra-SN subsequent CPAC and CHO can be configured together.

Not pursued.

[Huawei]: Think it was discussed in Rel-17. [CATT]: Agree with Huawei. It is captured in the stage 2 spec to cover both proposal 1 and 2. [Samsung]: Confirms if it is clear in the stage 2, we don’t need any RRC correction. [Apple]: Want to check if RAN2 has common understanding that Rel-17 option is also applied to Rel-18 intra-SN subsequent CPAC. It should be. [Ericsson]: Confirms it’s well covered in RAN3 spec, e.g. interface between SN and MN.

Proposal 2: To avoid the mismatch in CHO confguration between UE and MN and potential RLF, when intra-SN subsequent CPAC and CHO are configured together, consider the below options:

a. UE keeps the CHO configuration after subsequent CPAC execution stored in SCG VarConditionalReconfig (preferred).

b.Introduce new inter-node signalling from SN to MN to inform that subsequent CPAC execution is performed for intra-SN CPC.

Proposal 6: RAN2 confirms that if UE supports all ltm-MCG-NRDC-r18, ltm-MCG-IntraFreq-r18 and ltm-SCG-IntraFreq-r18, the UE is also able to support the configuration of LTM for MCG and LTM for SCG simultaneously.

RAN2 understands that if UE supports all ltm-MCG-NRDC-r18, ltm-MCG-IntraFreq-r18 and ltm-SCG-IntraFreq-r18, the UE is also able to support the configuration of LTM for MCG and LTM for SCG simultaneously.

[MediaTek, Ericsson, Apple, Huawei]: Although the sentence is correct, don’t see the real need to capture in the spec. It is NW configuration perspective. [Xiaomi]: It would be good to capture it in the minutes.

Proposal 7: RAN2 confirms that inter-frequency LTM will be supported between frequency bands which indicate supports of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18 respectively.

Not pursued.

[Huawei]: Don’t see the real difference between the original text and proposed text.

7. P1 and P2 in R2-2408523 (ZTE)

Proposal 1: 	From RAN2 perspective, CG-Config and CG-CandidateList are related to different configurations of intra-SN subsequent CPAC in MN format, but they are not required to must be sent together in the same SN Modification Required XnAP message.

Will indicate from RAN2 perspective, CG-Config and CG-CandidateList are different part of UE configurations, and not required to be sent together in the same SN Modification Required XnAP message.

[Qualcomm]: It would be good to add the clarification CG-Config is for initial execution condition and CG-CandidateList is for subsequent execution conditions. [ZTE, Nokia]: No need to indicate it to RAN3 since it is very clear to the current RRC.

Proposal 2: 	RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately. It requires some modification or clarification in the current stage-2 procedure. The detailed message and procedure can be up to RAN3 discussion.

[Samsung]: We don’t need to discuss proposal 2. First response is now clear that both RRC messages are not required to be sent together. [Nokia]: Think the first sentence “RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately.” is still helpful to RAN3. [Qualcomm] Agree with Nokia.

Will indicate RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately.

Proposal 3: 	RAN2 to approve the reply LS in the Annex.

[AT127bis][106][MOB] (ZTE)

Scope: To prepare the response LS to RAN3.

Intended outcome: Response LS in R2-2409359 to be approved. Email approval.

Deadline: Thursday 10:00am. => Completed.

R2-2409359	Reply LS on intra-SN SCPAC in MN format	LS out		To:RAN3

Approved.

8. P1 in R2-2408751 (Nokia)

Proposal 1 : The procedure text changes introduced in TS38.330 V 18.3.0 related to update of VarConditionalReconfig during SCPAC execution is modified to support subsequent CPA .  The description for subsequentCondExecutionCond updated to enable the above. [ Ref Annexure 1]

Not pursued.

[CATT]: Think MN Initiated Inter-SN SCPAC with subsequent CPA is supported but it is put into the legacy conditional execution IE. That’s align with what is specified in the field description. [Huawei]: Prefer not to change anything, it was discussed before and finally we agreed with what we have now. [Ericsson]: Agree with CATT and prefer keeping the agreement. [Nokia]: Then the consequence is that we now have not real intended MN initiated inter-SN SCPAC with subsequent CPA.

9. P3 in R2-2408819 (Ericsson)

“Another issue is the signaling of the fields subcarrierSpacing, absoluteFrequencyPointA, cyclicPrefix, and resourceType which, according to the current ASN.1 structure would need to be repeated for each of the CSI-RS resource configured in case the TRS configuration is present in LTM-TCI-Info. However, at least in some cases these field can be received already from the SpCellConfig and thus there is no need to signal them again.”

Not pursued.

[Huawei]: Proposed TP seems not really correct. Motivation is also not so clear. [Xiaomi]: Think it is not an essential correction. It’s for optimization. Prefer not to change anything. [Nokia]: Agree with Xiaomi. Think the gain is not large with the change.

10. P1 and P2 in R2-2408258 (MediaTek)

Proposal 1: RAN2 to clarify the understanding of maxNumberJointTCI-AcrossCells in FG45-3a:

-	Understanding 1: The volume of this component includes the legacy TCI state indicated by FG23-1-1 UnifiedJointTCI-r17, which represent a total maximum number of activated TCI states that UE can support.

-	Understanding 2: The volume of this component only includes LTM TCI states indicated in LTM-TCI-info. The value of UnifiedJointTCI-r17 is not taken into account.

Proposal 2: No matter which understanding, add a note in TS 38.306 for clarification.

Noted. We will wait for RAN1 decision.

[Nokia]: Understand RAN1 is clarifying it further. [Ericsson]: Has same understanding as Nokia.

11. P1 in R2-2409137 (Huawei)

Proposal 1: The UE that indicates support of interFreqL1-MeasConfig-r18 (R1 45-1a) can indicate that it can do L1 inter-frequency measurements of SSB in the serving BC, by indicating a new per UE capability interFreqL1-OnlyInBC-r18. defined e.g., as

| interFreqL1-OnlyInBC-r18 When included, indicates that, for each BC in which the UE indicates support of interFreqL1-MeasConfig-r18, the UE only supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) in the BC i.e., the UE does not support L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) out of the BC. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Proposal 1 is agreed.

LS is sent to RAN4 to inform the agreement.

We can start preparing the corresponding TP for next meeting.

[Session chair]: What is correct understanding on interFreqL1-MeasConfig-r18?

- Interpretation#1: While the UE reports the capability in BC {A+B+C}, and if the UE is in band A+B, any L1 inter-frequency measurement on the band A, B, and C are supported.

- Interpretation#2: While the UE reports the capability in BC {A+B+C}, and if the UE is in band A+B, any L1 inter-frequency measurement on any band, e.g. band D, E and F (even out of indicated BC) are supported.

[Huawei]: Understand interpretation#2, and with the new capability, want to achieve interpretation#1. [Huawei]: After offline discussion, the original RAN2 agreement is interpretation#2 and it is what the spec specifies. [Qualcomm]: If other companies want an additional capability to allow interpretation#1, it’s ok, but good to check with RAN4. [MediaTek, Apple]: Support the proposal. [Samsung]: To the previous RAN4 LS, it seems interpretation#2 is natural consequence, however for the need of additional capability bit, we may need to check with RAN4 (e.g. whether it is ok per UE). [Session chair]: Seems ok to send a LS to RAN4 to get confirmation. Question is whether we’re going to wait for their response LS before implementing it to RRC. [Qualcomm]: We should wait. [Apple]: We can inform it to RAN4, however we don’t have to wait for RAN4 response LS considering RAN4 couldn’t make decision and left to RAN2 to make a conclusion last time. [Ericsson]: Agree with Apple. We can start TP preparation. [Samsung]: Companies can internally check with their RAN4. We don’t wait for their response LS.

[AT127bis][107][MOB] (Huawei)

Scope: To prepare the LS to RAN4.

Intended outcome: Response LS in R2-2409360 to be approved. Email approval.

Deadline: Thursday 10:00am. => Completed.

R2-2409360	LS on UE capabilities for inter-frequency L1 measurements for LTM	LS out		To:RAN1, RAN4

Approved.

R2-2407993	Issue on the power control parameters after LTM cell switch	CATT	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407994	Miscellaneous corrections for SCPAC	CATT	discussion	Rel-18	NR_Mob_enh2-Core

R2-2408126	Correction on s-Measure for L1 measurement	vivo	draftCR	Rel-18	38.331	18.3.0	NR_Mob_enh2-Core

R2-2408258	Discussion and corrections on RRC and UE capability for LTM	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

R2-2408436	Remaining issues for Rel18 Mobility Enhancements	Samsung	discussion

R2-2408452	Misc state 2 corrections for LTM mobility	Vodafone, Ericsson	CR	Rel-18	38.300	18.3.0	0911	-	F	NR_Mob_enh2-Core

R2-2408522	Corrections for mobility enhancements in stage-2	ZTE Corporation, Ericsson	CR	Rel-18	37.340	18.3.0	0404	-	F	NR_Mob_enh2-Core

R2-2408632	TP for Stage 2 coexistence case	Lenovo	discussion	Rel-18	38.300	NR_Mob_enh2-Core

R2-2408751	SCPAC Corrections	Nokia	discussion

R2-2408818	Misc RRC corrections for feMob	Ericsson (Rapporteur)	CR	Rel-18	38.331	18.3.0	5037	-	F	NR_Mob_Ph4-Core	Late

R2-2408819	RRC corrections for feMob	Ericsson	CR	Rel-18	38.331	18.3.0	5038	-	F	NR_Mob_Ph4-Core

R2-2408820	Summary of RRC proposals for feMob	Ericsson (Rapporteur)	discussion	Rel-18	NR_Mob_Ph4-Core	Late

R2-2408945	Miscellaneous Rel-18 LTM Aspects and Corrections	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

R2-2409137	LTM UE capabilities for inter-frequency L1 measurements	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

R2-2408678	Clarification on conditional, DAPS and LTM candidate configurations in LTM configuration	Google	CR	Rel-18	38.331	18.3.0	5028	-	F	NR_Mob_enh-Core, NR_Mob_enh2-Core

=> Withdrawn

R2-2408756	Clarification on LTM capabilities	Google	CR	Rel-18	38.306	18.3.0	1183	-	F	NR_redcap-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core

=> Withdrawn

R2-2408325	Correction to LTM fast RRC processing	MediaTek Inc.	CR	Rel-18	38.331	18.3.0	4999	-	F	NR_Mob_enh2-Core

=> Withdrawn

### 7.4.3	User plane corrections

Including user plane (e.g. MAC) corrections. Minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note MAC CR rapporteur’s summary and suggestion (based on the submitted contributions) may be provided. Agreed changes may be merged into a single or multiple CRs containing similar issues.

1. Corrections that 1) “agreed” and 2) “not pursued” by MAC CR rapporteur

[AT127bis][108][MOB] (Huawei)

Scope: 1) To discuss and capture R2-2409138, R2-2408297, R2-2408784, and 1st change in R2-2408755 in MAC CR rapporteur’s miscellaneous CR. 2) To discuss 2nd change in R2-2408755 and R2-2408817.

Intended outcome: 1) MAC CR in R2-2409361 to be in principle agreed. Email approval. 2) Discussion summary in R2-2409362. Comeback in Thursday CB session.

Deadline: 1) Thursday 10:00am, 2) Thursday CB session. => Completed.

R2-2409362	Summary of [AT127b][108][MOB] (Huawei)	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

Noted. All related corrections are directly reflected into CR in R2-2409361.

R2-2409361	Miscellaneous corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	1968	1	F	NR_Mob_enh2-Core

Agreed in principle.

2. R2-2408262 (MediaTek)

Not pursued.

[Samsung]: To current spec, it handles a case when the UE receives PDCCH order while the UE performs RACH on the serving cell. For this case, it is RACH to the candidate cell. Think the scenario is not realistic. [Apple]: NW is already aware of that the UE performs RACH to the candidate cell, there is no reason NW sends PDCCH for BWP switching during the RACH procedure. [LG]: Share the view with Samsung and Apple. Also, the current spec specifies for the case UE initiated RACH procedure, which doesn’t happen for LTM. [Vivo]: Agree with Samsung. [MediaTek]: Do we need to send LS to RAN1 to ask this situation? [LG, Samsung]: No need to send LS to RAN1. The corresponding parts were discussed and decided in RAN2.

3. R2-2409139 (Huawei)

Noted. We will wait for the conclusion regarding whether LTM is supported by RedCap UEs.

[LG]: Question is whether LTM coexists with RedCap. It was discussed in the main session but not concluded yet. We should wait for that conclusion first. [Ericsson]: Confirms LG’s understanding.

4. R2-2408875 (Rakuten)

Proposal 1	RACH-less LTM cell switch is considered to be “ongoing” when the UE receives a PDCCH order from the serving gNB-DU.

Proposal 2	UE shall return to C-DRX cycle if the RACH-less LTM cell switch command is not received within a predefined time.

Not pursued.

[LG]: Think there is no issue/problem with the current spec. It’s optimization. [MediaTek]: Agree with LG. [Vivo]: It’s optimization. Not essential correction.

R2-2408262	Correction on BWP switching during PRACH triggered for LTM candidate cells	MediaTek Inc.	CR	Rel-18	38.321	18.3.0	1931	-	F	NR_Mob_enh2-Core

R2-2408297	MAC corrections for LTM	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_Mob_enh2-Core

R2-2408755	Clarification to 38.321 on LTM Cell Switch UP Aspect	ZTE Corporation	CR	Rel-18	38.321	18.3.0	1947	-	F	NR_Mob_enh2-Core

R2-2408784	Clarification on random access procedure initialization for early UL synchronization	Google	CR	Rel-18	38.321	18.3.0	1950	-	F	NR_Mob_enh2-Core

R2-2408817	MAC corrections for feMob	Ericsson	CR	Rel-18	38.321	18.3.0	1955	-	F	NR_Mob_Ph4-Core

R2-2408875	Coexistence of UE C-DRX and RACH-less LTM	Rakuten Mobile, Inc	discussion	Rel-18

R2-2409138	Miscellaneous corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	1968	-	F	NR_Mob_enh2-Core

R2-2409139	Correction to LTM MAC CE based CFRA with MSG1 repetition	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	1969	-	F	NR_Mob_enh2-Core, NR_redcap-Core, NR_cov_enh2-Core, NR_redcap_enh-Core

R2-2409140	MAC CR rapporteur summary	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core	Late

## 7.15	NR Sidelink evolution

(NR_SL_enh2; leading WG: RAN1; REL-18; WID: RP-230077)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.15.1	Organizational

Including incoming LSs and rapporteur inputs.

### 7.15.2	Corrections

Including corrections to all specifications.Minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note RRC and MAC CR rapporteurs’ summary and suggestion (based on the submitted contributions) may be provided. Agreed changes may be merged into a single or multiple CRs containing similar issues.

[Thursday CB session]:

[Session chair]: Do we need short email discussion for MAC/RRC rapporteur CRs? [LG, OPPO]: Not needed.

R2-2408643	Discussion on MAC correction for resource selection of MCSt	LG Electronics Inc.	discussion	NR_SL_enh2

Proposal 1. RAN2 agrees with the correction in R2-2406316 [2] regarding the resource selection procedure of the initial tramission loop for MCSt.

Agreed.

Proposal 2. RAN2 decides whether the UE uses the Re-TX loop or not during the resource selection procedure for MCSt.

[Qualcomm]: It was agreed that retransmission resource is not selected among MCSt if PSFCH is configured for transmission. The change may be only applicable to blind retransmission case. [CATT]: Agree with Qualcomm. That’s one of the reasons why we don’t put any in Re-TX loop. If we put any in Re-TX loop, we need more accurate based on number of retransmissions, number of consecutive resources and whether PSFCH is configured for transmissions. [Vivo]: Agree with CATT and Qualcomm. [OPPO]: We need to consider 2+2 MCSt resources for 1 TBs. [CATT]: Understand according to RAN1 LS, we will make sure single TB works but for multiple TBs, it’s done based on best efforts. [LG]: Re-TX loop still needs for single TB transmission, e.g. 2 slots for MCSt and if MAC selects total 4 transmissions (including initial and retransmissions), then we still need Re-TX loop.

Re-TX loop will be considered.

We need more accurate conditions when Re-TX loop will be used.

Detailed wordings can be discussed offline until Thursday CB session.

[AT127bis][101][V2X/SL] (LG)

Scope: To prepare detailed wordings for Re-TX loop.

Intended outcome: MAC CR in R2-2409351 to be agreed in principle. LG will inform the offline discussion schedule later.

Deadline: Comeback in Thursday CB session. => Completed.

Proposal 3. If it is decided that the Re-TX loop is used when selecting the resources for MCSt in the proposal 2, RAN2 can discuss the solutions (solution 2/3/4) listed in observation 2 for TP regarding the UE behavior in HARQ retransmission loop during the resource selection for MCSt.

R2-2409351	MAC correction for resource selection of MCSt	LG	CR	Rel-18	38.321	18.3.0	1973	-	F	NR_SL_enh2

Agreed in principle.

R2-2407969	Correction on selection of resources for MCSt	CATT, CICTCI	CR	Rel-18	38.321	18.3.0	1930	-	F	NR_SL_enh2

R2-2408105	Discussion on MCSt correction	vivo	discussion	Rel-18	NR_SL_enh2

R2-2408538	Clarification on single and multiple carrier selection	ZTE Corporation	discussion	Rel-18	38.470	NR_SL_enh2

Proposal 1	Regarding single/multiple carriers operation, it is suggested to discuss which understanding is the correct UE behaviour:

Understanding 1: When determining single or multiple carrier operation, MAC entity will take allowed carriers of all RLC channels into consideration, i.e. determine single/multiple carriers operation per UE. And adopt the TP-1.

Understanding 2:MAC entity determines single or multiple carrier operation per LCH. And adopt the TP-2.

Understanding 1 is confirmed. No need of further correction.

[OPPO]: Understand understanding 1 is correct. Note it is aligned with LTE V2X CA. [LG]: Have same understanding as OPPO. Also think there is no need of further correction for understanding 1.

Proposal 2	RAN2 is suggested to discuss which options is adopted for carrier selection for IUC MAC CEs:

	Option1: revert previous RAN2 agreement, adopt following modified version (without considering CBR based carrier selection) and TP-3:

The MAC entity shall select a carrier on which a Sidelink Inter-UE Coordination Request was received as a carrier for transmission of a Sidelink Inter-UE Coordination Information MAC CE.

The MAC entity shall select a carrier where the IUC resource set is required as a carrier for transmission of a Sidelink Inter-UE Coordination Information Request MAC CE.

	Option2: revert previous RAN2 agreement, adopt following modified version(following CBR based carrier selection) and TP-4:

The MAC entity shall select a carrier on which a Sidelink Inter-UE Coordination Request was received as a carrier for transmission of a Sidelink Inter-UE Coordination Information MAC CE, if the CBR of the carrier is higher than the CBR threshold associated to the priority of Sidelink Inter-UE Coordination Information MAC CE.

The MAC entity shall select a carrier where the IUC resource set is required as a carrier for transmission of a Sidelink Inter-UE Coordination Information Request MAC CE, if the CBR of the carrier is higher than the CBR threshold associated to the priority of Sidelink Inter-UE Coordination Request MAC CE.

Option1 is agreed.

TP3 is baseline and detailed wordings can be further discussed offline. We will have normative text for all IUC-REQ triggered IUC information transmission, condition triggered IUC information transmission and IUC REQ transmission.

Corresponding CR will be submitted next meeting.

[Apple]: IUC does not support cross-carrier operation. Prefer option1. [LG]: Option2 is correct based on our previous discussion. No need of additional correction. Single carrier for IUC will be selected based on the existing note, then MAC will see CBR condition based on MAC CE priority, if passed, the UE can select the carrier otherwise no transmission. [OPPO]: Note was added as compromised solution in the previous RAN2 meeting. Assume it will be difficult if a company intends to put it to normative text. [Qualcomm]: In the past discussion, we have RAN2 priority that is used for LCP and RAN1 priority that is used for resource allocations. What priority is used for CBR condition check for carrier selection? [LG]: Understand priority “1” is used for carrier selection. [Apple, Qualcomm]: Understand RAN1 priority is used. [Nokia]: Understand that IUC needs to wait for data until IUC transmission. [LG]: Condition-based IUC information or IUC request MAC CEs have different characteristics than SL CSI reporting and IUC request triggered IUC information. [Session chair]: Ask if a single carrier or multiple carriers can be considered for condition-based IUC information or IUC request MAC CE. [LG]: Only single carrier is a candidate carrier based on the existing note. [Session chair]: If single carrier is considered as candidate carrier, why do we have different approach between IUC REQ triggered IUC information transmission and condition triggered IUC information transmission/IUC REQ transmission? [ZTE, Apple, Qualcomm]: Agree with session chair and prefer option1. [Nokia]: Between normative text and note, prefer having a normative text for all cases.

R2-2407972	Co-configuration of random/partial-sensing resource selection and Co-Ex	OPPO	CR	Rel-18	38.331	18.3.0	4976	-	F	NR_SL_enh2

Agreed in principle.

R2-2408637	Correction on Co-channel coexistence for LTE sidelink and NR sidelink	LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	1942	-	F	NR_SL_enh2

Agreed in principle.

R2-2408252	TP for SL enhancemen in TS 38.321	NEC  Corporation	discussion	Rel-18

Proposal 1	It is suggested to remove the condition for SL-U and adopt the corresponding TP1.

Not pursed.

[LG]: Don’t consider this correction is needed. It is already covered by if conditions, e.g. “if multiple carrier frequencies are configured”.

Proposal 2	In section 5.31.2, it is suggested to change the IE sl-lbt-FailureInstanceMaxCount and sl-lbt-FailureDetectionTimer into sl-LBT-FailureInstanceMaxCount and sl-LBT-FailureDetectionTimer to keep align with TS 38.331

Proposal 3	It is suggested to adopt the corresponding TP2.

Proposal 2 is agreed.

MAC CR Rapporteur take the corresponding changes.

[Huawei]: “SL-LBT-FailureRecoveryConfig” should be “sl-LBT-FailureRecoveryConfig” [NEC]: Understand “SL-LBT-FailureRecoveryConfig” is correct to the current RRC. [Huawei]: Suggest to check it offline.

R2-2408363	Correction on carrier selection for SL evolution	Huawei, HiSilicon, LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	1936	-	F	NR_SL_enh2-Core

[Apple]: We can simply correct “a carrier” to “the carrier”. [Session chair]: Is triggering of TX carrier (re)selection for MAC CE transmission in general covered by other places? [LG]: Agrees with session chair, but good to have it.

Noted.

R2-2408638	Correction on MCSt	LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	1943	-	F	NR_SL_enh2

Move “larger than 1” to between “the” and “number” in both NOTE3Ae and NOTE7.

Remove check for radio access network should be removed in the cover page.

Consider Ericsson’s comment in both NOTE3Ae and NOTE7.

Agreed in principle with the changes above, in R2-2409371.

[Ericsson]: We need to add indicate to where, e.g. “L1 or lower layer”. [Apple]: “larger than 1” gives some confusion. [Huawei]: Prefer moving “larger than 1” between “the” and “number”. It cannot be removed since it is the key point. [Apple]: Check for radio access network should be removed in the cover page.

R2-2408686	Correction to MAC on LCH carrier mapping for sidelink	Ericsson	CR	Rel-18	38.321	18.3.0	1946	-	F	NR_SL_enh2

Noted. If needed, it can be discussed in positioning session.

[Huawei]: Prefer the existing sentence. No need for further correction. And if Ericsson still wants this change, it should be discussed in the positioning session. [Huawei]: This change does not impact on SL part, it impacts on SL PRS part, that’s why we think it needs to be discussed there.

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2407914	LS to RAN2 for on-demand SSB SCell operation (R1-2407438; contact: LGE)	RAN1	LS in	Rel-19	Netw_Energy_NR_enh-Core	To:RAN2

Noted.

R2-2408559	Updated workplan for Rel-19 network energy savings WI	Rapporteurs (Ericsson, Apple)	Work Plan	Rel-19	Netw_Energy_NR_enh-Core

Noted.

### 8.5.2	On-demand SSB SCell operation

Remaining issues from RAN2#127 and details of L3 measurements in case 1 and case 2 (taking measurement configuration and RAN1 agreement of including SSB periodicity in MAC-CE).

1. OD-SSB indication by RRC

P1 in R2-2408560 (Apple)

Proposal 1: Don’t support RRC reconfiguration of OD-SSB state (activation/deactivation), i.e. RRC based indication is only used to indicate the initial activation/deactivation state of OD-SSB.

P2 in R2-2409086 (Nokia)

Proposal 2 (modified): No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

P2 in R2-2409086 is agreed.

[Xiaomi]: UE configuration should be supported. Understand it is restricted only to initial status. [Ericsson]: RRC reconfiguration can be sent anytime and wonders how to specify do not support non-initial status. Will we capture a restriction to the field description? [Samsung]: Agree with Ericsson. Question is if we agree with P2 in Nokia, is there any specification impact? If not, support Nokia proposal. [Lenovo]: Agree with Nokia proposal. In fact, it would be same. [OPPO]: With P2 from Nokia, two kinds of status may be overlapped and the UE may be confused. [IDC]: Agree with Nokia proposal.

2. OD-SSB indication by MAC CE

P2 in R2-2408560 (Apple)

Proposal 2 (modified): Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

P2 in R2-2408560 is agreed.

P4 in R2-2408085 (CMCC)

Proposal 4: To support Scenario#2A, the new MAC CE should include the SCell activation/deactivation indication with OD-SSB related indication.

[LG]: Support Apple proposal. [Ericsson]: What’s separate MAC CE in P2 in Apple means? [Samsung]: It means for SCell activation/deactivation, only legacy MAC CE for SCell activation/deactivation is used. When OD-SSB indication is changed, new MAC CE for OD-SSB indication will be used. [Xiaomi]: Ok with Apple proposal. Question is raised if legacy MAC CE and OD-SSB MAC CE are sent together or in separate in time domain? [Apple]: Clarifies if OD-SSB MAC CE is only received, then it is scenario 2. If OD-SSB MAC CE and SCell activation/deactivation are received together, it’s scenario 2a. [CATT]: Understand one MAC CE is required for scenario 2a. [LG]: We are discussing DL MAC CE transmissions. There is no difficulty to send them together. [Fujitsu]: Prefer introducing a new MAC CE for scenario 2a. [Nokia]: Don’t think RAN2 has any restriction regarding whether to send both MAC CEs together. [OPPO]: Agree with Apple. Signalling overhead difference may be just 1 byte, which is not enough motivation to introduce a new MAC CE. [Session chairs]: Let’s check companies views. 9 companies prefer counting on legacy MAC CE and 6 companies prefer introducing new MAC CE.

P3 in R2-2408560 (Apple)

Proposal 3: The new OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Will discuss it later.

NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

[IDC]: It is more stage 3 issue. Ok with the proposal if we need to indicate OD-SSB indication for multiple SCells simultaneously. [LG]: Would like to see it in stage 3 discussion. [Nokia]: We can at least try to agree NW should be able to indicate OD-SSB indication for multiple SCells simultaneously. [Samsung, Apple]: Agree with Nokia. [Xiaomi]: Wonder if we need to send a LS to RAN4. [Samsung]: We already allow to send multiple MAC CEs via singe MAC PDU. Do not see any need to send a LS to RAN4. [Apple]: Would like to have more time to wait for other WG progress.

3. L3 RRM Framework

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

Need more time to think of.

P5 in R2-2408560 (Apple)

Proposal 5: Introduce a configuration within MeasObjectNR to include at least a list of SMTC for different OD-SSB patterns (e.g. different periodicity), which the UE applies upon reception of the OD-SSB indication.

P4 in R2-2408237 (Huawei)

Proposal 4: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the OD-SSB pattern is informed to the UE for performing L3 serving cell measurements. No need to modify the legacy SMTC or make up a dedicated SMTC for serving cell measurements.

[OPPO]: Considering OD-SSB transmission is dynamically changed and OD-SSB transmission periodicity is also dynamically changed, think it will be difficult to follow SMTC configuration. [Ericsson]: Many RAN1 things are not clear, we may need to wait for further RAN1 progress. [CATT] Support the Huawei proposal. [Nokia]: On Apple proposal, if multiple SMTCs are activated, how will impact to RAN4. On Huawei’s proposal, does it mean the UE will not measure OD-SSB? [Huawei]: Proposal is to measure OD-SSB based on OD-SSB transmission pattern (not based on SMTC configuration). [Nokia]: Then, with the UE autonomous measurements (out of SMTC window), will it impact on RAN4? [Qualcomm]: Original OD-SSB purpose was quick synchronization, now it is extending to L1/L3 RRM purpose. It is not clear how much gains will be brought? Maybe simplest one is just to measure based on periodic SSB. [Apple]: RAN1 already agreed L1/3 RRM based on OD-SSB, and left the detailed design to RAN2/4. On Nokia’s question, RAN2 originally introduced SMTC before RAN4. On Huawei proposal, it is controversial to the current RAN1/4 spec (The UE measurement is not required out of SMTC window). [Huawei]: For OD-SSB, it is so dynamic (with dynamic starting point A). Even with list of SMTC configurations, not sure how it works. [Xiaomi]: Reluctant to change traditional measurement configuration, however think single SMTC would be enough (instead of list of SMTCs). [ZTE]: Would like to have more time to think of.

4. Whether always-on SSB and/or OD-SSB are measured in case 2?

P1 in R2-2408295 (Samsung)

Proposal 1: For case #2, RAN2 to discuss which option is followed by UE for RRM measurements of SCell configured with on demand SSB.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Wait for next meeting.

[ZTE]: Understand option1. [OPPO]: Understand option1 can or cannot work case by case. Understand option1 works if offset or frequency is different. Otherwise we need to count on option2. [LG]: Why we need to measure both SSBs. Think option1 is baseline. [Ericsson]: Think this discussion up to RAN1/4. [Nokia]: Agree with Ericsson. Understand option2. Measure whatever sent by NW. [Huawei]: Agree with issue raised by OPPO. Understand OD-SSB and periodic SSB are on the same frequency, where SCell is located. [Apple]: Do not understand why we need to consider a case that OD-SSB and periodical SSB are located in the different frequencies. [Session chair]: Do we need to send a LS to RAN4 to ask our questions to help our future progress? [OPPO]: Ok to send a LS to RAN4. [ZTE]: If we have any consensus that OD-SSB and periodical SSB are located in the same frequency in RAN2 point of view, we can send a LS to RAN4. Otherwise, since it is anyway currently under RAN4 discussion, there is no point to send a LS. [Nokia]: Understand it is more reasonable to assume OD-SSB and periodic SSB are located in the different frequencies. [Nokia]: What’s RAN2 impact from this question? [Apple]: If different frequencies, it will bring more complications to support option2.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

R2-2407973	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407990	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408004	On-demand SSB SCell operation in connected mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408045	Further consideration for on-demand SSB	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408085	Discussion on on-demand SSB	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408100	Discussion on on-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408172	Discussion on on-demand SSB Scell operation	Spreadtrum Communications	discussion	Rel-19

R2-2408189	Discussion on on-demand SSB	Xiaomi	discussion

R2-2408237	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408295	On-demand SSB SCell Operation	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408321	Issues on the procedure of on-demand SSB SCell operation	Lenovo	discussion	Rel-18

R2-2408385	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408409	Discussion on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408560	Further Discussion on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408612	Further discussion on On-demand SSB for SCell	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408712	On-demand SSB Scell operation discussion	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408770	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408785	Discussion on OD-SSB	Qualcomm Incorporated	discussion

R2-2408904	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409053	Discussion on on-demand SSB SCell operation	Sharp	discussion

R2-2409086	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

### 8.5.3	On-demand SIB1

Procedures and signaling method for case 2, e.g. which SIB includes UL WUS config for cell A and NES cell, details of UL WUS configurations, access restriction for legacy UEs, details of the UE behaviors related to OD-SIB1 request and failure case, etc.

0. General

We will inherit all agreements made during SI phase to WI phase.

1. Whether a NES cell can include neighboring NES cell’s WUS configuration?

- Yes (P1 in R2-2408786, Qualcomm)

A NES cell can include neighbouring NES cell’s WUS configuration.

[Huawei]: Do not see real need. [Qualcomm]: UE power consumption should be also considered, otherwise the UE always need to go cell A before attempting any neighbouring NES cell. [Vivo]: Support the proposal. It will be included into OSI and it can operate based on OD manner. [Samsung]: Support the proposal. Do not see any additional specification efforts. [Rakuten]: Agree with Samsung and support the proposal. [Vodafone]: Understand any cell including SIB1 can include WUS configuration for any cell (including its own). [Apple]: Support the proposal. We still need to differentiate NES cell or cell A. [ZTE]: Understand a NES cell includes neighbouring cell’s WUS configuration once SIB1 is included. [Nokia]: For more detailed conditions, when to allow or when not to allow is more stage 3 discussion. Support the proposal.

2. NES Cell’s WUS configuration via existing SIB or new SIB?

- Option1: New SIB (P1 in R2-2408561, Apple)

- Option2: Existing SIB (P7 in R2-2408641, Lenovo) (P1 in R2-2407991, CATT)

NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

[Nokia]: It should not be in SIB1. No strong view between SIB3/4 and new SIB. SIB1 mainly includes serving cell configuration. [Vodafone]: If new SIB is used, can it delay the new SIB acquisition compared to existing SIB? [ZTE]: Agree with Nokia. Prefer new SIB. Using new SIB doesn’t mean more delay. [Huawei]: Ok with new SIB, however for its own WUS configuration, it may overlap with the existing IEs included in SIB1. [Nokia]: For its own WUS configuration, it only transmits when it is updated. There is no reason to transmit it regularly. Thus, including it to new SIB is ok. [Samsung, Lenovo, Xiaomi]: Do not really see the need to have different design for each case. [Xiaomi]: It is not clear how works if OD-SIB1 and OD-SIB coexist. [Huawei]: For its own WUS configuration, why we should include all overlapping parameters in both SIB1 and new SIB?

3. Failure of OD-SIB1 acquisition

P14, P15 in R2-2407952 (Xiaomi)

Proposal 14 (modified): In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

Agreed.

Proposal 15 (modified): The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

Agreed.

[IDC]: Support the proposal. Understand P15 was already agreed. [LG]: There is no specification regarding until the UE will attempt to get OD-SIB1. It is left to UE implementation. [Nokia]: Agree with LG regarding what the current spec specifies. [CATT, Xiaomi]: Think proposal 15 makes sense. Otherwise it will impact (or delay) for cell camping to the appropriate cell.

P6 in R2-2408461 (Google)

Proposal 5: If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred.

Wait for more RAN1 progress.

[LG, Apple]: Since it was not agreed even for RACH failure case, we can put this case as FFS too. [Xiaomi]: RAN1 is still discussing how it works. Good to wait for more RAN1 progress. [Nokia]: Window or how long to attempt SIB1 acquisition is RAN2 decision. Do not understand why RAN1 is discussing it. [Nokia]: Wonders if window is needed. To current spec, it is up to UE implementation how long to wait for SIB acquisition. [Apple]: Assume it is too late not to have window since it has been progressed in RAN1 during two meetings. [LG]: Agree with Nokia. [Session chair]: Are we ok with Nokia proposal? 6-7 companies are not ok. Then let’s wait for more RAN1 progress.

4. Cell reselection after OD-SIB1 acquisition failure

P8 in R2-2408410 (Fujitsu)

Proposal 8 (modified): The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

Agreed.

5. Broadcast status for OD-SIB1

P2, P3 in R2-2408294 (Samsung)

Proposal 2 (modified): A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

Agreed.

Proposal 3 (modified): If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Agreed.

[Samsung]: It is for mimicking the existing OD-SIB. [Lenovo]: Agree with proposals. [ZTE]: For proposal2, it is possible that NW broadcast SIB1 due to other UE’s REQ. For proposal3, do not need additional UE behaviour, the UE still requests OD-SIB1 and wait for SIB1. [Nokia]: Agree with proposals. [Vodafone]: Understand on-going SIB1 transmission can be aware by MIB. [CATT]: For proposal3, understand RAN1 is discussing an indication by MIB. [Huawei]: Agree with proposal 2, for proposal 3, we can wait for RAN1. [Google]: Assuming the indication is included in MIB, the UE is required to reacquire MIB once OD-SIB1 REQ is sent. [Ericsson]: For proposal3, it is required to the UE, not up to UE implementation.

6. Cell barring

P2, P3 in R2-2409087 (Nokia)

Proposal 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1.

Proposal 3: NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

Proposal 2 and proposal 3 are agreed.

No separate barring bit for R19 NES UEs is introduced if proposal 2 works according to RAN1.

[Ericsson]: Ok with proposal 2. Proposal 3 sounds optimization. [Nokia]: For proposal 3, yes it can be considered as optimization, however WID indicates to minimize legacy UE impacts. Last RAN plenary, it was also mentioned. [Session chair]: let’s see companies views. [Vodafone, Samsung]: Support the proposal 3. It would be desirable to minimize the legacy UE impact.

P5 in R2-2408772 (OPPO)

Proposal 5: For cell (re)selection, use cellbarred in MIB to prevent the legacy UE from accessing the on-demand SIB1 cell.

[Session chair]: It was agreed the we will bar legacy UEs. [Nokia, ZTE, Vodafone, Lenovo]: It is for the case when SIB1 is not periodically broadcast. If a NES UE requests OD-SIB1, the NW broadcast SIB1 periodically, then legacy UE should be able to camp.

P2 in R2-2408454 (Vodafone)

Proposal 2 (modified): The UE bars the cell as of today, but once reselecting to the Cell A and receiving the WUS, it would declare the SIB1-less cell as not barred anymore.

Let’s have more time to think of.

Related RAN2 agreement: If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

[Vodafone]: The previous agreement was for the case when the UE has valid WUS configuration. But if it is before the UE camping on cell A, what will happen? [Nokia]: Scenario is valid. [Lenovo]: Agree with Vodafone and Nokia. [LG]: Although the scenario is valid, don’t think anything new is needed. The current spec specifies “The UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds”. It’s may. [Apple]: Agree with LG. There are two kinds of barring case when the UE shall (e.g. when barring bit is set) and the UE may… (e.g. when the UE fails to acquire SIB1) blabla… Since “may” is specified, the UE still can attempt cell (re)selection even before 300s is not expired if the UE has WUS configuration from cell A now.

7. WUS Configuration update between cell A and NES cell

P5 in R2-2408410 (Fujitsu)

Proposal 5: RAN2 to discuss the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell.

- RAN2 will not start the discussion on this issue unless we’re asked to do that by other WG, e.g. RAN3.

[Fujitsu]: Had some offline discussion with companies. Some companies think it does not happen often and if happened, it can be left to NW implementation. And some companies think it’s more RAN3 issue, so RAN2 doesn’t need to start the discussion. [Vivo]: Consider it is RAN3 issue and it can be handled by NW implementation. [Qualcomm]: Understand it is transparent to the UE.

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

R2-2407952	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

R2-2407991	Consideration on on-demand SIB1 issues	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408005	Remaining issues of on-demand SIB1 in idle and inactive mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408086	Discussion on on-demand SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408101	Discussion on  on-demand SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408173	Discussion on on-demand SIB1 for NES	Spreadtrum Communications	discussion	Rel-19

R2-2408241	Discussion on on-demand SIB1	Sharp	discussion	Rel-19

R2-2408274	Discussion on on-demand SIB1 for NES	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408294	On-demand SIB1	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408386	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2407041

R2-2408410	Discussion on on-demand SIB1 procedure for NES	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408445	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408454	Barring and SIB1-less case 2	Vodafone, Deutsche Telekom	discussion	Rel-19

R2-2408461	Open issues for the on-demand SIB1 transmission	Google	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408561	Discussion on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408600	Discussion on On-demand SIB1 signalling details	NEC	discussion

R2-2408606	Discussion on Ondemand-SIB1	KDDI Corporation	discussion

R2-2408641	Remaining essential issues	Lenovo	discussion	Netw_Energy_NR_enh-Core

R2-2408677	On-demand SIB1 for Idle/Inactive mode UEs	III	discussion	Netw_Energy_NR_enh

R2-2408713	On-demand SIB1 for IDLE/INACTIVE UEs	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408771	On-demand transmission of SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408772	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408786	Discussion on OD-SIB1	Qualcomm Incorporated	discussion

R2-2408865	Detection and access of NES cells with OD-SIB1	Rakuten Mobile, Inc	discussion	Rel-19

R2-2408866	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc	discussion	Rel-19

R2-2408902	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409087	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409144	Discussion on On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2409156	On-demand SIB1 for IDLE/INACTIVE UEs	CEWiT	discussion

### 8.5.4	Adaptation of common signal/channel transmissions

Adapation of paging occasions in time domain (including the email discussion[POST127][109][NES]), legacy UE impact (including barring aspect for paging adaptation), configuration aspect for paging adaptation, RAN2 spec impact and solutions for RACH adaptation and SSB (with consideration of RAN1 progress), etc.

1. Paging adaptation

R2-2407974	Report for [POST127][109][NES] (OPPO)	OPPO	report	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Select option-b as baseline for R19 NES paging enhancement.

Proposal 2: R2 further discuss the impact of RRC configuration and the impact of legacy UE for option-b.

P9 in R2-2408238 (Huawei)

Proposal 9: Values ranges of nAndPagingFrameOffset and ns can be extended to allow configuring fewer PFs per paging cycle and more POs per PF.

P4 in R2-2408562 (Apple)

Proposal 4: For option-b, RAN2 is kindly suggested to only consider extending Ns value to 8 and 16 (i.e. no need to introduce large Ns vale like 32 or 64) while N value can be extended with 32, 64, 128, and 256.

P1 in R2-2408102 (Vivo)

Proposal 1: RAN2 to first study the signalling overhead issue and spec impact of option-b, and then compare it with option-a for further down-selection.

[AT127bis][110][NES] (OPPO)

Scope: To calculate signalling overheads between option-a and option-b. To check companies’ opinion if P1 in R2-2407974 is ok or if option-a should be considered instead.

Intended outcome: Discussion summary in R2-2409372. OPPO will announce the offline discussion schedule via email.

Deadline: Comebacks in Thursday CB session. => Completed.

R2-2409372	Summary of [110]	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Select option-b as baseline for R19 NES paging enhancement.

Agreed.

Proposal 2 (modified): R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

Agreed.

Proposal 3 (modified): Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

Agreed.

[Nokia, Samsung, OPPO]: Wonders why NW to allow legacy UE and R19 UEs to co-ex in the same PF/PO (or the other way around) is difficult. Understand it should be supported easily. [Samsung]: It is up to NW configuration. Assume we will not have any restriction for one or the other way around. [OPPO]: We can try to make conclusion that it’s allowed based on NW configuration. [ZTE]: Understand if NW configures both R18 and R19 paging configurations, co-ex is allowed. [Ericsson]: Question is if we have long enough PO offset not to overlap with a PF/PO that legacy UE monitors. [Lenovo]: Understand from NW point of view, it is more desirable to overlap between PFs/POs for legacy UEs and PFs/POs for R19 NES UEs. Otherwise it is not good for NW energy saving. [Samsung]: Current max PO’s offset length covers 16 frames. [Ericsson]: Want to double check if the current offset value is enough for P3.

2. Whether to bar legacy UEs?

- Yes: P3 in R2-2408006 (ZTE)

- No: P8 in R2-2408962 (Ericsson)

No.

3. Whether allow Rel-19 PF overlapping with legacy PF?

P3a in R2-2408562 (Apple)

Proposal 3a (modified): RAN2 confirm that it allows Rel-19 PF to overlap with legacy PF.

[Session chair]: Was already discussed above.

4. Whether Rel-19 UE only monitors Rel-19 configured PO?

P3b in R2-2408562 (Apple)

Proposal 3b (modified): Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

Agreed.

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

R2-2407992	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408006	Further consideration on paging occasion adaptation	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408102	Discussion on adaptation of common signal transmissions	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408136	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408149	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408190	Discussion on common signal adaptation	Xiaomi	discussion

R2-2408238	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408296	Adaptation of common signal channel transmissions	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408322	Discussion on the adaptation transmissions for NES operation	Lenovo	discussion	Rel-19

R2-2408562	Further discussion on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408601	PRACH and Paging adaptation	NEC	discussion

R2-2408684	Discussion on paging adaptation scheme for NES cell	ITRI	discussion	Netw_Energy_NR_enh-Core

R2-2408718	Considerations on adaptation for paging signals	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408794	Discussion on adaptation of common signal/channel transmission	Qualcomm Incorporated	discussion

R2-2408828	Discussion on common signal and channel adaptation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh

R2-2408903	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2408962	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409061	Discussion on RACH adaptation for NES	SHARP Corporation	discussion	Rel-19

R2-2409088	Common signal aspects of NES WI	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409146	Adaptation of Common Signals and Channels for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2409148	Adaptation of common signal/channel transmissions	III	discussion

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-242356)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Including incoming LSs, WI rapporteur inputs, stage 2 running CR, etc.

Additional CR rapporteur:

UE Capability CR rapporteur: CATT

R2-2408595	Important aspects of Rel-19 mobility enhancements WI	Rapporteurs (Apple, China Telecom)	discussion	NR_Mob_Ph4-Core

Noted.

R2-2408052	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	draftCR	Rel-19	37.340	18.3.0	B	NR_Mob_Ph4-Core

Noted.

R2-2408598	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.3.0	B	NR_Mob_Ph4-Core

Noted.

[Apple]: Ask companies to check running CRs and provide comments (if any). [Session Chair]: Plan to endorse them from the next meeting.

### 8.6.2	Inter-CU LTM

Remaining open issues for inter-CU LTM (SA). Further RAN2 spec impacts and details of inter-CU LTM in DC, e.g. signaling design for inter-CU SCG LTM configuration, network interaction on inter-CU SCG LTM which to be added to 37.340, further scenario or issue clarification on the coexistence of intra-MN/inter-MN MCG LTM and inter-SN/intra-SN SCG LTM, etc.

1. LTM fast recovery for inter-CU LTM (P3 in R2-2408860, Samsung)

Proposal 3 (modified): The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

Agreed.

[Apple]: If we start LTM fast recovery discussion after SA3 conclusion, it will be too late. Simplest approach would be for inter-CU case, no LTM fast recovery. [LG]: Ok with proposal 3. [Vivo, Xiaomi]: We can make conclusion it would be good to support it from RAN2 point of view, and informs SA3 by LS. [Lenovo, ZTE]: No need to send LS to SA3. [ZTE, Qualcomm]: Original proposal 3 is enough for now. [NEC]: In R18, if the first HO fails for inter-CU case, then fast recovery is not supported since PDCP is already re-established.

2. R19 Set ID and R18 NoResetID (P1 in R2-2408117, Vivo)

Proposal 1 (modified): For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

Agreed.

[IDC]: Is the ID associated per cell or per RB? [Vivo]: It is similar to Rel-18, ID is linked to serving cell and candidate configuration (basically per cell). [Samsung]: Agree with Vivo. [IDC]: For Rel-18 LTM, there is no case of PDCP re-establishment. Now for Rel-19, inter-CU LTM is supported and some bearer will be PDCP re-established and some are not (dependent on RB type). [MediaTek]: Not sure how it works. In LTM configuration, we don’t have RB configuration. It sounds very complicated to support RB level operation. [Huawei]: Think proposal is ok. Not sure how IDC’s comment is related to the proposal. [OPPO]: Understand IDC’s scenario is when RB type is changed upon inter-CU LTM. Not sure how it impacts on proposal, we can discuss it further later. [CATT]: Agree with OPPO, and we can agree with proposal 1 for now. [Apple]: Ok with proposal 1, we may comeback if real problem. [Qualcomm]: We can agree with proposal 1 for non-DC case.

3. Inter-CU SCG LTM

P1-P8 in R2-2408524, ZTE

Proposal 1:	On top of the current LTM signaling (i.e. LTM-Config IE), the SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId.

[Xiaomi]: WE may use separate IE for Rel-19 inter-SN LTM.

Proposal 2:	Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

•	The candidate and reference configuration for inter-CU SCG LTM can include both MCG and SCG part configurations, i.e. the candidate/reference configuration is modeled as an MN RRCReconfiguration message. It can be up to the NW implementation whether to include the MCG part.

•	The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

•	The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM.

•	The MN can request an SCG reference configuration from any of the involved SNs.

Proposal 3:	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. SSB or CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN via SN addition procedure.

Proposal 4:	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate cells.

Proposal 5:	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, e.g. via MN initiated SN modification procedure. And the source SN responds with the common CSI resource configuration to the MN.

Proposal 6:	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), e.g. via MN initiated SN modification procedure. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

Proposal 7:	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

Proposal 8:	The flow charts and procedures in Figure 1 are taken as baseline for SN initiated inter-CU SCG LTM .

P4 in R2-2408596, Apple

Proposal 4: Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

4. Inter-CU MCG LTM with SCG (P1 in R2-2408758, Ericsson)

RAN2 to confirm that when an inter-CU LTM cell switch is executed, the SCG addition is supported.

5. Inter-MN LTM with intra-SN LTM (P2 in R2-2408596, Apple)

Proposal 2: Support coexistence of inter-MN LTM and intra-SN LTM and no special handling is required.

[AT127bis][112][MOB] (ZTE)

Scope: To discuss P1-P8 in R2-2408524, ZTE, P4 in R2-2408596, Apple, P1 in R2-2408758, Ericsson, and P2 in R2-2408596, Apple. And to make agreeable proposals.

Intended outcome: Discussion summary in R2-2409374. Offline schedule will be announced.

Deadline: Comeback in Friday CB session. => Completed.

R2-2409374	[AT127bis][112][MOB] Inter-CU SCG LTM& Inter-CU MCG LTM with SCG (ZTE)	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

Proposal 2:	Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

•	The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

•	FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

•	The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

•	Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

•	The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

•	The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

•	The MN can request an SCG reference configuration from any of the involved SNs.

Proposal 3 (modified):	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

Proposal 4:	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

Proposal 5:	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

Proposal 6 (modified):	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

Proposal 7:	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

Proposal 8: Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM

Proposal 9: RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

-	inter-MN LTM and intra-SN LTM

-	Inter-SN LTM and intra-MN LTM

All proposals above are agreed.

Postpone the below proposal to next meeting:

Proposal 1(P1 in R2-2408758, Ericsson): RAN2 to confirm that when an inter-CU MCG LTM cell switch is executed, the SCG addition is supported.

[Samsung]: Some agreements will impact on RAN3, should we send LS to RAN3 to inform our agreements? [ZTE]: Agree with Samsung. We discuss what agreements need to be included in the LS and finalize the LS via short email discussion.

[POST127bis][115][MOB] (ZTE)

Scope: To prepare LS to RAN3 to inform RAN2 agreements that impact on RAN3.

Intended outcome: LS in R2-2409378 to be approved.

Deadline: Short email discussion.

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

R2-2409031	Multiple reference configuration for inter-CU LTM	KDDI Corporation, LG Electronics, Nokia, ZTE, Rakuten, KT, CMCC, ETRI, ITL, ITRI, Fujitsu, SK telecom, Uplus	discussion

Noted.

[Session chair]: With the consideration of comparison with the last RAN2 meeting, it seems not much changed in the point of companies’ positions. Hope to understand that we sometimes should make hard decision. Suggest noted without repeating the discussion. [LG]: Acceptable, however please note there may be similar discussion in RAN3 for their perspective.

R2-2407987	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408053	Discussion on inter-CU LTM in DC	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408061	Discussion on remaining issues of inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2408087	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408117	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408193	Discussion on inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408259	Discussion on Inter-CU LTM standalone and DC	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408279	Further discussion on inter-CU LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408292	Discussion on Inter CU LTM 	Lekha Wireless Solutions	discussion	Rel-19

R2-2408319	Discussion on Inter-CU LTM	Lenovo	discussion	Rel-19

R2-2408328	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408498	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408524	Discussion on inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408596	Open items for inter-CU LTM in SA and DC cases	Apple	discussion	NR_Mob_Ph4-Core

R2-2408607	Discussion on inter-CU LTM with DC	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408659	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408682	The Rel-19 ID for subsequent inter-CU LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2408714	LTM for Inter-CU	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2408752	On Inter-CU LTM Open Issues	Nokia	discussion

R2-2408758	DC aspects for inter-CU LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408860	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408867	Initial considerations for inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2408870	LTM in DC scenarios	Rakuten Mobile, Inc	discussion	Rel-19

=> Withdrawn

R2-2408876	LTM in DC scenarios	Rakuten Mobile, Inc	discussion	Rel-19

R2-2408957	Discussion on Inter-CU LTM	InterDigital, Europe, Ltd.	discussion	Rel-19

R2-2408959	Further Discussion to Support the inter -CU LTM	ETRI	discussion	Rel-19

R2-2408966	Inter-gNB LTM	Qualcomm Innovation Center Inc	discussion

R2-2409000	Discussion on Inter-CU LTM 	Kyocera 	discussion	Rel-19

R2-2409009	Discussion on fast LTM recovery support for Rel-19 LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409142	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.3	L1 event triggered measurement reporting

Details on report configuration signaling design associating with the resource configuration, event evaluation and triggering details, need of other condition to trigger measurement report (e.g. need of MR when leaving condition is met, need of event-triggered periodic MR), information and signaling structure of MR MAC CE, UE procedure to transmit MR MAC CE, etc.

1. Event MR when the leaving condition is met?

- Option1: Yes (P4 in R2-2408759, Ericsson)

- Option2: No (P4 in R2-2409096, Samsung)

Yes. It is configurable by NW.

[Samsung]: Concerns too frequent MRs if we have it. [LG]: NW needs to know whether the MR condition is still met or not. Support “Yes” [Nokia]: Support “Yes” and it will be configurable. If NW doesn’t want, NW can configure “No”. [ZTE]: Support “Yes.

2. Event triggered periodic MR?

- Option1: Yes (P5 in R2-2408759, Ericsson)

- Option2: No (P9 in R2-2408783, Huawei)

Yes. It is configurable by NW.

[Ericsson]: Note it is also configurable by NW. So one-shot is also possible by configuration. [LG]: For L3 MR case, it can be used for other purposes in addition to HO. For LTM cell switch purpose, don’t see real need. [OPPO]: Support “No”. [Xiaomi, Nokia, CATT]: Support “Yes”. With L1 measurement, it will provide more reliable decision to the NW. [MediaTek]: If needed, NW can configure R18 semi-persistent MR. Do not see real need of it.

3. Measurement resource configuration?

- Option1: R18 LTM CSI resource configuration (P2 in R2-2408118, Vivo)

- Option2: New R19 LTM CSI resource configuration (P1 in R2-2408150, Fujitsu)

R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

[Huawei]: It is stage 3 issue. We can further discuss in ASN.1 discussion. [Samsung]: Think option1 is baseline. Understand option2 is to extend for a set of CSI resources for measurement. [ZTE]: It is early to make a decision whether R18 IE extends to have CSI resources or we will have a separate new IE for CSI-RS resources. [Apple]: Still want to consider Rel-18 IE is as baseline.

4. Measurement reporting configuration?

- Option1: In R18 LTM-CSI-ReportConfig (P1 in R2-2408118, Vivo)

- Option2: In new R19 LTM CSI Report Configuration (P5 in R2-2408525, ZTE)

R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

[ZTE]: Existing mandatory IEs in R18 LTM CSI reporting configuration is not necessary for event triggered MR. [CATT]: Consider option2 is more flexible. [Samsung]: There are ways to handle existing mandatory IEs to be ignored for event triggered MR. It would be good to include all MR reporting configuration in one place. [Xiaomi]: Max number of MR is mandatory IE, however it may also need for event triggered MR. [MediaTek]: Agree with ZTE. [Apple]: Current CSI reporting structure is choice. There would be no problem to extend CHOICE structure to have event triggered MR configurations. [Huawei]: Option1 and option2 may not be so different. It’s also stage 3 issue.

5. Association between measurement resource configuration and measurement reporting configuration?

- Option1: R18 LTM Way (P10 in R2-2408597, Apple)

- Option2: Introduction of new id to link between them (P6 in R2-2408525, ZTE)

R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

6. Measurement event evaluation in MAC

- Option1: The entire event evaluation procedure is handled by MAC based on the latest L1 measure results reported by L1 (P5 in R2-2407988, CATT)

- Option2: L1 evaluates whether the latest measured result of one candidate beam fulfilled the event or not, and indicate the event evaluation instance to MAC (P5 in R2-2408783, Huawei)

Option1 is agreed.

[Vivo]: Option1 is simpler. RAN1 discussed option2 for MIMO, not for LTM. [Qualcomm]: Agree with option1. Common mechanism between MIMO and LTM was already broken. For MIMO, UCI is used and for LTM, MAC CE is used. [LG]: Not sure how option2 works with further filtering. Option1 is simple and works well. [OPPO, Lenovo]: Option2 is already used for BFD. [Xiaomi]: Prefer option1. For BFD, we don’t have TTT. With option2, we need more discussion. [Apple]: Good to make decision this meeting. Can accept option1.

7. TTT operation per beam or per cell (for a candidate cell)?

- Option1: Per beam (P1 in R2-2408260, MediaTek)

- Option2: Per cell (P2 in R2-2408525, ZTE)

We will make decision next meeting.

[LG]: With the previous RAN2 agreement, TTT per beam should be natural and sufficient. [Huawei]: LTM MR is associated with per CSI-RS set. Why not run TTT per the associated CSI-RS set. [Xiaomi]: The reason why we have TTT is to determine stable beam for MR. It should be per beam. [Nokia]: Don’t see the need of option2, however we may consider per beam or per CSI-RS set. [CATT, Samsung]: We already agreed beam level MR. It should be enough to have TTT per beam. Per CSI-RS set is also a kind of cell level TTT. [Qualcomm]: TTT per cell should be correct since for LTM switch purpose, NW doesn’t care what beam. TTT per CSI-RS resource set may be considered as compromise. [LG]: With option2, it brings immature MR. We need reliable MR. [Ericsson, Lenovo, IDC]: Agree with Qualcomm.

8. TTT operation on current beam changing

- Option1: TTT is not reset (P1 in R2-2408613, NEC)

- Option2: TTT is reset (P4 in R2-2408948, Nokia)

We will make decision next meeting.

[Xiaomi]: It is good to have same approach for both candidate cell and current beam. [CATT]: If we agree TTT is per beam, there is only one current beam. Considering current beam is dynamically changed, which is different than candidate beams, prefers option1. [LG]: If the current beam is changed, that means the changed beam is better than previous current beam. Then it should be more reasonable to reset TTT. [ZTE]: If the current beam is changed and considers better, but if it still meets the event condition, which means the candidate beam is better than the current beam. In the case, it is reasonable to report MR (instead of delaying MR because of TTT reset). [Apple]: Prefer to make decision for both current beam and candidate beam. We can make conclusion for both of them next meeting. [IDC]: Agree with ZTE.

9. TTT operation (P4 in R2-2408597, Apple)

- Option1: TTT (like L3 event triggered MR)

- Option2: Timer/window + counter (like R19 MIMO)

Option1 is agreed.

[ZTE]: Think it is related to the issue 7. If it is per beam, then it may be natural with option2. If it is per cell, option1 sounds better. [Samsung, Nokia, CATT]: Since it is agreed with option1 in the issue 6, it is more reasonable with option1.

10. Consideration of mTRP (P6-1, P6-2 in R2-2409096, Samsung)

- Option1: sTRP is prioritized in Rel-19

- Option2: mTRP is also considered in Rel-19

We can wait for RAN1 decision.

[Xiaomi]: mTRP is already supported in Rel-18 LTM. [Vivo]: We also support mTRP in Rel-19. [MediaTek]: RAN1 is discussing this issue. We can wait for RAN1 decision.

11. Measurement RS type alignment (P7 in R2-2409096, Samsung, P1 in R2-2408525, ZTE)

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

- Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement.

- Option 3: NW provides the offset which can be applied one type of RS to evaluate the different type of RS. For example, If the current beam is SSB and the measured beam for the candidate cell is CSI-RS, the UE applies the configured the offset to the measured CSI-RS value and then compare it with the current beam quality i.e. SSB measurement.

- Option 4: Rely on network configuration, i.e. same RS type is configured.

- Option 5 (new option proposed by CATT): Which type to be used in measurement will be configured by NW. Then the UE only consider the configured RS type in the measurement.

- Option6 (copied from P1, ZTE): The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

Proposal 7: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

Proposal 1: RAN2 confirms the working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). We will decide which option to go next meeting.

[Xiaomi]: Option4 is preferred. [CATT]: Consider which type to be used in measurement will be configured by NW. Then the UE only consider same type RS in the measurement. [Huawei]: Would like to have more time to think of. [Ericsson]: Option4 is enough. It is how works in Rel-18 LTM. [Nokia]: Option4 sounds too restrictive to NW. Better to have more time to think of.

12. Contents of MR MAC CE

12.1 Basic information (P1 in R2-2409033, NTT DCM)

Proposal 1 (modified): The baseline of contents for event-triggered L1 measurement is following;

・Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

・Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

・Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (whether the beam should satisfy the event or not is FFS), N is configured by NW.

Agreed.

[Samsung]: Is N configured by NW? [LG]: N should be configured by NW.

12.2 Additional information

A. Current beam quantity (P2 in R2-2409033, NTT DCM)

Proposal 2 (modified): UE can include the information and quantity of current beam in event-triggered L1 measurement reporting. It should be configurable by NW.

Agreed.

[Ericsson]: Why we need the current beam? [Xiaomi]: It gives more complete picture to NW. [Huawei]: Wonder if N should include this measured result. [Session chair]: That can be further discussed later in the stage 3.

B. Multiple serving beams (up to N) (P13 in R2-2408759, Ericsson)

Proposal 13: The network can configure the UE to report the serving cell in the event-triggered L1 measurement report.

Noted.

[Samsung]: Do not see a need. In event triggered MR, the UE doesn’t distinguish separate beams from the serving cell. UE just measure current beam whatever is. It’s different than Rel-18 LTM. [Apple]: Agree with Samsung. [Huawei]: Assume there is no real difference between Rel-18 LTM and Rel-19 LTM. [OPPO]: Agree with Samsung, Apple. [Xiaomi]: Feel sympathy on the proposal. It could be helpful if supported. [LG]: Assume no additional efforts to support it. [Apple]: For the serving cell, we already have beam management. If needed, we can count on it. [Nokia]: Agree with Samsung and Apple. [Qualcomm]: One thing clear is that we shouldn’t do additional measurements on the serving beams for event triggered MR. [MediaTek]: If multiple serving beams are included in LTM resource configuration, then anyway the UE will perform measurement on each serving beam. [Ericsson]: Proposal is made based on the assumption NW configures serving cell into a candidate configuration also.

C. Candidate beams (P12 in R2-2407988, CATT) (P6-7 in R2-2408499, OPPO)

- Candidate beams (i.e., beams associated to the same event configuration) which have fulfilled the event condition are included in the MAC CE measurement report according to limitation on the max number of candidate beams configured by NW.

- Other candidate beams (even though they haven’t fulfilled the event condition) can be included according to limitation on the max number of candidate beams configured by NW.

[Session chair]: It was already included as FFS in the previous discussion, so let’s skip it.

13. UE procedure to send a MR MAC CE (P6 in R2-2408759, Ericsson)

Proposal 6 (modified): The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

Agreed.

[Huawei]: Ok with SR, but not sure with BSR procedure. [Xiaomi]: Agree with Huawei. BSR doesn’t include the size of MAC CE. [Ericsson]: Clarifies the intention is to include some information of MR MAC CE size, otherwise if MR MAC CE size is variable and large, the NW should assign a grant to guarantee max size.

14. NW can configure a dedicated SR configuration?

Yes (P7 in R2-2408759, Ericsson)

NW can configure a dedicated SR configuration for MR MAC CE transmission.

15. LCP priority (P16b in R2-2407988, CATT)

Proposal 16b (modified): Measurement report MAC CE should be transmitted with a high LCP priority than data LCH. FFS the exact priority.

To be discussed in stage 3.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

R2-2407988	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408021	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408054	Discussion on L1 event triggered measurement reporting	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408062	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2408070	Discussion on L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408118	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408150	Discussions on event triggered L1 measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408164	Discussion on L1 event triggered measurement reporting	Spreadtrum Communications	discussion	Rel-19

R2-2408260	Measurement reporting discussion	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408280	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408320	L1 Measurement enhancements	Lenovo	discussion	Rel-19

R2-2408329	Discussion on issues for L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408384	Discussion on LTM measurement reporting configuration	Baicells	discussion

R2-2408455	L1 measurement event configuration and reporting	Panasonic	discussion	Rel-19

R2-2408492	Discussion on L1 event triggered measurement	Jio	discussion

R2-2408499	Open issues for event triggered  L1 measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408525	Discussion on L1 event triggered measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408597	LTM event triggered measurement reporting	Apple	discussion	NR_Mob_Ph4-Core

R2-2408613	Details of L1 event triggered measurement reporting	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408759	Event definition and MAC CE content for L1 event-triggered measurements	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408783	Discussion on event-triggered measurement report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408871	Event based L1 measurements triggered LTM candidate cell addition/release	Rakuten Mobile, Inc	discussion	Rel-19

R2-2408948	On L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2408965	Measurement enhancements for LTM	Qualcomm Incorporated	discussion

R2-2408996	L1 measurement reporting procedures for LTM enhancements 	Kyocera 	discussion	Rel-19

R2-2409033	Discussion on Event-triggered L1 measurement reporting	NTT DOCOMO, INC.	discussion	Rel-19

R2-2409065	Event LTM report 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

R2-2409096	Support of Event Triggered L1 Measurement Reporting	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409105	Discussion on event triggered L1 measurement reporting	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409127	Discussion on L1 event triggered measurement reporting	KDDI Corporation	discussion

R2-2409192	Event triggered L1 measurement reporting for LTM	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.4	Conditional intra-CU LTM

RAN2 spec impact and high-level solution, i.e. additional new aspect on top of intra-CU LTM.

R2-2407989	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Conditional LTM Preparation phase

Proposal 1: Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

Agreed.

Proposal 2 (modified): Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

Agreed.

[Lenovo]: For CHO, two conditions can be configured. What about C-LTM? [Ericsson]: We first L1 measurement is considered as baseline then we should see whether L3 measurement can be used.

Proposal 3a: To support subsequent conditional LTM, the execution conditions provided to UE includes the execution conditions for initial conditional LTM and the execution conditions for subsequent conditional LTM.

Noted.

[Lenovo]: Do not like to support subsequent C-LTM. [Nokia]: Note in Rel-18 LTM, we don’t specify the wording “subsequent LTM” in the spec anymore. That’s why proposal 3a sounds confusing.

Proposal 3b: Source cell and each candidate cell provides its own execution condition for conditional LTM.

Agreed.

[Apple]: Can we have single execution condition for all subsequent C-LTM?

Proposal 3c (modified):  It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

Agreed.

[Nokia]: Considering L3 measurement can be used for C-LTM, it can be CU.

Conditional LTM Early sync phase

Proposal 4 (modified): RACH-less Conditional intra-CU LTM is supported.

Agreed.

[Lenovo, Qualcomm, LG, Xiaomi, Nokia, Samsung, Huawei]: Support the proposal.

Proposal 5 (modified): RACH based conditional intra-CU LTM is supported.

Agreed.

[Apple]: It should be baseline.

Proposal 6 (modified): UE based TA measurement mechanism is supported for conditional intra-CU LTM.

Agreed.

Proposal 7a: PDCCH ordered early TA acquisition is supported for conditional LTM.

Agreed.

Proposal 7b: On how NW can determine candidate cells on which to perform early TA acquisition, down selection between the following options is needed,

-	Option 1: UE needs to be configured with legacy L3 measurement reporting or LTM L1 measurement reporting if conditional LTM is configured

-	Option 2: UE requests NW to trigger PDCCH ordered early TA on certain candidate cells

Noted.

[IDC]: Neither option1 or option2 is needed. We can count on event triggered MR. [Apple]: Even without event triggered MR, the NW can determine it based on NW implementation.

Proposal 7c: For PDCCH ordered early TA in conditional LTM, a new MAC CE is introduced to deliver the TA value and the corresponding candidate configuration ID to UE before the triggering of conditional LTM execution.

Proposal 7d: RAN2 discusses how to maintain the TA value at UE side before conditional LTM execution,

-	Option 1: TAT-like timer is maintained at UE side

-	Option 2: UE determines the TA value as valid if the L1 measurement result change is not above a threshold.

Skipped.

Proposal 8a (modified): Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

Agreed.

Proposal 8b: To perform early candidate TCI State activation/deactivation in conditional LTM, RAN2 down-selects between the following options,

-	Option 1: R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

-	Option 2: UE determines the TCI state to activate based on L1 measurement results without NW involvement. The detailed procedure is up to UE implementation.

Skipped.

Conditional LTM Evaluation and execution phase

Proposal 9: UE evaluates the execution condition for each candidate beam and triggers the LTM cell switch to the corresponding candidate cell when at least one beam of the candidate cell fulfills the associated execution condition.

Noted.

Proposal 10: MAC layer handles the event evaluation and determines to trigger the cell switch for conditional LTM.

Noted.

[Lenovo]: For C-LTM, L1 or RRC may handle it. [Qualcomm]: We already agreed MAC does it for L1 event MR.

Proposal 11: UE selects the beam which fulfils the associated execution condition as the target beam. If more than one beam fulfills the associated execution condition, it is up to UE implementation to select the target beam.

Noted.

[LG]: Why not just apply Rel-18 principle? For instance, if NW indicates beams via early activated TCI state MAC CE. [CATT]: Early activated TCI state MAC CE includes multiple beams so the UE still need to make decision.

Proposal 12: If conditional LTM is configured to UE, RAN2 down-selects between the following options to assist candidate cell to determine the target beam:

-	Option 1: NW determines the potential candidate cells/beams based on the L3 RRM reporting or L1 LTM measurement reporting from UE

-	Option 2: UE determines the potential candidate cells/beams and reports them to the source cell

Skipped.

Proposal 13 (modified): For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

Agreed.

[Samsung]: Ok with CG-based option, however it is not clear how DG-based option works. [Xiaomi]: With early TCI states activation, we may be able to apply Rel-18 principle to DG-based approach. [Apple]: NW does not know when the UE comes to the candidate cell. Not clear how DG-based approach works.

Conditional LTM completion phase

Proposal 14 (modified): The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Agreed.

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

R2-2408055	Discussion on conditional intra-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408063	Discussion on supporting conditional intra-CU LTM	Transsion Holdings	discussion	Rel-19

R2-2408088	Disucssion on conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408119	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408151	Discussion on conditional LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408165	Discussion on conditional intra-CU LTM	Spreadtrum Communications	discussion	Rel-19

R2-2408194	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408261	Initial thinking on conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408281	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408330	Discussion on conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408500	Discussion on conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408507	Considerations for conditional LTM	Panasonic	discussion	Rel-19

R2-2408526	Discussion on conditional intra-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408599	Scoping conditional LTM for Rel-19	Apple	discussion	NR_Mob_Ph4-Core

R2-2408608	Discussion on conditional LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408614	Discussion on conditional intra-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408640	Conditional LTM basics	Lenovo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408724	Conditional Intra-CU LTM	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2408760	Baseline assumptions for conditional LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2408928	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2408960	Discussion on conditional intra-CU LTM	ETRI	discussion	Rel-19

R2-2408964	Conditional intra-gNB LTM	Qualcomm Incorporated	discussion

R2-2408997	Discussion on Conditional LTM	KT Corp.	discussion

R2-2409001	Discussion on Conditional Intra-CU LTM 	Kyocera 	discussion	Rel-19

R2-2409035	Discussion on Conditional intra-CU LTM	NTT DOCOMO, INC.	discussion	Rel-19

R2-2409097	Support of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409106	Discussion on Conditional LTM	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409111	Discussion on Conditional mobility	CEWiT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409143	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409193	Conditional LTM	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core