3GPP TSG-RAN WG2 Meeting #127bis						R2-2409231

Hefei, China, Oct 14th – 18th, 2024

Agenda item:			8.9.3

Source:			NEC

Title:		Draft Report of [AT127bis][301][R19 IoT NTN] Working point for CB-msg3 (NEC)

Document for:	 	Discussion and decision

- Introduction

This is to discuss the following offline discussion:

[AT127bis][301][R19 IoT NTN] Working point for CB-msg3 (NEC)

Scope: Discuss the working point for the CB-msg3 EDT-like mechanism (i.e. which packet loss rate we need to target), e.g. based on the considerations in R2-2409170, R2-2408863 and R2-2408547

Intended outcome: Report of the offline discussion

F2F offline time and location:  Wednesday 2024-10-16 10:30-11:00 (morning coffee break) in Brk1 room

Deadline for rapporteur's summary (in R2-2409231):  Wednesday 2024-10-16 14:00

Discussion

R2-2409170 noted that an equivalent transport block error rate (BLER) of 0.1% is the standard target for satellite communications. For the CB-Msg3 discussion, BLER and Packet Loss Rate (PLR) are equivalent. The proposed working point is a PLR of less than 1% and 0.1% will be ideal.

For the NTN uplink coverage study, only Rel-18 covers the NR NTN evaluation and the coverage improvement target is the LEO-600 scenario, and there is no RAN1 evaluation yet on the link-level performance of the NPUSCH Msg3-EDT.

Q1: Do you agree that achieving a 0.1% BLER is possible for both LEO and GEO scenarios and do we need further verification of the link level performance of Msg3-EDT with RAN1?

Inmarsat: 8 repetitions could achieve the BLER / PLR 1%;

Qualcomm: setting the BLER as 0.1% achievable;

Ericsson: setting the BLER of either 0.1% or 1% is possible

ZTE: PLR/BLER is a user-level metric, which is not a suitable metric for CB-Msg3;

For extremely low PLR, other mechanisms like retransmission/large repetitions should be considered, which will bring in additional resource costs for the system.

Suggest to compare different mechanism around the 40% user load region.

VIVO: For the working point, PLR/BLER is not the only metric, and other system performance metrics should be considered.

ESA: The working point should consider the retransmission impacts.

MTK: Setting BLER as 10% is more reasonable. When comparing the BLER of CB and CF, the CB BLER will be higher.

NEC: The BLER of 0.1% is too ideal, which requires further check from RAN1.

Inmarsat: For CB-Msg3, there is no difference between BLER and PLR. Retransmission will lower the MAC layer throughput.

HW: The RAN1 evaluation assumption is 10% BLER. The throughput is more suitable as the working point metric.

ZTE: Clarify that retransmission is not preferred. BLER/PLR is not a suitable working point metric.

Novamint: Working point for less than 1%.

VIVO: If  BLER/PLR is chosen as the metric, it should be 10%. We can further online check which metric is more suitable as the working point.

Inmarsat: NPRACH target BLER is 1%.

Chair: Suggest to focusing the discussion on a working point where the PLR is lower than 10%.

R2-2409170 doesn't mention the relation between the chosen working point and the retransmission/HARQ.to

R2-2408863 proposes a working point of 1% PLR with up to 2 retransmissions.

R2-2408547 proposes that the working point for SA should be lower than 50% and the working point should follow the formula below,

PLR≤β1/N,

where β is MAC layer performance target and N is transmission times.

Of the companies that have provided performance comparisons between SA, DSA, and CRDSA on the CB-msg3, not all have taken retransmission into account.

Q2: Do you agree that retransmission should be taken into account when selecting the CB-Msg3 working point? If yes, do you agree to use PLR≤β1/N to determine the working point? And how many retransmissions will be feasible?

- Summary

Based on the input from companies, we have the following observations:

Different companies have different opinions on which metric or metrics are appropriate for the CB-Msg3 working point. Proposed metrics include system-level and user-level metrics.

In terms of the user-level metrics, different companies have different target BLERs or PLERs. Achieving 1% or 0.1% BLER is possible with enough repetitions, while 10% BLER is the current link-level evaluation target in RAN1. NPRACH and NPUSH have different L1 designs, and preamble detection could achieve 1% BLER. Contention-based transmission will have a higher BLER compared to contention-free transmission.

Based on the divergent views, we might try to focus the discussion on a working point where the PLR is lower than 10%.

- Reference

[1] R2-2409170	Opertor views on EDT enhancements and CRDSA	Inmarsat, Viasat

[2] R2-2408863	Discussion on throughput and delay performance of slotted ALOHA and diversity slotted ALOHA 	DLR, ESA

[3] R2-2408547	Repetitions and Delay Considerations about SA and DSA	ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot, Toyota ITC