3GPP TSG-RAN WG2 Meeting #127-bis	R2-2409312

# Hefei, China, Oct 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1176 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE capabilities for TCI state indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to RAN1 FG 23-1-1b, the unifiedJointTCI-multiMAC-CE-r17 is defined to indicate the unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management. The field minBeamApplicationTime-r17 indicates the minimum beam application time in Y symbols per SCS. It is reported in per band level, which is applicable for both FR1 and FR2. However, in current RAN2 spec, only one value can be indicated through the minBeamApplicationTime-r17 without differentiation between SCS. This is not aligned with RAN1 feature list. Besides, it is ambiguous which SCS the current capability field refers to. There is same issue with FG 23-10-1b, i.e. unifiedSeparateTCI-multiMAC-CE-r17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Clarify that the current minBeamApplicationTime-r17 is only applicable when there is same SCS for all the CCs of the corresponding band in all the band combinations. 2) Introduce new capability to indicate minimum beam application time per SCS for unifiedJointTCI-multiMAC-CE-r17 and unifiedSeparateTCI-multiMAC-CE-r17 respectively. The new capability is only reported when different SCS are supported for the CCs of the corresponding band. 3) Remove the restriction that minBeamApplicationTime-r17 is only for FR2 for unifiedJointTCI-multiMAC-CE-r17. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: Beam management Inter-operability: - If the network is implemented according to the CR and the UE is not, no inter-operability issue, but the consequences if the CR is not approved remains; - If the UE is implemented according to the CR and the network is not, the legacy network may misunderstand the unifiedJointTCI-multiMAC-CE-r17/unifiedSeparateTCI-multiMAC-CE-r17 is not supported by the UE, when the legacy minimum beam application time is absent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE is not allowed to indicate different minimum beam application time for different SCS, and the network cannot apply minimum beam application time per SCS since absence of UE capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.331 CR 5015 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17 Indicates whether the UE supports DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2. A UE supporting this feature shall also indicate support of ack-NACK-FeedbackForMulticast-r17 and dynamicMulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17 Indicates whether the UE supports DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signalling via DCI format 4_2. A UE supporting this feature shall also indicate support of ack-NACK-FeedbackForSPS-Multicast-r17 and sps-MulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| activeConfiguredGrant-r16 Indicates whether the UE supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. This field includes the following parameters: - maxNumberConfigsPerBWP-r16 indicates the maximum number of configured/active configured grant configurations in a BWP of a serving cell. - maxNumberConfigsAllCC-r16 indicates the maximum number of configured/active configured grant configurations across all serving cells in a MAC entity, and across MCG and SCG in case of NR-DC. The UE can include this feature only if the UE indicates support of either configuredUL-GrantType1 or configuredUL-GrantType1-v1650 and/or configuredUL-GrantType2 or configuredUL-GrantType2-v1650. NOTE: - For all the reported bands in FR1, a same X1 value is reported for maxNumberConfigsAllCC-r16. For all the reported bands in FR2, a same X2 value is reported for maxNumberConfigsAllCC-r16. - The total number of configured/active configured grant configurations across all serving cells in FR1 is no greater than X1. - The total number of configured/active configured grant configurations across all serving cells in FR2 is no greater than X2. - If the CA have some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of configured/active configured grant configurations across all serving cells is no greater than max(X1, X2). | Band | No | N/A | N/A |
| additionalActiveTCI-StatePDCCH Indicates whether the UE supports one additional active TCI-State for control in addition to the supported number of active TCI-States for PDSCH. The UE can include this field only if maxNumberActiveTCI-PerBWP in tci-StatePDSCH is set to n1. Otherwise, the UE does not include this field. | Band | No | N/A | N/A |
| antennaArrayType-r18 Indicates whether the UE supports the RF and RRM requirements with antenna array as specified in TS 38.101-1 [2] clause 6.1J, 7.1J and TS 38.133 [5]. If the field is absent, the RF and RRM requirements with omni-directional antenna applies as specified in TS 38.101-1 [2] clause 6.1J, 7.1J and TS 38.133 [5]. The UE indicating support of this feature shall also indicate support of airToGroundNetwork-r18. This field is only applicable for bands as specified for ATG in clause 5.2J of TS 38.101-1 [2]. | Band | CY | N/A | FR1 only |
| aperiodicBeamReport Indicates whether the UE supports aperiodic 'CRI/RSRP' or 'SSBRI/RSRP' reporting on PUSCH. The UE provides the capability for the band number for which the report is provided (where the measurement is performed). | Band | Yes | N/A | N/A |
| aperiodicCSI-RS-AdditionalBandwidth-r17 Indicates the UE supported TRS bandwidths for fast SCell activation, in addition to 52 RBs, for a 10MHz UE channel bandwidth. This field only applies for the BWPs configured with 52 RBs size and 15kHz SCS, in FDD bands and indicates the values: Value addBW-Set1 indicates 28, 32, 36, 40, 44, 48 RBs. Value addBW-Set2 indicates 32, 36, 40, 44, 48 RBs. The UE can include this feature only if the UE indicates support of aperiodicCSI-RS-FastScellActivation-r17. | Band | No | FDD only | FR1 only |
| aperiodicCSI-RS-FastScellActivation-r17 Indicates whether the UE supports aperiodic CSI-RS for tracking for fast SCell activation, i.e., 1) Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE; 2) Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell. This field includes the following parameters: - maxNumberAperiodicCSI-RS-PerCC-r17 indicates the maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE per CC in a reported band. Value n8 corresponds to 8, n16 corresponds to 16, and so on. - maxNumberAperiodicCSI-RS-AcrossCCs-r17 indicates the maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE across CCs in a reported band. Value n8 corresponds to 8, n16 corresponds to 16, and so on. NOTE: - maxNumberAperiodicCSI-RS-PerCC-r17 and maxNumberAperiodicCSI-RS-AcrossCCs-r17 values refer to the number of RS configurations for fast SCell activation that can be indicated by the MAC CE. - The NZP-CSI-RS configured as RS for tracking for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of CSI-RS and CSI-IM reception for CSI feedback. | Band | No | N/A | N/A |
| aperiodicTRS Indicates whether the UE supports DCI triggering aperiodic TRS associated with periodic TRS. | Band | No | N/A | Yes |
| asymmetricBandwidthCombinationSet Defines the supported asymmetric channel bandwidth combination for the band as defined in the TS 38.101-1 [2]. Field encoded as a bit map, where bit N is set to "1" if UE support asymmetric channel bandwidth combination set N for this band as defined in the TS 38.101-1 [2]. The leading / leftmost bit (bit 0) corresponds to the asymmetric channel bandwidth combination set 1, the next bit corresponds to the asymmetric channel bandwidth combination set 2 and so on. UE shall support asymmetric channel bandwidth combination set 0 if defined for the band in the TS 38.101-1 [2]. If the field is absent, the UE supports asymmetric channel bandwidth combination set 0 if defined for the band in the TS 38.101-1 [2]. | Band | No | N/A | N/A |
| bandNR Defines supported NR frequency band by NR frequency band number, as specified in TS 38.101-1 [2], TS 38.101-2 [3], and TS 38.101-5 [34]. | Band | Yes | N/A | N/A |
| beamCorrespondenceCSI-RS-based-r16 Indicates whether the UE support for beam correspondence based on CSI-RS has the ability to select its uplink beam based on measurement of CSI-RS. If a UE supports beam correspondence based on CSI-RS, then the network can expect the UE to also fulfil Rel-15 beam correspondence requirements. If UE supports neither beamCorrespondenceSSB-based-r16 nor beamCorrespondenceCSI-RS-based-r16, gNB can expect the UE to fulfill beam correspondence based on Rel-15 beam correspondence requirements. | Band | No | TDD only | FR2 only |
| beamCorrespondenceSSB-based-r16 Indicates whether the UE support for beam correspondence based on SSB has the ability to select its uplink beam based on measurement of SSB. If a UE supports beam correspondence based on SSB, then the network can expect the UE to also fulfil Rel-15 beam correspondence requirements. If UE supports neither beamCorrespondenceSSB-based-r16 nor beamCorrespondenceCSI-RS-based-r16, gNB can expect the UE to fulfil beam correspondence based on Rel-15 beam correspondence requirements. | Band | No | TDD only | FR2 only |
| beamCorrespondenceWithoutUL-BeamSweeping Indicates how UE supports FR2 beam correspondence as specified in TS 38.101-2 [3], clause 6.6. The UE that fulfils the beam correspondence requirement without the uplink beam sweeping (as specified in TS 38.101-2 [3], clause 6.6) shall set the field to supported. The UE that fulfils the beam correspondence requirement with the uplink beam sweeping (as specified in TS 38.101-2 [3], clause 6.6) shall not report this field. | Band | Yes | N/A | FR2 only |
| beamManagementSSB-CSI-RS Defines support of SS/PBCH and CSI-RS based RSRP measurements. The capability comprises signalling of - maxNumberSSB-CSI-RS-ResourceOneTx indicates maximum total number of configured one port NZP CSI-RS resources and SS/PBCH blocks that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). On FR2, it is mandatory to report >=8; On FR1, it is mandatory with capability signalling to report >=8. - maxNumberCSI-RS-Resource indicates maximum total number of configured NZP-CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] across all serving cells (see NOTE). It is mandated to report at least n8 for FR1. - maxNumberCSI-RS-ResourceTwoTx indicates maximum total number of two ports NZP CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). - supportedCSI-RS-Density indicates density of one RE per PRB for one port NZP CSI-RS resource for RSRP reporting, if supported. On FR2, it is mandatory to report either "three" or "oneAndThree"; On FR1, it is mandatory with capability signalling to report either "three" or "oneAndThree". - maxNumberAperiodicCSI-RS-Resource indicates maximum number of configured aperiodic CSI-RS resources across all serving cells (see NOTE). For FR1 and FR2, the UE is mandated to report at least n4. NOTE: If the UE sets a value other than n0 in an FR1 band, it shall set that same value in all FR1 bands. If the UE sets a value other than n0 in an FR2 band, it shall set that same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. | Band | Yes | N/A | FD |
| beamReportTiming, beamReportTiming-v1710 Indicates the number of OFDM symbols between the end of the last symbol of SSB/CSI-RS and the start of the first symbol of the transmission channel containing beam report. The UE provides the capability for the band number for which the report is provided (where the measurement is performed). The UE includes this field for each supported sub-carrier spacing. | Band | Yes | N/A | N/A |
| beamSweepingFactorReduction-r18 Indicates whether the UE supports beam sweeping factor reduction for FR2 unknown SCell activation. The capability comprises signalling of - reduceForCellDetection indicates reducing beam sweeping factor for cell detection if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure. - reduceForSSB-L1-RSRP-Meas indicates reducing beam sweeping factor for SSB based L1-RSRP measurement if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure. UE is required to meet the shortened SCell activation delay requirement in TS 38.133 [5] if the feature is supported. | Band | No | TDD only | FR2-1 only |
| beamSwitchTiming, beamSwitchTiming-v1710 Indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission. The number of OFDM symbols is measured from the end of the last symbol containing the indication to the start of the first symbol of CSI-RS. The UE includes this field for each supported sub-carrier spacing. NOTE: beamSwitchTiming of value (sym224 or sym336 for 60kHz and 120kHz SCS, sym896 or sym1344 for 480kHz SCS and sym1792 or sym2688 for 960kHz SCS) will be used to determine UE expectation/behaviour for aperiodic CSI-RS for tracking and latency requirements for L1-RSRP reporting as described in clause 5.1.6.1.1 of TS 38.214 [12], while UE behaviour/assumption regarding before or after beam switch timing is unspecified for measuring AP CSI-RS for CSI acquisition (without trs-Info and without repetition) and for beam management (with repetition 'off'). | Band | No | N/A | FR2 only |
| beamSwitchTiming-r16, beamSwitchTiming-r17 Indicates the minimum number of required OFDM symbols (sym224, sym336 for 60kHz and 120kHz SCS, sym896 or sym1344 for 480kHz SCS and sym1792 or sym2688 for 960kHz SCS) between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition 'ON' if enableBeamSwitchTiming-r16 is configured. For CSI-RS configured with repetition "off", the UE applies beam switch time of sym48 if beamSwitchTiming-r16 is reported and enableBeamSwitchTiming-r16 is configured. For CSI-RS configured without repetition and without trs-info, the UE applies beam switch time of sym48 if beamSwitchTiming-r16 is reported and enableBeamSwitchTiming-r16 is configured. | Band | No | N/A | FR2 only |
| bfd-Relaxation-r17 Indicates whether the UE supports BFD relaxation criteria and requirement as specified in TS 38.133 [5]. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. UE indicating support of this feature shall also indicate support of maxNumberCSI-RS-BFD, maxNumberSSB-BFD and maxNumberCSI-RS-SSB-CBD. | Band | No | N/A | N/A |
| bwp-DiffNumerology Indicates whether the UE supports BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer. Except for SUL, the UE only supports the same numerology for the active UL and DL BWP. For the UE that is capable of this feature but is not indicating supportOfRedCap-r17 nor supportOfERedCap-r18, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For the UE which is a (e)RedCap UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of the CORESET#0 (if configured) and SSB for PCell. For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). | Band | No | N/A | N/A |
| bwp-SameNumerology Indicates whether UE supports BWP adaptation (up to 2/4 BWPs) with the same numerology, via DCI and timer. Except for SUL, the UE only supports the same numerology for the active UL and DL BWP. For the UE that is capable of this feature but is not indicating supportOfRedCap-r17 nor supportOfERedCap-r18, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For the UE which is a (e)RedCap UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of the CORESET#0 (if configured) and SSB for PCell. For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). | Band | No | N/A | N/A |
| bwp-WithoutRestriction Indicates support of BWP operation without bandwidth restriction. The Bandwidth restriction in terms of DL BWP for PCell and PSCell means that the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of CORESET #0 (if configured) and SSB. For SCell(s), it means that the bandwidth of DL BWP may not include SSB. | Band | No | N/A | N/A |
| cancelOverlappingPUSCH-r16 Indicates whether UE supports the cancellation of the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to. If the UE supports this feature, the UE needs to report pa-PhaseDiscontinuityImpacts and ul-CancellationSelfCarrier-r16. | Band | No | N/A | N/A |
| cg-PUSCH-UTO-UCI-Ind-r18 Indicates whether the UE supports multiplexing of the unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH. The UE indicating support of this feature shall also indicate support of at least one of configuredUL-GrantType1, configuredUL-GrantType1-v1650, configuredUL-GrantType2, configuredUL-GrantType2-v1650. | Band | No | N/A | N/A |
| cg-SDT-r17 Indicates whether the UE supports transmission of data and/or signalling over allowed radio bearers in RRC_INACTIVE state via configured grant type 1 (i.e. CG-SDT), as specified in TS 38.331 [9]. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. UE supports multiple CG-SDT configurations when a UE indicates the support of this feature and activeConfiguredGrant-r16; otherwise UE only supports one CG-SDT configuration. | Band | No | N/A | N/A |
| cg-SDT-PeriodicityExt-r18 Indicates whether the UE supports to extend the range of CG-SDT periodicities for MO-SDT and/or MT-SDT, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of ra-InsteadCG-SDT-r18. A UE supporting this feature shall also indicate the support of cg-SDT-r17 or mt-CG-SDT-r18. | Band | No | N/A | N/A |
| channelBW-DL-IAB-r16 Indicates whether the IAB-MT supports channel bandwidth of 100 MHz for a given SCS in FR1 for DL or whether the IAB-MT supports channel bandwidth of 200 MHz for a given SCS in FR2 for DL. | Band | No | N/A | N/A |
| channelBW-DL-NCR-r18 Indicates whether the NCR-MT supports channel bandwidth of 100 MHz for a given SCS in FR1 for DL or whether the NCR-MT supports channel bandwidth of 200 MHz for a given SCS in FR2 for DL. | Band | No | N/A | N/A |
| channelBW-UL-IAB-r16 Indicates whether the IAB-MT supports channel bandwidth of 100 MHz for a given SCS in FR1 for UL or whether the IAB-MT supports channel bandwidth of 200 MHz for a given SCS in FR2 for UL. | Band | No | N/A | N/A |
| channelBW-UL-NCR-r18 Indicates whether the NCR-MT supports channel bandwidth of 100 MHz for a given SCS in FR1 for UL or whether the NCR-MT supports channel bandwidth of 200 MHz for a given SCS in FR2 for UL. | Band | No | N/A | N/A |
| channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-DL-IAB-r16. For NCR-MT, to determine whether the NCR-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-DL-NCR-r18. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT and NCR-MT, the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-DL-IAB-r16. To determine whether the NCR-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-DL-NCR-r18. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. For each band, (e)RedCap UEs shall indicate supporting the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. For each band, NTN capable UEs shall indicate the supported channel bandwidths for FR1, taking restrictions in TS 38.101-5 [34] into consideration. This feature is applicable only for FR1 and FR2-1 band, otherwise it is absent. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz for the band combination with other bandwidth combination set than BCS5, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, and supportedBandwidthCombinationSetIntraENDC-v1790. To determine whether the UE supports a channel bandwidth of 90 MHz for the band combination with BCS5, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, supportedAggBW-FR1-r17, and supportedBandwidthCombinationSetIntraENDC-v1790. To determine whether the UE supports a channel bandwidth of 400 MHz, the network may ignore this capability and validate the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the supportedBandwidthDL, and supportedBandwidthCombinationSetIntraENDC-v1790. For serving cell(s) with other channel bandwidths: - If supportedAggBW-FR1-r17 is reported, the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthDL-v1780, supportedMinBandwidthDL, supportedAggBW-FR1-r17, and supportedBandwidthCombinationSetIntraENDC-v1790. - Otherwise, the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthDL/supportedBandwidthDL-v1710, supportedMinBandwidthDL, supportedAggBW-FR2-r17, and supportedBandwidthCombinationSetIntraENDC-v1790. | Band | Yes | N/A | N/A |
| channelBWs-DL-SCS-120kHz-FR2-2-r17 Indicates the UE supported channel bandwidths in DL for the SCS 120kHz. The bits in channelBWs-DL-SCS-120kHz-FR2-2 starting from the leading / leftmost bit indicate 100 and 400MHz. 100 and 400 MHz are mandatory channel bandwidths if the UE supports 120 kHz SCS (i.e. the bit for 100 and 400MHz shall always be set to 1). UE supporting this feature shall also indicate support of dl-FR2-2-SCS-120kHz-r17. NOTE: To determine whether the UE supports a SCS 120kHz for a given band, the network validates the supportedSubCarrierSpacingDL. To determine the supported carrier bandwidths, the network validates the channelBWs-DL-SCS-120kHz-FR2-2-r17, the supportedBandwidthCombinationSet and the supportedBandwidthDL-v1710. | Band | CY | N/A | N/A |
| channelBWs-DL-SCS-480kHz-FR2-2-r17 Indicates the UE supported channel bandwidths in DL for the SCS 480kHz. The bits in channelBWs-DL-SCS-480kHz-FR2-2 starting from the leading / leftmost bit indicate 400, 800 and 1600MHz. 400 MHz is a mandatory channel bandwidth if the UE supports 480 kHz SCS (i.e. the bit for 400MHz shall always be set to 1). UE supporting this feature shall also indicate support of dl-FR2-2-SCS-480kHz-r17. NOTE: To determine whether the UE supports a SCS 480kHz for a given band, the network validates the supportedSubCarrierSpacingDL. To determine the supported carrier bandwidths, the network validates the channelBWs-DL-SCS-480kHz-FR2-2-r17, the supportedBandwidthCombinationSet and supportedBandwidthDL-v1710. | Band | CY | N/A | N/A |
| channelBWs-DL-SCS-960kHz-FR2-2-r17 Indicates the UE supported channel bandwidths in DL for the SCS 960kHz. The bits in channelBWs-DL-SCS-960kHz-FR2-2 starting from the leading / leftmost bit indicate 400, 800,1600 and 2000MHz. 400 MHz is a mandatory channel bandwidth if the UE supports 960 kHz SCS (i.e. the bit for 400MHz shall always be set to 1). UE supporting this feature shall also indicate support of dl-FR2-2-SCS-960kHz-r17. NOTE: To determine whether the UE supports a SCS 960kHz for a given band, the network validates the supportedSubCarrierSpacingDL. To determine the supported carrier bandwidths, the network validates the channelBWs-DL-SCS-960kHz-FR2-2-r17, the supportedBandwidthCombinationSet and supportedBandwidthDL-v1710. | Band | CY | N/A | N/A |
| channelBWs-UL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-UL-IAB-r16. For NCR-MT, to determine whether the NCR-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-UL-NCR-r18. For FR1, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT and NCR-MT, the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-UL-IAB-r16. To determine whether the NCR-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-UL-NCR-r18. For FR1, the leading/leftmost bit in channelBWs-UL-v1590 indicates 70 MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-UL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. For each band, (e)RedCap UEs shall indicate supporting the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. For each band, NTN capable UEs shall indicate the supported channel bandwidths for FR1, taking restrictions in TS 38.101-5 [34] into consideration. This feature is applicable only for FR1 and FR2-1 band, otherwise it is absent. NOTE 1: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingUL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz for the band combination with other bandwidth combination set than BCS5, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, and supportedBandwidthCombinationSetIntraENDC-v1790. To determine whether the UE supports a channel bandwidth of 90 MHz for the band combination with BCS5, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, supportedAggBW-FR1-r17, and supportedBandwidthCombinationSetIntraENDC-v1790. To determine whether the UE supports a channel bandwidth of 400 MHz, the network may ignore this capability and validate the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the supportedBandwidthUL, and supportedBandwidthCombinationSetIntraENDC-v1790. For serving cell(s) with other channel bandwidths: - If supportedAggBW-FR1-r17 is reported, the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthUL-v1780, supportedMinBandwidthUL, supportedAggBW-FR1-r17, and supportedBandwidthCombinationSetIntraENDC-v1790. - Otherwise, the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthUL/supportedBandwidthUL-v1710, supportedMinBandwidthUL, supportedAggBW-FR2-r17, and supportedBandwidthCombinationSetIntraENDC-v1790. NOTE 2: For SRS carrier switching to a PUSCH-less cell, to determine whether the UE supports a channel bandwidth 90MHz/400MHz for SRS configuration, the network validates the supported DL bandwidth, e.g. if the 90MHz is supported by the downlink, the network can configure SRS with 90MHz on the PUSCH-less carrier. SRS carrier switching on PUSCH-less SCells is not supported when channel bandwidth configured for DL is not supported in UL according to channelBWs-UL. | Band | Yes | N/A | N/A |
| channelBWs-UL-SCS-120kHz-FR2-2-r17 Indicates the UE supported channel bandwidths in UL for the SCS 120kHz. The bits in channelBWs-UL-SCS-120kHz-FR2-2 starting from the leading / leftmost bit indicate 100 and 400MHz. 100 and 400 MHz are mandatory channel bandwidths if the UE supports 120 kHz SCS (i.e. the bit for 100 and 400MHz shall always be set to 1). UE supporting this feature shall also indicate support of ul-FR2-2-SCS-120kHz-r17. NOTE: To determine whether the UE supports a SCS 120kHz for a given band, the network validates the supportedSubCarrierSpacingUL. To determine the supported carrier bandwidths, the network validates the channelBWs-UL-SCS-120kHz-FR2-2-r17, the supportedBandwidthCombinationSet and the supportedBandwidthUL-v1710. | Band | CY | N/A | N/A |
| channelBWs-UL-SCS-480kHz-FR2-2-r17 Indicates the UE supported channel bandwidths in UL for the SCS 480kHz. The bits in channelBWs-UL-SCS-480kHz-FR2-2 starting from the leading / leftmost bit indicate 400, 800 and 1600MHz. 400 MHz is a mandatory channel bandwidth if the UE supports 480 kHz SCS (i.e. the bit for 400MHz shall always be set to 1). UE supporting this feature shall also indicate support of ul-FR2-2-SCS-480kHz-r17. NOTE: To determine whether the UE supports a SCS 480kHz for a given band, the network validates the supportedSubCarrierSpacingUL. To determine the supported carrier bandwidths, the network validates the channelBWs-UL-SCS-480kHz-FR2-2-r17, the supportedBandwidthCombinationSet and supportedBandwidthUL-v1710. | Band | CY | N/A | N/A |
| channelBWs-UL-SCS-960kHz-FR2-2-r17 Indicates the UE supported channel bandwidths in UL for the SCS 960kHz. The bits in channelBWs-UL-SCS-960kHz-FR2-2 starting from the leading / leftmost bit indicate 400, 800, 1600 and 2000MHz. 400 MHz is a mandatory channel bandwidth if the UE supports 960 kHz SCS (i.e. the bit for 400MHz shall always be set to 1). UE supporting this feature shall also indicate support of ul-FR2-2-SCS-960kHz-r17. NOTE: To determine whether the UE supports a SCS 960kHz for a given band, the network validates the supportedSubCarrierSpacingUL. To determine the supported carrier bandwidths, the network validates the channelBWs-UL-SCS-960kHz-FR2-2-r17, the supportedBandwidthCombinationSet and supportedBandwidthUL-v1710. | Band | CY | N/A | N/A |
| codebookComboParameterMixedType-r17 Indicates the support of active CSI-RS resources and ports for mixed codebook types in any slot. The UE reports support active CSI-RS resources and ports for up to 4 mixed codebook combinations in any slot. The following are the possible mixed codebook combinations {Codebook1, Codebook2, Codebook3}: - type1SP-feType2PS-null-r17 indicates {Type 1 Single Panel, FeType II PS M=1, NULL} - type1SP-feType2PS-M2R1-null-r17 indicates {Type 1 Single Panel, FeType II PS M=2 R=1, NULL} - type1SP-feType2PS-M2R2-null-r17 indicates {Type 1 Single Panel, FeType II PS M=2 R=2, NULL} - type1SP-Type2-feType2-PS-M1-r17 indicates {Type 1 Single Panel, Type II, FeType II PS M=1} - type1SP-Type2-feType2-PS-M2R1-r17 indicates {Type 1 Single Panel, Type II, FeType II PS M=2 R=1} - type1SP-eType2R1-feType2-PS-M1-r17 indicates {Type 1 Single Panel, eType II R=1, FeType II PS M=1} - type1SP-eType2R1-feType2-PS-M2R1-r17 indicates {Type 1 Single Panel, eType II R=1, FeType II PS M=2 R=1} - type1MP-feType2PS-null-r17 indicates {Type 1 Multi Panel, FeType II PS M=1, NULL} - type1MP-feType2PS-M2R1-null-r17 indicates {Type 1 Multi Panel, FeType II PS M=2 R=1, NULL} - type1MP-feType2PS-M2R2-null-r17 indicates {Type 1 Multi Panel, FeType II PS M=2 R=2, NULL} - type1MP-Type2-feType2-PS-M1-r17 indicates {Type 1 Multi Panel, Type II, FeType II PS M=1} - type1MP-Type2-feType2-PS-M2R1-r17 indicates {Type 1 Multi Panel, Type II, FeType II PS M=2 R=1} - type1MP-eType2R1-feType2-PS-M1-r17 indicates {Type 1 Multi Panel, eType II R=1, FeType II PS M=1} - type1MP-eType2R1-feType2-PS-M2R1-r17 indicates {Type 1 Multi Panel, eType II R=1, FeType II PS M=2 R=1} For each mixed codebook supported by the UE, supportedCSI-RS-ResourceListAdd-r16 indicates the list of supported CSI-RS resources in a band by referring to codebookVariantsList. The following parameters are included for the supported CSI-RS resource: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource of a band. The minimum of maxNumberTxPortsPerResource is 'p4'; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs in a band; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs in a band. The minimum value of totalNumberTxPortsPerBand is 4. The UE supporting this feature shall indicate the support of individual codebook types in the reported mixed codebook combination among fetype2basic-r17, etype2R1-r16, CodebookComboParametersAddition-r16, supportedCSI-RS-ResourceList, fetype2R1-r17, fetype2R2-r17. | Band | No | N/A | N/A |
| codebookComboParameterMultiTRP-r17 Indicates the support of active CSI-RS resources and ports in the presence of multi-TRP CSI. Indicates the support of active CSI-RS resources and ports for mixed codebook types in any slot. The UE reports supported active CSI-RS resources and ports for up to 4 mixed codebook combinations. The following are the possible mixed codebook combinations {Codebook1, Codebook2, Codebook3}: - nCJT-null-null indicates {NCJT, NULL, NULL} - nCJT1SP-null-null indicates {NCJT+Type 1 SP for sTRP, NULL, NULL} - nCJT-Type2-null-r16 indicates {NCJT, Type 2, Null} - nCJT-Type2PS-null-r16 indicates {NCJT, Type 2 with port selection, Null} - nCJT-eType2R1-null-r16 indicates {NCJT, eType 2 with R=1, Null} - nCJT-eType2R2-null-r16 indicates {NCJT, eType 2 with R=2, Null} - nCJT-eType2R1PS-null-r16 indicates {NCJT, eType 2 with R=1 and port selection, Null} - nCJT-eType2R2PS-null-r16 indicates {NCJT, eType 2 with R=2 and port selection, Null} - nCJT-Type2-Type2PS-r16 indicates {NCJT, Type 2, Type 2 with port selection} - nCJT1SP-Type2-null-r16 indicates {NCJT+Type 1 SP for sTRP, Type 2, Null} - nCJT1SP-Type2PS-null-r16 indicates {NCJT+Type 1 SP for sTRP, Type 2 with port selection, Null} - nCJT1SP-eType2R1-null-r16 indicates {NCJT+Type 1 SP for sTRP, eType 2 with R=1, Null} - nCJT1SP-eType2R2-null-r16 indicates {NCJT+Type 1 SP for sTRP, eType 2 with R=2, Null} - nCJT1SP-eType2R1PS-null-r16 indicates {NCJT+Type 1 SP for sTRP, eType 2 with R=1 and port selection, Null} - nCJT1SP-eType2R2PS-null-r16 indicates {NCJT+Type 1 SP for sTRP, eType 2 with R=2 and port selection, Null} - nCJT1SP-Type2-Type2PS-r16 indicates {NCJT+Type 1 SP for sTRP, Type 2, Type 2 with port selection} - nCJT-feType2PS-null-r17 indicates {NCJT, FeType II PS M=1, NULL} - nCJT-feType2PS-M2R1-null-r17 indicates {NCJT, FeType II PS M=2 R=1, NULL} - nCJT-feType2PS-M2R2-null-r17 indicates {NCJT, FeType II PS M=2 R=2, NULL} - nCJT-Type2-feType2-PS-M1-r17 indicates {NCJT, Type II, FeType II PS M=1} - nCJT-Type2-feType2-PS-M2R1-r17 indicates {NCJT, Type II, FeType II PS M=2 R=1} - nCJT-eType2R1-feType2-PS-M1-r17 indicates {NCJT, eType II R=1, FeType II PS M=1} - nCJT-eType2R1-feType2-PS-M2R1-r17 indicates {NCJT, eType II R=1, FeType II PS M=2 R=1} - nCJT1SP-feType2PS-null-r17 indicates {NCJT+Type 1 SP for sTRP, FeType II PS M=1, NULL} - nCJT1SP-feType2PS-M2R1-null-r17 indicates {NCJT+Type 1 SP for sTRP, FeType II PS M=2 R=1, NULL} - nCJT1SP-feType2PS-M2R2-null-r17 indicates {NCJT+Type 1 SP for sTRP, FeType II PS M=2 R=2, NULL} - nCJT1SP-Type2-feType2-PS-M1-r17 indicates {NCJT+Type 1 SP for sTRP, Type II, FeType II PS M=1} - nCJT1SP-Type2-feType2-PS-M2R1-r17 indicates {NCJT+Type 1 SP for sTRP, Type II, FeType II PS M=2 R=1} - nCJT1SP-eType2R1-feType2-PS-M1-r17 indicates {NCJT+Type 1 SP for sTRP, eType II R=1, FeType II PS M=1} - nCJT1SP-eType2R1-feType2-PS-M2R1-r17 indicates {NCJT+Type 1 SP for sTRP, eType II R=1, FeType II PS M=2 R=1} For each mixed codebook supported by the UE, supportedCSI-RS-ResourceListAdd-r16 indicates the list of supported CSI-RS resources in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource of a band combination. - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs in a band combination. - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs in a band combination. NOTE 1: A CMR pair configured for NCJT will be counted as two activated resources, a CMR configured for sTRP will be counted as one activated resource for a triplet. NOTE 2: This capability is relevant only when UE is configured with NCJT CSI in at least one CSI report setting in at least one CC in the band and/or band combination. The UE indicating support of this feature shall also indicate the support of mTRP-CSI-EnhancementPerBand-r17. | Band | No | N/A | N/A |
| codebookComboParametersAddition-r16 Indicates the UE supports the mixed codebook combinations and the corresponding parameters supported by the UE. For mixed codebook types, UE reports support active CSI-RS resources and ports for up to 4 mixed codebook combinations in any slot. The following is the possible mixed codebook combinations: - {Type 1 Single Panel, Type 2, Null} - {Type 1 Single Panel, Type 2 with port selection, Null} - {Type 1 Single Panel, eType 2 with R=1, Null} - {Type 1 Single Panel, eType 2 with R=2, Null} - {Type 1 Single Panel, eType 2 with R=1 and port selection, Null} - {Type 1 Single Panel, eType 2 with R=2 and port selection, Null} - {Type 1 Single Panel, Type 2, Type 2 with port selection} - {Type 1 Multi Panel, Type 2, Null} - {Type 1 Multi Panel, Type 2 with port selection, Null} - {Type 1 Multi Panel, eType 2 with R=1, Null} - {Type 1 Multi Panel, eType 2 with R=2, Null} - {Type 1 Multi Panel, eType 2 with R=1 with port selection, Null} - {Type 1 Multi Panel, eType 2 with R=2 with port selection, Null} - {Type 1 Multi Panel, Type 2, Type 2 with port selection} Parameters for each mixed codebook supported by the UE: - supportedCSI-RS-ResourceListAdd-r16 indicates the list of supported CSI-RS resources in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: For supportedCSI-RS-ResourceListAdd-r16 related to the additional codebooks: - The minimum of maxNumberTxPortsPerResource is 'p4'; - The minimum value of totalNumberTxPortsPerBand is 4. If a UE reports one or more mixed codebook combinations, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations. For coexisting of mixed codebooks in any slot, gNB needs to consider the mixed codebook combination capability as well as per codebook capability of each codebook type in the mixed codebook combination. A UE that indicates support of a codebook type in the mixed codebook combination shall indicate support of the individual codebook type in the per band capability. | Band | No | N/A | N/A |
| CodebookComboParametersCJT-r18 Indicates the support of active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot. The UE reports supported active CSI-RS resources and ports for the following are the possible mixed codebook combinations {Codebook1, Codebook2, Codebook3}: - cjt-Type1SP-eType2R1-null indicates {Type I SP, eType-II-CJT R=1, NULL} - cjt-Type1SP-eType2R2-null indicates {Type I SP, eType-II-CJT R=2, NULL} - cjt-Type1SP-feType2R1M1-null indicates {Type I SP, FeType-II-CJT PS R=1 M=1, NULL} - cjt-Type1SP-feType2R1M2-null indicates {Type I SP, FeType-II-CJT PS R=1 M=2, NULL} - cjt-Type1SP-feType2R2M2-null indicates {Type I SP, FeType-II-CJT PS R=2 M=2, NULL} - cjt-Type1MP-eType2R1-null indicates {Type I MP, eType-II-CJT R=1, NULL} - cjt-Type1MP-eType2R2-null indicates {Type I MP, eType-II-CJT R=2, NULL} - cjt-Type1MP-feType2R1M1-null indicates {Type I MP, FeType-II-CJT PS R=1 M=1, NULL} - cjt-Type1MP-feType2R1M2-null indicates {Type I MP, FeType-II-CJT PS R=1 M=2, NULL} - cjt-Type1MP-feType2R2M2-null indicates {Type I MP, FeType-II-CJT PS R=2 M=2, NULL} For each mixed codebook supported by the UE, supportedCSI-RS-ResourceListAdd-r16 indicates the list of supported CSI-RS resources in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource of a band combination. The minimum of maxNumberTxPortsPerResource is 'p4'; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs in a band combination. - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs in a band combination. The minimum value of totalNumberTxPortsPerBand is 4. A UE supporting this feature shall also indicate support of individual codebook types in the reported mixed codebook combination among eType2CJT-r18, feType2CJT-r18, Type I single panel codebook and Type I multi-panel codebook. | Band | No | N/A | N/A |
| codebookParameters Indicates the codebooks and the corresponding parameters supported by the UE. Parameters for type I single panel codebook (type1 singlePanel) supported by the UE, which are mandatory to report: - supportedCSI-RS-ResourceList; - a UE shall support a maxNumberTxPortsPerResource minimum value of 4 for codebook type I single panel in FR1 in the case of a single active CSI-resource across all bands in a band combination, regardless of what it reports in supportedCSI-RS-ResourceList with maxNumberTxPortsPerResource; - a UE shall support a maxNumberTxPortsPerResource minimum value of 8 when configured with wideband CSI report for codebook type I single panel in FR1 in the case of a single active CSI-resource across all bands in a band combination, regardless of what it reports in supportedCSI-RS-ResourceList with maxNumberTxPortsPerResource; - a UE shall support a maxNumberTxPortsPerResource minimum value of 2 for codebook type I single panel in FR2 in the case of a single active CSI-resource across all bands in a band combination, regardless of what it reports in supportedCSI-RS-ResourceList with maxNumberTxPortsPerResource. - modes indicates supported codebook modes (mode 1, both mode 1 and mode 2); - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set. Parameters for type I multi-panel codebook (type1 multiPanel) supported by the UE, which are optional: - supportedCSI-RS-ResourceList; - modes indicates supported codebook modes (mode 1, mode 2, or both mode 1 and mode 2); - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set; - nrofPanels indicates supported number of panels. Parameters for type II codebook (type2) supported by the UE, which are optional: - supportedCSI-RS-ResourceList; - parameterLx indicates the parameter "Lx" in codebook generation where x is an index of Tx ports indicated by maxNumberTxPortsPerResource; - amplitudeScalingType indicates the amplitude scaling type supported by the UE (wideband or both wideband and sub-band); - amplitudeSubsetRestriction indicates whether amplitude subset restriction is supported for the UE. Parameters for type II codebook with port selection (type2-PortSelection) supported by the UE, which are optional: - supportedCSI-RS-ResourceList; - parameterLx indicates the parameter "Lx" in codebook generation where x is an index of Tx ports indicated by maxNumberTxPortsPerResource; - amplitudeScalingType indicates the amplitude scaling type supported by the UE (wideband or both wideband and sub-band). supportedCSI-RS-ResourceList includes list of the following parameters: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band simultaneously. For each codebook type, the UE may report another list of supported CSI-RS resources via supportedCSI-RS-ResourceListAlt in codebookParametersPerBand. For type I single panel codebook (type1 singlePanel) supportedCSI-RS-ResourceListAlt, - a UE shall report at least one triplet in supportedCSI-RS-ResourceListAlt with maxNumberTxPortsPerResource greater than or equal to 8 for FR1; - a UE shall report at least one triplet in supportedCSI-RS-ResourceListAlt with maxNumberTxPortsPerResource greater than or equal to 2 for FR2. | Band | FD | N/A | N/A |
| codebookParametersAddition-r16 Indicates the UE support of additional codebooks and the corresponding parameters supported by the UE. Codebook etype 2 R=1 support parameter combination 1 to 6 and rank 1 to 2. Parameters for etype 2 R=1 (etype2R1-r16) supported by the UE, which are optional: - supportedCSI-RS-ResourceListAdd-r16 indicates the list of supported CSI-RS resources in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource of a band; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs in a band, simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs in a band, simultaneously. - paramComb7-8-r16 indicates the support of parameter combinations 7-8 for etype 2 R=1 - rank3-4-r16 indicates the support of rank 3,4. - amplitudeSubsetRestriction-r16 indicates the support of amplitude subset restriction. Parameters for etype 2 R=2 (etype2R2-r16) supported by the UE, which are optional: - supportedCSI-RS-ResourceListAdd-r16; UE supporting etype2R2-r16supports also indicates support of etype2R1-r16. Codebook etype 2 R=1 with port selection supports 6 parameter combinations and rank 1,2. Parameters for etype 2 R=1 with port selection (etype2R1-PortSelection-r16) supported by the UE, which are optional: - supportedCSI-RS-ResourceListAdd-r16; - rank3-4-r16 indicates the support of rank 3,4 Parameters for etype 2 R=2 with port selection (etype2R2-PortSelection-r16) supported by the UE, which are optional: - supportedCSI-RS-ResourceListAdd-r16; UE supporting etype2R2-PortSelection-r16 also indicates support of etype2R1-PortSelection-r16. For supportedCSI-RS-ResourceListAdd-r16 related to the additional codebooks: - The minimum of maxNumberTxPortsPerResource is 'p4'; - The minimum value of totalNumberTxPortsPerBand is 4. | Band | No | N/A | N/A |
| codebookParametersetype2CJT-r18 Indicates the UE support of additional codebooks and the corresponding parameters supported by the UE of Enhanced Type II Codebook (eType-II) with refinement for multi-TRP CJT. The UE shall include eType2CJT-r18 to indicate basic features of eType-II codebook with refinement for multi-TRP CJT. This capability signalling comprises the following parameters: - supportedCSI-RS-ResourceList-r18 indicates the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - maxNumberResourcesPerBand indicates the maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - totalNumberTxPortsPerBand indicates the total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - scalingfactor-r18 indicates the scaling factor X for CPU occupation counting for CJT etype-II codebook - maxNumberNZP-CSI-RS-MultiTRP-CJT-r18 indicates the maximum number of NZP CSI-RS resources in one NZP CSI-RS resource set associated with multi-TRP CJT The UE indicating eType2CJT-r18 shall support N=N_TRP only, N_L=1 only, support mode 2 for eType-II codebook refinement for multi-TRP CJT, support for PMI subband R=1, support of parameter combinations with L=2,4, support rank 1,2, and support frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs. The UE indicating support of eType2CJT-r18 shall also indicate support of csi-ReportFramework and simultaneousCSI-ReportsAllCC. NOTE 1: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU = ceil(X * NTRP). NOTE 2: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on sp-CSI-ReportPUSCH. The UE optionally includes eType2CJT-FD-IO-r18 to indicate whether the UE supports mode 1 for CJT eType-II codebook with FD basis selection integer frequency offset. This capability signalling comprises the list of supported NZP CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The UE indicating eType2CJT-FD-IO-r18 shall also support frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset. The UE optionally indicates eType2CJT-FD-FO-r18 to indicate whether the UE supports frequency basis selection mode 1 with FD basis selection fractional frequency offset for eType-II based CJT codebook. The UE indicating eType2CJT-FD-FO-r18 shall also indicate support of eType2CJT-FD-IO-r18. The UE optionally indicates eType2CJT-R2-r18 to indicate whether the UE supports eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2. This capability signalling comprises the list of supported NZP CSI-RS resources with R=2 across all CCs in a band by referring to codebookVariantsList across all CCs. The UE optionally indicates eType2CJT-PV-Beta-r18 to indicate whether the UE supports eType-II codebook refinement for multi-TRP CJT with parameter combination pv={1/2,1/2,1/2,1/2} and beta=1/2. The UE optionally indicates eType2CJT-2NN1N2-r18 to indicate whether the UE supports 2NN1N2 >32 for eType-II CJT codebook. The UE indicates the maximum number of ports across all TRPs for one CJT CSI measurement. The UE optionally indicates eType2CJT-Rank3Rank4-r18 to indicate whether the UE supports eType-II codebook refinement for multi-TRP CJT with rank 3,4. The UE optionally indicates eType2CJT-L6-r18 to indicate whether the UE supports eType-II codebook refinement for multi-TRP CJT with parameter combination with L=6. The UE supports this capability only for N_TRP=1. The UE optionally indicates eType2CJT-NN-r18 to indicate whether the UE supports selection of N <= N_TRP CSI-RS resource by UE for multi-TRP CJT based on eType-II codebook. The UE optionally indicates eType2CJT-NL-SD-r18 to indicate whether the UE supports N_L>1 combinations of number of SD basis across CSI-RS resources for CJT eType-II codebook. The UE indicates the maximum number of lists for spatial basis selection, i.e., N_L, for multi-TRP CJT based on eType-II codebook. The UE optionally indicates eType2CJT-Unequal-r18 to indicate whether the UE supports unequal number of spatial basis selection configuration across CSI-RS resources for multi-TRP CJT including eType-II codebook refinement. For codebookVariantsList related to the eType-II: - The minimum of maxNumberTxPortsPerResource is 'p4'; - The minimum of maxNumberResourcesPerBand is 2; - The minimum value of totalNumberTxPortsPerBand is 4. | Band | No | N/A | N/A |
| codebookParametersetype2DopplerCSI-r18 Indicates the UE support of additional codebooks and the corresponding parameters supported by the UE of Enhanced Type II Codebook (eType-II) based on doppler CSI as specified in TS 38.214 [12]. The UE shall include eType2Doppler-r18 to indicate basic features of eType-II doppler codebook. This capability signalling comprises the following parameters: - supportedCSI-RS-ResourceList-r18 indicates the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource of a band - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs in a band, simultaneously - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs in a band, simultaneously - valueY-P-SP-CSI-RS-r18 indicates value of Y for CPU occupation (OCPU = Y* vectorLengthDD-r18), when P/SP-CSI-RS is configured for CMR - valueY-A-CSI-RS-r18 indicates value of Y for CPU occupation (OCPU = Y*K), when A-CSI-RS is configured for CMR - scalingfactor-r18 indicates scaling factor for active resource counting Kp The UE indicating eType2Doppler-r18 shall support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI='1-1'), support eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3, support parameter combinations with L=2,4, support for rank = 1,2, and support for the size of DD-basis, vectorLengthDD-r18 =1. The UE indicating support of eType2Doppler-r18 shall also indicate support of csi-ReportFramework and simultaneousCSI-ReportsAllCC. NOTE 1: When vectorLengthDD-r18 =1, OCPU =4. NOTE 2: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR. NOTE 3: when K=12, OCPU =8 NOTE 4: A UE that supports CSI enhancement for Rel-16 based type-II doppler must support this feature. The UE optionally includes eType2DopplerN4-r18 to indicate whether the UE supports doppler measurement with vectorLengthDD-r18 >1 for eType-II doppler codebook. This capability signalling comprises the following parameters: - supportedCSI-RS-ReportSettingList1-r18 indicates the list of supported combinations across all CCs in a band simultaneously by referring to supportedCSI-RS-ReportSettingList The following parameters are included in supportedCSI-RS-ReportSettingList-r18 - maxN4-r18 indicates the max number of vectorLengthDD-r18 - maxNumberTxPortsPerResource-r18 indicates the maximum number of Tx ports in a resource of a band - maxNumberResourcesPerBand-r18 indicates the maximum number of resources across all CCs in a band, simultaneously - totalNumberTxPortsPerBand-r18 indicates the total number of Tx ports across all CCs in a band, simultaneously - supportedCSI-RS-ReportSettingList2-r18 indicates the list of supported combinations for one CSI report setting by referring to supportedCSI-RS-ReportSettingList-r18. The UE indicating support of eType2DopplerN4-r18 shall also indicate support for the size of DD-basis, vectorLengthDD-r18 >1, and Value of unitDurationDD-r18=m for the DD unit size when A-CSI-RS is configured for CMR. The UE optionally includes ddUnitSize-A-CSI-RS-CMR-r18 to indicate the support of value of unitDurationDD-r18=1 for the DD unit duration when A-CSI-RS is configured for CMR. A UE supporting this feature shall also indicate support of eType2DopplerN4-r18. The UE optionally includes maxNumberAperiodicCSI-RS-Resource-r18 to indicate the maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for eType-II doppler measurement. The UE optionally includes eType2DopplerR2-r18 to indicate whether the UE supports R=2 for eType-II doppler codebook. This capability signalling comprises the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The UE optionally includes eType2DopplerX1-r18 to indicate whether the UE support X=1 based on first and last slot of WCSI, for eType-II doppler codebook. The UE optionally includes eType2DopplerX2-r18 to indicate whether the UE support X=2 CQI based on 2 slots for eType-II doppler codebook. The UE optionally includes eType2DopplerL-N4D1-r18 to indicate whether the UE support l = (n – nCSI,ref ) for CSI reference slot for eType-II doppler codebook. The UE optionally includes eType2DopplerL6-r18 to indicate whether the UE support L=6 for eType-II doppler codebook. The UE optionally includes eType2DopplerR3R4-r18 to indicate whether the UE support rank equals 3 and 4 for eType-II doppler codebook. For codebookVariantsList-r16 related to the eType-II: - The minimum of maxNumberTxPortsPerResource is 'p4'; - The minimum of maxNumberResourcesPerBand is 2, except for eType2DopplerR2-r18. - The minimum value of totalNumberTxPortsPerBand is 4. | Band | No | N/A | N/A |
| codebookParametersfetype2-r17 Indicates the UE support of additional codebooks and the corresponding parameters supported by the UE of Further Enhanced Port-Selection Type II Codebook (FeType-II) as specified in TS 38.214 [12] clause 5.2.2.2.7. The UE indicating this feature shall include fetype2basic-r17 to indicate basic features of FeType-II. This capability signalling comprises the following parameters: - indicates the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource of a band - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs in a band, simultaneously - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs in a band, simultaneously The UE indicating fetype2basic-r17 shall support parameter combinations with M=1 and support rank 1 and 2. UE indicating this feature shall also include csi-ReportFramework. The UE optionally includes fetype2R1-r17 to indicate whether the UE supports M=2 and R=1 for FeType-II. This capability signalling comprises the following parameters: - indicates the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The UE indicating support of fetype2R1-r17 shall also indicate support of fetype2basic-r17 and parameter combinations with M=2. The UE optionally includes fetype2R2-r17 to indicate whether the UE supports R=2 for FeType-II. This capability signalling comprises the following parameters: - indicates the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. UE indicating support of fetype2R2-r17 shall also indicate support of fetype2R1-r17. The UE optionally includes fetype2Rank3Rank4-r17 to indicate whether the UE supports rank = 3 and rank = 4 for FeType-II. UE indicating support of fetype2Rank3Rank4-r17 shall indicate support of fetype2basic-r17. For codebookVariantsList related to the FeType-II: - The minimum of maxNumberTxPortsPerResource is 'p4'; - The minimum value of totalNumberTxPortsPerBand is 4. | Band | No | N/A | N/A |
| codebookParametersfetype2CJT-r18 Indicates the UE support of additional codebooks and the corresponding parameters supported by the UE of Further Enhanced Type II Codebook (feType-II) with refinement for multi-TRP CJT. The UE shall include feType2CJT-r18 to indicate basic features of feType-II codebook with refinement for multi-TRP CJT. This capability signalling comprises the following parameters: - supportedCSI-RS-ResourceList-r18 indicates the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - maxNumberResourcesPerBand indicates the maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - totalNumberTxPortsPerBand indicates the total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - scalingfactor-r18 indicates the scaling factor X for CPU occupation counting for CJT fetype-II codebook - maxNumberNZP-CSI-RS-MultiTRP-CJT-r18 indicates the maximum number of NZP CSI-RS resources in one NZP CSI-RS resource set associated with multi-TRP CJT The UE indicating feType2CJT-r18 shall support N=N_TRP only, N_L=1 only, support mode 2 for FeType-II port selection codebook refinement for multi-TRP CJT, support for PMI subband R=1, support of parameter combinations with M=1, support rank 1,2, and support frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs. The UE indicating support of feType2CJT-r18 shall also indicate support of csi-ReportFramework and simultaneousCSI-ReportsAllCC. NOTE 1: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU = ceil(X * NTRP). NOTE 2: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on sp-CSI-ReportPUSCH. NOTE 3: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this feature. The UE optionally includes feType2CJT-FD-IO-r18 to indicate whether the UE supports FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. This capability signalling comprises the list of supported NZP CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The UE indicating feType2CJT-FD-IO-r18 shall also support frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset. The UE optionally indicates feType2CJT-FD-FO-r18 to indicate whether the UE supports frequency basis selection mode 1 with FD basis selection fractional frequency offset for FeType-II port selection based CJT codebook. The UE indicating feType2CJT-FD-FO-r18 shall also indicate support of feType2CJT-FD-IO-r18. The UE optionally indicates eType2CJT-M2R1-r18 to indicate whether the UE supports FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1. This capability signalling comprises the list of supported NZP CSI-RS resources with R=2 across all CCs in a band by referring to codebookVariantsList. The UE indicating feType2CJT-M2R1-r18 shall also indicate support of feType2CJT-r18 or feType2CJT-FD-IO-r18. The UE optionally indicates feType2CJT-R2-r18 to indicate whether the UE supports FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. This capability signalling comprises the list of supported NZP CSI-RS resources with R=2 across all CCs in a band by referring to codebookVariantsList. The UE indicating feType2CJT-R2-r18 shall also indicate support of feType2CJT-r18 or feType2CJT-FD-IO-r18. The UE optionally indicates feType2CJT-2NN1N2-r18 to indicate whether the UE supports 2NN1N2 >32 for FeType-II CJT codebook. The UE indicates the maximum number of ports across all TRPs for one CJT CSI measurement. The UE optionally indicates feType2CJT-Rank3Rank4-r18 to indicate whether the UE supports FeType-II port selection codebook refinement for multi-TRP CJT with rank 3,4. The UE optionally indicates feType2CJT-NN-r18 to indicate whether the UE supports selection of N <= N_TRP CSI-RS resource by UE for multi-TRP CJT based on FeType-II port selection codebook. The UE optionally indicates feType2CJT-NL-r18 to indicate whether the UE supports N_L>1 combinations of number of ports across CSI-RS resources for CJT Fetype-II codebook. The UE indicates the maximum number of lists for ports selection, i.e., NL, for multi-TRP CJT based on FeType-II port selection codebook. The UE optionally indicates feType2CJT-Unequal-r18 to indicate whether the UE supports unequal number of port selection configuration across CSI-RS resources for multi-TRP CJT including FeType-II port selection codebook refinement. For codebookVariantsList related to the FeType-II: - The minimum of maxNumberTxPortsPerResource is 'p4'; - The minimum of maxNumberResourcesPerBand is 2; - The minimum value of totalNumberTxPortsPerBand is 4. | Band | No | N/A | N/A |
| codebookParametersfetype2DopplerCSI-r18 Indicates the UE support of additional codebooks and the corresponding parameters supported by the UE of Further Enhanced Type II Codebook (FeType-II) based on doppler CSI as specified in TS 38.214 [12]. The UE shall include feType2Doppler-r18 to indicate basic features of FeType-II doppler codebook. This capability signalling comprises the following parameters: - supportedCSI-RS-ResourceList-r18 indicates the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource of a band - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs in a band, simultaneously - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs in a band, simultaneously - valueY-A-CSI-RS-r18 indicates value of Y for CPU occupation (OCPU = Y*K), when A-CSI-RS is configured for CMR - scalingfactor-r18 indicates scaling factor for active resource counting Kp The UE indicating feType2Doppler-r18 shall support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI, support FeType-II regular codebook refinement for predicted PMI with PMI subband R=1, support parameter combinations with M=1, support for rank = 1,2, and support vectorLengthDD-r18 =1. A UE indicating this feature shall also indicate the support of csi-ReportFramework. The UE indicating support of feType2Doppler-r18 shall also indicate support of eType2Doppler-r18 and, csi-ReportFramework and simultaneousCSI-ReportsAllCC. NOTE 1: OCPU = 4 when P/SP-CSI-RS is configured for CMR. NOTE 2: when K=12, OCPU =8. The UE optionally includes maxNumberAperiodicCSI-RS-Resource-r18 to indicate the maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for FeType-II doppler measurement. The UE optionally includes feType2DopplerM2R1-r18 to indicate whether the UE supports M=2 and R=1 for FeType-II doppler codebook. This capability signalling comprises the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The UE optionally includes feType2DopplerR2-r18 to indicate whether the UE supports R=2 for FeType-II doppler codebook. This capability signalling comprises the list of supported CSI-RS resources across all CCs in a band by referring to codebookVariantsList. The UE optionally includes feType2DopplerL-N4D1-r18 to indicate whether the UE support l = (n – nCSI,ref ) for CSI reference slot for FeType-II doppler codebook. The UE optionally includes feType2DopplerR3R4-r18 to indicate whether the UE support rank equals 3 and 4 for FeType-II doppler codebook. For codebookVariantsList-r16 related to the feType-II: - The minimum of maxNumberTxPortsPerResource is 'p4'; - The minimum of maxNumberResourcesPerBand is 2, except for eType2DopplerR2-r18. - The minimum value of totalNumberTxPortsPerBand is 4. | Band | No | N/A | N/A |
| codebookParametersHARQ-ACK-PUSCH-r18 Indicates whether the UE supports Multiplexing HARQ-ACK codebook in a PUSCH for PDSCH scheduled after UL grant. This capability signalling comprises the following parameters: - multiplexingType1-r18 indicates whether the UE supports multiplexing Type-1 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where ACK/NACK is generated for the HARQ-ACK codebook including HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. A UE supporting this feature shall also indicate support of semiStaticHARQ-ACK-Codebook. - multiplexingType2-r18 indicates whether the UE supports multiplexing Type-2 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where ACK/NACK is generated for the HARQ-ACK codebook including HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. A UE supporting this feature shall also indicate support of dynamicHARQ-ACK-Codebook. - multiplexingType3-r18 indicates whether the UE supports multiplexing Type-3 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where ACK/NACK is generated for the HARQ-ACK codebook including HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. A UE supporting this feature shall also indicate support of oneShotHARQ-feedback-r16. A UE shall also indicate support of one of pusch-RepetitionMultiSlots-r16 and pusch-RepetitionTypeB-r16. UE does not expect to determine a different codebook size in a PUCCH slot from the codebook size determined based on HARQ-ACK information associated with PDSCH reception(s) scheduled before a UL grant that schedules a PUSCH in a slot overlapping with the PUCCH slot unless the UE indicates support of diffCB-Size-PDSCH-r18. UE does not expect to determine a different PUCCH time domain resource in a slot from the PUCCH time domain resource determined based on HARQ-ACK information associated with PDSCH reception(s) scheduled before a UL grant that schedules a PUSCH in that slot unless the UE indicates support of pucch-DiffResource-PDSCH-r18. The UE optionally includes pucch-DiffResource-PDSCH-r18 to indicate whether the UE supports determining a different PUCCH resource in a slot from the PUCCH resource indicated by the last DCI format before a UL grant in the slot, to include HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling a PUSCH transmission with repetitions and the HARQ-ACK information are multiplexed on a repetition of the PUSCH transmission other than a first repetition in the same slot. The UE optionally includes diffCB-Size-PDSCH-r18 to indicate whether the UE supports determining different codebook size in a PUCCH slot from the size determined based on HARQ-ACK information associated with PDSCH reception(s) scheduled before a UL grant, to include HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling a PUSCH transmission with repetitions and the HARQ-ACK information are multiplexed on a repetition of the PUSCH transmission other than a first repetition in the same slot. | Band | No | N/A | N/A |
| commonTCI-MultiDCI-r18 Indicates whether the UE supports common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP. The UE also indicates the maximum number of CC list(s). A UE supporting this feature shall also indicate support of tci-JointTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18. | Band | No | N/A | N/A |
| commonTCI-SingleDCI-r18 Indicates whether the UE supports common multi-CC TCI state ID update and activation for single-DCI based multi-TRP. The UE also indicates the maximum number of CC list(s). A UE supporting this feature shall also indicate support of tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18. | Band | No | N/A | N/A |
| condHandover-r16 Indicates whether the UE supports conditional handover including execution condition, candidate cell configuration and maximum 8 candidate cells. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. | Band | No | N/A | N/A |
| condHandoverFailure-r16 Indicates whether the UE supports conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. | Band | No | N/A | N/A |
| condHandoverTwoTriggerEvents-r16 Indicates whether the UE supports 2 trigger events for same execution condition. This feature is mandatory supported if the UE supports condHandover-r16. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. | Band | CY | N/A | N/A |
| condHandoverWithCandSCG-change-r18 Indicates whether the UE supports conditional handover with candidate SCG, where conditional NR PSCell change is supported for FDD-FR1 bands, TDD-FR1 bands, TDD-FR2-1 bands and TDD-FR2-2 bands. The UE indicating support of this feature shall also indicate the support of condHandover-r16 and support of at least one NR-DC band combination. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| condPSCellChange-r16 Indicates whether the UE supports conditional PSCell change including execution condition, candidate cell configuration and maximum 8 candidate cells. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| condPSCellChangeTwoTriggerEvents-r16 Indicates whether the UE supports 2 trigger events for same execution condition. This feature is mandatory supported if the UE supports condPSCellChange-r16. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | CY | N/A | N/A |
| configuredUL-GrantType1-v1650 Indicates whether the UE supports Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. This applies only to non-shared spectrum channel access. For shared spectrum channel access, configuredUL-GrantType1-r16 applies. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The UE only includes configuredUL-GrantType1-v1650 if configuredUL-GrantType1 is absent. | Band | No | N/A | N/A |
| configuredUL-GrantType2-v1650 Indicates whether the UE supports Type 2 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. This applies only to non-shared spectrum channel access. For shared spectrum channel access, configuredUL-GrantType2-r16 applies. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The UE only includes configuredUL-GrantType2-v1650 if configuredUL-GrantType2 is absent. | Band | No | N/A | N/A |
| cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17 Indicates whether the UE supports CQI reporting with 4 bits per subband for NTN and shared spectrum channel access. | Band | No | N/A | N/A |
| crossCarrierScheduling-SameSCS Indicates whether the UE supports cross carrier scheduling for the same numerology with carrier indicator field (CIF) in carrier aggregation where numerologies for the scheduling cell and scheduled cell are same. | Band | No | N/A | N/A |
| csi-ReportFramework Indicates whether the UE supports CSI report framework. This capability signalling comprises the following parameters: - maxNumberPeriodicCSI-PerBWP-ForCSI-Report indicates the maximum number of periodic CSI report setting per BWP for CSI report; - maxNumberPeriodicCSI-PerBWP-ForBeamReport indicates the maximum number of periodic CSI report setting per BWP for beam report. - maxNumberAperiodicCSI-PerBWP-ForCSI-Report indicates the maximum number of aperiodic CSI report setting per BWP for CSI report; - maxNumberAperiodicCSI-PerBWP-ForBeamReport indicates the maximum number of aperiodic CSI report setting per BWP for beam report; - maxNumberAperiodicCSI-triggeringStatePerCC indicates the maximum number of aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC; - maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report indicates the maximum number of semi-persistent CSI report setting per BWP for CSI report; - maxNumberSemiPersistentCSI-PerBWP-ForBeamReport indicates the maximum number of semi-persistent CSI report setting per BWP for beam report; - simultaneousCSI-ReportsPerCC indicates the number of CSI report(s) for which the UE can measure and process reference signals simultaneously in a CC of the band for which this capability is provided. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsPerCC includes the beam report and CSI report. The UE is mandated to report csi-ReportFramework. | Band | Yes | N/A | N/A |
| csi-ReportFrameworkExt-r16 Indicates whether the UE supports the extension of the maximum number of configured aperiodic CSI report settings for all codebook types. The capability signalling comprises the following: maxNumberAperiodicCSI-PerBWP-ForCSI-ReportExt-r16 indicates the extended maximum number of aperiodic CSI report setting per BWP for CSI report. If present, the value of maxNumberAperiodicCSI-PerBWP-ForCSI-Report-r16 shall replace the corresponding value in csi-ReportFramework. | Band | No | N/A | N/A |
| csi-RS-ForTracking Indicates support of CSI-RS for tracking (i.e. TRS). This capability signalling comprises the following parameters: - maxBurstLength indicates the TRS burst length. Value 1 indicates 1 slot and value 2 indicates both of 1 slot and 2 slots. In this release UE is mandated to report value 2; - maxSimultaneousResourceSetsPerCC indicates the maximum number of TRS resource sets per CC which the UE can track simultaneously; - maxConfiguredResourceSetsPerCC indicates the maximum number of TRS resource sets configured to UE per CC. It is mandated to report at least 8 for FR1 and 16 for FR2; - maxConfiguredResourceSetsAllCC indicates the maximum number of TRS resource sets configured to UE across CCs. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. The UE is mandated to report at least 16 for FR1 and 32 for FR2. The UE is mandated to report csi-RS-ForTracking. | Band | Yes | N/A | N/A |
| csi-RS-IM-ReceptionForFeedback Indicates support of CSI-RS and CSI-IM reception for CSI feedback. This capability signalling comprises the following parameters: - maxConfigNumberNZP-CSI-RS-PerCC indicates the maximum number of configured NZP-CSI-RS resources per CC; - maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC indicates the maximum number of ports across all configured NZP-CSI-RS resources per CC; - maxConfigNumberCSI-IM-PerCC indicates the maximum number of configured CSI-IM resources per CC; - maxNumberSimultaneousNZP-CSI-RS-PerCC indicates the maximum number of simultaneous CSI-RS-resources per CC; - totalNumberPortsSimultaneousNZP-CSI-RS-PerCC indicates the total number of CSI-RS ports in simultaneous CSI-RS resources per CC. The UE is mandated to report csi-RS-IM-ReceptionForFeedback. | Band | Yes | N/A | N/A |
| csi-RS-ProcFrameworkForSRS Indicates support of CSI-RS processing framework for SRS. This capability signalling comprises the following parameters: - maxNumberPeriodicSRS-AssocCSI-RS-PerBWP indicates the maximum number of periodic SRS resources associated with CSI-RS per BWP; - maxNumberAperiodicSRS-AssocCSI-RS-PerBWP indicates the maximum number of aperiodic SRS resources associated with CSI-RS per BWP; - maxNumberSP-SRS-AssocCSI-RS-PerBWP indicates the maximum number of semi-persistent SRS resources associated with CSI-RS per BWP; - simultaneousSRS-AssocCSI-RS-PerCC indicates the number of SRS resources that the UE can process simultaneously in a CC, including periodic, aperiodic and semi-persistent SRS. | Band | No | N/A | N/A |
| cyclicShiftHoppingWithinSubset-r18 Indicates whether the UE supports configuration of subset of cyclic shifts for cyclic shift hopping. A UE supporting this feature shall also indicate the support of srs-cyclicShiftHopping-r18. | Band | No | N/A | N/A |
| defaultQCL-PerCORESETPoolIndex-r16 Indicates whether the UE supports default QCL assumption per CORESET pool index using multi-DCI based multi-TRP. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16 and simultaneousReceptionDiffTypeD-r16. | Band | No | N/A | FR2 only |
| defaultQCL-TwoTCI-r16 Indicates whether the UE supports default QCL assumption with two TCI states using single-DCI based multi-TRP. The UE can include this field only if simultaneousReceptionDiffTypeD-r16 is present. Otherwise, the UE does not include this field. | Band | No | N/A | FR2 only |
| dmrs-BundlingNonBackToBackTX-r17 Indicates whether the UE supports DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission as reported in dmrs-BundlingPUSCH-RepTypeA-r17, dmrs-BundlingPUSCH-RepTypeB-r17, dmrs-BundlingPUSCH-multiSlot-r17 or dmrs-BundlingPUCCH-Rep-r17. The UE is considered to support the feature in a band of a band combination if the UE indicates support of the feature for the corresponding band and for the band combination. UE indicating support of this feature shall also indicate support of at least one of dmrs-BundlingPUSCH-RepTypeA-r17, dmrs-BundlingPUSCH-RepTypeB-r17, dmrs-BundlingPUSCH-multiSlot-r17 or dmrs-BundlingPUCCH-Rep-r17. | Band | No | N/A | N/A |
| dmrs-BundlingPUCCH-Rep-r17 Indicates whether the UE supports DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 over consecutive symbols. The UE is considered to support the feature in a band of a band combination if the UE indicates support of the feature for the corresponding band and for the band combination. UE indicating support of this feature shall also indicate support of maxDurationDMRS-Bundling-r17 and pucch-Repetition-F1-3-4. | Band | No | N/A | N/A |
| dmrs-BundlingPUSCH-multiSlot-r17 Indicates whether the UE supports DM-RS bundling for TB processing over multi-slot PUSCH over consecutive symbols. The UE is considered to support the feature in a band of a band combination if the UE indicates support of the feature for the corresponding band and for the band combination. UE indicating support of this feature shall also indicate support of maxDurationDMRS-Bundling-r17 and tb-ProcessingMultiSlotPUSCH-r17. | Band | No | N/A | N/A |
| dmrs-BundlingPUSCH-RepTypeA-r17 Indicates whether the UE supports DM-RS bundling for PUSCH repetition type A over consecutive symbols. The UE is considered to support the feature in a band of a band combination if the UE indicates support of the feature for the corresponding band and for the band combination. UE indicating support of this feature shall also indicate support of maxDurationDMRS-Bundling-r17 and at least one of type1-PUSCH-RepetitionMultiSlots, type2-PUSCH-RepetitionMultiSlots or pusch-RepetitionMultiSlots. | Band | No | N/A | N/A |
| dmrs-BundlingPUSCH-RepTypeB-r17 Indicates whether the UE supports DM-RS bundling for PUSCH repetition type B over consecutive symbols. The UE is considered to support the feature in a band of a band combination if the UE indicates support of the feature for the corresponding band and for the band combination. UE indicating support of this feature shall also indicate support of maxDurationDMRS-Bundling-r17 and pusch-RepetitionTypeB-r16. | Band | No | N/A | N/A |
| dmrs-BundlingRestart-r17 Indicates whether the UE supports restarting DM-RS bundling after the events triggered by DCI or MAC CE that violate power consistency and phase continuity. The UE is considered to support the feature in a band of a band combination if the UE indicates support of the feature for the corresponding band and for the band combination. UE indicating support of this feature shall also indicate support of maxDurationDMRS-Bundling-r17. NOTE: Events which are triggered by DCI or MAC CE, but do not require UE capability to resume maintaining power consistency and/or phase continuity as specified in clause 6.1.7 of TS 38.214 [12] are excluded from this feature. | Band | No | N/A | N/A |
| dmrs-PortEntrySingleDCI-SDM-r18 Indicates whether the UE supports UL DMRS port entry {0, 2, 3} for single DCI based SDM scheme for Rel-15 DMRS port and/or Rel-18 DMRS port. A UE indicates supporting of this feature shall also indicate support of pusch-CB-SingleDCI-STx2P-SDM-r18 or pusch-NonCB-SingleDCI-STx2P-SDM-r18. | Band | No | N/A | FR2 only |
| dynamicMulticastDCI-Format4-2-r17 Indicates whether the UE supports DCI format 4_2 with CRC scrambled with G-RNTI for multicast in RRC_CONNECTED. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. | Band | No | N/A | N/A |
| dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17 Indicates the maximum number of supported dynamic slot-level repetitions for group-common PDSCH for multicast in RRC_CONNECTED for NTN and shared spectrum channel access. Value n8 corresponds to 8, and value n16 corresponds to 16. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. | Band | No | N/A | N/A |
| dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17 Indicates the maximum number of supported dynamic slot-level repetitions for group-common PDSCH for multicast in RRC_CONNECTED for TN and non-shared spectrum channel access. Value n8 corresponds to 8, and value n16 corresponds to 16. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2 bands respectively. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. | Band | No | N/A | N/A |
| dynamicWaveformSwitch-r18 Indicates whether the UE supports dynamic waveform switching for DCI format 0_1/0_2 when configured with only 1 UL carrier in the band. If UE supporting this feature also supports dci-Format1-2And0-2-r16, the UE supports this feature with DCI format 0_2. | Band | No | N/A | N/A |
| dynamicWaveformSwitchIntraCA-r18 Indicates whether the UE supports dynamic waveform switching for DCI format 0_1/0_2 for intra-band UL CA with up to X CCs in the band. A UE supporting this feature shall also indicate support of dynamicWaveformSwitch-r18. | Band | No | N/A | N/A |
| dynamicWaveformSwitchPHR-r18 Indicates whether the UE supports reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH. A UE supporting this feature shall also indicate support of dynamicWaveformSwitch-r18. NOTE: A UE can be configured to use either the single entry PHR with assumed PUSCH MAC CE or the multiple entry PHR with assumed PUSCH MAC CE for a cell group if the UE indicates support for this feature in any one cell of the cell group. | Band | No | N/A | N/A |
| enhancedChannelRaster-r18 Indicates whether the UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1 [2] and TS 38.101-5 [34]. It is mandatory with capability signalling for all Rel-18 UEs for certain bands as defined in TS 38.101-1 [2] and TS 38.101-5 [34]. Otherwise, it is optional. | Band | CY | N/A | FR1 only |
| enhancedSkipUplinkTxConfigured-v1660 Indicates whether the UE supports skipping UL transmission for a configured uplink grant only if no data is available for transmission and no UCI is multiplexed on the corresponding PUSCH of the uplink grant as specified in TS 38.321 [8]. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The UE only includes enhancedSkipUplinkTxConfigured-v1660 if enhancedSkipUplinkTxConfigured-r16 is absent. | Band | No | N/A | N/A |
| enhancedSkipUplinkTxDynamic-v1660 Indicates whether the UE supports skipping UL transmission for an uplink grant addressed to a C-RNTI only if no data is available for transmission and no UCI is multiplexed on the corresponding PUSCH of the uplink grant as specified in TS 38.321 [8]. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The UE only includes enhancedSkipUplinkTxDynamic-v1660 if enhancedSkipUplinkTxDynamic-r16 is absent. | Band | No | N/A | N/A |
| enhancedType3-HARQ-CodebookFeedback-r17 Indicates whether the UE supports enhanced type 3 HARQ-ACK codebook feedback based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2 as indicated in dci-Format1-2And0-2-r16) and also supports transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config as indicated in twoHARQ-ACK-Codebook-type1-r16). The capability signalling comprises the following parameters: - enhancedType3-HARQ-Codebooks-r17 indicates the maximum number of supported enhanced type 3 HARQ-ACK codebooks; - maxNumberPUCCH-Transmissions-r17 indicates the maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot. UE only supports feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2 as indicated in dci-Format1-2And0-2-r16) if the UE supports more than one enhanced type 3 HARQ-ACK codebook to be configured (as indicated in enhancedType3-HARQ-Codebooks-r17). The UE indicates support of this capability shall also indicate support of oneShotHARQ-feedback-r16. | Band | No | N/A | N/A |
| enhancedUL-TransientPeriod-r16 Indicates whether the UE supports enhanced UL performance for the transient period as specified in clause 6.3.3 of TS 38.101-1 [2] and in clause 6.3.3 of TS 38.101-5 [34]. If not reported, the UE supports transient period of 10us. | Band | No | N/A | FR1 only |
| eventA4BasedCondHandover-r17 Indicates whether the UE supports Event A4 based conditional handover in NTN bands, i.e., CondEvent A4 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. | Band | No | N/A | N/A |
| eventA4BasedCondHandoverNES-r18 Indicates whether the UE supports Event A4 based conditional handover for NES, i.e., CondEvent A4 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of nesBasedCondHandoverWithDCI-r18. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| extendedCP Indicates whether the UE supports 60 kHz subcarrier spacing with extended CP length for reception of PDCCH, and PDSCH, and transmission of PUCCH, PUSCH, and SRS. | Band | No | N/A | N/A |
| fastBeamSweepingMultiRx-r18 Indicates whether the UE supports beam sweeping factor reduction for SSB-based layer-1 measurement for activated serving cell when the UE is in multi-Rx operation. NOTE: It is only supported for power class 3. | Band | No | TDD only | FR2-1 only |
| groupBeamReporting Indicates whether UE supports RSRP reporting for the group of two reference signals. | Band | No | N/A | N/A |
| groupBeamReporting-STx2P-r18 Indicates whether the UE supports grouped-based beam reporting for STx2P. This capability signalling comprises the following parameters: - groupL1-RSRP-Reporting-r18 indicates the supported group based L1-RSRP reporting for STx2P based transmission. - maxNumberBeamGroups-r18 indicates the maximum number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets. - maxNumberResWithinSlotAcrossCC-r18 indicates the maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs in a band. - maxNumberResAcrossCC-r18 indicates the maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs in a band. A UE supporting this feature shall also indicate support of mTRP-GroupBasedL1-RSRP-r17. NOTE: maxNumberResWithinSlotAcrossCC-r18 and maxNumberResAcrossCC-r18 are also counted in maxTotalResourcesForOneFreqRange-r16, maxTotalResourcesForAcrossFreqRanges-r16, and mTRP-GroupBasedL1-RSRP-r17. | Band | No | N/A | FR2 only |
| groupSINR-reporting-r16 Indicates whether UE supports group based L1-SINR reporting. UE indicates support of this feature shall indicate support of ssb-csirs-SINR-measurement-r16. | Band | No | N/A | N/A |
| handoverUTRA-FDD-r16 Indicates whether the UE supports NR to UTRA-FDD CELL_DCH CS handover for the PCell on the band. It is mandatory to support both UTRA-FDD measurement and event B triggered reporting, and periodic UTRA-FDD measurement and reporting if the UE supports HO to UTRA-FDD. If this field is included, then UE shall support IMS voice over NR. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| interCellCrossTRP-PDCCH-OrderCFRA-r18 Indicates whether the UE supports cross-TRP PDCCH order based on CFRA for inter-cell multi-DCI based mTRP. A UE supporting this feature shall also indicate support of multiDCI-InterCellMultiTRP-TwoTA-r18. | Band | No | N/A | N/A |
| interSlotFreqHopInterSlotBundlingPUSCH-r17 Indicates whether the UE supports enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH. UE indicating support of this feature shall also indicate support of at least one of dmrs-BundlingPUSCH-RepTypeA-r17, dmrs-BundlingPUSCH-RepTypeB-r17 or dmrs-BundlingPUSCH-multiSlot-r17. | Band | No | N/A | N/A |
| interSlotFreqHopPUCCH-r17 Indicates whether the UE supports enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling. UE indicating support of this feature shall also indicate support of dmrs-BundlingPUCCH-Rep-r17. | Band | No | N/A | N/A |
| intraCellCrossTRP-PDCCH-OrderCFRA-r18 Indicates whether the UE supports cross-TRP PDCCH order based on CFRA for intra-cell multi-DCI based mTRP. A UE supporting this feature shall also indicate support of multiDCI-IntraCellMultiTRP-TwoTA-r18. | Band | No | N/A | N/A |
| intraSlot-PDSCH-MulticastInactive-r18 Indicates whether the UE supports TDM between one unicast PDSCH (e.g., small data transmission PDSCH) and one group-common PDSCH for multicast in a slot. This capability indicates, for any two consecutive slots n and n+1, if there are more than 1 multicast/unicast PDSCH in either slot, whether to require the minimum time separation (4 OFDM symbols for 30kHz and 7 OFDM symbols for 60kHz) between starting time of any two multicast/unicast PDSCHs within the duration of these slots. A UE indicating support of this feature shall also indicate support of multicastInactive-r18 and any of ra-SDT-r17, ra-SDT-NTN-r17, cg-SDT-r17, mt-SDT-r18, mt-SDT-NTN-r18 or mt-CG-SDT-r18. | Band | No | N/A | N/A |
| jointConfigDMRSPortDynamicSwitching-r18 Indicates whether the UE supports joint configuration of DMRS ports and dynamic switching between DFT-S-OFDM and CP-OFDM for PUSCH. A UE supporting this feature shall also indicate the support of pusch-TypeA-DMRS-r18 or pusch-TypeB-DMRS-r18, and dynamicWaveformSwitch-r18. | Band | No | N/A | N/A |
| jointReleaseConfiguredGrantType2-r16 Indicates whether the UE supports joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell. The UE can include this feature only if the UE indicates support of activeConfiguredGrant-r16. | Band | No | N/A | N/A |
| jointReleaseDCI-r18 Indicates whether the UE supports joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell. A UE supporting this feature shall also indicate support of one of multiPUSCH-CG-r18 and multiPUSCH-ActiveConfiguredGrant-r18. NOTE: For the case of joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell, the reporting of this feature applies, i.e., ignore irrespective of jointReleaseConfiguredGrantType2-r16. If UE supports jointReleaseConfiguredGrantType2-r16 but does not support this feature, the UE does not expect to be indicated for joint release including multi-PUSCH CG configuration(s). | Band | No | N/A | N/A |
| jointReleaseSPS-r16 Indicates whether the UE supports joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell. The UE can include this feature only if the UE indicates support of sps-r16. | Band | No | N/A | N/A |
| k1-RangeExtension-r17 Indicates whether the UE supports extended K1 value range of (0..31) for unpaired spectrum. This field is only applicable for bands in Table 5.2.2-1 and Table 5.2.3-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |
| locationBasedCondHandover-r17 Indicates whether the UE supports location based conditional handover, i.e., CondEvent D1 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. | Band | No | N/A | N/A |
| locationBasedCondHandoverATG-r18 Indicates whether the UE supports location based conditional handover, i.e., CondEvent D1, CondEvent A3, CondEvent A4 and CondEvent A5 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for bands as specified for ATG in clause 5.2J of TS 38.101-1 [2] and the support of airToGroundNetwork-r18. UE shall set the capability value consistently for all bands as specified for ATG in clause 5.2J of TS 38.101-1 [2]. | Band | No | N/A | FR1 only |
| locationBasedCondHandoverEMC-r18 Indicates whether the UE supports location based conditional handover for an NTN Earth-moving cell, i.e. condEventD2 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. | Band | No | N/A | N/A |
| lowerMSD-r18, lowerMSD-ENDC-r18 Indicates whether the UE supports lower maximum sensitivity degradation when the band is the victim band with sensitivity degradation as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. The victim band and associated aggressor band(s) are within at least one of inter-band CA or EN-DC band combinations supported by the UE. This feature includes following parameters: - aggressorband1-r18 indicates the aggressor band which causes sensitivity degradation to the victim band. It is an NR band for inter-band CA band combination and LTE band for EN-DC band combination. - aggressorband2-r18 indicates the additional aggressor band only when the sensitivity degradation to the victim band is caused by IMD of another two bands, i.e. aggressorband1-r18 and aggressorband2-r18 together (i.e. if aggressorband2-r18 is the victim band, it does not have to be indicated). - msd-Type-r18 indicates the MSD type, including harmonic, harmonic mixing, cross band isolation, IMD2, IMD3, IMD4, IMD5 and 'all'. Value 'all' indicates the MSD capability class is applicable for all MSD types defined in this release, which are applicable to the associated victim band/aggressor band(s). - msd-PowerClass-r18 indicates the applicable power class applied for the aggressor band(s) of the CA configuration for the lower MSD capability class reported in msd-Class-r18. - msd-Class-r18 indicates the lower MSD capability class as specified in 7.3A.7 in TS 38.101-1 [2] and in 7.3B2.3.7 in TS 38.101-3 [4]. The victim band and aggressor band(s) only consist of the bands requested by the network in frequencyBandListFilter. | Band | No | N/A | FR1 only |
| lowPAPR-DMRS-PDSCH-r16 Indicates whether the UE supports low PAPR DMRS for PDSCH. | Band | No | N/A | N/A |
| lowPAPR-DMRS-PUCCH-r16 Indicates whether the UE supports low PAPR DMRS for PUCCH format 3 and format 4 with transform precoding and with pi/2 BPSK modulation. UE indicates support of this feature shall indicate support of pucch-F3-4-HalfPi-BPSK and any combination of support of pucch-F3-WithFH, pucch-F4-WithFH and pucch-F1-3-4WithoutFH. It is mandatory with capability signalling. | Band | Yes | N/A | N/A |
| lowPAPR-DMRS-PUSCHwithoutPrecoding-r16 Indicates whether the UE supports low PAPR DMRS for PUSCH without transform precoding. | Band | No | N/A | N/A |
| lowPAPR-DMRS-PUSCHwithPrecoding-r16 Indicates whether the UE supports low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK modulation. It is mandatory with capability signalling. UE indicates support of this feature shall indicate support of pusch-HalfPi-BPSK. | Band | Yes | N/A | N/A |
| ltm-BeamIndicationJointTCI-r18 Indicates whether the UE supports unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, indicating and activating a single joint LTM TCI state in a cell switch command. This capability comprises the following parameters: - maxNumberJointTCI-PerCell-r18 indicates the maximum number of configured joint LTM TCI state(s) per candidate cell - qcl-Resource-r18 indicates of the supported QCL source RS in the LTM TCI-state- configuration. - maxNumberJointTCI-AcrossCells-r18 indicates index N of the maximum number of configured separate DL LTM TCI state(s) across candidate cells. The maximum number of configured joint LTM TCI state(s) across candidate cells is N*8, where N={1..128}. - maxNumberCells-r18 indicates the maximum number of configured cells for joint LTM TCI state(s). A UE supporting this feature shall also indicate support of unifiedJointTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. | Band | No | N/A | N/A |
| ltm-BeamIndicationSeparateTCI-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI-state indication for LTM procedure and indicating/activating a pair of UL/DL TCI-state in a cell switch command. This capability comprises the following parameters: - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of configured DL TCI state(s) per candidate cell. - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of configured UL TCI state(s) per candidate cell. - qcl-Resource-r18 indicates the supported QCL source RS in the LTM TCI-state configuration. - maxNumberDL-TCI-AcrossCells-r18 indicates value N of the maximum number of configured separate DL LTM TCI state(s) across candidate cells. The maximum number of configured separate DL LTM TCI state(s) across candidate cells is N*8, where N={1..128}. - maxNumberUL-TCI-AcrossCells-r18 indicates value N of the maximum number of configured separate UL LTM TCI state(s) across candidate cells. The maximum number of configured separate UL LTM TCI state(s) across candidate cells is N*8, where N={1..64}. - maxNumberCells-r18indicates the maximum number of configured cells for separate DL/UL LTM TCI states A UE supporting this feature shall also indicate support of unifiedSeparateTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. | Band | No | N/A | N/A |
| ltm-FastProcessingConfig-r18 Indicates whether the UE supports fast processing of LTM candidate cell RRC configuration. This capability signalling comprises the following parameters: - maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133 [5]. A UE supporting this capability shall also indicate support of ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18. UE shall set the capability values for maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18 consistently for all bands. These capability values represent the maximum number across all the supported bands. NOTE: The conditions for fast processing of an LTM candidate cell RRC configuration is defined in clause 6.3 in TS 38.133 [5]. | Band | No | N/A | N/A |
| ltm-MAC-CE-JointTCI-r18 Indicates whether the UE supports MAC-CE activated joint LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberJointTCI-PerCell-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states per candidate cell - maxNumberJointTCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cells A UE supporting this feature shall also indicate support of ltm-BeamIndicationJointTCI-r18. NOTE: The maximum number of MAC-CE activated joint TCI states across all servings cells is limited by of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| ltm-MAC-CE-SeparateTCI-r18 Indicates whether the UE supports MAC-CE activated DL/UL LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated DL TCI states per candidate cell - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated UL TCI states per candidate cell. - maxNumberDL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated DL TCI states across all candidate cells and serving cells - maxNumberUL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated UL TCI states across all candidate cells and serving cells A UE supporting this feature shall also indicate support of ltm-BeamIndicationSeparateTCI-r18. The maximum number of MAC-CE activated DL/UL TCI states across all servings cells is limited by unifiedSeparateTCI-r17. | Band | No | N/A | N/A |
| ltm-MCG-IntraFreq-r18 Indicates whether the UE supports intra-frequency LTM for MCG with RACH as defined in TS 38.331 [9] and TS 38.321 [8] without NR-DC configured. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. UE supporting this feature shall also indicate support for ltm-BeamIndicationJointTCI-r18 or ltm-BeamIndicationSeparateTCI-r18. | Band | No | N/A | N/A |
| ltm-SCG-IntraFreq-r18 Indicates whether the UE supports intra-frequency LTM for SCG with RACH as defined in TS 38.331 [9] and TS 38.321 [8]. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. UE supporting this feature shall also indicate support for ltm-BeamIndicationJointTCI-r18 or ltm-BeamIndicationSeparateTCI-r18. | Band | No | N/A | N/A |
| maxDurationDMRS-Bundling-r17 Indicates whether the UE supports the maximum duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH. NOTE: DM-RS bundling is only applicable for UL transmissions with pi/2 BPSK, BPSK, and QPSK modulation orders for the corresponding physical channels. | Band | No | N/A | N/A |
| maxDynamicSlotRepetitionForSPS-Multicast-r17 Indicates maximum number of dynamic slot-level repetitions for SPS group-common PDSCH for multicast. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE that indicates support of this feature shall indicate support of sps-Multicast-r17. | Band | No | N/A | N/A |
| max-HARQ-ProcessNumber-r17 Indicates the maximal supported HARQ process numbers for UL and for DL respectively. For each value of max-HARQ-ProcessNumber-r17, value u16d32 indicates the maximal supported HARQ process number is 16 for UL and 32 for DL, value u32d16 indicates the maximal supported HARQ process number is 32 for UL and 16 for DL, value u32d32 indicates the maximal supported HARQ process number is 32 for UL and 32 for DL. This field is only applicable for bands in Table 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |
| maxMIMO-LayersForMulti-DCI-mTRP-r16 Indicates the interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP. If this field is included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH for multi-DCI multi-TRP operation. If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. The UE that indicates support of this feature shall support overlapPDSCHsFullyFreqTime-r16. NOTE 1: For data rate calculation in clause 4.1.2, if this feature is indicated, each multi-DCI based multi-TRP CC is counted two times toward J. | Band | No | N/A | N/A |
| maxModulationOrderForMulticast-r17 Defines the maximal modulation order for multicast PDSCH in RRC_CONNECTED. If not reported, UE supports the same modulation order as unicast. - For FR1, up to 1024QAM is supported. - For FR2, up to 256QAM is supported. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. NOTE: A UE shall support the corresponding mandatory maximum modulation for unicast. | Band | No | N/A | N/A |
| maxNumberActivatedTCI-States-r16 Indicates maximum number of activated TCI states. This capability signalling includes the following: - maxNumberPerCORESET-Pool-r16 indicates maximal number of activated TCI states per CORESETPoolIndex per BWP per CC including data and control - maxTotalNumberAcrossCORESET-Pool-r16 indicates maximal total number of activated TCI states across CORESETPoolIndex per BWP per CC including data and control The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16. | Band | No | N/A | N/A |
| maxNumberCSI-RS-BFD Indicates maximal number of CSI-RS resources across all CCs, and across MCG and SCG in case of NR-DC, for UE to monitor PDCCH quality. In this release, the maximum value that can be signalled is 16. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. It is mandatory with capability signalling for FR2 and optional for FR1. | Band | CY | N/A | N/A |
| maxNumberCSI-RS-SSB-CBD Defines maximal number of different CSI-RS [and/or SSB] resources across all CCs, and across MCG and SCG in case of NR-DC, for new beam identifications. In this release, the maximum value that can be signalled is 128. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. It is mandatory with capability signalling for FR2 and optional for FR1. The UE is mandated to report at least 32 for FR2. | Band | CY | N/A | N/A |
| maxNumberG-CS-RNTI-r17 Defines maximum number of G-CS-RNTIs for SPS multicast. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE supporting this feature shall also indicate support of sps-Multicast-r17. | Band | No | N/A | N/A |
| maxNumberG-RNTI-r17 Defines maximum number of G-RNTIs for multicast in RRC_CONNECTED. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. For the UE indicating support of multicastInactive-r18, this capability is also applicable to multicast reception in RRC_INACTIVE, as specified in TS 38.331 [9]. | Band | No | N/A | N/A |
| maxNumber-NGSO-SatellitesPerCarrier-r17 Indicates the number of target NGSO satellites the UE can monitor per carrier. For serving carrier, the number of target NGSO satellites also includes the serving satellite. If this field is not included, the number of target satellites UE can monitor per carrier is 2. The value shall be larger than or equal to the reported value on maxNumber-NGSO-SatellitesWithinOneSMTC-r17. | Band | No | FDD only | FR1 only |
| maxNumber-NGSO-SatellitesWithinOneSMTC-r17 Indicates the number of different NGSO satellites for target cells that the UE supports of simultaneous measurements within a SMTC with value n1 corresponds to 1, value n2 corresponds to 2 and so on. | Band | No | FDD only | FR1 only |
| maxNumberNonGroupBeamReporting Defines support of non-group based RSRP reporting using N_max RSRP values reported. | Band | Yes | N/A | N/A |
| maxNumberPUSCH-TypeA-Repetition-r17 Indicates whether the UE supports the increased maximum number of PUSCH Type A repetitions to 32. A UE that indicates support of this feature shall support type1-PUSCH-RepetitionMultiSlots, type2-PUSCH-RepetitionMultiSlots, pusch-RepetitionTypeA-r16 or pusch-RepetitionTypeA-v16c0. NOTE: For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-v1710. For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-v1710. | Band | No | N/A | N/A |
| maxNumberRxBeam, maxNumberRxBeam-v1720 Defines whether UE supports receive beamforming switching using NZP CSI-RS resource. UE shall indicate a single value for the preferred number of NZP CSI-RS resource repetitions per CSI-RS resource set. Support of Rx beam switching is mandatory for FR2. | Band | CY | N/A | N/A |
| maxNumberRxTxBeamSwitchDL, maxNumberRxTxBeamSwitchDL-v1710 Defines the number of Tx and Rx beam changes UE can perform on this band within a slot. UE shall report one value per each subcarrier spacing supported by the UE. In this release, the number of Tx and Rx beam changes for scs-15kHz and scs-30kHz are not included. | Band | No | N/A | FR2 only |
| maxNumberSCellBFR-r16 Defines the maximum number of SCells configured for SCell beam failure recovery simultaneously. The UE indicating support of this also indicates the capabilities of maxNumberCSI-RS-BFD, maxNumberSSB-BFD and maxNumberCSI-RS-SSB-CBD. | Band | No | N/A | N/A |
| maxNumberSSB-BFD Defines maximal number of different SSBs across all CCs, and across MCG and SCG in case of NR-DC, for UE to monitor PDCCH quality. In this release, the maximum value that can be signalled is 16. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. It is mandatory with capability signalling for FR2 and optional for FR1. | Band | CY | N/A | N/A |
| maxOutputPowerATG-r18 Indicates the maximum output power rating at maximum modulation order and full RB allocation as specified in clause 6.2J of TS 38.101-1 [2]. Value 1 indicates 23dBm, value 2 indicates 24dBm and so on. If present, the ue-PowerClass is not included, and default UE power class is not applicable. The UE indicating support of this feature shall also indicate support of airToGroundNetwork-r18. This field is only applicable for bands as specified for ATG in clause 5.2J of TS 38.101-1 [2]. | Band | CY | N/A | FR1 only |
| maxPeriodicityCMR-r18 Indicates the maximum periodicity of periodic CSI-RS (in slots) UE can handle for Type-II-Doppler CSI report. The UE supporting this feature shall also indicate support of at least one of eType2Doppler-r18 and feType2Doppler-r18. NOTE: A UE that supports at least one of eType2Doppler-r18 and feType2Doppler-r18 must signal this feature. | Band | CY | N/A | N/A |
| maxUplinkDutyCycle-PC2-FR1 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is applicable for FR1 power class 2 UE and also applicable for FR1 power class 1.5 UE as specified in clause 6.2.1 of TS 38.101-1 [2]. If the field and maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 are both absent, 50% shall be applied as the upper limit of the UL duty cycle for power class 2. Value n60 corresponds to 60%, value n70 corresponds to 70% and so on. This capability is not applicable to IAB-MT. | Band | No | N/A | FR1 only |
| maxUplinkDutyCycle-FR2 Indicates the maximum percentage of symbols during 1s that can be scheduled for uplink transmission at the UE maximum transmission power, so as to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. This field is applicable for all power classes UE in FR2 as specified in TS 38.101-2 [3]. Value n15 corresponds to 15%, value n20 corresponds to 20% and so on. If the field is absent or the percentage of uplink symbols transmitted within any 1s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE behaviour is specified in TS 38.101-2 [3]. This capability is not applicable to IAB-MT. | Band | No | N/A | FR2 only |
| maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for FR1 power class 1.5 UE as specified in clause 6.2.1 of TS 38.101-1 [2]. If the field and maxUplinkDutyCycle-PC2-FR1 are both absent, 25% shall be applied as the upper limit of the UL duty cycle for power class 1.5. | Band | No | N/A | FR1 only |
| measEnhCAInterFreqFR2-r18 Indicates whether the UE supports the RRM requirement for intra-band CA operation in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 [5] and the RRM requirement for enhanced inter-frequency measurements in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 [5]. A UE supporting this feature shall also indicate support of PC6 in ue-PowerClass-v1700. | Band | No | N/A | FR2 only |
| measValidationReportEMR-r18 Indicates whether the UE supports measurement validation and report based on EMR measurement during connection setup/resume for fast CA/DC setup. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. A UE supporting this feature shall also indicate support of idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16. | Band | No | N/A | N/A |
| measValidationReportReselectionMeasurements-r18 Indicates whether the UE supports measurement validation based on reselection measurements during IDLE/INACTIVE state and reporting for fast CA/DC setup. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| mixCodeBookSpatialAdaptation-r18 Indicates whether the UE supports active CSI-RS resources and ports for mixed codebook types in any slot. The following codebook combination is a possible mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null } for UE supporting CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration. A UE supporting this feature shall also indicate support of spatialAdaptation-CSI-Feedback-r18, or spatialAdaptation-CSI-FeedbackPUSCH-r18, or spatialAdaptation-CSI-FeedbackPUCCH-r18, or spatialAdaptation-CSI-FeedbackAperiodic-r18. | Band | No | N/A | N/A |
| mn-InitiatedCondPSCellChangeNRDC-r17 Indicates whether the UE supports MN initiated conditional PSCell change in NR-DC, which is configured by NR conditionalReconfiguration using MN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in NR-DC. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. | Band | No | N/A | N/A |
| modifiedMPR-Behaviour Indicates whether UE supports modified MPR behaviour defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 38.101-5 [34]. | Band | No | N/A | N/A |
| mpe-Mitigation-r17 Indicates the support of enhanced PHR reporting which includes pairs of (P-MPR, SSBRI/CRI). This feature also includes following parameters: - maxNumP-MPR-RI-pairs-r17 indicates the maximum number of reported P-MPR and SSBRI/CRI pairs; - maxNumConfRS-r17 indicates the maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation. NOTE: maxNumConfRS-r17 is also counted in maxTotalResourcesForOneFreqRange-r16/ maxTotalResourcesForAcrossFreqRanges-r16. | Band | No | N/A | FR2 only |
| mpr-PowerBoost-FR2-r16 Indicates whether UE supports uplink transmission power boost by suspension of in-band emission (IBE) requirements as specified in TS 38.101-2 [3]. | Band | No | TDD only | FR2 only |
| mt-CG-SDT-r18 Indicates whether the UE supports initiating MT-SDT procedure over configured grant type 1, as specified in TS 38.331 [9]. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. Except for NTN, a UE supporting this feature shall also support mt-SDT-r18. For NTN, a UE supporting this feature shall also support mt-SDT-NTN-r18. | Band | No | N/A | N/A |
| mTRP-BFD-RS-MAC-CE-r17 Indicates the support of MAC-CE based update of explicit BFD-RS for mTRP BFR with maximum number of configured candidate BFD-RS per BWP for MAC-CE based update. The UE indicating support of this feature shall also indicate the support of mTRP-BFR-twoBFD-RS-Set-r17. | Band | No | N/A | N/A |
| mTRP-BFR-association-PUCCH-SR-r17 Indicates whether the UE supports association between a BFD-RS resource set on SpCell and a PUCCH SR resource. The UE indicating support of this feature shall support mTRP-BFR-PUCCH-SR-perCG-r17. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| mTRP-BFR-PUCCH-SR-perCG-r17 Indicates the maximum number of supported PUCCH-SR resources for MTRP BFR per cell group. A UE that supports mTRP-BFR-twoBFD-RS-Set-r17 shall indicate support of this feature with at least 1 PUCCH-SR resources for MTRP BFR per cell group. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| mTRP-BFR-twoBFD-RS-Set-r17 Indicates whether the UE supports mTRP BFR based on two BFD-RS sets. The capability signalling comprises the following parameters: - maxBFD-RS-resourcesPerSetPerBWP-r17 indicates the maximum number of supported measured BFD-RS resources per set per BWP. - maxBFR-r17 indicates the maximum number of CCs per band configured with BFR (including spCell/SCell/MTRP BFR). - maxBFD-RS-resourcesAcrossSetsPerBWP-r17 indicates the supported maximum number of measured BFD-RS resources across two BFD-RS sets per BWP. maxBFD-RS-resourcesAcrossSetsPerBWP-r17 is also counted in maxTotalResourcesForOneFreqRange-r16 and maxTotalResourcesForAcrossFreqRanges-r16. | Band | No | N/A | N/A |
| mTRP-CSI-additionalCSI-r17 Indicates the maximum value of numberOfSingleTRP-CSI-Mode1. The UE indicating support of this feature shall also indicate 'mode1' or 'both' in cSI-Report-mode-r17 of mTRP-CSI-EnhancementPerBand-r17. | Band | No | N/A | N/A |
| mTRP-CSI-CMR-r17 Indicates the support of a NZP CSI-RS resource referred by both a CMR pair configured for Rel-17 Multi-TRP CSI enhancement and a single CMR configured for Single-TRP measurement in a CSI reporting setting. The UE indicating support of this feature shall also indicate the support of mTRP-CSI-EnhancementPerBand-r17. | Band | No | N/A | FR2 only |
| mTRP-CSI-EnhancementPerBand-r17 Indicates support of CSI enhancements for multi-TRP including support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis with N=1. This feature also includes following parameters: - maxNumNZP-CSI-RS-r17 indicates the maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - cSI-Report-mode-r17 indicates the CSI report mode selection. Mode1 indicates mode 1 with X=0, mode2 indicates mode 2, both indicate the support of both mode 1 with X=0 and mode 2. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination includes: - maxNumTx-Ports-r17 indicates the maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - maxTotalNumCMR-r17 indicates the maximum total number of CMRs for NCJT measurement - maxTotalNumTx-PortsNZP-CSI-RS-r17 indicates the maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses - codebookModeNCJT-r17 indicates the supported codebook modes for NCJT CSI. | Band | No | N/A | N/A |
| mTRP-CSI-N-Max2-r17 Indicates the support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting. The UE indicating support of this feature shall also indicate the support of mTRP-CSI-EnhancementPerBand-r17. | Band | No | N/A | N/A |
| mTRP-CSI-numCPU-r17 Indicates the number of CSI processing units (CPUs) occupied by a pair of CMRs for NCJT CSI hypotheses. Maximum number of CPUs is reported in csi-ReportFramework. The UE indicating support of this feature shall also indicate the support of mTRP-CSI-EnhancementPerBand-r17. | Band | No | N/A | N/A |
| mTRP-GroupBasedL1-RSRP-r17 Indicates the support of group based L1-RSRP reporting enhancements. This feature also includes following parameters: - maxNumBeamGroups-r17 indicates the maximum number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets. - maxNumRS-WithinSlot-r17 indicates the maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs. - maxNumRS-AcrossSlot-r17 indicates the maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs. maxNumRS-WithinSlot-r17 and maxNumRS-AcrossSlot-r17 are also counted in maxTotalResourcesForOneFreqRange-r16 and maxTotalResourcesForAcrossFreqRanges-r16. | Band | No | N/A | N/A |
| mTRP-inter-Cell-r17 Indicates the support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info. This feature also includes following parameters: - maxNumAdditionalPCI-Case1-r17 indicates the maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI. - maxNumAdditionalPCI-Case2-r17 indicates the maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1. The UE indicating support of this feature shall also indicate the support of multiDCI-MultiTRP-r16. | Band | No | N/A | N/A |
| mTRP-PDCCH-anySpan-3Symbols-r17 Indicates support of PDCCH repetition for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot. It is applicable to 15kHz SCS only. The UE indicating support of this feature shall also indicate support of pdcchMonitoringSingleOccasion and mTRP-PDCCH-Repetition-r17. | Band | No | N/A | FR1 only |
| mTRP-PDCCH-individual-r17 Indicates the support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET. The UE indicating support of this feature shall also indicate support of mTRP-PDCCH-Repetition-r17. | Band | No | N/A | N/A |
| mTRP-PDCCH-TwoQCL-TypeD-r17 Indicates the support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition. The UE indicating support of this feature shall also indicate support of mTRP-PDCCH-Repetition-r17. | Band | No | N/A | FR2 only |
| mTRP-PUCCH-CyclicMapping-r17 Indicates whether the UE supports cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2. The UE that indicates support of this feature shall also indicate support of mTRP-PUCCH-InterSlot-r17. | Band | No | N/A | N/A |
| mTRP-PUCCH-InterSlot-r17 Indicates whether the UE supports the following features: - support of PUCCH repetition scheme 1 (inter-slot repetition) with sequential mapping for repetitions larger than 2 and with cyclic mapping for 2 repetitions. - support of up to two PUCCH power control parameter sets/spatial relation information per PUCCH resource. The power control parameter sets only apply to FR1 and spatial relation information only applies to FR2. - supported PUCCH formats for PUCCH repetition scheme 1. | Band | No | N/A | N/A |
| mTRP-PUCCH-MAC-CE-r17 Indicates the support of updating two Spatial Relation Info's and two sets of power control parameters for a group of PUCCH resources in a CC by MAC-CE. The UE indicates support of this feature shall also indicate support of mTRP-PUCCH-InterSlot-r17. | Band | No | N/A | N/A |
| mTRP-PUCCH-maxNum-PC-FR1-r17 Indicates the maximum number of power control parameter sets configured for multi-TRP PUCCH repetition in FR1. The UE indicating support of this feature shall also indicate the support of mTRP-PUCCH-InterSlot-r17. | Band | No | N/A | FR1 only |
| mTRP-PUCCH-SecondTPC-r17 Indicates whether the UE supports second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2. The UE that indicates support of this feature shall also indicate support of mTRP-PUCCH-InterSlot-r17. | Band | No | N/A | N/A |
| mTRP-PUSCH-A-CSI-r17 Indicates the support of A-CSI report on two PUSCH repetitions. The UE indicating support of this feature shall also indicate the support of mTRP-PUSCH-TypeA-CB-r17 or mTRP-PUSCH-RepetitionTypeA-r17. | Band | No | N/A | N/A |
| mTRP-PUSCH-CG-r17 Indicates the support of CG PUSCH transmission towards M-TRPs using a single CG configuration. The UE uses same beam mapping principals as dynamic grant PUSCH repetition scheme. The UE indicating support of this feature shall also indicate the support of mTRP-PUSCH-TypeA-CB-r17 or mTRP-PUSCH-RepetitionTypeA-r17. | Band | No | N/A | N/A |
| mTRP-PUSCH-CSI-RS-r17 Indicates the support of CSI-RS processing framework for SRS with two associated CSI-RS resources. This feature also includes following parameters: - maxNumPeriodicSRS-r17 indicates the maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. - maxNumAperiodicSRS-r17 indicates the maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. - maxNumSP-SRS-r17 indicates the maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. - numSRS-ResourcePerCC-r17: UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes Periodic/Semi-Persistent/Aperiodic SRS. - numSRS-ResourceNonCodebook-r17: UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously. The UE indicating support of this feature shall also indicate the support of mTRP-PUSCH-twoCSI-RS-r17. | Band | No | N/A | N/A |
| mTRP-PUSCH-cyclicMapping-r17 Indicates the support of cyclic mapping when the number of repetitions is larger than 2 with repetition type. The UE indicating support of this feature shall also indicate the support of mTRP-PUSCH-TypeA-CB-r17 or mTRP-PUSCH-RepetitionTypeA-r17. | Band | No | N/A | N/A |
| mTRP-PUSCH-secondTPC-r17 Indicates the support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 and 0_2. The UE indicating support of this feature shall also indicate the support of mTRP-PUSCH-TypeA-CB-r17 or mTRP-PUSCH-RepetitionTypeA-r17. | Band | No | N/A | N/A |
| mTRP-PUSCH-SP-CSI-r17 Indicates the support of SP-CSI report on two PUSCH repetitions. The UE indicating support of this feature shall also indicate the support of mTRP-PUSCH-TypeA-CB-r17 or mTRP-PUSCH-RepetitionTypeA-r17. | Band | No | N/A | N/A |
| mTRP-PUSCH-twoCSI-RS-r17 Indicates whether the UE supports up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH. The UE that indicates support of this feature shall also indicate support of srs-AssocCSI-RS, csi-RS-IM-ReceptionForFeedbackPerBandComb and mTRP-PUSCH-RepetitionTypeA-r17. | Band | No | N/A | N/A |
| mTRP-PUSCH-twoPHR-Reporting-r17 Indicates the support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion corresponding to each SRS resource set, and report two PHRs). The UE indicating support of this feature shall also indicate the support of mTRP-PUSCH-TypeA-CB-r17 or mTRP-PUSCH-RepetitionTypeA-r17. | Band | No | N/A | N/A |
| multicastInactive-r18 Indicates whether the UE supports multicast reception in RRC_INACTIVE as specified in TS 38.331 [9], comprised of the following functional components: - Supports group-common PDCCH/PDSCH for multicast with CRC scrambled by Multicast MCCH-RNTI; - Supports group-common PDCCH/PDSCH for multicast with CRC scrambled by G-RNTI; - Supports DCI format 4_0 with CRC scrambled with Multicast MCCH-RNTI for multicast MCCH; - Supports DCI format 4_1 with CRC scrambled with G-RNTI for multicast MTCH; - Supports multicast MCCH change notification indication via DCI; - Supports CFR configuration for multicast; - Supports CORESET and common search space configuration for multicast; - Supports one G-RNTI for multicast reception; - Supports RRC configured slot-level repetition up to 8 for multicast MTCH; - Supports inter-slot TDM between group-common PDSCH for multicast MCCH and group-common PDSCH for multicast MTCH, or among group-common PDSCH for multicast MCCH, group-common PDSCH for multicast MTCH and other PDSCHs in different slots; - Supports up to 64QAM for FR1/FR2; - Supports 12-bit length of PDCP sequence number; - Supports ROHC profiles 0x0000, 0x0001 and 0x0002; - Supports 4 ROHC header compression context sessions; - Supports UM MRB with 12-bit length of RLC sequence number; - Supports UM MRB with 6-bit length of RLC sequence number; - Supports long DRX cycle for MBS multicast reception as specified in TS 38.321 [8]. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. A UE supporting this feature and supporting Mission Critical Services as described in clause 5.16.6 in TS 23.501 [37] shall also indicate the support of thresholdBasedMulticastResume-r18. | Band | No | N/A | N/A |
| multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 Indicates whether the UE supports multi-PDSCH scheduling by single DCI for the operation with 120kHz SCS in FR2-1 and HARQ enhancements for both type 1 and type 2 HARQ codebook. | Band | No | N/A | N/A |
| multipleRateMatchingEUTRA-CRS-r16 Indicates whether the UE supports multiple E-UTRA CRS rate matching patterns, which is supported only for FR1. The capability signalling comprises the following parameters: - maxNumberPatterns-r16 indicates the maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS. The UE can report the value larger than 2 only if UE reports the value of maxNumberNon-OverlapPatterns-r16 is larger than 1. - maxNumberNon-OverlapPatterns-r16 indicates the maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS. The UE can include this feature only if the UE indicates support of rateMatchingLTE-CRS. | Band | No | N/A | FR1 only |
| multipleTCI Indicates whether UE supports more than one TCI state configurations per CORESET. UE is only required to track one active TCI state per CORESET. UE is required to support minimum between 64 and number of configured TCI states indicated by tci-StatePDSCH. This field shall be set to supported. | Band | Yes | N/A | N/A |
| multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17 Indicates whether the UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE supporting this feature shall also indicate support of priorityIndicatorInDCI-Multicast-r17 and twoHARQ-ACK-CodebookForUnicastAndMulticast-r17. | Band | No | N/A | N/A |
| multiPUSCH-ActiveConfiguredGrant-r18 Indicates whether the UE supports multiple active multi-PUSCHs configured grant configurations for a BWP of a serving cell. This feature also includes following parameters: - maxNumberConfigsPerBWP indicates the supported maximum number of configured/active configured grant configurations in a BWP of a serving cell. - maxNumberConfigsAllCC-FR1 indicates the supported maximum number of configured/active configured grant configurations across all serving cells, and across MCG and SCG in case of NR-DC in FR1. - maxNumberConfigsAllCC-FR2 indicates the supported maximum number of configured/active configured grant configurations across all serving cells, and across MCG and SCG in case of NR-DC in FR2. A UE supporting this feature shall also indicate support of multiPUSCH-CG-r18. When UE supports both activeConfiguredGrant-r16 and multiPUSCH-ActiveConfiguredGrant-r18, the total number which can be configured for CG with single-PUSCH TO in one CG period and CG with multi-PUSCH TO in one CG period should not exceed the value reported by activeConfiguredGrant-r16. For all the reported bands in FR1, a same value is reported for maxNumberConfigsAllCC. For all the reported bands in FR2, a same value is reported for maxNumberConfigsAllCC. The total number of configured/active configured grant configurations across all serving cells in FR1 is no greater than maxNumberConfigsAllCC in FR1. The total number of configured/active configured grant configurations across all serving cells in FR2 is no greater than maxNumberConfigsAllCC in FR2. If there are some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of configured/active configured grant configurations across all serving cells is no greater than max(maxNumberConfigsAllCC-FR1, maxNumberConfigsAllCC-FR2). NOTE: Separate release of different multi-PUSCHs configuration grant Type 2 configuration, i.e., one DCI release one multi-PUSCHs configured grant Type 2 configuration is supported with this feature. | Band | No | N/A | N/A |
| multiPUSCH-CG-r18 Indicates whether the UE supports multi-PUSCHs for configured grant by indicating whether the UE supports the determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG and also the maximum supported number of consecutive slots configured for CG-PUSCG TOs in one CG period. This feature also includes following parameters: - n16 indicates the maximum supported number of consecutive slots configured for CG-PUSCH TOs in one CG period is 16. - n32 indicates the maximum supported number of consecutive slots configured for CG-PUSCH TOs in one CG period is 32. A UE supporting this feature shall also indicate support of at least one of configuredUL-GrantType1, configuredUL-GrantType1-v1650, configuredUL-GrantType2, and configuredUL-GrantType2-v1650. | Band | No | N/A | N/A |
| multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 Indicates whether the UE supports multi-PUSCH scheduling by single DCI for the operation with 120kHz SCS in FR2-1 with non-contiguous allocation. | Band | No | N/A | N/A |
| multiPUSCH-SingleDCI-NonConsSlots-r18 Indicates support of Multi-PUSCH scheduling by single DCI format 0_1 for the operation with non-contiguous allocation. A UE supporting this feature shall also indicate support of multiPUSCH-UL-grant-r16. | Band | No | N/A | FR1 only |
| mux-HARQ-ACK-DiffPriorities-r17 Indicates whether the UE supports HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH, comprised of the following functional components: - Supports multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Supports separate coding for the two HARQ-ACKs; - Supports multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH; - Supports multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Supports separate beta_offset values for this priority combination; - Supports multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Supports separate beta_offset values for this priority combination; - Supports multiplexing a low-priority HARQ-ACK, a high-priority PUSCH, a high-priority HARQ-ACK and/or CSI; - Supports multiplexing a high-priority HARQ-ACK, a low-priority PUSCH, a low-priority HARQ-ACK and/or CSI. The UE indicating support of this feature shall also indicate the support of twoHARQ-ACK-Codebook-type1-r16. | Band | No | N/A | N/A |
| nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17 Indicates whether the UE supports DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2. A UE supporting this feature shall also indicate support of nack-OnlyFeedbackForMulticast-r17 and dynamicMulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| nack-OnlyFeedbackForSPS-MulticastWithDCI-Enabler-r17 Indicates whether the UE supports DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI by RRC signalling via DCI format 4_2. A UE that indicates support of this feature shall indicate support of nack-OnlyFeedbackForSPS-Multicast-r17 and sps-MulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| ncd-SSB-BWP-Wor-r18 Indicates whether the UE supports RLM/BM/BFD and gapless L3 intra-frequency measurements based on NCD-SSB within active BWP. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell. NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. UE performs L3 intra-frequency measurements without gaps based on NCD-SSB, where the NCD-SSB is within the active DL BWP. NOTE: This feature applies only to PCell and PSCell (if configured). It is not applicable to RedCap or eRedCap UEs. | Band | No | N/A | N/A |
| nesBasedCondHandoverWithDCI-r18 Indicates whether the UE supports DCI-based enabling/disabling NES-specific CHO execution condition, i.e. NES-specific CHO execution condition based on source cell NES mode indicated via DCI format 2_9 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| nes-CellDTX-DRX-r18 Indicates whether the UE supports cell DTX and/or DRX operation by RRC configuration. The supported number of cell DTX/DRX patterns per cell group is 2, regardless of each pattern is for cell DTX only, cell DRX only, or both. A UE setting this field to the value 'cellDTXonly' or 'both' shall also indicate support of longDRX-Cycle. | Band | No | N/A | N/A |
| nes-CellDTX-DRX-DCI2-9-r18 Indicates whether the UE supports cell DTX/DRX configuration activation and deactivation via DCI 2_9. A UE supporting this feature shall also indicate support of nes-CellDTX-DRX-r18. | Band | No | N/A | N/A |
| nonGroupSINR-reporting-r16 Indicates N_max L1-SINR values reported when UE supports non-group based L1-SINR reporting. UE indicates support of this feature shall indicate support of ssb-csirs-SINR-measurement-r16. | Band | No | N/A | N/A |
| nr-PDCCH-OverlapLTE-CRS-RE-r18 Indicates whether the UE supports reception of NR PDCCH candidates that overlap with LTE CRS REs within a NR carrier using 15 kHz SCS. The UE is provided with LTE CRS RM pattern by configuration of one CRS rate matching pattern via lte-CRS-ToMatchAround. NR PDCCH that overlaps with LTE CRS REs is in Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS that are monitored within the first 3 OFDM symbols of a slot. This feature comprises following components: - overlapInRE-r18 indicates reception of a NR PDCCH candidate in REs that overlap with LTE CRS: Value oneSymbolNoOverlap indicates when at least one symbol of the NR PDCCH candidate and the DMRS for demodulation of the NR PDCCH candidateis not overlapped with LTE CRS. Value someOrAllSymOverlap indicates when some or all of symbols of NR PDCCH candidate overlap with LTE CRS. - overlapInSymbol-r18 indicates reception of NR PDCCH candidates that overlap with LTE CRS REs on the X-th symbols of an NR slot: Value symbol2 indicates only 2nd symbol, Value symbol1And2 indicates 1st and 2nd symbols; The UE supporting this feature shall also indicate support of rateMatchingLTE-CRS. NOTE: This feature is supported by UE performing channel estimation with a regular Rel-15 DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Band | No | N/A | FR1 only |
| nr-PDCCH-OverlapLTE-CRS-RE-MultiPatterns-r18 Indicates whether the UE supports reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of one or multiple non-overlapping CRS rate matching patterns via lte-CRS-PatternList1-r16 if the UE supports multipleRateMatchingEUTRA-CRS-r16 or lte-CRS-PatternList3-r18 if the UE supports nr-PDCCH-OverlapLTE-CRS-RE-MultiPatterns-r18. The UE supporting of this feature shall also indicate support of nr-PDCCH-OverlapLTE-CRS-RE-r18 and at least one of multipleRateMatchingEUTRA-CRS-r16 and twoRateMatchingEUTRA-CRS-patterns-3-4-r18. NOTE: The feature is supported by UE performing channel estimation with a regular Rel-15 DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Band | No | N/A | FR1 only |
| nr-PDCCH-OverlapLTE-CRS-RE-Span-3-4-r18 Indicates whether the UE supports NR PDCCH that overlaps with LTE CRS REs is in Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS that are monitored within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot. The UE supporting of this feature shall also indicate support of nr-PDCCH-OverlapLTE-CRS-RE-r18 and pdcch-MonitoringSingleSpanFirst4Sym-r16. | Band | No | N/A | FR1 only |
| nr-UE-TxTEG-ID-MaxSupport-r17 Indicates the maximum number of UE TxTEG for SRS resource for positioning, which is supported and reported by UE for UL TDOA. The UE can include this field only if the UE supports srs-AllPosResources-r16. | Band | No | N/A | N/A |
| ntn-DMRS-BundlingNGSO-r18 Indicates whether the UE supports DM-RS bundling for PUSCH over consecutive slots in NGSO scenarios and pre-compensation to keep phase rotation due to timing drift within the phase difference limit. The UE indicates the maximum duration during which UE is able to maintain power consistency and phase continuity to support NTN DM-RS bundling for PUSCH over consecutive slots. A UE supporting this feature shall indicate support of uplinkPreCompensation-r17 and at least one of dmrs-BundlingPUSCH-RepTypeA-r17, dmrs-BundlingPUSCH-RepTypeB-r17 or dmrs-BundlingPUSCH-RepTypeC-r17. NOTE 1: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in Clause 5.2 of TS 38.104 [35]. NOTE 2: A UE that does not report support of this feature and reports support of maxDurationDMRS-Bundling-r17 for an NTN band can perform DMRS bundling only in GSO scenario in the NTN band. NOTE 3: DM-RS bundling is only applicable for UL transmissions with pi/2 BPSK, BPSK, and QPSK modulation orders. NOTE 4: For bands in Table 5.2.2-1 in TS 38.101-5 [34], reported value in maxDurationDMRS-Bundling-r17 is applied only for GSO scenario. | Band | No | N/A | N/A |
| olpc-SRS-Pos-r16 Indicates whether the UE supports OLPC for SRS for positioning. The capability signalling comprises the following parameters. - olpc-SRS-PosBasedOnPRS-Serving-r16 indicates whether the UE supports OLPC for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability-r16 defined in TS 37.355 [22], and srs-PosResources-r16. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnSSB-Neigh-r16 indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnPRS-Neigh-r16 indicates whether the UE supports OLPC for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving-r16. Otherwise, the UE does not include this field; NOTE: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. - maxNumberPathLossEstimatePerServing-r16 indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissios. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving-r16, olpc-SRS-PosBasedOnSSB-Neigh-r16 and olpc-SRS-PosBasedOnPRS-Neigh-r16. Otherwise, the UE does not include this field. | Band | No | N/A | N/A |
| olpc-SRS-PosRRC-Inactive-r17 Indicates whether the UE supports OLPC for SRS for positioning in RRC_INACTIVE. The capability signalling comprises the following parameters. - olpc-SRS-PosBasedOnPRS-Serving-r16 indicates whether the UE supports OLPC for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability-r16 defined in TS 37.355 [22], and srs-PosResourcesRRC-Inactive-r17. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnSSB-Neigh-r16 indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResourcesRRC-Inactive-r17. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnPRS-Neigh-r16 indicates whether the UE supports OLPC for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving-r16. Otherwise, the UE does not include this field; NOTE: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. - maxNumberPathLossEstimatePerServing-r16 indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving-r16, olpc-SRS-PosBasedOnSSB-Neigh-r16 and olpc-SRS-PosBasedOnPRS-Neigh-r16. Otherwise, the UE does not include this field. | Band | No | N/A | N/A |
| oneShotHARQ-feedbackPhy-Priority-r17 Indicates whether the UE supports transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI. A UE supporting this feature shall also indicate support of oneShotHARQ-feedback-r16 and twoHARQ-ACK-Codebook-type1-r16. | Band | No | N/A | N/A |
| oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 Indicates whether the UE supports one-shot HARQ ACK feedback triggered by DCI format 1_2, comprised of the following functional components: - Supports feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH; - Supports feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value. A UE supporting this feature shall also indicate support of oneShotHARQ-feedback-r16 and dci-Format1-2And0-2-r16. | Band | No | N/A | N/A |
| oneSlotPeriodicTRS-r16 Indicates whether the UE supports one-slot periodic TRS configuration only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated. If the UE supports this feature, the UE needs to report csi-RS-ForTracking. | Band | No | TDD only | FR1 only |
| outOfOrderOperationDL-r16 Indicates whether the UE supports out of order operation for DL. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16. The capability signalling comprises the following parameters: - supportPDCCH-ToPDSCH-r16 indicates support out-of-order operation for PDCCH to PDSCH; - supportPDSCH-ToHARQ-ACK-r16 indicates support out-of-order operation for PDSCH to HARQ-ACK. | Band | No | N/A | N/A |
| outOfOrderOperationUL-r16 Indicates whether the UE supports out of order operation for UL. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16. Note: Same closed loop index for power control across PUSCHs associated with different CORESETPoolIndex values is not supported by a UE indicating the support of this feature when TPC accumulation is enabled. | Band | No | N/A | N/A |
| overlapPDSCHsFullyFreqTime-r16 Indicates the maximal number of PDSCH scrambling sequences per serving cell when the UE supports PDSCHs with fully overlapping Resource Elements. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16. Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a Cyclic Prefix | Band | No | N/A | N/A |
| overlapPDSCHsInTimePartiallyFreq-r16 Indicates whether the UE supports PDSCHs with partially overlapping Resource Elements. The UE that indicates support of this feature shall support overlapPDSCHsFullyFreqTime-r16. | Band | No | N/A | N/A |
| overlapRateMatchingEUTRA-CRS-r16 Indicates whether the UE supports two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz SCS overlapping with a LTE carrier. If the UE supports this feature, the UE needs to report multipleRateMatchingEUTRA-CRS-r16 and multiDCI-MultiTRP-r16. | Band | No | N/A | FR1 only |
| overlapRateMatchingEUTRA-CRS-Patterns-3-4-Diff-CS-Pool-r18 Indicates whether the UE supports two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier for the case when crs-RateMatchPerCoresetPoolIndex is configured. UE supporting this feature shall support twoRateMatchingEUTRA-CRS-patterns-3-4-r18 and multiDCI-MultiTRP-r16. | Band | No | N/A | FR1 only |
| overlapUL-TransReduction-r18 Indicates whether the UE supports reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is not configured with UL STx2P for multi-DCI based multi-TRP operation with two TA enhancement. A UE supporting this feature shall indicate support of multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18. NOTE: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions). | Band | No | N/A | N/A |
| parallelMeasurementWithoutRestriction-r17 Indicates whether the UE supports measurements on cells belonging to different satellites as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release. If not reported, for measurements in parallel with normal operation of serving cell scheduling restrictions shall apply. | Band | No | FDD only | FR1 only |
| parallelPRS-MeasRRC-Inactive-r17 Indicates whether the UE supports performing RRM measurement and PRS measurement in parallel. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively | Band | No | N/A | N/A |
| pdcch-MonitoringResumptionAfterUL-NACK-r18 Indicates whether the UE supports PDCCH monitoring resumption after UL NACK. The UE indicating support of this feature shall also indicate support of pdcch-SkippingWithoutSSSG-r17. | Band | No | N/A | N/A |
| pdcch-SkippingWithoutSSSG-r17 Indicates whether the UE supports up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured as specified in TS 38.213 [11], clause 10.4. | Band | No | N/A | N/A |
| pdcch-SkippingWithSSSG-r17 Indicates whether the UE supports 2-bit indication of SSSG switching between 2 SSSGs, PDCCH skipping by scheduling DCI, and timer based SSSG switching as specified in TS 38.213 [11], clause 10.4. UE supports search space set group switching capability-1 according to Table 10.4-1 of TS 38.213 [11]. UE indicating support of this feature shall also indicate support of pdcch-SkippingWithoutSSSG-r17 and sssg-Switching-1bitInd-r17. | Band | No | N/A | N/A |
| pdc-maxNumberPRS-ResourceProcessedPerSlot-r18 Indicates the maximum number of single-symbol DL-PRS resources used in RTT-based Propagation delay compensation that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. A UE which supports pdc-maxNumberPRS-ResourceProcessedPerSlot-r18 shall support single-symbol DL-PRS for PDC with the comb sizes from {2,4,6,12}. A UE supporting this feature shall also indicate support of rtt-BasedPDC-PRS-r17. | Band | No | N/A | N/A |
| pdsch-1024QAM-2MIMO-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1 and shall not indicate support of pdsch-1024QAM-FR1-r17. | Band | No | N/A | FR1 only |
| pdsch-1024QAM-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1 and shall not indicate support of pdsch-1024QAM-2MIMO-FR1-r17. | Band | No | N/A | FR1 only |
| pdsch-256QAM-FR2 Indicates whether the UE supports 256QAM modulation scheme for PDSCH for FR2 as defined in 7.3.1.2 of TS 38.211 [6]. | Band | No | N/A | FR2 only |
| pdsch-MappingTypeB-Alt-r16 Indicates whether the UE supports PDSCH Type B scheduling of length 9 and 10 OFDM symbols, and DMRS shift for length-10 symbols. If the UE supports this feature, the UE needs to report pdsch-MappingTypeB. | Band | No | N/A | FR1 only |
| periodicBeamReport Indicates whether UE supports periodic 'CRI/RSRP' or 'SSBRI/RSRP' reporting using PUCCH formats 2, 3 and 4 in one slot. | Band | Yes | N/A | N/A |
| posJointTriggerBySingleDCI-RRC-Connected-r18 Indicates whether UE supports a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers for SRS bandwidth aggregation in RRC_CONNECTED state. A UE indicating support of this feature shall also indicate support of posSRS-BWA-RRC-Connected-r18. | Band | No | N/A | N/A |
| posSRS-BWA-RRC-Inactive-r18 Indicates the UE capability for support of positioning SRS bandwidth aggregation in RRC_INACTIVE and the support of the same SRS power reduction across aggregated carriers. The capability signalling comprises the following parameters: - numOfCarriersIntraBandContiguous-r18 indicates the number of supported aggregated carriers in intra band contiguous carriers, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR1-r18 indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR2-r18 indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR1-r18 indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR2-r18 indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedResourceSet-r18 indicates the max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodic-r18 indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceSemi-r18 indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodicPerSlot-r18 indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceSemiPerSlot-r18 indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - guardPeriod-r18 indicates the guard period in microseconds before and after aggregated SRS transmission. - powerClassForTwoAggregatedCarriers-r18 indicates the power class of supported two aggregated carriers in intra band contiguous carriers. - powerClassForThreeAggregatedCarriers-r18 indicates the power class of supported three aggregated carriers in intra band contiguous carriers. NOTE: The power class is only applicable for FR1 bands. UE indicating support of this feature shall also indicate support of posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17. | Band | No | N/A | N/A |
| posSRS-PreconfigureRRC-InactiveInitialUL-BWP-r18 Indicates whether the UE supports preconfigured SRS with validity area in RRC_INACTIVE for initial UL BWP. UE indicating support of this feature shall also indicate support of posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18. | Band | No | N/A | N/A |
| posSRS-PreconfigureRRC-InactiveOutsideInitialUL-BWP-r18 Indicates whether the UE supports preconfigured SRS with validity area in RRC_INACTIVE outside initial UL BWP. UE indicating support of this feature shall also indicate support of posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP-r18. | Band | No | N/A | N/A |
| posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 Indicates support of Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP. The capability signalling comprises the following parameters: - maxSRSposBandwidthForEachSCS-withinCC-FR1-r17 Indicates the maximum SRS bandwidth supported for each SCS that UE supports within a single CC for FR1; - maxSRSposBandwidthForEachSCS-withinCC-FR2-r17 indicates the maximum SRS bandwidth supported for each SCS that UE supports within a single CC for FR2; - maxNumOfSRSposResourceSets-r17 indicates the max number of SRS Resource Sets for positioning supported by UE; - maxNumOfPeriodicSRSposResources-r17 indicates the max number of periodic SRS Resources for positioning; - maxNumOfPeriodicSRSposResourcesPerSlot-r17 indicates the max number of periodic SRS Resources for positioning per slot; - differentNumerologyBetweenSRSposAndInitialBWP-r17 indicates the support of different numerology between the SRS and the initial UL BWP; - srsPosWithoutRestrictionOnBWP-r17 indicates the support of SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB; - maxNumOfPeriodicAndSemipersistentSRSposResources-r17 indicates the max number of P/SP SRS Resources for positioning; - maxNumOfPeriodicAndSemipersistentSRSposResourcesPerSlot-r17 indicates the max number of P/SP SRS Resources for positioning per slot; - differentCenterFreqBetweenSRSposAndInitialBWP-r17 indicates the support of a different center frequency between the SRS for positioning and the initial UL BWP; - switchingTimeSRS-TX-OtherTX-r17 indicates the switching time between SRS TX and other TX in initial UL BWP or RX in initial DL BWP - maxNumOfSemiPersistentSRSposResources-r17 indicates the max number of semi-persistent SRS Resources for positioning; - maxNumOfSemiPersistentSRSposResourcesPerSlot-r17 indicates the max number of semi-persistent SRS Resources for positioning per slot. The UE can include this field only if the UE supports srs-PosResourcesRRC-Inactive-r17. Otherwise, the UE does not include this field; NOTE 1: The BWP with SRS for positioning is defined by the parameters locationAndBandwidth, SCS, CP in the same way as other BWPs. NOTE 2: If differentCenterFreqBetweenSRSposAndInitialBWP-r17 is not signalled, the UE only supports same center frequency between the SRS for positioning and initial UL BWP. NOTE 3: If differentNumerologyBetweenSRSposAndInitialBWP-r17 is not signalled, the UE only supports same numerology between the SRS and the initial UL BWP. NOTE 4: If srsPosWithoutRestrictionOnBWP-r17 is not signalled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB. NOTE 5: The fields of maxNumOfSemiPersistentSRSposResources-r17 and maxNumOfSemiPersistentSRSposResourcesPerSlot-r17 shall be reported together if supported by UE. One of the fields between maxSRSposBandwidthForEachSCS-withinCC-FR1-r17 and maxSRSposBandwidthForEachSCS-withinCC-FR2-r17, and the fields of maxNumOfSRSposResourceSets-r17, maxNumOfPeriodicSRSposResources-r17, maxNumOfPeriodicSRSposResourcesPerSlot-r17, maxNumOfPeriodicAndSemipersistentSRSposResources-r17, maxNumOfPeriodicAndSemipersistentSRSposResourcesPerSlot-r17, and switchingTimeSRS-TX-OtherTX-r17 shall be reported together if supported by UE. NOTE 6: srsPosWithoutRestrictionOnBWP-r17 is not applicable to FDD or SUL bands. | Band | No | N/A | N/A |
| posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18 Indicates whether the UE support SRS for positioning configuration in multi cells in RRC_INACTIVE for initial UL BWP. UE indicating support of this feature shall also indicate support of posSRS-RRC-Inactive-InInitialUL-BWP-r17. | Band | No | N/A | N/A |
| posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP-r18 Indicates whether the UE supports SRS for positioning configuration in multi cells in RRC_INACTIVE outside initial UL BWP. UE indicating support of this feature shall also indicate support of posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 and posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18. | Band | No | N/A | N/A |
| posUE-TA-AutoAdjustment-r18 Indicates whether the UE supports autonomous TA adjustment when cell-reselection happens. UE indicating support of this feature shall also indicate support of posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18. | Band | No | N/A | N/A |
| powerAdaptation-CSI-Feedback-r18 Indicates whether the UE supports power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting and single-panel type 1 codebook. The UE supports CSI feedback based on CSI report sub-configuration(s), each containing one power offset for periodic CSI reporting. This capability signalling comprises the following parameters: - maxNumberLmax-r18 indicates the max number of sub-configurations Lmax in one CSI report configuration; - maxNumberCSI-ResourcePerCC-r18 indicates the maximum number of simultaneous NZP-CSI-RS resources per CC. - maxNumberTotalCSI-ResourcePerCC-r18 indicates the maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC. - totalNumberCSI-Reporting-r18 indicates total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP. NOTE 1: For maxNumberCSI-ResourcePerCC-r18 and maxNumberTotalCSI-ResourcePerCC-r18, NZP-CSI-RS resource and CSI-RS ports are counted for reporting settings with and without sub-configurations. NOTE 2: If a UE reports more than one capability from spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the above reported features, then the supported maximum of NZP-CSI-RS resources/ports across all periodic, semi-persistent, aperiodic CSI report settings with sub-configurations corresponding to all of spatial and power domain adaptations and without sub-configurations is determined by the minimum of the reported values from that subset. NOTE 3: If a UE reports both spatialAdaptation-CSI-Feedback-r18 and powerAdaptation-CSI-Feedback-r18, and if the UE is configured with CSI report settings with sub-configurations corresponding to both spatialAdaptation-CSI-Feedback-r18 and powerAdaptation-CSI-Feedback-r18, then the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP is determined by the minimum of the reported values from both spatialAdaptation-CSI-Feedback-r18 and powerAdaptation-CSI-Feedback-r18. A UE indicating support of this feature shall also indicate support of csi-ReportFramework and powerAdaptation-CSI-FeedbackPerBC-r18. | Band | No | N/A | N/A |
| powerAdaptation-CSI-FeedbackAperiodic-r18 Indicates whether the UE supports power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting and single-panel type 1 codebook. The UE supports CSI feedback based on CSI report sub-configuration(s), each containing one power offset for aperiodic CSI reporting. This capability signalling comprises the following parameters: - maxNumberLmax-r18 indicates the max number of sub-configurations Lmax in one CSI report configuration; - subReportCSI-r18 indicates N number of report of CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. - maxNumberCSI-ResourcePerCC-r18 indicates the maximum number of simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - maxNumberTotalCSI-ResourcePerCC-r18 indicates the maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - totalNumberCSI-Reporting-r18 total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP. NOTE 1: For maxNumberCSI-ResourcePerCC-r18 and maxNumberTotalCSI-ResourcePerCC-r18, NZP-CSI-RS resource and CSI-RS ports are counted for reporting settings with and without sub-configurations. NOTE 2: If a UE reports more than one capability from spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the above reported features, then the supported maximum of NZP-CSI-RS resources/ports across all periodic, semi-persistent, aperiodic CSI report settings with sub-configurations corresponding to all of spatial and power domain adaptations and without sub-configurations is determined by the minimum of the reported values from that subset. NOTE 3: If a UE reports both spatialAdaptation-CSI-FeedbackAperiodic-r18 and powerAdaptation-CSI-FeedbackAperiodic-r18, and if the UE is configured with CSI report settings with sub-configurations corresponding to both spatialAdaptation-CSI-FeedbackAperiodic-r18 and powerAdaptation-CSI-FeedbackAperiodic-r18, then the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP is determined by the minimum of the reported values from both spatialAdaptation-CSI-FeedbackAperiodic-r18 and powerAdaptation-CSI-FeedbackAperiodic-r18. A UE indicating support of this feature shall also indicate support of csi-ReportFramework and powerAdaptation-CSI-FeedbackAperiodicPerBC-r18. | Band | No | N/A | N/A |
| powerAdaptation-CSI-FeedbackPUCCH-r18 Indicates whether the UE supports power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting on PUCCH and single-panel type 1 codebook. The UE supports CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting on PUCCH. This capability signalling comprises the following parameters: - maxNumberLmax-r18 indicates the max number of sub-configurations Lmax in one CSI report configuration; - subReportCSI-r18 indicates N number of report of CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. - maxNumberCSI-ResourcePerCC-r18 indicates the maximum number of simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - maxNumberTotalCSI-ResourcePerCC-r18 indicates the maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - totalNumberCSI-Reporting-r18 indicates total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across semi-persistent CSI report settings with sub-configurations per BWP. NOTE 1: For maxNumberCSI-ResourcePerCC-r18 and maxNumberTotalCSI-ResourcePerCC-r18, NZP-CSI-RS resource and CSI-RS ports are counted for reporting settings with and without sub-configurations. NOTE 2: If a UE reports more than one capability from spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the above reported features, then the supported maximum of NZP-CSI-RS resources/ports across all periodic, semi-persistent, aperiodic CSI report settings with sub-configurations corresponding to all of spatial and power domain adaptations and without sub-configurations is determined by the minimum of the reported values from that subset. NOTE 3: If a UE reports more than one capability from spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-FeedbackPUSCH-r18 and powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the reported features, then the supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across semi-persistent CSI report settings with sub-configurations per BWP is determined by the minimum of the reported values from that subset. A UE indicating support of this feature shall also indicate support of csi-ReportFramework, sp-CSI-ReportPUCCH and powerAdaptation-CSI-FeedbackPUCCH-PerBC-r18. | Band | No | N/A | N/A |
| powerAdaptation-CSI-FeedbackPUSCH-r18 Indicates whether the UE supports power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting on PUSCH and single-panel type 1 codebook. The UE supports CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting. This capability signalling comprises the following parameters: - maxNumberLmax-r18 indicates the max number of sub-configurations Lmax in one CSI report configuration; - subReportCSI-r18 indicates N number of report of CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. - maxNumberCSI-ResourcePerCC-r18 indicates the maximum number of simultaneous NZP-CSI-RS resources per CC. - maxNumberTotalCSI-ResourcePerCC-r18 indicates the maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC. - totalNumberCSI-Reporting-r18 indicates total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across semi-persistent CSI report settings with sub-configurations per BWP. NOTE 1: For maxNumberCSI-ResourcePerCC-r18 and maxNumberTotalCSI-ResourcePerCC-r18, NZP-CSI-RS resource and CSI-RS ports are counted for reporting settings with and without sub-configurations. NOTE 2: If a UE reports more than one capability from spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the above reported features, then the supported maximum of NZP-CSI-RS resources/ports across all periodic, semi-persistent, aperiodic CSI report settings with sub-configurations corresponding to all of spatial and power domain adaptations and without sub-configurations is determined by the minimum of the reported values from that subset. NOTE 3: If a UE reports more than one capability from spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-FeedbackPUSCH-r18 and powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the reported features, then the supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across semi-persistent CSI report settings with sub-configurations per BWP is determined by the minimum of the reported values from that subset. A UE indicating support of this feature shall also indicate support of csi-ReportFramework, sp-CSI-ReportPUSCH and powerAdaptation-CSI-FeedbackPUSCH-PerBC-r18. | Band | No | N/A | N/A |
| powerBoosting-pi2BPSK Indicates whether UE supports power boosting for pi/2 BPSK, when applicable as defined in 6.2 of TS 38.101-1 [2] v16.9.0. It is mandatory with capability signalling. This capability is not applicable to IAB-MT. | Band | CY | TDD only | FR1 only |
| prach-CoverageEnh-r18 Indicates whether the UE supports {2, 4, 8} for the number of multiple PRACH transmissions with same Tx spatial filter. | Band | No | N/A | N/A |
| prach-Repetition-r18 Indicates whether the UE supports transmitting two PRACH repetitions when a gap between the last symbol of a PRACH repetition in the first slot and the first symbol of a PRACH repetition in the second slot is less than N symbols, where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the SCS configuration for the UL BWP with the PRACH. A UE supporting this feature shall also indicate support of prach-CoverageEnh-r18. | Band | No | N/A | N/A |
| priorityIndicatorInDCI-Multicast-r17 Indicates whether the UE supports DL priority indication for multicast in DCI, comprised of the following functional components: - Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast; - Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE supporting this feature shall also indicate support of ack-NACK-FeedbackForMulticast-r17 and dynamicMulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| priorityIndicatorInDCI-SPS-Multicast-r17 Indicates whether the UE supports priority indicator field configured in DCI format 4_2 for multicast HARQ-ACK feedback of SPS multicast. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE supporting this feature shall also indicate support of ack-NACK-FeedbackForSPS-Multicast-r17 and sps-MulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| prs-MeasurementWithoutMG-r17 Indicates whether the UE supports using the threshold to compare the Rx time difference between the serving cell and a neighbour cell/TRP for PRS measurements, as defined in clause 9.9.1.2 of TS 38.133 [5], to determine whether the PRS from the non-serving cell satisfy the condition of PRS measurement outside MG. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A-r17, prs-ProcessingWindowType1B-r17 and prs-ProcessingWindowType2-r17. | Band | No | N/A | N/A |
| prs-ProcessingCapabilityOutsideMGinPPW-r17 Indicates the DL-PRS Processing Capability outside MG of each of the supported PRS Processing Window (PPW) Type in the case the UE supports multiple PPW Types in a band and comprises the following parameters: - prsProcessingType-r17: Indicates the PPW Type for which the prs-ProcessingCapabilityOutsideMGinPPW-r17 are provided. - ppw-dl-PRS-BufferType-r17: Indicates DL-PRS buffering capability. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering. - ppw-durationOfPRS-Processing1-r17: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth-r17 and comprises the following parameters: - ppw-durationOfPRS-ProcessingSymbolsN-r17: This field specifies the values for N with values msDot125 indicates 0.125ms, msDot25 indicates 0.25ms, and so on - ppw-durationOfPRS-ProcessingSymbolsT-r17: This field specifies the values for T with values ms1 indicates 1ms, ms2 indicates 2ms, and so on. - ppw-durationOfPRS-Processing2-r17: Indicates the duration of DL-PRS symbols N2 in units of ms a UE can process every T2 ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth-r17 and comprises the following parameters: - ppw-durationOfPRS-ProcessingSymbolsN2-r17: This field specifies the values for N2 with values msDot125 indicates 0.125ms, msDot25 indicates 0.25ms, and so on. - ppw-durationOfPRS-ProcessingSymbolsT2-r17: This field specifies the values for T2 with values ms4 indicates 4ms, ms5 indicates 5ms, and so on. - ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17: Indicates the maximum number of DL PRS bandwidth in MHz, which is supported and reported by UE for PRS measurement outside MG within the PPW. - ppw-maxNumOfDL-Bandwidth-r17: Indicates the maximum number of DL PRS bandwidth in MHz for FR1 and FR2, which is supported and reported by UE for PRS measurement outside MG within the PPW. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A-r17, prs-ProcessingWindowType1B-r17 and prs-ProcessingWindowType2-r17. Otherwise, the UE does not include this field. NOTE 1: A UE that supports one of prs-ProcessingWindowType1A-r17, prs-ProcessingWindowType1B-r17 or prs-ProcessingWindowType2-r17 shall always include the prs-ProcessingCapabilityOutsideMGinPPW-r17. NOTE 2: The (N, T) in ppw-durationOfPRS-Processing1-r17 is interpreted as in (N,T) in durationOfPRS-Processing-r16 in TS 37.355 [22], and the UE is expected to receive the DL-PRS within the PPW but the processing of the received DL-PRS may be outside a PPW NOTE 3: The (N2, T2) in ppw-durationOfPRS-Processing2-r17 is interpreted such that the UE is capable of measuring up to N2 ms DL-PRS within a PPW and is capable of completing the DL-PRS processing within the PPW, e.g., if the time duration from the last symbol of the measured DL-PRS resource(s) inside the PPW to the end of PPW is not smaller than T2 ms. NOTE 4: A UE which supports prs-ProcessingCapabilityOutsideMGinPPW-r17 shall support either ppw-durationOfPRS-Processing1-r17 or ppw-durationOfPRS-Processing2-r17, but not both for each supported PPW type in a band. | Band | No | N/A | N/A |
| prs-ProcessingRRC-Inactive-r17 Indicates whether the UE supports PRS processing in RRC_INACTIVE. | Band | No | N/A | N/A |
| prs-ProcessingWindowType1A-r17 Indicates whether the UE supports PRS processing Type 1A, subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window and the priority handling options of PRS as follows: - Option 1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [12]. - Option 2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [12]. NOTE 1: Void. - Option 3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [12]. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList-r16 defined in TS 37.355 [22]. A UE supporting this feature shall also indicate support of prs-ProcessingCapabilityOutsideMGinPPW-r17. NOTE 2: Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR. NOTE 3: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP. NOTE 4: Support of configuration of PRS processing window in RRC and support of using DL MAC CE to activate/deactivate the PRS processing window for PRS measurements is part of the feature. NOTE 5: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS. | Band | No | N/A | N/A |
| prs-ProcessingWindowType1B-r17 Indicates whether the UE supports PRS processing Type 1B, subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window and the priority handling options of PRS as follows: - Option 1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [12]. - Option 2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [12]. NOTE 1: Void. - Option 3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [12]. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList-r16 defined in TS 37.355 [22]. A UE supporting this feature shall also indicate support of prs-ProcessingCapabilityOutsideMGinPPW-r17. NOTE 2: Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected. NOTE 3: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP. NOTE 4: Support of configuration of PRS processing window in RRC and support of using DL MAC CE to activate/deactivate the PRS processing window for PRS measurements is part of the feature. NOTE 5: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS. | Band | No | N/A | N/A |
| prs-ProcessingWindowType2-r17 Indicates whether the UE supports PRS processing Type 2, subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window and the priority handling options of PRS as follows: - Option 1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [12]. - Option 2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [12]. NOTE 1: Void. - Option 3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [12]. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList-r16 defined in TS 37.355 [22]. A UE supporting this feature shall also indicate support of prs-ProcessingCapabilityOutsideMGinPPW-r17. NOTE 2: Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window. NOTE 3: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP. NOTE 4: Support of configuration of PRS processing window in RRC and support of using DL MAC CE to activate/deactivate the PRS processing window for PRS measurements is part of the feature. NOTE 5: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS. | Band | No | N/A | N/A |
| ptrs-DensityRecommendationSetDL For each supported sub-carrier spacing, indicates preferred threshold sets for determining DL PTRS density. It is mandated for FR2. For each supported sub-carrier spacing, this field comprises: - two values of frequencyDensity; - three values of timeDensity. | Band | CY | N/A | N/A |
| ptrs-DensityRecommendationSetUL For each supported sub-carrier spacing, indicates preferred threshold sets for determining UL PTRS density. For each supported sub-carrier spacing, this field comprises: - two values of frequencyDensity; - three values of timeDensity; - five values of sampleDensity. | Band | No | N/A | N/A |
| pucch-RepetitionDynamicIndicationSFN-r18 Indicates whether the UE supports STx2P SFN PUCCH scheme together with pucch-Repetition-F0-1-2-3-4-DynamicIndication-r17. A UE supporting this feature shall also indicate support of pucch-SingleDCI-STx2P-SFN-r18 and slotBasedDynamicPUCCH-Rep-r17. | Band | No | N/A | FR2 only |
| pucch-Repetition-F0-2-r17 Indicates whether the UE supports transmission of a PUCCH format 0 and 2 over multiple slots with the repetition factor 2, 4 or 8. A UE supporting this feature shall also indicate support of pucch-Repetition-F1-3-4. | Band | No | N/A | N/A |
| pucch-SpatialRelInfoMAC-CE Indicates whether the UE supports indication of PUCCH-spatialrelationinfo by a MAC CE per PUCCH resource. It is mandatory for FR2 and optional for FR1. | Band | CY | N/A | N/A |
| pusch-256QAM Indicates whether the UE supports 256QAM modulation scheme for PUSCH as defined in 6.3.1.2 of TS 38.211 [6]. | Band | No | N/A | N/A |
| pusch-CB-2PTRS-SingleDCI-STx2P-SDM-r18 Indicates whether the UE supports 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH codebook. A UE supporting this feature shall also indicate support of pusch-CB-SingleDCI-STx2P-SDM-r18. | Band | No | N/A | FR2 only |
| pusch-CB-2PTRS-SingleDCI-STx2P-SFN-r18 Indicates whether the UE supports 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH codebook. A UE supporting this feature shall also indicate support of pusch-CB-SingleDCI-STx2P-SFN-r18. | Band | No | N/A | FR2 only |
| pusch-NonCB-2PTRS-SingleDCI-STx2P-SDM-r18 Indicates whether the UE supports 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook. A UE supporting this feature shall also indicate support of pusch-NonCB-SingleDCI-STx2P-SDM-r18. | Band | No | N/A | FR2 only |
| pusch-NonCB-2PTRS-SingleDCI-STx2P-SFN-r18 Indicates whether the UE supports 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook. A UE supporting this feature shall also indicate support of pusch-NonCB-SingleDCI-STx2P-SFN-r18. | Band | No | N/A | FR2 only |
| pusch-NonCB-SingleDCI-STx2P-SDM-CSI-RS-SRS-r18 Indicates whether the UE supports up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STx2P SDM scheme for PUSCH. This capability comprises: - maxNumberPeriodicSRS-Resource-PerBWP-r18 indicates the maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. - maxNumberAperiodicSRS-Resource-PerBWP-r18 indicates the maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. - maxNumberSemiPersistentSRS-ResourcePerBWP-r18 indicates the maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. - valueY-SRS-ResourceAssociate-r18 indicates UE can process (Y) SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS - valueX-CSI-RS-ResourceAssociate-r18 indicates UE can process up to (X) CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously A UE supporting this feature shall also indicate support of srs-AssocCSI-RS and pusch-NonCB-SingleDCI-STx2P-SDM-r18. | Band | No | N/A | FR2 only |
| pusch-NonCB-SingleDCI-STx2P-SFN-CSI-RS-SRS-r18 Indicates whether the UE supports up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STx2P SFN scheme for PUSCH. This capability comprises: - maxNumberPeriodicSRS-Resource-PerBWP-r18 indicates the maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. - maxNumberAperiodicSRS-Resource-PerBWP-r18 indicates the maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. - maxNumberSemiPersistentSRS-ResourcePerBWP-r18 indicates the maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. - valueY-SRS-ResourceAssociate-r18 indicates UE can process (Y) SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS - valueX-CSI-RS-ResourceAssociate-r18 indicates UE can process up to (X) CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously A UE supporting this feature shall also indicate support of srs-AssocCSI-RS and pusch-NonCB-SingleDCI-STx2P-SFN-r18. | Band | No | N/A | FR2 only |
| pusch-RepetitionMsg3-r17 Indicates whether the UE supports repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | Band | No | N/A | N/A |
| pusch-RepetitionMultiSlots-v1650 Indicates whether the UE supports transmitting PUSCH scheduled by DCI format 0_1 when configured with pusch-AggregationFactor > 1, as defined in clause 6.1.2.1 of TS 38.214 [12]. This applies only to non-shared spectrum channel access. For shared spectrum channel access, pusch-RepetitionMultiSlots-r16 applies. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The UE only includes pusch-RepetitionMultiSlots-v1650 if pusch-RepetitionMultiSlots is absent. | Band | Yes | N/A | N/A |
| pusch-RepetitionTypeA-v16c0 Indicates whether the UE supports the dynamic indication of the number of repetitions for PUSCH transmission as specified in TS 38.214 [12], clause 6.1.2.1. Support of this field is reported for shared spectrum channel access and non-shared spectrum channel access, respectively. UE indicating support of this feature shall support of at least one of type2-PUSCH-RepetitionMultiSlots and pusch-RepetitionMultiSlots for shared spectrum and non-shared spectrum respectively. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. The UE only includes pusch-RepetitionTypeA-v16c0 if pusch-RepetitionTypeA-r16 is absent. | Band | No | N/A | N/A |
| pusch-TransCoherence Defines support of the uplink codebook subset by the UE for UL precoding for PUSCH transmission as described in clause 6.1.1.1 of TS 38.214 [12]. UE indicated support of partial coherent codebook subset shall also support non-coherent codebook subset. UE indicated support of full coherent codebook subset shall also support partial and non-coherent codebook subset. | Band | No | N/A | N/A |
| puschTypeA-RepetitionsAvailSlot-r17 Indicates whether UE supports dynamic and configured grant PUSCH repetitions based on available slots. Transmission occasions for the repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. A UE that indicates support of this feature shall support type1-PUSCH-RepetitionMultiSlots, type2-PUSCH-RepetitionMultiSlots or pusch-RepetitionMultiSlots. | Band | No | N/A | N/A |
| rach-EarlyTA-Measurement-r18 Indicates the maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure before receiving cell switch command MAC-CE. Power ramping for PRACH retransmission based on PDCCH order indication. UE also supports dropping the serving cell UL to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s). A UE supporting this feature shall also indicate support of ta-IndicationCellSwitch-r18 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. | Band | No | N/A | N/A |
| rach-LessHandoverCG-r18 Indicates whether the UE supports RACH-less handover with configured grant for SpCell, as specified in TS 38.321 [8]. In this release, FR1-FR2 and FDD-TDD RACH-less handovers with configured grant are not supported. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. For NTN bands, a UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. If an NTN UE indicates the support of both timeBasedCondHandover-r17 and rach-LessHandoverCG-r18, the UE supports time based RACH-less CHO with configured grant. | Band | No | N/A | N/A |
| rach-LessHandoverDG-r18 Indicates whether the UE supports RACH-less handover with dynamic grant for SpCell, as specified in TS 38.321 [8]. In this release, FR1-FR2 and FDD-TDD RACH-less handovers with dynamic grant are not supported. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. For NTN bands, a UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. If an NTN UE indicates the support of both timeBasedCondHandover-r17 and rach-LessHandoverDG-r18, the UE supports time based RACH-less CHO with dynamic grant. | Band | No | N/A | N/A |
| rateMatchingLTE-CRS Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS, as specified in TS 38.214 [12]. | Band | Yes | N/A | N/A |
| releaseSPS-MulticastWithCS-RNTI-r17 Indicates whether UE supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE that indicates the support of this feature shall indicate support of sps-Multicast-r17 and sps-r16. | Band | No | N/A | N/A |
| re-LevelRateMatchingForMulticast-r17 Indicates whether the UE supports group-common PDSCH RE-level rate matching for multicast, comprised of the following functional components: - Supports SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns; - Supports P ZP-CSI-RS for group-common PDSCH RE-mapping patterns; - Supports p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config; - Supports AP ZP-CSI-RS for group-common PDSCH RE-mapping patterns. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. A UE supporting this feature in FR1 bands shall also indicate support of pdsch-RE-MappingFR1-PerSymbol or pdsch-RE-MappingFR1-PerSlot. A UE supporting this feature in FR2 bands shall also indicate support of pdsch-RE-MappingFR2-PerSymbol or pdsch-RE-MappingFR2-PerSlot. NOTE: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16. | Band | No | N/A | N/A |
| rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 Indicates whether the UE supports RLM/BM/BFD measurements based on CSI-RS, when CD-SSB is outside active DL BWP. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell. The UE also supports CSI-RS within active DL BWP for RLM/BM/BFD measurements can be QCLed with CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s). The UE supporting this feature shall also indicate support of csi-RS-RLM, beamManagementSSB-CSI-RS and maxNumberCSI-RS-BFD,maxNumberSSB-BFD, maxNumberCSI-RS-SSB-CBD. The UEs indicating the support of this feature group shall not indicate the support of bwp-WithoutRestriction. NOTE: The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig. It is not applicable to RedCap or eRedCap UEs. | Band | No | N/A | N/A |
| rlm-Relaxation-r17 Indicates whether the UE supports RLM relaxation criteria and requirement as specified in TS 38.133 [5]. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. UE indicating support of this feature shall also indicate support of ssb-RLM and/or csi-RS-RLM. | Band | No | N/A | N/A |
| searchSpaceSetGrp-switchCap2-r17 Indicates whether UE supports search space set group switching capability 2 for FR1 according to Table 10.4-1 of TS 38.213 [11] for SSSG switching. UE indicating support of this feature shall also indicate support of sssg-Switching-1bitInd-r17. NOTE: For UE supporting this feature and also sssg-Switching-1BitInd-r17, sssg-Switching-2BitInd-r17, and/or pdcch-SkippingWithSSSG-r17, search space set group switching Capability-2 is applied to sssg-Switching-1BitInd-r17, sssg-Switching-2BitInd-r17, and/or pdcch-SkippingWithSSSG-r17. | Band | No | N/A | FR1 only |
| semi-PersistentL1-SINR-Report-PUCCH-r16 Indicates whether the UE supports semi-persistent L1-SINR report on PUCCH. The UE indicating support of this feature shall include at least one of the following capabilities: - supportReportFormat1-2OFDM-syms-r16 indicates support of report on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) - supportReportFormat4-14OFDM-syms-r16 indicates support of report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH). The UE indicating support of this feature shall also indicate support of ssb-csirs-SINR-measurement-r16. | Band | No | N/A | N/A |
| semi-PersistentL1-SINR-Report-PUSCH-r16 Indicates whether the UE supports semi-persistent L1-SINR report on PUSCH. The UE indicating support of this feature shall also indicate support of ssb-csirs-SINR-measurement-r16. | Band | No | N/A | N/A |
| separateCRS-RateMatching-r16 Indicates whether the UE supports rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16 and overlapRateMatchingEUTRA-CRS-r16. This is only applicable for 15kHz SCS. | Band | No | N/A | FR1 only |
| sfn-DefaultDL-BeamSetup-r17 Indicates whether the UE supports the following features: - For FR2 only, PDSCH reception using default beam for enhanced SFN scheme when PDSCH is scheduled with offset less than threshold. - For FR1 and FR2, PDSCH reception using default beam for enhanced SFN scheme when TCI field is not present in DCI format 1_0/1_1/1_2 when PDSCH is scheduled with offset equal or larger than the threshold, if applicable. - For FR2 only, aperiodic CSI-RS reception using default beam for enhanced SFN scheme when scheduling offset is less than threshold. The UE indicating support of this feature shall also indicate sfn-schemeA-r17 or sfn-schemeB-r17. | Band | No | N/A | N/A |
| sfn-DefaultUL-BeamSetup-r17 Indicates whether the UE supports the following features: - Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured. - Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured. - Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured. The UE indicating support of this feature shall also indicate sfn-schemeA-r17 or sfn-schemeB-r17 or sfn-SchemeA-PDCCH-only-r17. | Band | No | N/A | FR2 only |
| sfn-ImplicitRS-twoTCI-r17 Indicates whether the UE supports RS(s) with two TCI states configured implicitly for beam failure detection enhancement for HST. | Band | No | N/A | N/A |
| sfn-QCL-TypeD-Collision-twoTCI-r17 Indicates whether the UE supports identification of two QCL-TypeD properties for multiple overlapping CORESETs when a CORESET is activated with two TCI states which overlaps with another CORESET. | Band | No | N/A | N/A |
| sfn-SimulTwoTCI-AcrossMultiCC-r17 Indicates whether the UE supports simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE. The UE indicating support of this feature shall also indicate sfn-schemeA-r17 or sfn-schemeB-r17 or sfn-SchemeA-PDCCH-only-r17. The UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| simul-SpatialRelationUpdatePUCCHResGroup-r16 Indicates whether the UE support PUCCH resource groups per BWP for simultaneous spatial relation update. The UE indicating support of this also indicates the capabilities of supported SRS resources and maximum supported spatial relations for the supported bands using supportedSRS-Resources, maxNumberConfiguredSpatialRelations and pucch-SpatialRelInfoMAC-CE. | Band | No | N/A | N/A |
| simulSRS-MIMO-TransWithinBand-r16 Indicates the number of SRS resources for positioning and SRS resource for MIMO on a symbol within a band across multiple CCs. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field. | Band | No | N/A | N/A |
| simulSRS-TransWithinBand-r16 Indicates the number of SRS resources for positioning on a symbol within a band across multiple CCs. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field. | Band | No | N/A | N/A |
| simultaneousCSI-SubReportsPerCC-r18 Indicates the number of CSI report(s) for which the UE can measure and process reference signals simultaneously in a CC of the band for which this capability is provided. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types, and includes the beam report, and CSI report without sub-configurations plus CSI sub-report across CSI reports. NOTE 1: UE shall report the value in this capability being equal to or larger than that in simultaneousCSI-ReportsPerCC. NOTE 2: UE supporting at least one of spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, and powerAdaptation-CSI-FeedbackPUCCH-r18 shall report this feature. A UE supporting this feature shall also indicate support of csi-ReportFramework. | Band | No | N/A | N/A |
| simultaneousReceptionDiffTypeD-r16 Indicates whether the UE supports simultaneous reception with different QCL Type D reference signal as specified in TS 38.213 [11]. | Band | No | N/A | FR2 only |
| simultaneousReceptionTwoQCL-r18 Indicates whether the UE supports enhanced RF requirement to support FR2-1 PC6 UEs with simultaneous DL signals reception with two different QCL TypeD RSs and enhanced RRM requirement to support FR2-1 PC6 UEs with simultaneous DL signals reception associated with two different QCL TypeD RSs. This feature is applied when highSpeedDeploymentTypeFR2-r17 is configured by network as bidirectional. A UE supporting this feature shall also indicate support of PC6 in ue-PowerClass-v1700. | Band | No | N/A | FR2 only |
| simulTX-SRS-AntSwitchingIntraBandUL-CA-r16 Indicates whether the UE support simultaneous transmission of SRS on different CCs for intra-band UL CA. The UE indicating support of this feature shall include at least one of the following capabilities: - supportSRS-xTyR-xLessThanY-r16 indicates support transmission of SRS for xTyR (x<y) based antenna switching and SRS for CB/NCB/BM on different CCs in overlapped symbol(s) for intra-band UL CA. - supportSRS-xTyR-xEqualToY-r16 indicates support transmission of SRS for xTyR (x=y) based antenna switching and SRS for CB/NCB/BM on different CCs in overlapped symbol(s) for intra-band UL CA. - supportSRS-AntennaSwitching-r16 Indicates whether the UE support simultaneous transmission of SRS for antenna switching on different CCs in overlapped symbol(s) for intra-band UL CA. NOTE: For simultaneously antenna switching and antenna switching SRS in intra-band CAs with bands whose UL are switched together according to the reported supportSRS-AntennaSwitching-r16, the UE expects the same configuration of xTyR across the different CCs and the SRS resources overlapped in time domain from UE perspective are from the same UE antenna ports. | Band | No | N/A | N/A |
| sn-InitiatedCondPSCellChangeNRDC-r17 Indicates whether the UE supports SN initiated inter-SN conditional PSCell change in NR-DC, which is configured by NR conditionalReconfiguration using SN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in SN initiated inter-SN conditional PSCell change in NR-DC. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. | Band | No | N/A | N/A |
| spatialAdaptation-CSI-Feedback-r18 Indicates whether the UE supports spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting and single-panel type 1 codebook. This capability signalling comprises the following parameters: - csiFeedbackType-r18 indicates CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS resource IDs for periodic CSI reporting. Value sdType1 indicates support of SD-type1, value sdType2 indicates support of SD-type2, value both indicates support of both SD-type1 and SD-type2; NOTE 1: SD-type1 refers to all sub-configurations that contain one port subset. NOTE 2: SD-type2 refers to all sub-configurations that contain list of CSI-RS resource IDs. - maxNumberLmax-r18 indicates the max number of sub-configurations Lmax in one CSI report configuration; - maxNumberCSI-ResourcePerCC-r18 indicates the maximum number of simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - maxNumberTotalCSI-ResourcePerCC-r18 indicates the maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - totalNumberCSI-Reporting-r18 indicates total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP. NOTE 3: For maxNumberCSI-ResourcePerCC-r18 and maxNumberTotalCSI-ResourcePerCC-r18, NZP-CSI-RS resource and CSI-RS ports are counted for reporting settings with and without sub-configurations. NOTE 4: If a UE reports more than one capability from spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the above reported features, then the supported maximum of NZP-CSI-RS resources/ports across all periodic, semi-persistent, aperiodic CSI report settings with sub-configurations corresponding to all of spatial and power domain adaptations and without sub-configurations is determined by the minimum of the reported values from that subset. NOTE 5: If a UE reports both spatialAdaptation-CSI-Feedback-r18 and powerAdaptation-CSI-Feedback-r18, and if the UE is configured with CSI report settings with sub-configurations corresponding to both spatialAdaptation-CSI-Feedback-r18 and powerAdaptation-CSI-Feedback-r18, then the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP is determined by the minimum of the reported values from both spatialAdaptation-CSI-Feedback-r18 and powerAdaptation-CSI-Feedback-r18. A UE indicating support of this feature shall also indicate support of csi-ReportFramework and spatialAdaptation-CSI-FeedbackPerBC-r18. | Band | No | N/A | N/A |
| spatialAdaptation-CSI-FeedbackAperiodic-r18 Indicates whether the UE supports spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting and single-panel type 1 codebook. This capability signalling comprises the following parameters: - csiFeedbackType-r18 indicates CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS resource IDs for periodic CSI reporting. Value sdType1 indicates support of SD-type1, value sdType2 indicates support of SD-type2, value both indicates support of both SD-type1 and SD-type2; NOTE 1: SD-type1 refers to all sub-configurations that contain one port subset. NOTE 2: SD-type2 refers to all sub-configurations that contain list of CSI-RS resource IDs. - maxNumberLmax-r18 indicates the max number of sub-configurations Lmax in one CSI report configuration; - subReportCSI-r18 indicates N number of report of CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. - maxNumberCSI-ResourcePerCC-r18 indicates the maximum number of simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - maxNumberTotalCSI-ResourcePerCC-r18 indicates the maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - totalNumberCSI-Reporting-r18 indicates total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP. NOTE 3: For maxNumberCSI-ResourcePerCC-r18 and maxNumberTotalCSI-ResourcePerCC-r18, NZP-CSI-RS resource and CSI-RS ports are counted for reporting settings with and without sub-configurations. NOTE 4: If a UE reports more than one capability from spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the above reported features, then the supported maximum of NZP-CSI-RS resources/ports across all periodic, semi-persistent, aperiodic CSI report settings with sub-configurations corresponding to all of spatial and power domain adaptations and without sub-configurations is determined by the minimum of the reported values from that subset. NOTE 5: If a UE reports both spatialAdaptation-CSI-FeedbackAperiodic-r18 and powerAdaptation-CSI-FeedbackAperiodic-r18, and if the UE is configured with CSI report settings with sub-configurations corresponding to both spatialAdaptation-CSI-FeedbackAperiodic-r18 and powerAdaptation-CSI-FeedbackAperiodic-r18, then the supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP is determined by the minimum of the reported values from both spatialAdaptation-CSI-FeedbackAperiodic-r18 and powerAdaptation-CSI-FeedbackAperiodic-r18. A UE indicating support of this feature shall also indicate support of csi-ReportFramework and spatialAdaptation-CSI-FeedbackAperiodicPerBC-r18. | Band | No | N/A | N/A |
| spatialAdaptation-CSI-FeedbackPUCCH-r18 Indicates whether the UE supports spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting on PUCCH and single-panel type 1 codebook. This capability signalling comprises the following parameters: - csiFeedbackType-r18 indicates the support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS resource IDs for semi-persistent CSI reporting on PUCCH. Value sdType1 indicates support of SD-type1, value sdType2 indicates support of SD-type2, value both indicates support of both SD-type1 and SD-type2; NOTE 3: SD-type1 refers to all sub-configurations that contain one port subset. NOTE 4: SD-type2 refers to all sub-configurations that contain list of CSI-RS resource IDs. - maxNumberLmax-r18 indicates the max number of sub-configurations Lmax in one CSI report configuration; - subReportCSI-r18 indicates N number of report of CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. - maxNumberCSI-ResourcePerCC-r18 indicates the maximum number of simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - maxNumberTotalCSI-ResourcePerCC-r18 indicates the maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC for SD-type 1 and/or SD-type 2. - totalNumberCSI-Reporting-r18 indicates total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across semi-persistent CSI report settings with sub-configurations per BWP. NOTE 5: For maxNumberCSI-ResourcePerCC-r18 and maxNumberTotalCSI-ResourcePerCC-r18, NZP-CSI-RS resource and CSI-RS ports are counted for reporting settings with and without sub-configurations. NOTE 6: If a UE reports more than one capability from spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the above reported features, then the supported maximum of NZP-CSI-RS resources/ports across all periodic, semi-persistent, aperiodic CSI report settings with sub-configurations corresponding to all of spatial and power domain adaptations and without sub-configurations is determined by the minimum of the reported values from that subset. NOTE 7: If a UE reports more than one capability from spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-FeedbackPUSCH-r18 and powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the reported features, then the supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across semi-persistent CSI report settings with sub-configurations per BWP is determined by the minimum of the reported values from that subset. A UE indicating support of this feature shall also indicate support of csi-ReportFramework, sp-CSI-ReportPUCCH and spatialAdaptation-CSI-FeedbackPUCCH-PerBC-r18. NOTE 1: Void NOTE 2: Void | Band | No | N/A | N/A |
| spatialAdaptation-CSI-FeedbackPUSCH-r18 Indicates whether the UE supports spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting on PUSCH and single-panel type 1 codebook. This capability signalling comprises the following parameters: - csiFeedbackType-r18 indicates CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS resource IDs for semi-persistent CSI reporting on PUSCH. Value sdType1 indicates support of SD-type1, value sdType2 indicates support of SD-type2, value both indicates support of both SD-type1 and SD-type2; NOTE 1: SD-type1 refers to all sub-configurations that contain one port subset. NOTE 2: SD-type2 refers to all sub-configurations that contain list of CSI-RS resource IDs. - maxNumberLmax-r18 indicates the max number of sub-configurations Lmax in one CSI report configuration; - subReportCSI-r18 indicates N number of report of CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. - maxNumberCSI-ResourcePerCC-r18 indicates the maximum number of simultaneous NZP-CSI-RS resources per CC. - maxNumberTotalCSI-ResourcePerCC-r18 indicates the maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per. - totalNumberCSI-Reporting-r18 indicates total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across semi-persistent CSI report settings with sub-configurations per BWP. NOTE 3: For maxNumberCSI-ResourcePerCC-r18 and maxNumberTotalCSI-ResourcePerCC-r18, NZP-CSI-RS resource and CSI-RS ports are counted for reporting settings with and without sub-configurations. NOTE 4: If a UE reports more than one capability from spatialAdaptation-CSI-Feedback-r18, spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackAperiodic-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-Feedback-r18, powerAdaptation-CSI-FeedbackPUSCH-r18, powerAdaptation-CSI-FeedbackAperiodic-r18, powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the above reported features, then the supported maximum of NZP-CSI-RS resources/ports across all periodic, semi-persistent, aperiodic CSI report settings with sub-configurations corresponding to all of spatial and power domain adaptations and without sub-configurations is determined by the minimum of the reported values from that subset. NOTE 5: If a UE reports more than one capability from spatialAdaptation-CSI-FeedbackPUSCH-r18, spatialAdaptation-CSI-FeedbackPUCCH-r18, powerAdaptation-CSI-FeedbackPUSCH-r18 and powerAdaptation-CSI-FeedbackPUCCH-r18 and if the UE is configured with CSI report settings with sub-configurations corresponding to a subset of the reported capabilities, then the supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across semi-persistent CSI report settings with sub-configurations per BWP is determined by the minimum of the reported values from that subset. A UE indicating support of this feature shall also indicate support of csi-ReportFramework, sp-CSI-ReportPUSCH and spatialAdaptation-CSI-FeedbackPUSCH-PerBC-r18. | Band | No | N/A | N/A |
| spatialRelations, spatialRelations-v1640 Indicates whether the UE supports spatial relations. The capability signalling comprises the following parameters. - maxNumberConfiguredSpatialRelations indicates the maximum number of configured spatial relations per CC for PUCCH and SRS. It is not applicable to FR1 and applicable to FR2 only. The UE is mandated to report 16 or higher values. maxNumberConfiguredSpatialRelations-v1640 indicates the maximum number of configured spatial relations per CC for PUCCH and SRS with UE supporting the configuration of maximum 64 PUCCH spatial relations per BWP per CC; - maxNumberActiveSpatialRelations indicates the maximum number of active spatial relations with regarding to PUCCH and SRS for PUSCH, per BWP per CC. It is not applicable to FR1 and applicable and mandatory to report one or higher value for FR2 only; - additionalActiveSpatialRelationPUCCH indicates support of one additional active spatial relation for PUCCH. It is mandatory with capability signalling if maxNumberActiveSpatialRelations is set to n1; - maxNumberDL-RS-QCL-TypeD indicates the maximum number of downlink RS resources used for QCL type D in the active TCI states and active spatial relation information, which is optional. The UE is mandated to report spatialRelations for FR2. if maxNumberConfiguredSpatialRelations-v1640 is reported, UE shall report value n96 in maxNumberConfiguredSpatialRelations. | Band | FD | N/A | FD |
| spatialRelationsSRS-Pos-r16 Indicates whether the UE supports spatial relations for SRS for positioning. The capability signalling comprises the following parameters. - spatialRelation-SRS-PosBasedOnSSB-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnPRS-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports any of DL PRS Resources for DL AoD, DL PRS Resources for DL-TDOA or DL PRS Resources for Multi-RTT defined in TS 37.355 [22], or srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnSRS-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnSSB-Neigh-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnPRS-Neigh-r16 indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnPRS-Serving-r16. Otherwise, the UE does not include this field; NOTE: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. | Band | No | N/A | FR2 only |
| spatialRelationsSRS-PosRRC-Inactive-r17 Indicates whether the UE supports spatial relations for SRS for positioning in RRC_INACTIVE. The capability signalling comprises the following parameters: - spatialRelation-SRS-PosBasedOnSSB-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE indicating support of this feature shall also indicate support of srs-PosResourcesRRC-Inactive-r17; - spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE indicating support of this feature shall also indicate support of spatialRelation-SRS-PosBasedOnSSB-Serving-r16; - spatialRelation-SRS-PosBasedOnPRS-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the serving cell in the same band. The UE indicating support of this feature shall also indicate support any of DL PRS Resources for DL AoD, DL PRS Resources for DL-TDOA or DL PRS Resources for Multi-RTT defined in TS 37.355 [22], or srs-PosResourcesRRC-Inactive-r17; - spatialRelation-SRS-PosBasedOnSRS-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE indicating support of this feature shall also indicate support of srs-PosResourcesRRC-Inactive-r17; - spatialRelation-SRS-PosBasedOnSSB-Neigh-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE indicating support of this feature shall also indicate support of spatialRelation-SRS-PosBasedOnSSB-Serving-r16; - spatialRelation-SRS-PosBasedOnPRS-Neigh-r16 indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE indicating support of this feature shall also indicate support of spatialRelation-SRS-PosBasedOnPRS-Serving-r16. NOTE: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. | Band | No | N/A | FR2 only |
| sp-BeamReportPUCCH Indicates support of semi-persistent 'CRI/RSRP' or 'SSBRI/RSRP' reporting using PUCCH formats 2, 3 and 4 in one slot. | Band | No | N/A | N/A |
| sp-BeamReportPUSCH Indicates support of semi-persistent 'CRI/RSRP' or 'SSBRI/RSRP' reporting on PUSCH. | Band | No | N/A | N/A |
| spCell-TAG-Ind-r18 Indicates whether the UE supports indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE. A UE that indicates support of this feature shall indicate support of multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18. | Band | No | N/A | N/A |
| sps-MulticastDCI-Format4-2-r17 Indicates whether the UE supports transmission and retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. A UE that indicates support of this feature shall indicate support of sps-Multicast-r17. | Band | No | N/A | N/A |
| sps-MulticastMultiConfig-r17 Indicates whether the UE supports up to 8 SPS group-common PDSCH configurations per CFR for multicast on PCell. The value indicates the maximum number of activated SPS group-common PDSCH configurations per CFR for multicast. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE that indicates support of this feature shall indicate support of sps-Multicast-r17. | Band | No | N/A | N/A |
| sps-r16 Indicates whether the UE support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group. This field includes the following parameters: - maxNumberConfigsPerBWP-r16 indicates the maximum number of active SPS configurations in a BWP of a serving cell. - maxNumberConfigsAllCC-r16 indicates the maximum number of active SPS configurations across all serving cells in a MAC entity, and across MCG and SCG in case of NR-DC. The UE can include this feature only if the UE indicates support of downlinkSPS. NOTE: - For all the reported bands in FR1, a same X1 value is reported for maxNumberConfigsAllCC-r16. For all the reported bands in FR2, a same X2 value is reported for maxNumberConfigsAllCC-r16. - The total number of active SPS configurations across all serving cells in FR1 is no greater than X1. - The total number of active SPS configurations across all serving cells in FR2 is no greater than X2. - If the CA have some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of active SPS configurations across all serving cells is no greater than max(X1, X2). | Band | No | N/A | N/A |
| srs-AssocCSI-RS Parameters for the calculation of the precoder for SRS transmission based on channel measurements using associated NZP CSI-RS resource (srs-AssocCSI-RS) as described in clause 6.1.1.2 of TS 38.214 [12]. UE supporting this feature shall also indicate support of non-codebook based PUSCH transmission. This capability signalling includes list of the following parameters: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band simultaneously. | Band | No | N/A | N/A |
| srs-combEight-r17 Indicates whether the UE supports comb-8 for SRS other than for positioning. | Band | No | N/A | N/A |
| srs-combOffsetCombinedGroupSequence-r18 Indicates whether the UE supports SRS comb offset hopping combined with group/sequence hopping. The UE supporting this feature shall also indicate the support of srs-combOffsetHopping-r18. | Band | No | N/A | N/A |
| srs-combOffsetHopping-r18 Indicates whether the UE supports SRS comb offset hopping. The UE supporting this feature shall also indicate the support of supportedSRS-Resources. | Band | No | N/A | N/A |
| srs-combOffsetHoppingWithinSubset-r18 Indicates whether the UE supports configuration of subset of comb offsets for comb offset hopping. A UE supporting this feature shall also indicate support of srs-combOffsetHopping-r18. | Band | No | N/A | N/A |
| srs-combOffsetInTime-r18 Indicates whether the UE supports comb offset hopping granularity in time when repetition factor R>1 is configured. Value srs indicates the granularity is per SRS symbol, Value rsrs indicates the granularity is per R SRS symbols, Value both indicates both of per SRS symbol and per R SRS symbols are supported. The UE supporting this feature shall also indicate the support of srs-combOffsetHopping-r18. | Band | No | N/A | N/A |
| srs-cyclicShiftCombinedCombOffset-r18 Indicates whether the UE supports SRS cyclic shift hopping combined SRS comb offset hopping. The UE supporting this feature shall also indicate the support of srs-combOffsetHopping-r18 and srs-cyclicShiftHopping-r18. | Band | No | N/A | N/A |
| srs-cyclicShiftCombinedGroupSequence-r18 Indicates whether the UE supports SRS cyclic shift hopping combined with group/sequence hopping. The UE supporting this feature shall also indicate the support of srs-cyclicShiftHopping-r18. | Band | No | N/A | N/A |
| srs-cyclicShiftHopping-r18 Indicates whether the UE supports SRS cyclic shift hopping. A UE supporting this feature shall also indicate support of supportedSRS-Resources. | Band | No | N/A | N/A |
| srs-cyclicShiftHoppingSmallGranularity-r18 Indicates whether the UE supports configuration of cyclic shift hopping with smaller granularity (with factor K=2). A UE supporting this feature shall also indicate the support srs-cyclicShiftHopping-r18. | Band | No | N/A | N/A |
| srs-increasedRepetition-r17 Indicates whether the UE supports increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource. The UE supporting this feature shall also indicate the support of srs-StartAnyOFDM-Symbol-r16. | Band | No | N/A | N/A |
| srs-partialFreqSounding-r17 Indicates the support of partial frequency sounding for SRS for non-frequency hopping case. The UE indicating support of this feature shall also indicate the support of srs-partialFrequencySounding-r17. | Band | No | N/A | N/A |
| srs-partialFrequencySounding-r17 Indicates whether the UE supports partial frequency sounding for SRS with frequency hopping. | Band | No | N/A | N/A |
| srs-PortReport-r17 Indicates the maximum number of SRS ports for each UE reported quantity in reportQuantity-r17. | Band | No | N/A | N/A |
| srs-PortReportSP-AP-r17 Indicates that the UE supports the maximum number of SRS ports with semi-persistent/aperiodic capability value reporting. The UE supporting this feature shall also indicate support of srs-PortReport-r17 and one of aperiodicBeamReport, sp-BeamReportPUCCH, sp-BeamReportPUSCH, ssb-csirs-SINR-measurement-r16, semi-PersistentL1-SINR-Report-PUCCH-r16 or semi-PersistentL1-SINR-Report-PUSCH-r16. | Band | No | N/A | N/A |
| srs-PosResourcesRRC-Inactive-r17 Indicates support of positioning SRS transmission in RRC_INACTIVE for initial UL BWP. The capability signalling comprises the following parameters: - maxNumberSRS-PosResourceSetPerBWP-r17 Indicates the max number of SRS Resource Sets for positioning supported by UE; - maxNumberSRS-PosResourcesPerBWP-r17 indicates the max number of P/SP SRS Resources for positioning; - maxNumberSRS-ResourcesPerBWP-PerSlot-r17 indicates the max number of P/SP SRS Resources for positioning per slot; - maxNumberPeriodicSRS-PosResourcesPerBWP-r17 indicates the max number of periodic SRS Resources for positioning; - maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r17 indicates the max number of periodic SRS Resources for positioning per slot. NOTE: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this feature. No dedicated capability signalling is intended for this component | Band | No | N/A | N/A |
| srs-SemiPersistent-PosResourcesRRC-Inactive-r17 Indicates support of positioning SRS transmission in RRC_INACTIVE for initial UL BWP with semi-persistent SRS. UE indicating support of this feature shall indicate support of srs-PosResourcesRRC-Inactive-r17. The capability signalling comprises the following parameters: - maxNumOfSemiPersistentSRSposResources-r17 indicates the max number of semi-persistent SRS Resources for positioning; - maxNumOfSemiPersistentSRSposResourcesPerSlot-r17 indicates the max number of semi-persistent SRS Resources for positioning per slot. | Band | No | N/A | N/A |
| srs-startRB-locationHoppingPartial-r17 Indicates whether the UE supports start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS. The UE supporting this feature shall also indicate the support of srs-partialFrequencySounding-r17. | Band | No | N/A | N/A |
| srs-TriggeringDCI-r17 Indicates whether the UE supports triggering SRS in DCI 0_1/0_2 without data and without CSI. | Band | No | N/A | N/A |
| srs-TriggeringOffset-r17 Indicates the maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot. | Band | No | N/A | N/A |
| ssb-csirs-SINR-measurement-r16 Indicates the limitations of the UE support of SSB/CSI-RS for L1-SINR measurement. This capability signalling includes list of the following parameters: Per slot limitations: - maxNumberSSB-CSIRS-OneTx-CMR-r16 indicates the maximum number of SSB/CSI-RS (1TX) across all CCs within a band for Channel Measurement Report - maxNumberCSI-IM-NZP-IMR-res-r16 indicates the maximum number of CSI-IM/NZP-IMR resources across all CCs within a band - maxNumberCSIRS-2Tx-res-r16 indicates the maximum number of CSI-RS (2TX) resources across all CCs within a band for Channel Measurement Report Memory limitations: - maxNumberSSB-CSIRS-res-r16 indicates the max number of SSB/CSI-RS resources across all CCs within a band as Channel Measurement Report - maxNumberCSI-IM-NZP-IMR-res-mem-r16 indicates the maximum number of CSI-IM/NZP-IMR resources across all CCs within a band Other limitations: - supportedCSI-RS-Density-CMR-r16 indicates supported density of CSI-RS for Channel Measurement Report. - maxNumberAperiodicCSI-RS-Res-r16 indicates the maximum number of aperiodic CSI-RS resources across all CCs within a band configured to measure L1-SINR (including CMR and IMR) - supportedSINR-meas indicates the supported SINR measurements. - supportedSINR-meas-r16 contains values {ssbWithCSI-IM, ssbWithNZP-IMR, csirsWithNZP-IMR, csi-RSWithoutIMR} representing {SSB as CMR with dedicated CSI-IM, SSB as CMR with dedicated NZP IMR, CSI-RS as CMR with dedicated NZP IMR configured, CSI-RS as CMR without dedicated IMR configured}. - supportedSINR-meas-v1670 indicates a 4-bit bitmap {ssbWithCSI-IM, ssbWithNZP-IMR, csirsWithNZP-IMR, csi-RSWithoutIMR}, where the leftmost bit corresponds to ssbWithCSI-IM, the next bit corresponds to ssbWithNZP-IMR and so on. UE indicating supportedSINR-meas-v1670 shall always indicate supportedSINR-meas-r16. UE supporting this feature shall also indicate support of CSI-RS as CMR with dedicated CSI-IM. UE indicating support of this feature shall also indicate support of periodicBeamReport and aperiodicBeamReport or sp-BeamReportPUCCH and sp-BeamReportPUSCH. UE indicating support of ssb-csirs-SINR-measurement-r16 shall support periodic and aperiodic L1-SINR report. NOTE 1: The reference slot duration is the shortest slot duration defined for the frequency range where the reported band belongs. NOTE 2: For maxNumberSSB-CSIRS-res-r16 and maxNumberCSI-IM-NZP-IMR-res-mem-r16 the configured CSI-RS resources for both active and inactive BWPs are counted. NOTE 3: For maxNumberSSB-CSIRS-OneTx-CMR-r16, maxNumberCSI-IM-NZP-IMR-res-r16 and maxNumberCSIRS-2Tx-res-r16, CSI-RS resources configured as CMR without dedicated IMR are counted both as CMR and IMR. NOTE 4: For maxNumberSSB-CSIRS-OneTx-CMR-r16, maxNumberCSI-IM-NZP-IMR-res-r16, maxNumberCSIRS-2Tx-res-r16, maxNumberAperiodicCSI-RS-Res-r16, a SSB/CSI-RS resource is counted within the duration of a reference slot in which the corresponding reference signals are transmitted. NOTE 5: For maxNumberSSB-CSIRS-OneTx-CMR-r16, maxNumberCSI-IM-NZP-IMR-res-r16, maxNumberCSIRS-2Tx-res-r16, maxNumberAperiodicCSI-RS-Res-r16, if one resource used for L1-SINR measurement is referred N times by one or more CSI reporting settings with reportQuantity-r16 = ssb-Index-SINR-r16 or cri-SINR-r16, it is counted N times. NOTE 6: If more than one type of SINR measurement is indicated in supportedSINR-meas-v1670, it is left to UE implementation which SINR measurement to indicate in supportedSINR-meas-r16. | Band | No | N/A | N/A |
| sssg-Switching-1BitInd-r17 Indicates whether the UE supports 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based SSSG switching, if pdcch-SkippingDurationList is not configured as specified in TS 38.213 [11], clause 10.4. UE supports search space set group switching capability-1 according to Table 10.4-1 of TS 38.213 [11]. | Band | No | N/A | N/A |
| sssg-Switching-2BitInd-r17 Indicates whether the UE supports 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching, if pdcch-SkippingDurationList is not configured as specified in TS 38.213 [11], clause 10.4. UE supports search space set group switching capability-1 according to Table 10.4-1 of TS 38.213 [11]. UE indicating support of this feature shall also indicate support of sssg-Switching-1bitInd-r17. | Band | No | N/A | N/A |
| support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |
| support64CandidateBeamRS-BFR-r16 Indicates UE support of configuring maximum 64 candidate beam RSs per BWP per CC. UE indicating support of this feature shall also indicate support of maxNumberCSI-RS-BFD, maxNumberSSB-BFD and maxNumberCSI-RS-SSB-CBD. | Band | No | N/A | N/A |
| supportCodeWordSoftCombining-r16 Indicates whether UE supports codeword soft combining for FDMSchemeB. UE indicates support of this feature depends on whether the supportFDM-SchemeB-r16 is also supported. | Band | No | N/A | N/A |
| supportFDM-SchemeA-r16 Indicates whether UE supports single DCI based FDMSchemeA. | Band | No | N/A | N/A |
| supportInter-slotTDM-r16 Indicates whether UE supports single-DCI based inter-slot TDM. This capability signalling includes the following: - supportRepNumPDSCH-TDRA-r16 indicates support of repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16 and the maximum value of repetitionNumber-r16 - maxTBS-Size-r16 indicates maximum TBS size. - maxNumberTCI-states-r16 indicates the maximum number of TCI states. | Band | No | N/A | N/A |
| supportNewDMRS-Port-r16 Indicates whether UE supports new DMRS port entry {0,2,3}. UE supports this feature should indicate support singleDCI-SDM-scheme-r16 for the band. | Band | No | N/A | N/A |
| supportOf2RxXR-r18 Indicates that the UE is 2Rx XR UE as specified in TS 38.101-1 [2] (see "two antenna port XR UE"). A UE reporting this parameter shall not indicate support of supportOfRedCap-r17 or supportOfERedCap-r18. | Band | No | N/A | N/A |
| supportRepNumPDSCH-TDRA-DCI-1-2-r17 Indicates support of repetitionNumber-v1730 in PDSCH-TimeDomainResourceAllocation for DCI format 1_2 and the maximum value of repetitionNumber-v1730. The UE indicating support of this field shall also indicate support of dci-Format1-2And0-2-r16. | Band | No | N/A | N/A |
| supportTDM-SchemeA-r16 Indicates whether UE supports single DCI based TDMSchemeA. The capability signalling includes the maximum TBS size. | Band | No | N/A | N/A |
| supportTwoPortDL-PTRS-r16 Indicates whether UE supports 2-port DL PT-RS. UE supports this feature should indicate support singleDCI-SDM-scheme-r16 for the band. | Band | No | N/A | N/A |
| ta-BasedPDC-NTN-SharedSpectrumChAccess-r17 Indicates whether the UE supports propagation delay compensation based on Rel-15 TA procedure for NTN and shared spectrum channel access. | Band | No | N/A | N/A |
| ta-IndicationCellSwitch-r18 Indicates whether the UE supports TA indication in cell switch command. A UE supporting this feature shall also indicate support of at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. | Band | No | N/A | N/A |
| tb-ProcessingMultiSlotPUSCH-r17 Indicates whether UE supports TB processing over multi-slot PUSCH for DG and Type 2 CG without repetition in RRC connected mode. | Band | No | N/A | N/A |
| tb-ProcessingRepMultiSlotPUSCH-r17 Indicates whether UE supports repetition of TB processing over multi-slot PUSCH in RRC connected mode. UE supporting this feature shall also indicate support of tb-ProcessingMultiSlotPUSCH-r17. | Band | No | N/A | N/A |
| tci-StatePDSCH Defines support of TCI-States for PDSCH. The capability signalling comprises the following parameters: - maxNumberConfiguredTCI-StatesPerCC indicates the maximum number of configured TCI-states per CC for PDSCH. For FR2, the UE is mandated to set the value at least to 64 (i.e. value 128 is an optional value). For FR1, the UE is mandated to set these values at least to the maximum number of allowed SSBs in the supported band; - maxNumberActiveTCI-PerBWP indicates the maximum number of activated TCI-states per BWP per CC, including control and data. If a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. The UE shall include this field. NOTE: the UE is required to track only the active TCI states. The UE is mandated to report tci-StatePDSCH. | Band | Yes | N/A | N/A |
| tci-StateSwitchInd-r18 Indicates whether the UE supports enhanced one-shot large UL transmit timing adjustment requirement to support FR2-1 PC6 Ues and enhanced TCI state switching delay requirements based on [the cross-RRH TCI state indication for UE-specific PDCCH MAC CE] in HST FR2 scenario, as specified in TS 38.133 [5]. A UE supporting this feature shall also indicate support of PC6 in ue-PowerClass-v1700. | Band | No | N/A | FR2 only |
| tci-JointTCI-UpdateMultiActiveTCI-PerCC-r18 Indicates whether the UE supports unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC. The capability signalling comprises the following parameters: - tci-StateInd-r18 indicates TCI state indication for update and activation. Value withAssignment corresponds to MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment, value withoutAssignment corresponds to MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment; - maxNumberActiveJointTCI-PerCC-r18 indicates the maximum number of activated joint TCI states per CC. A UE supporting this feature shall also indicate support tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18 and unifiedJointTCI-multiMAC-CE-r17. NOTE: defaultQCL-TwoTCI-r16 can be used to indicate support of two default beams | Band | No | N/A | N/A |
| tci-JointTCI-UpdateMultiActiveTCI-PerCC-PerCORESET-r18 Indicates whether the UE supports unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC. The capability indicates the maximum number of MAC-CE activated joint TCI states per CORESETPoolIndex per CC. The TCI state indication for update and activation includes: - MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment; - MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment. A UE supporting this feature shall also indicate support of tci-JointTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18 and unifiedJointTCI-multiMAC-CE-r17. | Band | No | N/A | N/A |
| tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18 Indicates whether the UE supports Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC. The capability signalling comprises the following parameters: - maxNumberConfigJointTCIPerCC-PerBWP-r18 indicates the maximum number of configured joint TCI states per CC per BWP; - maxNumberActiveJointTCI-AcrossCC-r18 indicates the maximum number of activated joint TCI states across all CCs in a band. A UE supporting this feature shall also indicate support of unifiedJointTCI-r17. NOTE: defaultQCL-TwoTCI-r16 can be used to indicate support of two default beams. | Band | No | N/A | N/A |
| tci-JointTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18 Indicates whether the UE supports unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC. UE supporting this feature supports one MAC-CE activated joint TCI-states per CC in a band for a TRP associated with a 'coresetPoolIndex' value. The capability signalling comprises the following parameters: - mTRP-Operation-r18 indicates mTRP operation for M-DCI with joint TCI state. - maxNumberConfigJointTCIPerCC-PerBWP-r18 indicates the maximum number of configured joint TCI states per BWP per CC. - maxNumberActiveJointTCIAcrossCC-PerCORESET-r18 indicates the maximum number of activated joint TCI states across all CCs in a band per 'coresetPoolIndex' value. A UE supporting this feature shall also indicate support of unifiedJointTCI-r17. NOTE 1: Activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions. NOTE 2: defaultQCL-PerCORESETPoolIndex-r16 can be used to indicate support of two default beams. | Band | No | N/A | N/A |
| tci-SelectionAperiodicCSI-RS-r18 Indicates whether the UE supports per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP. The UE supporting this feature shall also indicate support of tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18. NOTE: When the UE supports NCJT CSI under mTRP-CSI-EnhancementPerBand-r17 or CJT CSI under twoTCI-StatePDSCH-CJT-TxScheme-r18, UE is expected to support "per resource" when the corresponding NCJT CSI or CJT CSI is configured. | Band | No | N/A | N/A |
| tci-SelectionAperiodicCSI-RS-M-DCI-r18 Indicates whether the UE supports per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP. The UE supporting this feature shall also indicate support of tci-JointTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18. | Band | No | N/A | N/A |
| tci-SelectionDCI-r18 Indicates whether the UE supports DCI format 1_1 and if supported 1_2 configured with TCI selection field. The UE supporting this feature shall also indicate support of at least one of tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18, tci-JointTCI-UpdateMultiActiveTCI-PerCC-r18, tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-r18, and tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-r18. | Band | No | N/A | N/A |
| tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC. TCI state indication for update and activation includes: - MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment; - MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment. The capability signalling comprises the following parameters: - maxNumActiveDL-TCI-AcrossCC-r18 indicates the maximum number of activated DL TCI states across all CCs in a band, - maxNumActiveUL-TCI-AcrossCC-r18 indicates the maximum number of activated UL TCI states across all CCs in a band. The UE supporting this feature shall also indicate support of tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-r18. NOTE: defaultQCL-TwoTCI-r16 can be used to indicate support of two default beams. | Band | No | N/A | N/A |
| tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-PerCORESET-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC. TCI state indication for update and activation includes: - MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment; - MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment. The capability signalling comprises the following parameters: - maxNumConfigDL-TCI-PerCC-PerBWP-r18 indicates the maximum number of configured DL TCI states per CC per BWP , - maxNumConfigUL-TCI-PerCC-PerBWP-r18 indicates the maximum number of configured UL TCI states per CC per BWP. A UE supporting this feature shall also indicate support of tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18 and unifiedSeparateTCI-multiMAC-CE-r17. | Band | No | N/A | N/A |
| tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC. The capability signalling comprises the following parameters: - maxNumConfigDL-TCI-PerCC-PerBWP-r18 indicates the maximum number of configured DL TCI states per CC per BWP , - maxNumConfigUL-TCI-PerCC-PerBWP-r18 indicates the maximum number of configured UL TCI states per CC per BWP. - maxNumActiveDL-TCI-AcrossCC-r18 indicates the maximum number of activated DL TCI states across all CCs in a band, - maxNumActiveUL-TCI-AcrossCC-r18 indicates the maximum number of activated UL TCI states across all CCs in a band. A UE supporting this feature shall also indicate support of tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18 and unifiedJointTCI-commonUpdate-r17. NOTE: defaultQCL-TwoTCI-r16 can be used to indicate support of two default beams | Band | No | N/A | N/A |
| tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC. UE supporting this feature supports one MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a 'coresetPoolIndex' value and one MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a 'coresetPoolIndex' value. The capability signalling comprises the following parameters: - mTRP-Operation-r18 indicates the mTRP operation for M-DCI with separate DL/UL TCI state. - maxNumConfigDL-TCI-PerCC-PerBWP-r18 indicates the maximum number of configured DL TCI states per CC per BWP, - maxNumConfigUL-TCI-PerCC-PerBWP-r18 indicates the maximum number of configured UL TCI states per CC per BWP. - maxNumActiveDL-TCI-AcrossCC-r18 indicates the maximum number of activated DL TCI states across all CCs in a band, - maxNumActiveUL-TCI-AcrossCC-r18 indicates the maximum number of activated UL TCI states across all CCs in a band. A UE supporting this feature shall also indicate support of tci-JointTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18 and unifiedSeparateTCI-r17. | Band | No | N/A | N/A |
| tci-TRP-BFR-r18 Indicates whether the UE supports TRP-specific BFR with unified TCI framework with Unified TCI. A UE supporting this feature shall also indicate support of mTRP-BFR-twoBFD-RS-Set-r17. | Band | No | N/A | N/A |
| tdcp-Report-r18 Indicates whether the UE supports Y=1 delay value for TDCP report and amplitude report. The UE also supports to configure KTRS = 1 TRS resource set. This capability signalling comprises the following parameters: - valueX-r18 indicates CPU occupation (OCPU=(Y+1)*X). - maxNumberActiveResource-r18 indicates the index N of the maximum number of simultaneously active CSI-RS resources for TDCP across all CCs within a band. The maximum number of simultaneously active CSI-RS resources for TDCP across all CCs within a band is N*2, where N = {2..32}. A UE supporting this feature shall also indicate support of csi-ReportFramework and simultaneousCSI-ReportsAllCC. NOTE: Counting of simultaneously active CSI-RS resources follows existing specification TS 38.214 [12]. | Band | No | N/A | N/A |
| tdcp-Resource-r18 Indicates the number of CSI-RS resources for TDCP that the UE supports. This capability signalling comprises the following parameters: - maxNumberConfigPerCC-r18 indicates the maximum number of configured CSI-RS resources for TDCP per CC. - maxNumberConfigAcrossCC-r18 indicates the index N the maximum number of configured CSI-RS resources for TDCP across all CCs within a band. The maximum number of configured CSI-RS resources for TDCP across all CCs within a band is N*2, where N = {1..32}. - maxNumberSimultaneousPerCC-r18 indicates the maximum number of simultaneously active CSI-RS resources for TDCP per CC. A UE supporting this feature shall indicate support of tdcp-Report-r18. NOTE: Counting of simultaneously active CSI-RS resources follows existing specification TS 38.214 [12]. | Band | No | N/A | N/A |
| thresholdBasedMulticastResume-r18 Indicates whether the UE supports thresholdMBS-List-r18 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate support of multicastInactive-r18. | Band | No | N/A | N/A |
| timeBasedCondHandover-r17 Indicates whether the UE supports time based conditional handover, i.e., CondEvent T1 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. | Band | No | N/A | N/A |
| timelineRelax-CJT-CSI-r18 Indicates whether the UE supports timeline relaxation parameter for regular eType-II-CJT CSI, or for port selection FeType-II-CJT CSI. Value n0 indicates 0, value n2 indicates Z2. A UE supporting this feature shall also indicate support of eType2CJT-r18 or feType2CJT-r18. NOTE: A UE that supports eType2CJT-r18 or feType2CJT-r18 must signal this feature. | Band | CY | N/A | N/A |
| triggeredHARQ-CodebookRetx-r17 Indicates whether the UE supports triggered HARQ-ACK codebook re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2 as indicated in dci-Format1-2And0-2-r16) and support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config as indicated in twoHARQ-ACK-Codebook-type1-r16). The capability signalling comprises the following parameters: - minHARQ-Retx-Offset-r17 indicates minimum value for the HARQ re-tx offset. Value n-7 corresponds to -7, value n-5 corresponds to -5, and so on. - maxHARQ-Retx-Offset-r17 indicates maximum value for the HARQ re-tx offset. NOTE: The minimum requirement for minHARQ-Retx-Offset-r17 and maxHARQ-Retx-Offset-r17 is valid for HARQ CBs consisted of HARQ Processes with a single HARQ bit per HARQ Process ID. | Band | No | N/A | N/A |
| triggeredHARQ-CodebookRetxDCI-1-3-r18 Indicates whether the UE supports triggered HARQ-ACK codebook re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_3 and supports the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in simultaneous-2-1-HARQ-ACK-CB-r18). The capability signalling comprises the following parameters: - minHARQ-Retx-Offset-r18 indicates minimum value for the HARQ re-tx offset. Value n-7 corresponds to -7, value n-5 corresponds to -5, and so on. If the UE also supports triggeredHARQ-CodebookRetx-r17, the same values as minHARQ-Retx-Offset-r17 is reported. - maxHARQ-Retx-Offset-r18 indicates maximum value for the HARQ re-tx offset. If the UE also supports triggeredHARQ-CodebookRetx-r17, the same values as maxHARQ-Retx-Offset-r17 is reported. A UE supporting this feature shall also indicate support of at least one of multiCell-PDSCH-DCI-1-3-SameSCS-r18 and multiCell-PDSCH-DCI-1-3-DiffSCS-r18. NOTE: The minimum requirement for minHARQ-Retx-Offset-r18 and maxHARQ-Retx-Offset-r18 is valid for HARQ CBs consisting of HARQ Processes with a single HARQ bit per HARQ Process ID. | Band | No | N/A | N/A |
| trs-AdditionalBandwidth-r16 Indicates the UE supported TRS bandwidths, in addition to 52 RBs, for a 10MHz UE channel bandwidth. This field only applies for the BWPs configured with 52 RBs size and 15kHz SCS, in FDD bands. Value trs-AddBW-Set1 indicates 28, 32, 36, 40, 44, 48 RBs. Value trs-AddBW-Set2 indicates 32, 36, 40, 44, 48 RBs. | Band | No | FDD only | FR1 only |
| twoHARQ-ACK-CodebookForUnicastAndMulticast-r17 Indicates whether the UE supports two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. A UE supporting this feature shall also indicate support of priorityIndicatorInDCI-Multicast-r17. | Band | No | N/A | N/A |
| twoPHR-Reporting-r18 Indicates whether the UE supports two PHR reporting related to STx2P. A UE supporting this feature shall also indicate support of at least one of pusch-CB-SingleDCI-STx2P-SDM-r18, pusch-NonCB-SingleDCI-STx2P-SDM-r18, pusch-CB-SingleDCI-STx2P-SFN-r18, pusch-NonCB-SingleDCI-STx2P-SFN-r18, twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18, and twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. NOTE: If gNB does not configure corresponding RRC parameter for this feature, UE will report a PHR for an actual PUSCH transmission and PHR for the first indicated TCI state or PHR associated with coresetPoolIndex0 is reported if actual PUSCH transmission is based on STx2P schemes. | Band | No | N/A | FR2 only |
| twoPortsPTRS-UL Defines whether UE supports PT-RS with 2 antenna ports for UL transmission. | Band | No | N/A | N/A |
| twoPUSCH-CB-MultiDCI-STx2P-CG-CG-r18 Indicates whether the UE supports multi-DCI based STx2P CG-PUSCH+CG-PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-CB-MultiDCI-STx2P-CG-DG-r18 Indicates whether the UE supports multi-DCI based STx2P DG-PUSCH+CG-PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-CB-MultiDCI-STx2P-FullTimeFullFreqOverlap-r18 Indicates whether the UE supports fully overlapping PUSCHs in time and fully overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-CB-MultiDCI-STx2P-FullTimePartialFreqOverlap-r18 Indicates whether the UE supports fully overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18 | Band | No | N/A | FR2 only |
| twoPUSCH-CB-MultiDCI-STx2P-PartialTimeFullFreqOverlap-r18 Indicates whether the UE supports partially overlapping PUSCHs in time and fully overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18 | Band | No | N/A | FR2 only |
| twoPUSCH-CB-MultiDCI-STx2P-PartialTimeNonFreqOverlap-r18 Indicates whether the UE supports the partially overlapping PUSCHs in time, non-overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18 | Band | No | N/A | FR2 only |
| twoPUSCH-CB-MultiDCI-STx2P-PartialTimePartialFreqOverlap-r18 Indicates whether the UE supports the partially overlapping PUSCHs in time, partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18 | Band | No | N/A | FR2 only |
| twoPUSCH-NonCB-MultiDCI-STx2P-CG-CG-r18 Indicates whether the UE supports multi-DCI based STx2P CG-PUSCH+CG-PUSCH for noncodebook. A UE supporting this feature shall also indicate support of twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-NonCB-MultiDCI-STx2P-CG-DG-r18 Indicates whether the UE supports multi-DCI based STx2P DG-PUSCH+CG-PUSCH for noncodebook. A UE supporting this feature shall also indicate support of twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-NonCB-Multi-DCI-STx2P-CSI-RS-Resource-r18 Indicates whether the UE supports up to two NZP CSI-RS resources associated with the two SRS resource sets for multi-DCI non-codebook based STx2P scheme for PUSCH. The capability signalling comprises the following parameters: - maxNumberPeriodicSRS-r18 indicates the maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. - maxNumberAperiodicSRS-r18 indicates the maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. - maxNumberSemiPersistentSRS-r18 indicates the maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. - simultaneousSRS-PerCC-r18 indicates the number of SRS resources associated with first and second CSI-RS resources simultaneously in a CC that the UE can process. The number of SRS resources includes P/SP/A SRS. - simultaneousCSI-RS-NonCB-r18 indicates the maximum number of CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously that the UE can process. A UE supporting this feature shall also indicate support of srs-AssocCSI-RS, csi-RS-IM-ReceptionForFeedbackPerBandComb and twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-NonCB-MultiDCI-STx2P-FullTimeFullFreqOverlap-r18 Indicates whether the UE supports fully overlapping PUSCHs in time and fully overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-NonCB-MultiDCI-STx2P-FullTimePartialFreqOverlap-r18 Indicates whether the UE supports fully overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimeFullFreqOverlap-r18 Indicates whether the UE supports partially overlapping PUSCHs in time and fully overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimeNonFreqOverlap-r18 Indicates whether the UE supports partially overlapping PUSCHs in time, non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimePartialFreqOverlap-r18 Indicates whether the UE supports partially overlapping PUSCHs in time, partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH. A UE supporting this feature shall also indicate support of twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18. | Band | No | N/A | FR2 only |
| twoRateMatchingEUTRA-CRS-patterns-3-4-r18 Indicates whether the UE supports two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier (regardless of support or configuration of multi-TRP) for the case when crs-RateMatchPerCoresetPoolIndex is not configured. The capability signalling comprises the following parameters: - maxNumberPatterns-r18 indicates the maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS. - maxNumberNon-OverlapPatterns-r18 indicates the maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS. UE supporting this feature shall support rateMatchingLTE-CRS. NOTE: If a UE supports this feature and multipleRateMatchingEUTRA-CRS-r16, multipleRateMatchingEUTRA-CRS-r16 is reported for lte-CRS-PatternList1-r16 and lte-CRS-PatterList2-r16 and twoRateMatchingEUTRA-CRS-patterns-3-4-r18 is reported for lte-CRS-PatternList3-r16 and lte-CRS-PatternList4-r16. | Band | No | N/A | FR1 only |
| twoTCI-StatePDSCH-CJT-TxScheme-r18 Indicates whether the UE supports two TCI states for CJT Tx scheme for PDSCH. Value cjtSchemeA corresponds to PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA, value cjtSchemeB corresponds to PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state. Value both corresponds to the supporting of both cjtSchemeA and cjtSchemeB. A UE supporting this feature shall also indicate support of tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18. | Band | No | N/A | N/A |
| txDiversity-r16 Indicates whether the UE supports transparent Tx diversity requirements for 2Tx as specified in the suffix G clauses of TS 38.101-1 [2] (see also clauses 4.2 and 4.3 of TS 38.101-1 [2]). This field is only applicable for single CC case (i.e. non-CA). | Band | No | N/A | FR1 only |
| type1-HARQ-Codebook-r17 Indicates whether the UE supports Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes. UE indicating support of this feature shall also indicate support of harq-FeedbackDisabled-r17. This field is only applicable for bands in Table 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |
| type1-PUSCH-RepetitionMultiSlots-v1650 Indicates whether the UE supports Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value equal to 2, 4, or 8 with a single repetition of the transport block within each slot, and redundancy version pattern as indicated by UL-TWG-RV-rep. A UE supporting this feature shall also support Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. This applies only to non-shared spectrum channel access. For shared spectrum channel access, type1-PUSCH-RepetitionMultiSlots-r16 applies. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The UE only includes type1-PUSCH-RepetitionMultiSlots-v1650 if type1-PUSCH-RepetitionMultiSlots is absent | Band | No | N/A | N/A |
| type2-HARQ-Codebook-r17 Indicates whether the UE supports Type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes. UE indicating support of this feature shall also indicate support of harq-FeedbackDisabled-r17. This field is only applicable for bands in Table 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |
| type2-PUSCH-RepetitionMultiSlots-v1650 Indicates whether the UE supports Type 2 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value equal to 2, 4, or 8 with a single repetition of the transport block within each slot, and redundancy version pattern as indicated by UL-TWG-RV-rep. A UE supporting this feature shall also support Type 2 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. This applies only to non-shared spectrum channel access. For shared spectrum channel access, type2-PUSCH-RepetitionMultiSlots-r16 applies. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The UE only includes type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCH-RepetitionMultiSlots is absent | Band | No | N/A | N/A |
| type3-HARQ-Codebook-r17 Indicates whether the UE supports Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes. UE indicating support of this feature shall also indicate support of harq-FeedbackDisabled-r17. This field is only applicable for bands in Table 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |
| ue-OneShotUL-TimingAdj-r17 Indicates whether the UE supports one shot large UL timing adjustment. UE indicating support of this feature shall indicate support of ue-PowerClass-v1700 set to 'pc6'. | Band | No | N/A | FR2 only |
| ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 For FR1, if the UE supports the different UE power class than the default UE power class as defined in clause 6.2 of TS 38.101-1 [2], or in clause 6.2 of TS 38.101-5 [34], the UE shall report the supported UE power class in this field. For FR2, UE shall report the supported UE power class as defined in clause 6 and 7 of TS 38.101-2 [3] in this field. UE indicating support for pc6 supports the enhanced intra-NR RRM and demodulation processing requirements for FR2 to support high speed up to 350 km/h as specified in TS 38.133 [5]. This capability is not applicable to IAB-MT or NCR-MT. The power class pc7 is only applicable for RedCap UEs operation in FR2. This capability is not applicable for UEs indicating support of maxOutputPowerATG-r18. | Band | Yes | N/A | N/A |
| ue-specific-K-Offset-r17 Indicates whether the UE supports the reception of UE-specific K-offset comprised of the following functional components: - Support of reception of Differential K-offset via MAC-CE - Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with Differential K-offset UE indicating support of this feature shall also indicate support of uplinkPreCompensation-r17 and uplink-TA-Reporting-r17 for this band. This field is only applicable for bands in Table 5.2.2-1 and Table 5.2.3-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |
| ue-TA-Measurement-r18 Indicates whether the UE supports UE-based TA measurement by indicating the maximum number of candidate cells that the UE maintains the TA for. A UE supporting this feature shall also indicate the support of at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. | Band | No | N/A | N/A |
| ul-GapFR2-r17 Indicates whether the UE supports FR2 UL gap to perform BPS sensing for Tx power management by the use of uplink gap patterns as specified in TS 38.133 [5] if UE supports a band in FR2. | Band | No | No | FR2 only |
| unifiedJointTCI-r17 Indicates the support of unified TCI state operation with joint DL/UL TCI update for intra-cell beam management including the support of: - One MAC-CE activated joint TCI state per CC in a band - TCI state indication for update and activation of MAC CE based TCI state indication for one active TCI state The capability signalling comprises the following parameters: - maxConfiguredJointTCI-r17 indicates the maximum number of configured joint TCI states per BWP per CC in a band - maxActivatedTCIAcrossCC-r17 indicates the maximum number of MAC-CE activated joint TCI states across all CC(s) in a band If a UE supports unifiedJointTCI-InterCell-r17, the signalled component values (except additionalMAC-CE-AcrossCC-r17) also apply to inter-cell beam management, NOTE: Activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions | Band | No | N/A | N/A |
| unifiedJointTCI-BeamAlignDLRS-r17 Indicates the support of beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS. The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. | Band | No | N/A | FR2 only |
| unifiedJointTCI-commonMultiCC-r17 Indicates the support of common multi-CC TCI state ID update and activation. The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| unifiedJointTCI-InterCell-r17 Indicates the support of Unified TCI with joint DL/UL TCI update for inter-cell beam management including following parameters: - additionalMAC-CE-PerCC-r17 indicates the number of K additional MAC-CEs to indicate joint TCI states per CC in a band. - additionalMAC-CE-AcrossCC-r17 indicates the number of K additional MAC-CE activated joint TCI states across all CC(s) in a band. A UE indicating support of this shall also indicate support of unifiedJointTCI-r17 and unifiedJointTCI-mTRP-InterCell-BM-r17. NOTE: A UE that supports unifiedJointTCI-InterCell-r17 supports K additional MAC-CE activated joint TCI states across all CC(s) in a band in addition to the maximum number of MAC-CE activated joint TCI states across all CC(s) in a band signalled in unifiedJointTCI-r17. The signalled value in additionalMAC-CE-AcrossCC-r17 plus the signalled value in maxActivatedTCIAcrossCC-r17 determine the maximum number of MAC-CE activated joint TCI states across all CC(s) in a band that are applied to intra and inter-cell beam management jointly. | Band | No | N/A | N/A |
| unifiedJointTCI-Legacy-r17 Indicates the support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH (except for TRS and for CORESET #0 and the respective PDSCH reception) reusing the Rel-15/16 signalling/configuration design(s). The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| unifiedJointTCI-Legacy-CORESET0-r17 Indicates the support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signalling/configuration design(s). The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| unifiedJointTCI-Legacy-SRS-r17 Indicates the support of indication/configuration of R17 TCI states for SRS (except for periodic/semi-persistent SRS for BM) reusing the Rel-15/16 signalling/configuration design(s). The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| unifiedJointTCI-ListSharingCA-r17 Indicates the support of reference BWP/serving cell index to indicate reference TCI state list shared by multiple BWPs/serving cells. The value indicates the maximum number of configured joint TCI state lists across all BWPs and all Serving cells in a band. The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. A UE that supports CA and unifiedJointTCI-r17 shall indicate support of this feature. | Band | No | N/A | N/A |
| unifiedJointTCI-mTRP-InterCell-BM-r17 Indicates the support of inter-cell beam measurement and reporting for inter-cell BM and mTRP. This feature includes support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI (additional PCI) and support of up to K SSBRI-RSRP pairs in one report where a pair is associated with a PCI different from serving cell PCI can be reported, where K is equal to maxNumberNonGroupBeamReporting. This feature also includes following parameters: - maxNumAdditionalPCI-L1-RSRP-r17 indicates the maximum number of RRC-configured] PCI(s) different from serving cell PCI for L1-RSRP measurement. - maxNumSSB-ResourceL1-RSRP-AcrossCC-r17 indicates the maximum number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI [across all CC]. NOTE: maxNumSSBResource-L1-RSRP-AcrossCC-r17 is also counted in maxTotalResourcesForOneFreqRange-r16/ maxTotalResourcesForAcrossFreqRanges-r16. | Band | No | N/A | N/A |
| unifiedJointTCI-multiMAC-CE-r17, unifiedJointTCI-multiMAC-CE-v17xy Indicates the support of unified TCI state operation with joint DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC with MAC CE and DCI based TCI state indication in DCI formats 1_1 and 1_2 with and without DL assignment. This capability signalling includes the following parameters: - minBeamApplicationTime-r17 indicates the minimum beam application time in Y symbols per SCS indicated only for FR2. - maxNumMAC-CE-PerCC-r17 indicates the maximum number of MAC-CE activated joint TCI states per CC in a band. The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. unifiedJointTCI-multiMAC-CE-r17 is included only when the UE supports a single SCS for the band in all the supported band combinations. unifiedJointTCI-multiMAC-CE-v17xy is only included when unifiedJointTCI-multiMAC-CE-r17 is absent. NOTE 1: The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band for more than one MAC-CE activated joint TCI state is signaled in unifiedJointTCI-r17. NOTE 2: Activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH. | Band | No | N/A | N/A |
| unifiedJointTCI-multiMAC-CE-DCI-1-3-r18 Indicates whether the UE supports unified TCI with joint DL/UL TCI update by DCI format 1_3 for intra-cell and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC. The UE also supports using TCI state indication for update and activation, i.e. MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3). The capability signalling comprises the following parameters: - minBeamApplicationTime-r18 indicates the minimum beam application time in symbols per SCS. If the UE also support unifiedJointTCI-multiMAC-CE-r17, same values as minBeamApplicationTime-r17 for unifiedJointTCI-multiMAC-CE-r17 are reported. - maxActivatedTCI-PerCC-r18 indicates the maximum number of MAC-CE activated joint TCI states per CC in a band. If the UE also support unifiedJointTCI-multiMAC-CE-r17, same values as maxActivatedTCIAcrossCC-r17 for unifiedJointTCI-multiMAC-CE-r17 are reported. NOTE 1: The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band for more than one MAC-CE activated joint TCI state is signalled in maxActivatedTCIAcrossCC-r17 of unifiedJointTCI-r17. NOTE 2: Activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH. A UE supporting this feature shall also indicate support of unifiedJointTCI-r17, at least one of multiCell-PDSCH-DCI-1-3-SameSCS-r18 and multiCell-PDSCH-DCI-1-3-DiffSCS-r18. | Band | No | N/A | N/A |
| unifiedJointTCI-PC-association-r17 Indicates the support of association between TCI state and UL PC settings except for PL RS for PUCCH, PUSCH, and SRS. The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| unifiedJointTCI-perBWP-CA-r17 Indicates the support of TCI state list configuration per BWP when CA is configured. The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| unifiedJointTCI-SCellBFR-r17 Indicates the support of SCell BFR with unified TCI operation. The maximum number of CCs configured with SCell BFR with unified TCI framework in a band with SpCell BFR is given by maxNumberSCellBFR-r16. The UE supporting this feature assumes that maxNumberSCellBFR-r16 includes SpCell. | Band | No | N/A | N/A |
| unifiedSeparateTCI-r17 Indicates the support of unified TCI state operation with joint DL/UL TCI update for intra-cell beam management including the support of: - One MAC-CE activated DL TCI state per CC in a band - One MAC-CE activated UL TCI state per CC in a band - TCI state indication for update and activation including MAC CE based TCI state indication for one active DL/UL TCI state The capability signalling comprises the following parameters: - maxConfiguredDL-TCI-r17 indicates the maximum number of configured DL TCI states per BWP per CC - maxConfiguredUL-TCI-r17 indicates the maximum number of configured UL TCI states per BWP per CC - maxActivatedDL-TCIAcrossCC-r17 indicates the maximum number of MAC-CE activated DL TCI states across all CC(s) in a band - maxActivatedUL-TCIAcrossCC-r17 indicates the maximum number of MAC-CE activated UL TCI states across all CC(s) in a band The UE indicating support of this feature shall also indicate support of unifiedJointTCI-r17. If a UE supports unifiedSeparateTCI-InterCell-r17, the maxConfiguredDL-TCI-r17 and maxConfiguredUL-TCI-r17 apply to intra- and inter-cell beam management jointly. | Band | No | N/A | N/A |
| unifiedSeparateTCI-commonMultiCC-r17 Indicates the Common multi-CC DL/UL-TCI state ID update and activation. The UE indicating support of this feature shall also indicate support of unifiedSeparateTCI-r17. | Band | No | N/A | N/A |
| unifiedSeparateTCI-InterCell-r17 Indicates the support of unified TCI with separate DL/UL TCI update for inter-cell beam management with more than one MAC-CE activated separate TCI state per CC. This feature also includes following parameters: - k-DL-PerCC-r17 indicates the number of additional MAC-CE activated DL TCI states per CC in a band - k-UL-PerCC-r17 indicates the number of additional MAC-CE activated UL TCI states per CC in a band - k-DL-AcrossCC-r17 indicates the number of additional MAC-CE activated DL TCI states across all CC(s) in a band - k-UL-AcrossCC-r17 indicates the number of additional MAC-CE activated UL TCI states across all CC(s) in a band The UE indicating support of this feature shall also indicate support of unifiedSeparateTCI-r17. NOTE: A UE that supports this feature supports K additional MAC-CE activated DL and K additional MAC-CE activated UL TCI states across all CC(s) in a band in addition to the maximum number of MAC-CE activated DL and UL TCI states across all CC(s) in a band signalled in unifiedSeparateTCI-r17. The signalled value in k-DL-AcrossCC-r17 (k-UL-AcrossCC-r17) plus the signalled value in maxActivatedDL-TCIAcrossCC-r17 (maxActivatedUL-TCIAcrossCC-r17) determine the maximum number of MAC-CE activated DL (UL) TCI states across all CC(s) in a band that are applied to intra and inter-cell beam management jointly. | Band | No | N/A | N/A |
| unifiedSeparateTCI-ListSharingCA-r17 Indicates the support of reference BWP/serving cell configured with reference TCI state pool shared by a set of BWPs/serving cells. The value indicates the maximum number of configured DL/UL TCI state pools across all BWPs and all serving cells in a band. | Band | No | N/A | N/A |
| unifiedSeparateTCI-multiMAC-CE-r17, unifiedSeparateTCI-multiMAC-CE-v17xy Indicates TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) And b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment). This capability signalling includes the following parameters: - minBeamApplicationTime-r17 indicates the minimum beam application time in Y symbols per SCS. - maxActivatedDL-TCIPerCC-r17 indicates the maximum number of MAC-CE activated DL TCI states per CC in a band - maxActivatedUL-TCIPerCC-r17 indicates the maximum number of MAC-CE activated UL TCI states per CC in a band The UE indicating support of this feature shall also indicate support of unifiedSeparateTCI-r17. unifiedSeparateTCI-multiMAC-CE-r17 is included only when there is same SCS for all the CCs of the band in all the band combinations. unifiedSeparateTCI-multiMAC-CE-v17xy is only included when unifiedSeparateTCI-multiMAC-CE-r17 is absent. | Band | No | N/A | N/A |
| unifiedSeparateTCI-MultiMAC-CE-IntraCell-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI update by DCI format 1_3 for intra-cell beam management with more than one MAC-CE activated separate TCI state per CC. This capability also indicates TCI state indication for update and activation, i.e. MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3). The capability signalling comprises the following parameters: - minBeamApplicationTime-r18 indicates the minimum beam application time in symbols per SCS. If the UE also support unifiedJointTCI-multiMAC-CE-r17, same values as minBeamApplicationTime-r17 for unifiedJointTCI-multiMAC-CE-r17 are reported. - maxActivatedDL-TCI-PerCC-r18 indicates the maximum number of MAC-CE activated DL TCI states per CC in a band. - maxActivatedUL-TCI-PerCC-r18 indicates the maximum number of MAC-CE activated UL TCI states per CC in a band. If a UE supports unifiedSeparateTCI-InterCell-r17, the signalled component values also apply to inter-cell beam management. A UE supporting this feature shall also indicate support of unifiedSeparateTCI-r17, at least one of multiCell-PDSCH-DCI-1-3-SameSCS-r18 and multiCell-PDSCH-DCI-1-3-DiffSCS-r18. | Band | No | N/A | N/A |
| unifiedSeparateTCI-perBWP-CA-r17 Indicates the support of DL/UL TCI state pool configuration per BWP for CA mode. The UE indicating support of this feature shall also indicate support of unifiedSeparateTCI-r17. | Band | No | N/A | N/A |
| uplinkBeamManagement Defines support of beam management for UL. This capability signalling comprises the following parameters: - maxNumberSRS-ResourcePerSet-BM indicates the maximum number of SRS resources per SRS resource set configurable for beam management, supported by the UE. - maxNumberSRS-ResourceSet indicates the maximum number of SRS resource sets configurable for beam management, supported by the UE. If the UE does not set beamCorrespondenceWithoutUL-BeamSweeping to supported, the UE shall report this capability. This feature is optional for the UE that supports beam correspondence without uplink beam sweeping as defined in clause 6.6, TS 38.101-2 [3]. NOTE: The network uses maxNumberSRS-ResourceSet to determine the maximum number of SRS resource sets that can be configured to the UE for periodic/semi-persistent/aperiodic configurations as below: Maximum number of SRS resource sets across all time domain behaviour (periodic/semi-persistent/aperiodic) reported in maxNumberSRS-ResourceSet; Additional constraint on the maximum number of SRS resource sets configured to the UE for each supported time domain behaviour (periodic/semi-persistent/aperiodic). 1; 1. 2; 1. 3; 1. 4; 2. 5; 2. 6; 2. 7; 4. 8; 4. | Band | No | N/A | FR2 only |
| uplinkPreCompensation-r17 Indicates whether the UE supports the uplink time and frequency pre-compensation and timing relationship enhancements comprised of the following functional components: - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of pre-compensation of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of UE receiving cell-specific K_offset/K_mac in system information Support of this feature in NTN bands is mandatory for UE supporting nonTerrestrialNetwork-r17. This field is only applicable for bands in Table 5.2.2-1 and Table 5.2.3-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | CY | N/A | N/A |
| uplink-TA-Reporting-r17 Indicates whether the UE supports UE reporting of information related to TA pre-compensation as specified in TS 38.321 [8]. UE indicating support of this feature shall also indicate support of uplinkPreCompensation-r17 for this band. This field is only applicable for bands in Table 5.2.2-1 and Table 5.2.3-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |