3GPP TSG-RAN WG2 Meeting #127bis	R2-2409393

Hefei, China, 14 – 18 October 2024

Agenda item:	8.1.2.2

Source:	Nokia

Title:	[017] Report of Offline Discussion on LCM Definitions

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Proposals from papers regarding LCM definitions submitted by Nokia and Interdigital were discussed during the afternoon session on AI/ML Air on 15/10/24. The aim of this offline discussion is to agree on basic definitions for activation, deactivation, and fallback. To ease the discussion, we will avoid the term “functionality” as to allow RAN1 to determine the definition and consider configurations for AI/ML Beam Management and legacy Beam Management instead.

# 2	Background

## 2.1	Proposals Discussed

The following proposals were discussed.

R2-2408553	LCM for UE-side Beam Management	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: A deactivated functionality is an AI/ML functionality for which the UE has received a radio configuration via a CSI-ReportConfig, but which is neither configured to perform inference nor to report its outputs to the NW.

Proposal 3: Functionality fallback to non-AI/ML means deactivating one AI/ML-enabled functionality and activating a non-AI/ML feature of the same type, e.g., Beam Management.

Proposal 4: Functionality switching means simultaneously deactivating one AI/ML-enabled functionality and activating another of the same type, e.g., Beam Management.

R2-2408921	Remaining details of applicable functionality reporting	InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 1:	Upon AI/ML functionality deactivation, UE falls back to legacy (i.e., non-AI/ML) operation as baseline.

During the discussion, it was raised by Huawei that for CSI measurement reporting, we already have the following:

- Aperiodic – CSI reporting activated via a DCI indication

- Semipersistent – CSI reporting activated via a MAC CE indication

- Periodic – CSI reporting activated via an RRCReconfiguration message

Definitions for switching will be left out of this discussion.

## 2.2	Legacy Activation and Deactivation of CSI Reports

However, it is still unclear what the term activated means in the context of BM LCM. As a starting point for the discussion, we suggest discussing the behaviour of the legacy mechanisms. CSI-ReportConfigs are configured, i.e., added, modified, or released via CSI-MeasConfig in lists named csi-ReportConfigToAddModList and csi-ReportConfigToReleaseList.

The following table, 5.2.1.4-1 from TS38.214 summarizes the different CSI-RS configuration and Reporting combinations.

The following sections go into further details for each CSI Reporting Type. Further details can be found in TS38.214 and TS38.331.

Aperiodic

The UE receives a configuration for aperiodic reporting via CSI-ReportConfig with a ReportConfigType set to aperiodic.

The NW is not required to send any CSI-RS associated an aperiodic CSI-RS configuration until the triggering of an aperiodic reporting occurs. When triggered by a DCI, the NW begins to transmit CSI-RS, if the CSI-RS are also aperiodic, the UE makes measurements, and then the UE is expected to produce one report over PUCCH or PUSCH and to produce no further reports until triggered again.

Semipersistent on PUCCH

The UE receives a configuration for semipersistent reporting via CSI-ReportConfig with a ReportConfigType set to semiPersistentOnPUCCH. When triggered by an “SP CSI reporting on PUCCH Activation/Deactivation” or “SP CSI reporting on PUSCH Activation/Deactivation” MAC CE, the UE is expected to produce reports when activated until deactivated. An index into a list derived from the list of CSI-ReportConfigs of the ReportConfigType set to semiPersistentOnPUCCH.

Semipersistent on PUSCH

The UE receives a configuration for semipersistent reporting via CSI-ReportConfig with a ReportConfigType set to semiPersistentOnPUSCH. A semipersistent on PUSCH reporting configuration is activated via a DCI, with an index configured in the CSI-Request field, and continues to report until deactivated by a DCI, setting the CSI-Request field to 0.

Periodic

The UE receives a configuration for periodic reporting via CSI-ReportConfig with a ReportConfigType set to periodic. Upon reception of a periodic CSI reporting configuration, it is expected that the UE will periodically report CSI measurements of the type configured in reportQuantity until the configuration is released. Therefore, according to the legacy procedures, a periodic configuration for CSI measurement reporting should begin reporting when “added” or “modified” and should cease reporting when “released”.

# 3	Discussion

## 3.1	What is Activation?

Question 1: Is AI/ML BM activated simply having been provided an aperiodic, semipersistent, or periodic configuration, or, for at least the first two, is an activation command also required?

Qualcomm – can follow CSI reporting as a baseline. Configuration can be provided and will not be activated until specific activation has been received from the network.

Samsung – assumption is that the operation is based on the CSI reporting framework, so the existing framework should be supported.

Huawei – if it’s a layer 1 command, then wait for R1

Question 2: Is a periodic AI/ML BM configuration initially activated and can it be configured in an initially deactivated state?

Nokia – Similar view to Qualcom and Samsung to follow CSI framework.

Qualcomm – wait for periodic case since we have an LS to RAN1 regarding what to do with Step 3.

Samsung – similar view to QC

Interdigital – activation needed or not is part of the LS.

Given the following agreement made in RAN2#127, it seems that reporting must occur to be considered “activated”.

- Activated functionalities refers to functionalities already enabled for performing inference

In the previous section, four different CSI reporting configuration types from legacy CSI reporting were described, all but one of which are initially deactivated.

Question 3: Is a configured AI/ML BM CSI-ReportConfig that is not producing reports considered an activated functionality?

Vivo – RAN1 discussion necessary

Oppo – similar view to Samsung with regard to R1 LS

Huawei – similar view to wait for R1 progress. CSI framework should be part of the inference configuration

Question 4: Is a configured AI/ML BM CSI-ReportConfig, which has been triggered to produce reports considered an activated functionality?

Interdigital – inference is reported to the NW then it is activated

Samsung – periodic reporting is simple, but for aperiodic there may be something to clarify, but maybe not possible to conclude right now

Qualcomm – can we use the term configuration instead of functionality?

Moderator (Nokia): [ Is it possible to drop functionality and instead refer to configurations? ]

## 3.2	What is Deactivation?

As noted in Section 2.2, aperiodic and semipersistent CSI reporting configurations are initially deactivated, until triggered by a DCI or a DCI or MAC CE, respectively. Additionally, an aperiodic CSI reporting configuration automatically stops reporting after a report has been sent.

Question 5: For an aperiodic AI/ML BM CSI reporting configuration, is it considered to be automatically deactivated after having sent the report which it was triggered to send?

Interdigital – the UE doesn’t stop using the model after it sends the report. Between reports ,the model could be considered active.

Asustek – link the activation/deactivation with the model or the inference operation instead of whether reports are being transmitted. This is related to the AI/ML model itself.

Qualcomm – CSI resource configuration is included. There isn’t a situation where the UE is measuring but not reporting. Activation means the CSI reporting configuration is activated by the NW

Ericsson - Activation should be related to the UE performing inference, but not to the reporting itself.

Lenovo – agree that reporting config is related. Reporting configuration comes with CSI RS configuration. What matters is what we can see in the spec.

ZTE – with activation, the UE should report something to the NW

Vivo – activation should refer to inference

Qualcomm – map to the configuration whether a CSI reporting configuration is activated by the NW or not.

Semipersistent and Periodic AI/ML BM CSI reporting configurations report until deactivated either by a DCI or MAC CE, or by RRC Reconfiguration, respectively.

Question 6: Is a semipersistent AI/ML BM CSI reporting configuration considered to be deactivated after having received a DCI or MAC CE deactivation command?

Question 7: Is there an equivalent deactivation possible for a Periodic AI/ML CSI reporting configuration since the act of Releasing the configuration destroys the context for a deactivated state?

Samsung – maybe we can have a small step, instead of changing activation. We can say that on top of that, the activation means that a UE is enabled to perform inference and provide CSI reporting based on the CSI framework.

Nokia – create observation

## 3.3	What is Fallback?

The following examples are provided for discussion on what procedural steps could define the fallback operation, mapped to legacy procedures. These procedures are provided for discussion only, and to begin to achieve a general sense for what will be required. Whether fallback could be pre-configured or whether a UE could be triggered to automatically fallback is out of scope for this short offline discussion.

- Aperiodic

  - DCI trigger Aperiodic AI/ML BM report

  - Poor performance detected by network

  - DCI trigger Aperiodic Legacy CSI-RS measurement report

- Semipersistent PUCCH

  - MAC activation of Semipersistent AI/ML BM reporting

  - Poor performance detected by network

  - One Step

    - MAC deactivation of Semipersistent AI/ML BM reporting

    - MAC activation of Semipersistent Legacy CSI-RS measurement reporting

- Semipersistent PUSCH

- Periodic

  - RRC Reconfiguration with ToAddMod, adding AI/ML BM Periodic reporting configuration

  - Poor performance detected by network

  - RRC Reconfiguration

    - ToAddMod, adding Legacy BM Periodic reporting configuration

    - ToRelease, releasing AI/ML BM Periodic reporting configuration

# 4	Way Forward / Proposals

Here, we will capture discussion notes and proposals that we develop during the offline.

During the discussion, it became clear that there are aspects that require RAN1 input since we sent an LS in the last meeting, RAN2#127. There were differing opinions on whether a functionality is required to produce reports in order to be considered to be activated. There was not sufficient time to discuss Section 3.3 on fallback. Based on discussion during the session and after the session, a potential way forward is proposed to aid in avoiding ambiguity until RAN1 produces an LS response for our consideration.

Proposal 1: For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations.

Proposal 2: RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.