3GPP TSG-RAN WG2 Meeting #128                                                               R2-2409540

Orlando, USA, November 18th– 22nd, 2024

Agenda item:	8.9.2

Source:	vivo

Title:	Further Discussion on S&F Operation

Document for:	Discussion and Decision

- Introduction

In RAN2#127bis [1], RAN2 discussed the issue on S&F operation indication and cell barred issue, the following agreements were made correspondingly.

Agreements: 1. The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed) 2. RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided. 3. An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed 4. If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure 5. When present, the “S&F operation” indication has two possible settings: ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In the contribution, we mainly discuss the following open aspects of S&F:

- FFS additional information except the S&F operation indication is needed;

- Satellite ID list provided via NAS.

- Discussion

  - Additional S&F information

Figure 1: cell mode switching between regeneration payload mode and S&F mode

As shown in Figure 1, depending on the satellite movement, the cell may operate in S&F mode at some times and normal regeneration payload mode at other times. The regeneration payload mode provides better performance than the S&F mode, and UEs supporting the S&F mode can work well in the cell with the regeneration payload mode. Thus, in case the cell’s operation mode changes from S&F mode to regeneration payload mode, no additional information needs to be provided to UE since no specific handling is required for UEs.

If the cell is currently working on regeneration payload mode, both the S&F capable UEs and the S&F not capable UEs may work in the cell. Then, when the cell changes its operation mode from regeneration payload to S&F mode, UEs without S&F capability may not work well in the cell. If S&F not capable UE is in RRC CONNECTED, it can rely on NW implementation to handle these UEs, i.e., NW can trigger the handover of the UEs to cells in regeneration payload mode or release the UE. For S&F not capable UE in RRC IDLE, it is beneficial if UEs can camp on other cells before the cell changes to S&F mode. To enable this, we think the simple way is to reuse the new S&F operation indication, i.e., up to NW implementation, NW indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation, then the legacy UEs and R19 UEs not supporting the S&F operation consider the cell is barred and perform the cell reselection, while the S&F capable UE can continue work in the cell.

Observation 1: For S&F not capable UE in RRC CONNECTED state, NW can release the UE or the trigger handover to a cell in regeneration payload mode before the serving cell changes from regeneration payload mode to S&F mode.

Observation 2: For S&F not capable UE in RRC IDLE state, NW can indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation.

Proposal 1: Up to NW implementation, NW can enable the S&F not capable UE camps/works in a regeneration payload mode cell before the serving cell changes from regeneration payload mode to S&F mode, no further spec is foreseen.

Proposal 2: No additional information except the S&F operation indication needs to be provided by NW.

  - Satellite ID list provided via NAS

According to the TR23.700-29 [2], MME can provide a list of satellite IDs to UE via the NAS message. The satellite IDs are used for UE to decide which cell/satellite can re-attempt to perform the subsequent attach/TAU procedure after the wait time expires. From this perspective, it is clear that the satellite ID provided by NAS is globally unique.

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

Observation 3: To assist UE perform subsequent attach/TAU, the satellite ID provided by the NAS is globally unique.

Regarding the satellite ID used in SIB, according to the discussion in RAN2#122 [3], the number of the satellite ID depends on the maximum number of satellites to be considered for mobility, but not the deployed satellite in the same or neighbour orbits.

5. The satellite ID in the new SIB is an integer of X bits wherein X depends on the maximum number of satellites to be considered for mobility.

Based on the above agreements, our understanding is that the satellite ID used in SIBs is unique within the cell level, i.e., different cells may assign different satellite ID for the same satellite.

Observation 4: The satellite ID used in the RAN is not a world global ID, i.e., it is allocated by the eNB and it uniquely identifies a satellite within the cell.

However, according to the SA2, the satellite IDs in NAS are based on the SIB information broadcasted by eNB. This is not possible since the satellite IDs used in NAS by SA2 are globally unique while the satellite IDs used in eNB are unique within the cell. Further, before SA2 made the initial conclusion, SA2 may not have been aware of the issue, i.e., the satellite ID used in RAN is unique within one cell. So it is proposed that RAN2 sends LS to SA2 telling them the issue to trigger SA2 to discuss how to handle it.

Proposal 3: RAN2 confirms that UE cannot recognize a unique satellite marked with monitoring satellite ID provided by NAS, considering that only local satellite ID is used in Uu interface.

Proposal 4: RAN2 to send an LS to indicate the issue to SA2 and ask SA2 whether and how to solve the issue.

- Conclusion

In this contribution, we have further discussed the S&F operation. And we make the following observations and proposals:

Additional S&F information:

Observation 1: For S&F not capable UE in RRC CONNECTED state, NW can release the UE or the trigger handover to a cell in regeneration payload mode before the serving cell changes from regeneration payload mode to S&F mode.

Observation 2: For S&F not capable UE in RRC IDLE state, NW can indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation.

Proposal 1: Up to NW implementation, NW can enable the S&F not capable UE camps/works in a regeneration payload mode cell before the serving cell changes from regeneration payload mode to S&F mode, no further spec is foreseen.

Proposal 2: No additional information except the S&F operation indication needs to be provided by NW.

Satellite ID list provided via NAS:

Observation 3: To assist UE perform subsequent attach/TAU, the satellite ID provided by the NAS is globally unique.

Observation 4: The satellite ID used in the RAN is not a world global ID, i.e., it is allocated by the eNB and it uniquely identifies a satellite within the cell.

Proposal 3: RAN2 confirms that UE cannot recognize a unique satellite marked with monitoring satellite ID provided by NAS, considering that only local satellite ID is used in Uu interface.

Proposal 4: RAN2 to send an LS to indicate the issue to SA2 and ask SA2 whether and how to solve the issue.

- Reference

- R2-2409213, Report from Break-out session on NR-NTN and IoT-NTN.

- TR 23.700-29, Study on integration of satellite components in the 5G architecture.

- R2-2306642	[offline-102] Mobility enhancements Qualcomm	discussion	Rel-18	IoT_NTN_enh-Core.