3GPP TSG-RAN WG2 Meeting #128	                                                                                   R2-2409595

Orlando, USA, Nov.  18th – 22nd , 2024

Source:	CATT

Title:	Discussion on Conditional Intra-CU LTM

Agenda Item:    8.6.4

Document for:  Discussion and Decision

Introduction

This paper further discusses the remaining issues on the conditional intra-CU LTM [1]. In detail, the following aspects are covered by our discussion,

- Conditional LTM preparation

- Conditional LTM early sync

- Conditional LTM evaluation

- Conditional LTM execution

The detailed discussion is provided in Section 2, and followed by Section 3 that summarizes the contribution of this paper.

Discussion

- Conditional LTM preparation

Whether to support CHO conditions for conditional LTM

During last meeting discussion, it was agreed that Event LTM3-like and LTM5-like are used as the conditional LTM execution condition,

Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

Whether to reuse the CHO conditions for conditional LTM still remains open, e.g., CondEvent A3 and CondEvent A5 for CHO.

As per the definition of Event LTM3 and LTM5 used for the event triggered L1 measurement reporting,

Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

The intention is to use the beam-level measurement results to evaluate the execution condition and determine the suitable beam to report. So for conditional LTM, since we reuse the Event LTM3-like and LTM5-like execution conditions, it should also be the beam level measurement results used for the evaluation of execution conditions and the determination of the suitable beam. However, the evaluation of CondEvent A3 and CondEvent A5 for CHO are based on cell-level measurement results, which is filtered among the beams configured by RRC and thus cannot determine the trigger of the LTM execution. If we support CHO conditions for conditional LTM, in order to select the suitable beam based on CHO conditions more spec impact will be introduced.

Besides, there is no strong motivation to support the L3 measurement-based conditional LTM, as the NW can trigger the L3 measurement based LTM if needed or just configure UE with CHO.

Observation 1: Conditional LTM should be not only determines the target cell but also the target beam, but the CHO conditions can only determine the target cell.

Furthermore, if CHO events are supported for conditional LTM, it means L3 measurement based conditional LTM is supported. On whether the conditional LTM can be based on L3 measurement, it is still discussed in RAN4, and RAN4 will follow RAN2 conclusion if technically allowed,

o On whether L3 measurement based conditional LTM in FR1 can be supported - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached. o As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

So from RAN2 perspective, we can make a conclusion in RAN2 that the conditional LTM is based on L1 measurement and do not support the L3 measurement-based conditional LTM. Then we can inform this RAN2 conclusion to RAN4.

Proposal 1a: CHO conditions are not used as conditional LTM execution condition.

Proposal 1b: Conditional LTM is only based on L1 measurement (i.e., L3 measurement-based conditional LTM is not supported). RAN2 sends LS to RAN4 to inform this agreement.

Whether to configure two conditions for conditional LTM

In CHO, the execution condition of a candidate cell can be associated to up to two triggering events. In our understanding, the intention is that the NW may configure different measurement quantities associated with each execution condition, which provides information from more dimensions for UE to make the CHO decision.

For LTM, currently only L1-RSRP is supported for SSB based L1 measurement. Furthermore, there is also no consensus in RAN1 on supporting L1-SINR measurement based on CSI-RS, according to RAN1 agreement as follows

- There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells.

Based on the current state, i.e., only L1-RSRP is supported, it seems not motivated to configure two triggering events with different thresholds. This can be revisited if RAN1 changes their minds.

Proposal 2: The execution condition of a candidate cell is associated to only one triggering event. It can be revisited if RAN1 decides to support more quantities (e.g., L1-SINR or L1-RSRQ) later.

The initialization of conditional LTM preparation

The Rel-19 conditional LTM only supports intra-CU scenario. For the preparation of conditional LTM, similar as NW-triggered LTM, it should be the CU to initiate the conditional LTM preparation based on the L3 measurement.

Proposal 3: The CU initiates the conditional LTM preparation.

The generation of execution condition

On the generation of execution condition for conditional LTM, we reached the following agreement,

Source cell and each candidate cell provides its own execution condition for conditional LTM.

It needs to further clarify how the source cell and each candidate cell provide the execution condition for conditional LTM.

The CHO principle can be taken as baseline. In CHO, it is the serving cell to generate the execution condition for each candidate cell, since the serving cell is aware of the deployment information for neighbor cells. Based on this principle, in conditional LTM, it should be the source cell to determine the execution condition for each conditional LTM candidate for initial evaluation and the candidate cell to determine the execution condition for each other conditional LTM candidate for subsequent evaluation when the candidate cell becomes the serving cell.

Proposal 4a: Source cell provides the initial execution conditions for conditional LTM candidates.

Proposal 4b: The candidate cell provides the execution conditions for other conditional LTM candidates. They are used when the candidate becomes the new serving cell.

The co-existence between conditional LTM and NW-triggered LTM

The co-existence between conditional LTM and Rel-18 intra-CU LTM

Similar as CHO, the NW can perform legacy L3 handover although the NW has configured UE with CHO. Therefore, we should not put much restriction on the NW implementation, i.e., the conditional LTM and Rel-18 intra-CU LTM can be configured together.

Regarding how to implement such co-existence, from signalling point of view, we can reuse the Rel-18 LTM candidate list to provide the candidate configuration used for both conditional LTM and Rel-18 intra-CU LTM. For each LTM candidate in the candidate list, it can be used for NW-triggered LTM by default. For the candidates associated with the execution condition, it can be used by UE for the evaluation of conditional LTM.

Proposal 5a: Rel-18 LTM candidate list is reused to include conditional LTM candidates. If one LTM Candidate is associated with the execution condition, it could be used as conditional LTM, otherwise it is NW based LTM candidate.

Proposal 5b: NW can trigger a LTM execution to any LTM candidates including the conditional LTM candidates.

The co-existence between conditional LTM and Rel-19 inter-CU LTM

As per the WID, the conditional LTM only supports intra-CU scenario in non-DC. Then whether to support the co-existence between conditional LTM and Rel-19 inter-CU LTM should be further discussed. As per our Proposal 5a, if at least one candidate cell is inter-CU LTM candidate (i.e., mixture of intra-CU and inter-CU LTM candidates), UE may firstly perform intra-CU conditional LTM or inter-CU LTM triggered by the NW.

However, for conditional LTM, as per our Proposal 4a/4b, the execution condition is provided by the current serving cell for its corresponding candidate cells. Thus, it could be ensured that only the candidate cells belonging to the same CU with the current serving cell are provided with the execution condition by NW implementation.

For example, UE is in serving cell 0 belonging to CU1, and is configured with candidate cell 1, candidate cell 2 belonging to CU1, and candidate cell 3, candidate cell 4 belonging to CU2.

| Current serving cell | Which cells can be configured as conditional LTM candidates |
| --- | --- |
| Serving cell 0 | Cell 1 and cell 2 |
| Candidate cell 1 | Cell 0 and cell 2 |
| Candidate cell 3 | Cell 4 |

e.g., if serving cell is cell0, UE only evaluate the candidate cell which is associated with execution condition, i.e. cell 1 and cell2, hence it could ensure the conditional LTM is within intra-CU. Therefore, it is proposed that,

Proposal 6: The co-existence between conditional LTM and Rel-19 inter-CU LTM could be supported by NW implementation, i.e., the serving cell only provides execution condition for the candidate cells belongs to the same CU.

- Conditional LTM early sync

Need of assistance information from UE to network

In Rel-18 LTM, NW selects the candidate beams based on measurement results reported from UE and performs the candidate TCI activation as well as PDCCH ordered early TA on these candidate beams. Upon the execution of LTM, the NW determines the target beam and sends the cell switch command to indicate the target beam and TA value for RACH-less LTM. The beam information is also informed to the target cell to facilitate the efficient CG/DG scheduling.

When considering conditional LTM, if the NW is not aware of such assistance information, e.g., L1 measurement results or the potential beams for candidate TCI activation, PDCCH ordered early TA or conditional LTM execution, the NW cannot perform early sync or efficient scheduling from target cell. To solve this concern, UE should report some assistance information to the NW before conditional LTM execution.

Observation 2: Some assistance information from UE on the potential target cells is necessary for the UE to perform the following procedures,

- Early UL sync

- candidate TCI activation/deactivation

- first UL transmission scheduling

Regarding the detailed assistance information, it can be the L1 measurement reporting, for example, triggered by the event that are nearby the conditional LTM execution so that the reported assistance information is valid for cell switch. Another way is to directly report the potential target cell and beam information by UE.

Therefore, it is proposed that,

Proposal 7: To support early UL sync, TCI activation/deactivation and efficient first UL transmission scheduling for conditional LTM execution, UE reports assistance information to the NW based on the event configured by the NW. FFS down selection between the options for the assistance information reporting,

- Option 1: L1 measure reporting triggered by event

- Option 2: potential target cells and target beams

Early DL sync

Based on the assistance information reported by UE before conditional LTM execution, the NW can further determine the candidate TCI for activation/deactivation, i.e., the Rel-18 candidate TCI activation/deactivation mechanism can be reused by the NW for conditional LTM.

Proposal 8: Rel-18 candidate TCI activation/deactivation mechanism is reused in conditional LTM.

Early UL sync

For the early UL sync, the NW can determine the target beam/cell to perform PDCCH ordered early TA based on the assistance information reported from UE. Upon the acquisition of early TA, the following issues should be further discussed.

How to deliver the TA value from NW to UE before the triggering of conditional LTM

Firstly, we still prefer that the source DU delivers the TA value to UE, i.e., the unified procedure with the NW triggered LTM for PDCCH ordered early TA., because the RAR-based solution on candidate cell will increase the interruption of data transmission in the serving cell.

Proposal 9a: For PDCCH ordered early TA in conditional LTM, the source DU sends the TA value of the candidate(s) to UE before the triggering of conditional LTM execution.

Secondly, to deliver the TA value from NW to UE before the triggering of conditional LTM, only the candidate ID and TA value is needed, it seems obvious if we reuse the legacy cell switch command MAC CE, redundant information, e.g., TCI state index will be included. How to handle this redundant information and how to distinguish it is used for conditional LTM or legacy LTM will cause complexity for spec. New MAC CE could be introduced to include the TA value used for conditional LTM.

Proposal 9b: For PDCCH ordered early TA in conditional LTM, a new MAC CE is used to deliver the TA value of the candidate(s) to UE before the triggering of conditional LTM execution.

How to maintain the valid TA value at UE side before conditional LTM execution

UE needs to maintain the TA value and ensures that the stored TA value is valid when to use it upon conditional LTM execution.

The potential solutions are as follows,

- Option 1: No need to check the validity of TA value at UE side

As suggested by our Proposal 7, the assistance information reporting is the event-triggered, and such event is nearby the execution condition for the triggering of conditional LTM. The TA value is used only when UE performs LTM execution to this candidate cell, so upon the execution of conditional LTM, the received TA value should still remain valid as the UE does not move much compared to the location that triggers the assistance information reporting. So it doesn’t need any mechanism to maintain the TA value.

- Option 2: TAT-like timer is maintained at UE side

In option 2, a TAT-like timer is started when UE receives the TA value of a candidate cell. UE treats the TA value of the target cell as valid if the TAT is still running when conditional LTM is triggered by UE.

- Option 3: UE determines the TA value as valid if the L1 measurement result change is not above a threshold.

In option 3, a TA-specific threshold is configured to UE. UE stores the TA value when receiving it. If the L1 measurement result change is above a threshold, the UE consider the stored TA is invalid.

RAN2 needs to discuss and down selects between the potential options above. Therefore, it is proposed that,

Proposal 10: On how to maintain the TA at UE side before conditional LTM execution, RAN2 down selects between the following options,

- Option 1: No TA maintaining at UE side, i.e., UE always uses the acquired TA value upon conditional LTM execution;

- Option 2: TAT-like timer is used;

- Option 3: The Validity of TA value is determined based on a configured threshold of the L1 measurement result change.

- Conditional LTM Evaluation

Which layer to evaluate

In the event-triggered measurement reporting, it had been agreed that,

- MAC layer handles the event evaluation and measurement report triggering.

We prefer a unified solution for conditional LTM and event-triggered measurement reporting. And the Rel-18 LTM is also based on MAC CE to perform cell switch. Hence it is reasonable to let MAC entity to determine the cell switch execution.

Proposal 11: MAC layer handles the event evaluation and determines to trigger the cell switch for conditional LTM.

When to start evaluation

In CHO, the UE starts the evaluation of the execution condition upon receiving the CHO configuration. Some companies propose to evaluate the conditional LTM since UE acquired TA value. However, considering both RACH-based and RACH-less based conditional LTM are supported, TA value is not the essential condition for conditional LTM execution. Hence, for conditional LTM, it could follow the principle used for CHO.

Proposal 12: UE starts the conditional LTM evaluation upon reception of the execution condition for conditional LTM.

The beam used for evaluation

In order to avoid ping-pong, some companies proposed to evaluate multiple beams for conditional LTM, i.e. when at least N beams fulfilled the execution condition for one candidate cell, the UE could trigger the conditional LTM execution. However, it has already agreed to support TTT for the event configuration, thus we can assume the beam quality of the candidate cell could be stable. Hence it is not necessary to evaluate multiple beams for one triggering event. The UE evaluates the execution condition for each candidate cell/beam, and when at least one of the candidate cell/beam fulfills the associated execution condition, the UE should trigger the LTM cell switch procedure.

Proposal 13: UE evaluates the execution condition for each candidate beam and triggers the LTM cell switch procedure when at least one beam fulfills the associated execution condition.

- Conditional LTM execution

Target beam determination for RACH-less conditional LTM

In the NW-based LTM, the UE is indicated with the TCI state in the LTM cell switch command MAC CE. For RACH-less LTM, the UE will use the indicated TCI state to receive the dynamic grant or to select the type-1 configured grant.

For conditional RACH-less LTM, how to determine the selected cell and beam is based on UE. As mentioned above, UE evaluates the execution condition for each candidate cell/beam, and determines to trigger the cell switch upon at least one of the candidate cell/beam fulfills the associated execution condition. Thus, the candidate cell/beam which fulfills the execution condition should be the target cell/beam. If more than one candidate cell/beam fulfills the execution condition, it is up to UE to select which one as the target cell/beam, similar as CHO.

Proposal 14: The target beam is the beam which fulfils the associated execution condition. If more than one beam fulfills the associated execution condition, it is up to UE implementation to select the target beam.

FFS DG-based scheduling for first UL transmission in RACH-less conditional LTM

As suggested by our Proposal 7, the UE will report the assistance information to the NW before the conditional LTM execution. That means the source cell has the potential beam information selected by UE to be used in the target cell in RACH-less conditional LTM. Therefore, the source cell can send the association information to the target cell to facilitate the DG scheduling in the target cell. Hence it is feasible for target cell to start DG scheduling upon reception of the assistance information from the source cell.

Proposal 15: In RACH-less conditional LTM, DG-based scheduling for the first UL transmission is supported.

Conclusion

Based on the analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: Conditional LTM should be not only determines the target cell but also the target beam, but the CHO conditions can only determine the target cell.

Observation 2: Some assistance information from UE on the potential target cells is necessary for the UE to perform the following procedures,

- Early UL sync

- candidate TCI activation/deactivation

- first UL transmission scheduling

Conditional LTM preparation

Proposal 1a: CHO conditions are not used as conditional LTM execution condition.

Proposal 1b: Conditional LTM is only based on L1 measurement (i.e., L3 measurement-based conditional LTM is not supported). RAN2 sends LS to RAN4 to inform this agreement.

Proposal 2: The execution condition of a candidate cell is associated to only one triggering event. It can be revisited if RAN1 decides to support more quantities (e.g., L1-SINR or L1-RSRQ) later.

Proposal 3: The CU initiates the conditional LTM preparation.

Proposal 4a: Source cell provides the initial execution conditions for conditional LTM candidates.

Proposal 4b: The candidate cell provides the execution conditions for other conditional LTM candidates. They are used when the candidate becomes the new serving cell.

Proposal 5a: Rel-18 LTM candidate list is reused to include conditional LTM candidates. If one LTM Candidate is associated with the execution condition, it could be used as conditional LTM, otherwise it is NW based LTM candidate.

Proposal 5b: NW can trigger a LTM execution to any LTM candidates including the conditional LTM candidates.

Proposal 6: The co-existence between conditional LTM and Rel-19 inter-CU LTM could be supported by NW implementation, i.e., the serving cell only provides executions conditions for the candidate cells belongs to the same CU.

Conditional LTM early sync

Proposal 7: To support early UL sync, TCI activation/deactivation and efficient first UL transmission scheduling for conditional LTM execution, UE reports assistance information to the NW based on the event configured by the NW. FFS down selection between the options for the assistance information reporting,

- Option 1: L1 measure reporting triggered by event

- Option 2: potential target cells and target beams

Proposal 8: Rel-18 candidate TCI activation/deactivation mechanism is reused in conditional LTM.

Proposal 9a: For PDCCH ordered early TA in conditional LTM, the source DU sends the TA value of the candidate(s) to UE before the triggering of conditional LTM execution.

Proposal 9b: For PDCCH ordered early TA in conditional LTM, a new MAC CE is used to deliver the TA value of the candidate(s) to UE before the triggering of conditional LTM execution.

Proposal 10: On how to maintain the TA at UE side before conditional LTM execution, RAN2 down selects between the following options,

- Option 1: No TA maintaining at UE side, i.e., UE always uses the acquired TA value upon conditional LTM execution;

- Option 2: TAT-like timer is used;

- Option 3: The Validity of TA value is determined based on a configured threshold of the L1 measurement result change.

Conditional LTM Evaluation

Proposal 11: MAC layer handles the event evaluation and determines to trigger the cell switch for conditional LTM.

Proposal 12: UE starts the conditional LTM evaluation upon reception of the execution condition for conditional LTM.

Proposal 13: UE evaluates the execution condition for each candidate beam and triggers the LTM cell switch procedure when at least one beam fulfills the associated execution condition.

Conditional LTM execution

Proposal 14: The target beam is the beam which fulfils the associated execution condition. If more than one beam fulfills the associated execution condition, it is up to UE implementation to select the target beam.

Proposal 15: In RACH-less conditional LTM, DG-based scheduling for the first UL transmission is supported.

Reference

[1] Draft_RAN2_127b_Meeting_Report_v1