3GPP TSG-RAN WG2 Meeting #128                                                                    R2-2409596

Orlando, USA, 18th - 22nd November, 2024

Source:	CATT

Title:	Consideration on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

In the RAN2#127 bis meeting [1], on-demand SSB was discussed, with the following agreements:

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we share our view on L3 measurement and MAC CE design for OD-SSB in section 2. Our proposals are summarized in Section 3.

Discussion

OD-SSB measurement of SCell

- L3 RRM Framework

During the last RAN2 meeting, the details on L3 RRM framework were discussed and the following two options were summarized.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

For option1, UE measures OD-SSB within SMTC after receiving OD-SSB transmission indication. The SMTC applied to OD-SSB may be a new SMTC that is different from the SMTC of always-on SSB. However, there is a question about whether the new SMTC used for OD-SSB can also be used for always-on SSB. If the new SMTC is only used for OD-SSB, always-on SSB can not be measured when OD-SSB is transmitted.

In addition, currently one cell can only be configured with one SMTC. If the SCell is configured with always-on SSB and OD-SSB, the legacy SMTC is not applicable to OD-SSB. Especially when multiple OD-SSB periodicities are configured, introducing one additional SMTC for OD-SSB may not be sufficient. And if more than one additional SMTC for OD-SSB are introduced for RRM measurement on OD-SSB, the relationship between additional SMTC for OD-SSB and the received new MAC CE for OD-SSB transmission needs to be introduced. More specification impacts are required.

Furthermore, some companies proposed to introduce a new MO for OD-SSB, but according to the current specification, when the OD-SSB and always-on SSB are in the same frequency, they should be configured in the same MO, we do not see the necessity of introducing a new MO for OD-SSB.

For option2, UE measures the OD-SSB within the transmission duration based on the configuration and transmission indication of the OD-SSB for each SCell, which conflicts with existing spec and may impact RAN4 as well as measurement time for the SCell is updated upon receiving MAC CE indication. Compared to option 1, option 2 does not change the existing MO configuration, and a separate measurement method is used for OD-SSB.

In Table 1, we summarize the pros and cons of the two options.

Table 1 Comparison of two options

|  | Pros | Cons |
| --- | --- | --- |
| Option1 | The restriction for SSB measurement within SMTC are not affected | - Modified the configuration of MO - The relationship between additional SMTC for OD-SSB and the received new MAC CE for OD-SSB transmission needs to be introduced - Always-on SSB may not be measured - May have an impact on RAN4 |
| Option2 | - The configurations of MO are not affected - The measurement of always-on SSB is not affected | - Modified the restriction for SSB measurement within SMTC - May have an impact on RAN4 |

According the above analyses, we slightly prefer option 2.

Proposal 1: The RRM measurement of OD-SSB can be performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

In general, both option1 and option2 will have an impact on RAN4. RAN2 needs to send relevant LS to RAN4 after determining the specific options.

Proposal 2: RAN2 sends an LS to RAN4 to ask RAN4 to define requirements for the RRM measurement of OD-SSB.

- Measurement of Case #2

For Case #2, the question of whether always-on SSB and/or OD-SSB are measured was discussed at the last RAN2 meeting. First of all, we think that the question of whether there will be L3 measurements of OD-SSB after receiving the OD-SSB transmission indication needs to be discussed first. If the purpose is to immediately measure and report the measurement result, there will be L3 measurement after receiving the OD-SSB transmission indication, otherwise, L3 measurement is not required, e.g. the transmission of OD-SSB is for L1 measurement. Therefore, RRM measurement based on OD-SSB should be configurable.

Proposal 3: Whether to perform RRM measurement based on OD-SSB is configured by the network. The signalling detail  is FFS.

And then we provide further analysis and proposals of the following options for RRM measurements of SCell configured with on demand SSB.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

The RRM measurement of always-on SSB and/or OD-SSB is affected by the frequency of OD-SSB and always-on SSB. If OD-SSB and always-on SSB are on the different frequency, option1 should be supported. If OD-SSB and always-on SSB are on the same frequency, it is beneficial to go with option 2, i.e., UE measures always-on SSB when the time locations or spatial location of always-on SSB and OD-SSB do not overlap.

Regarding the relation (including frequency, time location and periodicity) between always-on SSB and on-demand SSB, RAN4 has already sent an LS [2] to RAN1 for clarification.

Obj. 1 related to on-demand SSB SCell operation_ Based on the discussion, RAN4 identified that the relation between always-on SSB and on-demand SSB (i.e. Case#2 as agreed in RAN1) has impacts on UE’s behavior and requirements in SCell. RAN4 would like RAN1 to clarify at least the following aspects: - Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB? - Q2: What is the relation in terms of time location between always-on SSB and OD-SSB? - Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB? - Q4: What is the spatial relation between the always-on SSB and OD-SSB?

Therefore, RAN2 can wait until the relation between always-on SSB and on-demand SSB is clarified by RAN1. By then RAN2 can decide which option to go.

Proposal 4: RAN2 waits RAN1 conclusion about the relation between always-on SSB and OD-SSB.

- L3 measurement reporting

- Scenario #2

According to RAN4[3], if the SCell is an unknown SCell, after UE receives SCell activation command, UE has to perform AGC, synchronization and L1 measurement/report based on SSB, and then gNB activate TCI state for data transmission according to UE’s report. If the SCell is a known SCell, gNB can directly activate the TCI state for data transmission. Therefore, larger activation delay is required for the unknown SCell.

Observation 1: The latency of activating a SCell is different between a known cell and an unknown cell.

For Case #1, at RAN2#127 meeting, it was agreed that the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. Therefore, the measurement reporting is triggered only after receiving the OD-SSB transmission indication.

Observation 2: In Scenario #2 with Case #1, the SCell may be an unknown cell because the measurement reporting is triggered only after the indication of the OD-SSB transmission.

According to RAN4 [3], for known cells, there is a limit to the time between the measurement reporting and UE receiving the SCell activation command.

8.3.2	SCell Activation Delay Requirement for Deactivated Scell

SCell in FR1 is known if it has been meeting the following conditions:

- During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified

in clause 9.2 and 9.3.

- the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

- During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time

when UE receives MAC-CE command for TCI activation

- During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

…

For Case #2, UE can measure the always-on SSB, and UE can measure the OD-SSB after OD-SSB is activated. If the measurement reporting of always-on SSB is triggered too early, and the time between the measurement reporting and UE receiving the SCell activation command exceeds the limit, it is beneficial to quickly measure and report the measurement results after receiving the OD-SSB transmission indication, which can change the SCell from unknown cell to known cell to accelerate the activation of SCell. However, according to the existing specification, if the event triggered reporting is configured, UE may not trigger the measurement reporting after receiving the OD-SSB transmission indication, e.g. the measurement reporting for the cell is triggered long along and no new applicable cell is fulfilled the event triggered condition.

Observation 3: For Scenario #2 with Case #2, the SCell may be an unknown cell because UE may not trigger the measurement reporting after receiving the OD-SSB transmission indication if the event triggered reporting is configured.

In order for UE to quickly measure and report measurement results after receiving the OD-SSB transmission indication, turn an unknown cell into a known cell, a new trigger conditions of measurement reporting should be introduced, i.e., UE triggers measurement reporting after the OD-SSB is activated via MAC CE.

Proposal 5: For Scenario #2, a measurement reporting is triggered after the OD-SSB is activated via MAC CE.

- Scenario #2A

Enhancement for multiple unknown SCells’ activation was discussed in R18. The RRC CR [4] for introduction of FR2 SCell enhancements is shown below.

| Reason for change: | This CR is to introduce the FR2 unknown SCell activation enhancement as requested by RAN4 in their LS in R2-2304636 and R2-2307039. In the enhancement of network triggering report (of L3 measurement result) during FR2 unknown SCell activation, when SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported on the unkown SCell, the RRC measurement report will be intiated which includes the SCell with beam level measurement result for that SCell. RAN2 agreed to reuse the existing RRM measurement framework with a new report type to support this enhancement. In RAN2#123bis meeting, RAN2 made the following agreements on multiple SCell activation case: - If the network activates multiple Sells within same MAC CE the UE may send only one measurement report. |
| --- | --- |

Based on the CR, after receiving the SCell activation command, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated.

Observation 4: During SCell activation procedure, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated.

For Scenario #2A with Case #1, RAN2 can discuss whether the similar functions can be reused, i.e. after receiving the SCell activation command and OD-SSB transmission indication, the L3 measurement report will be initiated if UE has valid L3 measurement results available. For Scenario #2A with Case #2, measurement results of alway-on SSB before Scell activation may not be available, so RAN2 can also discuss whether a similar function can be introduced.

Proposal 6: For Scenario #2A, RAN2 discusses whether to trigger the L3 measurement reporting after receiving the SCell activation command and OD-SSB transmission indication.

OD-SSB measurement of neighbor cell

According to the agreement of “the UE expects to measure SSB when on-demand SSB transmission is activated”, there is a question about whether UE can measure the OD-SSB of a neighbor cell when the OD-SSB transmission of the neighbor cell is activated. According to the WID [5], OD-SSB transmission is only applicable to SCell. Indicating the OD-SSB transmission of the neighbor cell to the UE is not supported.

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

- Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

- Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In addition, during last RAN4 # 112 bis meeting [6], following agreements have been achieved on neighbour cell measurement.

Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neigboring cell, the legacy meaurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. This agreement can be revisited if it is not aligned with further agreement in other WGs.

Based on the agreement, RAN4 will not update the neighbour cell L3 measurement requirement due to the OD-SSB transmission. This means that UE will continue to measure SSB using the always-on SSB measurement configuration, even if OD-SSB transmission is activated. Therefore, OD-SSB SCell operation has no impacts to RRM measurement on neighbor cell.

Proposal 7: OD-SSB SCell operation has no impacts to RRM measurement of neighbor cells.

Contents of OD-SSB MAC CE

In last RAN2 meeting, it was agreed that NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.  In the existing specification [7], the SCell Activation/Deactivation MAC CE support 1 octet (7 SCells) or 4 octets (31 SCells l). Therefore, the new MAC CE should also support sending OD-SSB indication for 7 or 31 SCells simultaneously.

Proposal 8: The new OD-SSB MAC CE can indicate up to 7 or up to 31 SCells.

Based on RAN1 discussion, multiple candidate values can be configured by RRC, e.g., periodicity of OD-SSB, and the applicable value can be indicated by MAC CE for OD-SSB transmission indication. Therefore, 1bit indication may not be suitable for OD-SSB transmission.

In the last RAN1#118 bis meeting, deactivation of on-demand SSB transmission was discussed and no consensus was achieved. Furthermore, during RAN1#118 meeting, RAN1 sent an LS [8] to RAN2, RAN1 respectfully requests RAN2 to design MAC CE based signaling to indicate OD-SSB transmission for Scenarios #2 and #2A, and RAN1 is working on the further details needed for RRC and MAC CE based signaling to indicate OD-SSB transmission and will inform RAN2 as further agreements are made.

Observation 5: RAN1 is already working on the further details of the new MAC CE content for OD-SSB transmission.

Therefore, RAN2 can wait for RAN1 further details on the content of the new MAC CE for OD-SSB transmission.

Proposal 9: RAN2 waits RAN1 progress on the further details of the new MAC CE content for OD-SSB transmission.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: The latency of activating a SCell is different between a known cell and an unknown cell.

Observation 2: In Scenario #2 with Case #1, the SCell may be an unknown cell because the measurement reporting is triggered only after the indication of the OD-SSB transmission.

Observation 3: For Scenario #2 with Case #2, the SCell may be an unknown cell because UE may not trigger the measurement reporting after receiving the OD-SSB transmission indication if the event triggered reporting is configured.

Observation 4: During SCell activation procedure, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated.

Observation 5: RAN1 is already working on the further details of the new MAC CE content for OD-SSB transmission.

OD-SSB measurement of SCell

Proposal 1: The RRM measurement of OD-SSB can be performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

Proposal 2: RAN2 sends an LS to RAN4 to ask RAN4 to define requirements for the RRM measurement of OD-SSB.

Proposal 3: Whether to perform RRM measurement based on OD-SSB is configured by the network. The signalling detail is FFS.

Proposal 4: RAN2 waits RAN1 conclusion about the relation between always-on SSB and OD-SSB.

Proposal 5: For Scenario #2, a measurement reporting is triggered after the OD-SSB is activated via MAC CE.

Proposal 6: For Scenario #2A, RAN2 discusses whether to trigger the L3 measurement reporting after receiving the SCell activation command and OD-SSB transmission indication.

OD-SSB measurement of neighbor cell

Proposal 7: OD-SSB SCell operation has no impacts to RRM measurement of neighbor cells.

Contents of OD-SSB MAC CE

Proposal 8: The new OD-SSB MAC CE can indicate up to 7 or up to 31 SCells.

Proposal 9: RAN2 waits RAN1 progress on the further details of the new MAC CE content for OD-SSB transmission.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #RAN2 127 bis

- R4-2416913, LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN4#112 bis

- 3GPP TS 38.133, “Requirements for support of radio resource management (Release 18)”, V18.4.0 (2023-12)

- R2-2313936, Introduction of FR2 SCell enhancements

- RP-242354 Revised WID: Enhancements of network energy savings for NR

- RAN4 #112bis RRM session meeting report

- 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification (Release 18)”, V18.2.0 (2024-06)

- R1-2407438, LS to RAN2 for on-demand SSB SCell operation, RAN1#118