3GPP TSG-RAN2#128 	R2-2409641

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:		8.12.3 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Impact from UEI beam reporting

Document for:	Discussion and Decision

1.	Introduction

Based on the Rel-19 MIMO WID [1], in the last meeting, RAN2 started to discuss UE-initiated/event-driven beam management (UEI beam management).

In this contribution, we analyse RAN2 impacts of UEI beam reporting based on [1] and RAN1 agreements.

2.	Discussion

According to [1], one of objectives for Rel-19 MIMO WI is to support UEI beam management and RAN1 made following agreements for this aspect.

[RAN1#116] Agreement On UE-initiated/event-driven beam report, at least of following aspects should be included: - Trigger-event detection for beam reporting by UE - UE monitors RS to assess if a beam-reporting trigger condition has been met - FFS: Trigger condition for declaring beam-reporting event - Beam-report transmission by UE - Signaling contents in the beam report - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission - MAC-CE - UCI - Others are not precluded. On UE-initiated/event-driven beam report, the following aspects may be included: - UE requesting UL resource(s) for the beam report - UE notifying transmission of beam report - gNB preconfigured resources Other procedure(s) as required

Based on the above agreement, we can see that UEI beam management is modeled in two steps, i.e., Step 1) event detection/trigger and Step 2) beam reporting transmission.

For Step 1) event detection/trigger, some points need to be determined: Which event triggers beam reporting, Which layer evaluates event for beam reporting, Which RS is used to detect event(s), How to measure RS and Which RS/parameter needs to be configured/updated.

Regarding Which event triggers beam reporting, Which RS is used to detect event(s) and How to measure RS, we think that this is RAN1 scope, and RAN1 already make some progress. Thus, RAN2 don’t need to discuss these points again.

Regarding Which layer evaluates event for beam reporting, in the last meeting, some companies think that the event evaluation procedure should capture in MAC and RAN2 should design a common event evaluation procedure for both UEI beam reporting and LTM L1 MR. This is because RAN2 agreed that MAC layer already handles the event evaluation procedure for LTM and both procedures are based on L1 measurement.

Agreements on L1 MR event evaluation - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering.

However, whether the event evaluation procedure for UEI beam reporting is in MAC or PHY is still remained as FFS in RAN1. Thus, it would be better to wait RAN1 further progress whether the event evaluation procedure is captured in MAC or not.

In addition, even if RAN1 concludes to specify the event evaluation procedure in MAC specification, it is not sure whether a common event evaluation procedure can be designed for both UEI beam reporting and LTM. This is because the RS configuration for L1 measurement is different, and reporting signaling is different, i.e., UCI for UEI beam reporting and MAC CE for LTM L1 MR. Thus, it would be better to design the event evaluation procedure for UEI beam reporting after details are concluded in RAN1.

Therefore, we propose to wait RAN1 further progress on UEI beam reporting before starting to discuss MAC impact by UEI beam reporting.

Proposal 1. RAN2 waits for RAN1 further progress on UEI beam reporting before starting to discuss MAC impact by UEI beam reporting.

For the configuration/update of RS/parameter for beam reporting, RAN1 made some agreements and sent LS [2] to inform of RRC parameter list in the last meeting.

However, the RRC parameter list for UEI beam reporting is incomplete and many things are still under discussion in RAN1. We think that it is sufficient to start RAN2 discussion on RRC configuration after RAN1 makes further progress.

Proposal 2. RAN2 start the discussion on the configuration/update of RS/parameter for UEI beam reporting after RAN1 makes further progress.

For Step 2) beam reporting transmission, RAN1 agreed to support Mode-A and Mode-B as follows.

[RAN1#116bis] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode-A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This option is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode-B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all options For above procedures, cross-CC beam reporting is supported for both options. - FFS: Details.

RAN1 made further agreements in the past several meetings, and based on the RAN1 agreements, Mode-A and Model-B can be modeled as follows:

- Mode-A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH to request a resource for a second UL channel to carry beam report. (FFS: dedicated SR or a new UCI, FFS: one-bit or multi-bit)

  - Step 2: UE detects the UL-grant DCI format (at least DCI format 0_1/0_2) to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel - PUSCH. (FFS: the second UL channel is PUCCH).

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH notifying a second UL channel to carry beam report. (FFS: dedicated SR or a new UCI, one-bit or multi-bit)

  - Step 2: UE transmits the beam report in the second UL channel - Type 1 CG PUSCH. (FFS: the second UL channel is PUCCH, FFS: whether Type 1 CG PUSCH can carry UL-SCH or is dedicated to beam reporting)

For Step 1 of Mode-A and Mode-B, if RAN1 agreed to support dedicated SR, there is RAN2 impact to introduce SR procedure in the MAC specification. However, this is still FFS, so RAN2 don’t need to start the discussion until there is RAN1 input.

Observation 1. For beam reporting transmission, depending on whether the first PUCCH is dedicated SR or not, there may be RAN2 impact to introduce SR procedure, but RAN2 don’t need to start the discuss until there is RAN1 input.

For Step 2 and Step 3 of Mode-A, the UE receives a DCI to indicate to transmit beam reporting, then the UE transmits beam reporting. This is similar to the legacy aperiodic CSI reporting and we think there is no RAN2 impact, except RRC configuration.

Observation 2. For Step 2 and Mode 3 of Mode-A, there is no RAN2 impact except RRC configuration.

For Step 2 of Mode-B, if RAN1 agreed to support Type 1 CG PUSCH dedicated to beam reporting, there may be RAN2 impact. For example, RAN2 need to discuss how to check whether CG occasion can be used for UL-SCH or for beam reporting only and how to handle the overlapping case between the dedicated CG PUSCH and other PUSCH/PUCCH. However, this is still FFS, so RAN2 don’t need to start the discussion until there is RAN1 input.

Observation 3. For Step 2 of Mode-B, depending on whether Type 1 CG PUSCH is dedicated to beam reporting or not, there may be RAN2 impact, but RAN2 don’t need to start discussion until there is RAN1 input.

Based on Observation 1, 2 and 3, we propose that RAN2 don’t start to discuss beam reporting transmission until there is RAN1 input

Proposal 3. RAN2 do not start the discussion on beam reporting transmission until there is RAN1 input.

3.	Conclusion

In this document, we analysis RAN2 impacts of UEI beam reporting, and made observations and proposals as follows.

Observation 1. For beam reporting transmission, depending on whether the first PUCCH is dedicated SR or not, there may be RAN2 impact to introduce SR procedure, but RAN2 don’t need to start the discuss until there is RAN1 input.

Observation 2. For Step 2 and Mode 3 of Mode-A, there is no RAN2 impact except RRC configuration.

Observation 3. For Step 2 of Mode-B, depending on whether Type 1 CG PUSCH is dedicated to beam reporting or not, there may be RAN2 impact, but RAN2 don’t need to start discussion until there is RAN1 input.

Proposal 1. RAN2 waits for RAN1 further progress on UEI beam reporting before starting to discuss MAC impact by UEI beam reporting.

Proposal 2. RAN2 start the discussion on the configuration/update of RS/parameter for UEI beam reporting after RAN1 makes further progress.

Proposal 3. RAN2 do not start the discussion on beam reporting transmission until there is RAN1 input.

4.	Reference

- RP-242394	WID revision: NR MIMO Phase 5	Samsung

- R2-2407906	LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5	RAN1