3GPP TSG-RAN WG2 Meeting #128	R2-2409667

Orlando, USA, Nov. 18th – 22nd, 2024

Source:	vivo

Title:	Updated simulation results for RRM measurement prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#127bis meeting, the following agreements were made based on the initial simulation results:

Agreements on intra-frequency

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.  RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy

- Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used.   The companies should at least provide the results of only one value it should be the last value at the end of the PW.   We will add two columns in the spreadsheet to capture the last value and the average value.

Agreements on inter-frequency

- Companies need to report whether earlier predicted results are also used as inputs for future RRM prediction.

- Companies should report with their simulation the correlation coefficient

- Higher-to-lower and lower-to-higher frequency prediction is comparable

- For co-located scenario,  the UE speed in the inter-frequency case has minor impact on prediction accuracy

In this contribution, we will provide updated simulation results on the agreed scenarios and agreed assumptions.

- Discussion

  - FR1 to FR1 intra-frequency (temporal domain)

    - Clarification on L1 filtering option

At the RAN#127 meeting, RAN2 discussed and agreed on two L1 filtering options, i.e., sliding and non-sliding L1 filtering.

Agreements 1 To keep two filtering options on the table and up to company to report.

Figure 1 Sliding L1/L3 filtering Figure 2: non-sliding L1/L3 filtering In sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every sample period. In non-sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every measurement period. In both L1/L3 filtering options, the filtered L1 measurement result is obtained based on the non-filtered L1 measurement results within one measurement period. And the filtered L3 measurement result is obtained as specified in section 5.5.3.2 of TS 38.331

However, we found that certain combinations of L1/L3 filtering options and the sub-use cases for measurement reduction Case B may lead to feasibility issues.

Sliding L1/L3 filtering

Figure 2.1-1 Sub-use case 1 (input & output: L1 sample without L1 filtering)

In sub-use case 1, L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated. For this sub-use case, if both input and output are L1 beam-level measurements without L1 filtering, the sliding L1/L3 filtering is feasible and the pattern is in Figure 2.1-1.

Figure 2.1-2 Sub-use case 2

Figure 2.1-3 Sub-use case 3 (input: L1 sample without L1 filtering)

In sub-use cases 2 and 3, cell-level measurement result(s) is predicted based on actual cell-level or L1 beam-level measurement result(s). For these two sub-use cases, the sliding L1/L3 filtering is not feasible as illustrated in Figure 2.1-2 and Figure 2.1-3.

Proposal 1: For measurement reduction Case B, sliding L1/L3 filtering is only applicable for sub-use case 1 when both input and output are L1 beam-level measurements without L1 filtering.

Non-sliding L1/L3 filtering

Figure 2.1-4 Sub-use case 1 (input & output: L1 filtered RSRP)

Figure 2.1-5 Sub-use case 2 (input: L3 filtered RSRP)

Figure 2.1-6 Sub-use case 3 (input: L1 filtered RSRP)

In all sub-use cases, the non-sliding L1/L3 filtering is feasible for measurement reduction Case B, and the patterns are shown in Figure 2.1-4 to Figure 2.1-6.

Proposal 2: For measurement reduction Case B, non-sliding L1/L3 filtering is applicable for all sub-use cases.

    - Clarification on L3 filtering option

In the email discussion [POST127bis][022], the following three options for L3 filtering are provided as candidates:

Observation 1: For temporal domain prediction case B, the following filtering scheme can be considered: - Filtering option 1: L3 filtering is based on the L1 filtered result and the last L3 filtered cell result; - Filtering option 2: L3 filtering is based on the L1 filtered result if the last L3 filtered result is from prediction; - Filtering option 3: L3 filtering is based on the L1 filtered result and the L3 filtered result from last actual measurement.

Within the above three options, Option 1 is similar to the current L3 filtering mechanism, with the difference being that it considers predictive results when calculating the L3 result. Therefore, Option 1 is selected in our simulation.

Furthermore, for Sub-use case 2, as the input is L3 filtered RSRP, inaccurate predictive results can introduce errors into the next inference, thereby affecting the accuracy of the next inference, i.e., the errors will accumulate. For Sub-use cases 1 and 3, since the input is L1 RSRP, Option 1 will not have an impact on error accumulation for other sub-use cases.

Observation 1: For measurement reduction Case B, L3 filtering Option 1 (i.e., L3 filtering is based on the L1 filtered result and the last L3 filtered cell result) may cause prediction error accumulation in sub-use case 2.

    - Sliding L1/L3 filtering (Sub-use case 1)

For sub-use case 1 with sliding L1/L3 filtering, we select the pattern of inputs and outputs shown in Figure 2.1-1, where the PW length is the L1 sample period multiplied by the number of L1 samples, i.e., 40ms*N. For MRRT of 1/2, N=1.

For the non-AI benchmark, ‘sample and hold’ is selected for this scenario, i.e., the predicted value equals the previous actual measured value.

The results of prediction accuracy (L1 sample RSRP) for different combinations of UE speed and OW length under MRRT of 1/2 are shown in Table 2.1-1.

Table 2.1-1 L1/L3 RSRP of sub-use case 1 (sliding L1/L3 filtering, MRRT=1/2)

| OW length (ms) |  | 6*40 | 8*40 | 10*40 | 20*40 |
| --- | --- | --- | --- | --- | --- |
| Number of samples within OW |  | 6 | 8 | 10 | 20 |
| Number of samples as input |  | 3 | 4 | 5 | 10 |
| L1 beam RSRP difference (30km/h) | AI | 0.2 | 0.19 | 0.19 | 0.18 |
|  | Non-AI | 0.19 | 0.19 | 0.19 | 0.19 |
| L3 cell RSRP difference (30km/h) | AI | 0.112 | 0.105 | 0.102 | 0.100 |
|  | Non-AI | 0.108 | 0.108 | 0.108 | 0.108 |
| L1 beam RSRP difference (60km/h) | AI | 0.324 | 0.316 | 0.314 | 0.302 |
|  | Non-AI | 0.347 | 0.347 | 0.347 | 0.347 |
| L3 cell RSRP difference (60km/h) | AI | 0.175 | 0.168 | 0.167 | 0.160 |
|  | Non-AI | 0.192 | 0.192 | 0.192 | 0.192 |
| L1 beam RSRP difference (90km/h) | AI | 0.483 | 0.470 | 0.471 | 0.467 |
|  | Non-AI | 0.525 | 0.525 | 0.525 | 0.525 |
| L3 cell RSRP difference (90km/h) | AI | 0.245 | 0.235 | 0.236 | 0.231 |
|  | Non-AI | 0.275 | 0.275 | 0.275 | 0.275 |

It can be observed that for sliding L1 filtering, the prediction accuracy is mostly quite small, i.e., less than 1dB. This is because the RSRP change is relatively small within a short prediction window. And the Non-AI approach can also achieve comparable performance. From our understanding, this is because the L1 sample values are unstable and the corresponding rule of RSRP change is not clear, so a longer observation window does not provide more information to improve prediction accuracy. That is, the AI model primarily relies on the last RSRP measurement in the observation window to generate the prediction.

Observation 2a: For temporal domain prediction Case B, if the model input is L1 samples (without filtering), the prediction accuracy is quite close to the non-AI approach.

    - Non-Sliding L1/L3 filtering (Sub-use cases 1/2/3)

For the non-sliding L1/L3 filtering scenario, all the Cases are considered:

- For Sub-use case 1, the pattern of inputs and outputs is shown in Figure 2.1-4, and the PW length is the measurement of L1 filtered RSRP measurement period multiplied by the number of L1 filtered RSRP, i.e., 200ms*N, For MRRT of 1/2, N=1; for MRRT of 4/5, N=4.

- For Sub-use case 2, the pattern of inputs and outputs is shown in Figure 2.1-5, and the PW length is the measurement of L3 filtered RSRP measurement period multiplied by the number of L3 filtered RSRP, i.e., 200ms*N, For MRRT of 1/2, N=1; for MRRT of 4/5, N=4.

- For Sub-use case 3, the pattern of inputs and outputs is shown in Figure 2.1-6, and the PW length is the measurement of L3 filtered RSRP measurement period multiplied by the number of L3 filtered RSRP, i.e., 200ms*N, For MRRT of 1/2, N=1; for MRRT of 4/5, N=4.

For Case 1, the simulation results of prediction accuracy for different combinations of UE speed and OW length under MRRT of 1/2 are shown in Table 2.1-2.

Table 2.1-2 Simulation results of sub-use case 1 (Non-Sliding L1/L3 filtering, MRTT=1/2)

| OW length (ms) |  | 4*200 | 6*200 | 8*200 | 10*200 |
| --- | --- | --- | --- | --- | --- |
| Number of samples within OW |  | 4 | 6 | 8 | 10 |
| Number of samples as input |  | 2 | 3 | 4 | 5 |
| L1 beam RSRP difference (30km/h) | AI | 0.901 | 0.794 | 0.781 | 0.779 |
|  | Non-AI | 0.904 | 0.904 | 0.904 | 0.904 |
| L3 cell RSRP difference (30km/h) | AI | 0.534 | 0.457 | 0.445 | 0.446 |
|  | Non-AI | 0.538 | 0.538 | 0.538 | 0.538 |
| L1 beam RSRP difference (60km/h) | AI | 1.239 | 1.140 | 1.135 | 1.136 |
|  | Non-AI | 1.266 | 1.266 | 1.266 | 1.266 |
| L3 cell RSRP difference (60km/h) | AI | 0.744 | 0.684 | 0.680 | 0.678 |
|  | Non-AI | 0.77 | 0.77 | 0.77 | 0.77 |
| L1 beam RSRP difference (90km/h) | AI | 1.871 | 1.767 | 1.757 | 1.759 |
|  | Non-AI | 1.881 | 1.881 | 1.881 | 1.881 |
| L3 cell RSRP difference (90km/h) | AI | 0.946 | 0.892 | 0.876 | 0.881 |
|  | Non-AI | 0.955 | 0.955 | 0.955 | 0.955 |

For sub-use case 1 with non-sliding L1/L3 filtering, unlike the sliding approach, L1 filtered RSRP is selected as input and output. From the above Table 2.1-2, it can be observed that with sufficient long OW length, the AI-based approach is much better than that of the non-AI approach.

Observation 2b: For temporal domain prediction Case B, if the model input is L1 filtered RSRP, the prediction accuracy is better than non-AI approach.

Proposal 3: The L1 sample (without filtering) is not expected to be the model input for RRM measurement prediction.

To analyze the impact of MRRT on prediction accuracy, the simulation results of prediction accuracy for different combinations of UE speed and OW length under MRRT of 4/5 are shown in Table 2.1-3.

Table 2.1-3 Simulation results of sub-use case 1 (Non-Sliding L1/L3 filtering, MRTT=4/5)

| OW length (ms) |  | 10*200 |
| --- | --- | --- |
| Number of samples within OW |  | 10 |
| Number of samples as input |  | 2 |
| L1 beam RSRP difference (30km/h) | AI | 2.016 |
|  | Non-AI | 2.093 |
| L3 cell RSRP difference (30km/h) | AI | 1.641 |
|  | Non-AI | 1.733 |
| Multiple predicted point L3 cell RSRP differences (30km/h) | AI | [0.994,1.396,1.859,2.315] |
|  | Non-AI | [1.076,1.485,1.956,2.417] |
| L1 beam RSRP difference (60km/h) | AI | 3.362 |
|  | Non-AI | 3.391 |
| L3 cell RSRP difference (60km/h) | AI | 2.676 |
|  | Non-AI | 2.72 |
| Multiple predicted point L3 cell RSRP differences (60km/h) | AI | [1.706,2.248,2.99,3.761] |
|  | Non-AI | [1.747,2.29,3.034,3.811] |
| L1 beam RSRP difference (90km/h) | AI | 4.663 |
|  | Non-AI | 4.703 |
| L3 cell RSRP difference (90km/h) | AI | 3.217 |
|  | Non-AI | 3.539 |
| Multiple predicted point L3 cell RSRP differences (90km/h) | AI | [2.028,2.712,3.601,4.528] |
|  | Non-AI | [1.528,2.714,4.22,5.698] |

By comparing Table 2.1-2 and Table 2.1-3, it can be seen that as the MRRT increases, i.e., more skipped measurement samples, there is a significant decrease in prediction accuracy. For example, in sub-use case 1, corresponding to speeds of 30km/h to 90km/h, the prediction accuracy range of L3 cell RSRP is 1.641~3.217 dB, which is quite close to the actual RSRP difference (1.733~3.539 dB). This is mainly due to two reasons: first, as the MRRT increases, the prediction window becomes longer, and the gap between the predicted value and the actual measured value in the temporal domain also increases; second, as the MRRT increases, the actual measurements within the same observation window length decrease, resulting in a decrease in the input information.

Observation 3: For temporal domain prediction Case B, as the MRRT increases, prediction accuracy decreases.

The simulation results for sub-use case 2 and sub-use case 3 (MRTT=1/2) are shown in Tables 2.1-4 and Tables 2.1-5.

Table 2.1-4 Simulation results of sub-use case 2 (Non-Sliding L1/L3 filtering, MRTT=1/2)

| OW length (ms) |  | 4*200 | 6*200 | 8*200 | 10*200 |
| --- | --- | --- | --- | --- | --- |
| Number of samples within OW |  | 4 | 6 | 8 | 10 |
| Number of samples as input |  | 2 | 3 | 4 | 5 |
| Prediction accuracy (30km/h) | AI | 1.685 | 1.731 | 1.708 | 1.730 |
|  | Non-AI | 0.538 | 0.538 | 0.538 | 0.538 |
| Prediction accuracy (60km/h) | AI | 2.656 | 2.684 | 2.718 | 2.743 |
|  | Non-AI | 0.77 | 0.77 | 0.77 | 0.77 |
| Prediction accuracy (90km/h) | AI | 3.763 | 3.832 | 3.802 | 3.915 |
|  | Non-AI | 0.955 | 0.955 | 0.955 | 0.955 |

Table 2.1-5 Simulation results of sub-use case 3 (Non-Sliding L1/L3 filtering, MRTT=1/2)

| OW length (ms) |  | 4*200 | 6*200 | 8*200 | 10*200 |
| --- | --- | --- | --- | --- | --- |
| Number of samples within OW |  | 4 | 6 | 8 | 10 |
| Number of samples as input |  | 2 | 3 | 4 | 5 |
| Prediction accuracy (30km/h) | AI | 0.575 | 0.454 | 0.455 | 0.453 |
|  | Non-AI | 0.538 | 0.538 | 0.538 | 0.538 |
| Prediction accuracy (60km/h) | AI | 0.964 | 0.8 | 0.796 | 0.796 |
|  | Non-AI | 0.77 | 0.77 | 0.77 | 0.77 |
| Prediction accuracy (90km/h) | AI | 1.258 | 1.182 | 1.181 | 1.178 |
|  | Non-AI | 0.955 | 0.955 | 0.955 | 0.955 |

For the same OW length, the performance of sub-use case 1 is better than that of sub-use case 2 and sub-use case 3. From our understanding, the poor performance of sub-use case 2 is the prediction error accumulation issue in Observation 1 of section 2.1.2. In addition, the performance of sub-use cases 2 and 3 is worse than the Non-AI approach. Therefore, for measurement reduction Case B, sub-use case 1 should be prioritized.

Observation 4: For temporal domain prediction Case B, the performance of sub-use case 1 is better than that of sub-use case 2 and sub-use case 3.

Proposal 4: For measurement reduction Case B, sub-use case 1 should be prioritized.

  - FR1 to FR1 inter-frequency (frequency domain)

    - Pattern of inter-frequency prediction

For frequency domain prediction with measurement reduction purposes, the pattern/ timing relationship between the measurement (on frequency 1) and prediction (on frequency 2) should be clarified.

From our understanding, the following two options can be considered:

-	Option 1: The input is historical measurement values on Frequency 1(e.g., instances 1-10) and the output is the value on Frequency 2 at the last time instance (e.g., instances 10) of measurement on Frequency 1.

-	Option 2: The input is historical measurement values on frequency 1 (e.g., instances 1-10) and the output is the values on Frequency 2 at the same time instances (e.g., instances 1-10) of measurement on Frequency 1.

Figure 2.2-1(a) Pattern of inter-frequency prediction of Option 1

Figure 2.2-1(b) Pattern of inter-frequency prediction of Option 2

The advantage of Option 2 is that multiple instances on Frequency 2 can be out for one single model inference. On the contrary, multiple model inferences are needed for Option 1 to obtain multiple predicted instances on Frequency 2. However, the typical usage of inter-frequency prediction is inter-frequency handover/redirection, in which Option 2 is not applicable. For instance, extra latency of historical prediction at Frequency 2 may lead to too-late handover.

Therefore, the pattern of Option 1 should be the baseline for inter-frequency prediction.

Proposal 5: For frequency domain prediction for measurement reduction, the input is a set of historical measurement instances on Frequency 1 and the output is the predicted value on Frequency 2 at the time of the last measurement instance on Frequency 1.

For the inter-frequency simulation, we adopt the pattern of inputs and outputs shown in Figure 2.2-1(a).

    - Simulation results

For the prediction from low-frequency (2GHz) to high-frequency (4GHz), the results of prediction accuracy for different combinations of UE speed and OW length are shown in Table 2.2-1.

Table 2.2-1 Simulation results of prediction from 2GHz to 4GHz

| OW length (ms) | 5*40 | 10*40 | 20*40 | 30*40 | 40*40 | 50*40 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of samples within OW | 5 | 10 | 20 | 30 | 40 | 50 |
| Prediction accuracy (30km/h) | 1.746 | 1.483 | 1.153 | 0.932 | 0.823 | 0.721 |
| Prediction accuracy (60km/h) | 2.218 | 1.980 | 1.599 | 1.364 | 1.215 | 1.099 |
| Prediction accuracy (90km/h) | 2.419 | 2.186 | 1.867 | 1.603 | 1.418 | 1.356 |

For the non-AI approach, a simple approach is to calculate the average RRM measurement difference between two frequencies (i.e., frequency of serving cell and frequency for prediction) in the training dataset. In the inference phase, the prediction can be derived based on the measurement results of the serving cell and the average RRM measurement difference. The results of the Non-AI approach are shown in Table 2.2-2.

Table 2.2-2 Results of prediction from 2GHz to 4GHz based on basic Non-AI approach

|  | average RRM measurement difference (dB) | Prediction accuracy (dB) |
| --- | --- | --- |
| UE speed (30km/h) | 2.221 | 3.278 |
| UE speed (60km/h) | 2.653 | 3.252 |
| UE speed (90km/h) | 2.780 | 3.167 |

By comparing Table 2.2-1 and Table 2.2-2, it can be seen that the performance of AI-based approach is much better than that of basic non-AI approach. From our understanding, it is due to the RRM measurements of difference frequencies for the co-located cells having a high correlation coefficient (shown in Table 2.2-3).

Table 2.2-3 Correlation coefficient

|  | Channel correlation coefficient |
| --- | --- |
| UE speed (30km/h) | 0.923 |
| UE speed (60km/h) | 0.915 |
| UE speed (90km/h) | 0.926 |

In addition, if some of the instances can be measured in the frequency for prediction, the prediction accuracy of the non-AI approach can be improved significantly. Specifically, if the Inter-frequency RRM measurement results are available for each N instances, the enhanced non-AI approach can use the updated RRM measurement difference to derive the prediction. The corresponding results are shown in Table 2.2-4, and the performance is much better than AI-based approach with a small value of N.

Table 2.2-4 Results of prediction from 2GHz to 4GHz based on enhanced Non-AI approach

| Inter-frequency measurement is available every N instances | 5 | 10 | 20 | 30 | 40 | 50 |
| --- | --- | --- | --- | --- | --- | --- |
| Prediction accuracy (30km/h) | 0.087 | 0.193 | 0.394 | 0.579 | 0.740 | 0.892 |
| Prediction accuracy (60km/h) | 0.146 | 0.318 | 0.641 | 0.914 | 1.150 | 1.354 |
| Prediction accuracy (90km/h) | 0.194 | 0.421 | 0.812 | 1.134 | 1.403 | 1.643 |

Observation 5: For FR1 to FR1 inter-frequency prediction, the correlation coefficient of the RRM measurements on the two frequencies of co-located cells is quite good (0.915~0.926).

Observation 6: For FR1 to FR1 inter-frequency prediction, if part of the Inter-frequency RRM measurement results are available, non-AI approach can achieve good performance.

Proposal 6: For FR1 to FR1 inter-frequency prediction, non-colocated neighbouring cells should also be considered.

  - FR2 to FR2 intra-frequency (temporal domain)

In the updated TR [1], for intra-frequency temporal domain case A, the ratio between OW and PW is at least limited to the value range {5,4,3,2,1,1/2,1/3,1/4,1/5}.

As the PW length is 400 ms and the sampling rate is 80 ms for FR2, the applicable value range of OW length is {80,160,240,320,400,800,1200,1600,2000} ms. And the corresponding ratio between OW and PW is {1/5, 2/5, 3/5, 4/5, 1, 2, 3, 4, 5}. Therefore, we propose:

Proposal 7: Update the ratio between OW and PW to the value range {1/5, 2/5, 3/5, 4/5, 1, 2, 3, 4, 5}.

At RAN2#127bis meeting, it was observed that among sub-use cases 1, 2, and 3, at least with a shorter prediction window sub-use case 2 demonstrates the highest prediction accuracy. The simulation results of FR2 intra-frequency for the sub-use case 2 are illustrated in the following Table 2.3-1:

Table 2.3-1 Simulation results of sub-use case 2

| OW length (ms) |  | 80 | 160 | 240 | 320 | 400 |
| --- | --- | --- | --- | --- | --- | --- |
| OW length/PW length |  | 1/5 | 2/5 | 3/5 | 4/5 | 1 |
| L3 cell RSRP difference (60km/h) | AI | 2.095 | 1.302 | 1.198 | 1.155 | 1.146 |
|  | Non-AI | 2.096 | 2.096 | 2.096 | 2.096 | 2.096 |
| Multiple predicted point L3 cell RSRP differences (60km/h) | AI | [0.737, 1.448, 2.133, 2.776, 3.38] | [0.278, 0.7, 1.236, 1.834, 2.462] | [0.268, 0.609, 1.113, 1.689, 2.312] | [0.246, 0.586, 1.069, 1.631, 2.242] | [0.232, 0.586, 1.065, 1.621, 2.225] |
|  | Non-AI | [0.732, 1.446, 2.133, 2.78, 3.388] | [0.732, 1.446, 2.133, 2.78, 3.388] | [0.732, 1.446, 2.133, 2.78, 3.388] | [0.732, 1.446, 2.133, 2.78, 3.388] | [0.732, 1.446, 2.133, 2.78, 3.388] |
| L3 cell RSRP difference (90km/h) | AI | 2.562 | 1.753 | 1.591 | 1.538 | 1.531 |
|  | Non-AI | 2.569 | 2.569 | 2.569 | 2.569 | 2.569 |
| Multiple predicted point L3 cell RSRP differences (90km/h) | AI | [0.92, 1.804, 2.624, 3.386, 4.078] | [0.408, 0.974, 1.679, 2.458, 3.245] | [0.339, 0.846, 1.5, 2.246, 3.026] | [0.33, 0.813, 1.44, 2.168, 2.938] | [0.328, 0.812, 1.434, 2.156, 2.923] |
|  | Non-AI | [0.918, 1.804, 2.628, 3.396, 4.098] | [0.918, 1.804, 2.628, 3.396, 4.098] | [0.918, 1.804, 2.628, 3.396, 4.098] | [0.918, 1.804, 2.628, 3.396, 4.098] | [0.918, 1.804, 2.628, 3.396, 4.098] |
| L3 cell RSRP difference (120km/h) | AI | 2.784 | 1.986 | 1.807 | 1.766 | 1.723 |
|  | Non-AI | 2.792 | 2.792 | 2.792 | 2.792 | 2.792 |
| Multiple predicted point L3 cell RSRP differences (120km/h) | AI | [1.016, 1.969, 2.858, 3.672, 4.407] | [0.464, 1.118, 1.923, 2.786, 3.637] | [0.392, 0.977, 1.72, 2.547, 3.4] | [0.388, 0.947, 1.673, 2.487, 3.333] | [0.375, 0.922, 1.626, 2.426, 3.267] |
|  | Non-AI | [1.013, 1.969, 2.862, 3.685, 4.429] | [1.013, 1.969, 2.862, 3.685, 4.429] | [1.013, 1.969, 2.862, 3.685, 4.429] | [1.013, 1.969, 2.862, 3.685, 4.429] | [1.013, 1.969, 2.862, 3.685, 4.429] |

The results show that within the prediction window, the accuracy of the later prediction instances is worse. The specific impact can be analyzed in the simulation of measurement event prediction and/or system-level performance.

Observation 7: Within the prediction window, the prediction accuracy of the later prediction instances is worse.

  - Beam-level RRM measurement prediction

At the RAN2#127 meeting, in addition to cell-level prediction, 3 sub-use cases are considered for beam-level RRM measurement prediction:

Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and should focus on FR2 intra-frequency temporal domain case A

We assume that some of the observations of cell-level prediction on FR2 intra-frequency temporal domain case A are also valid for Beam-level, e.g.,

-	For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

-	Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits.

Therefore, both OW and PW length are set as 400 ms in the simulation and the simulation results are shown in Table 2.4-1.

Table 2.4-1 Simulation results of beam-level RRM measurement prediction

|  |  | Sub-use case 4 | Sub-use case 5 | Sub-use case 6 |
| --- | --- | --- | --- | --- |
| L3 beam RSRP difference (60km/h) | AI | 1.307 | 1.299 | 1.271 |
|  | Non-AI | 1.993 | 2.287 | 1.993 |
| Multiple predicted point L3 beam RSRP differences (60km/h) | AI | [0.614, 0.811, 1.19, 1.68, 2.241] | [0.46, 0.742, 1.2, 1.745, 2.353] | [0.606, 0.783, 1.145, 1.623, 2.196] |
|  | Non-AI | [0.904, 1.331, 1.929, 2.575, 3.224] | [0.798, 1.577, 2.329, 3.036, 3.699] | [0.904, 1.331, 1.929, 2.575, 3.224] |
| L3 beam RSRP difference (90km/h) | AI | 1.786 | 1.71 | 1.68 |
|  | Non-AI | 2.536 | 2.792 | 2.536 |
| Multiple predicted point L3 beam RSRP differences (90km/h) | AI | [0.842, 1.143, 1.647, 2.289, 3.008] | [0.588, 0.974, 1.585, 2.312, 3.095] | [0.717, 1.025, 1.54, 2.191, 2.929] |
|  | Non-AI | [1.186, 1.738, 2.471, 3.257, 4.025] | [0.995, 1.958, 2.856, 3.695, 4.46] | [1.186, 1.738, 2.471, 3.257, 4.025] |
| L3 beam RSRP difference (120km/h) | AI | 2.09 | 1.947 | 1.909 |
|  | Non-AI | 2.831 | 3.033 | 2.831 |
| Multiple predicted point L3 beam RSRP differences (120km/h) | AI | [0.981, 1.354, 1.956, 2.681, 3.475] | [0.631, 1.138, 1.826, 2.643, 3.501] | [0.814, 1.175, 1.763, 2.492, 3.304] |
|  | Non-AI | [1.364, 1.964, 2.772, 3.62, 4.437] | [1.095, 2.134, 3.109, 4.008, 4.819] | [1.364, 1.964, 2.772, 3.62, 4.437] |

It can be observed in Table 2.4-1 that the performance of prediction accuracy of sub-use cases 4/5/6 is comparable.

Observation 8: For beam-level temporal domain prediction, the prediction accuracy of sub-use cases 4/5/6 is comparable.

Note that the above prediction accuracy is calculated based on the prediction for all the beams. However, the intention for beam-level prediction is for RACH resource allocation. Thus only the prediction of best K (K>= 1) beams are needed and the performance analysis should focus on them, which is quite similar to beam management.

The KPI for beam management is copied as follows:

Beam prediction accuracy related KPIs, including: - Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - Specific Rx beam(s) are to be reported. Note: specific Rx beams are a subset of all Rx beams. - Top-1 genie-aided Tx-Rx beam pair considers the following definitions: - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options not precluded and can be reported - Average L1-RSRP difference of Top-1 predicted beam: - The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam - Beam prediction accuracy (%): - Top-1 (%): the percentage of "the Top-1 genie-aided beam is Top-1 predicted beam" - Top-K/1 (%): the percentage of "the Top-1 genie-aided beam is one of the Top-K predicted beams" - Top-1/K (%) (Optional): the percentage of "the Top-1 predicted beam is one of the Top-K genie-aided beams" - Where K >1 and values can be reported - CDF of L1-RSRP difference for Top-1 predicted beam - Beam prediction accuracy (%) with 1dB margin for Top-1 beam - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam" - Other beam prediction accuracy related KPIs are not precluded and can be reported

Within them, we think the average L1-RSRP difference of Top-1 predicted beam and beam prediction accuracy (%) with 1dB margin for Top-1 beam can be reused for beam-level temporal domain prediction.

Proposal 8: For beam-level temporal domain prediction, reuse the following KPIs of AI/ML-based beam management:

- Average RSRP difference of Top-1 predicted beam: The RSRP difference between the Top-1 predicted beam and the actual Top-1 beam.

- Beam prediction accuracy (%) with 1dB margin for Top-1 beam: the percentage of the Top-1 predicted beam whose prediction accuracy is within 1dB.

  - FR2 to FR2 intra-frequency spatial domain

In the previous RAN2 meetings, an issue was raised that, it is unclear how to derive cell quality based on both predicted beam qualities and measured beam qualities in the case of spatial RRM prediction. From our understanding, predicted beam qualities are different from measured beam qualities considering predicted beam qualities may be inaccurate compared to measured beam qualities. It is unclear whether predicted beam qualities can directly be used together with measured beam qualities for beam consolidation/selection. We therefore suggest RAN2 to discuss this issue.

Proposal 9: For spatial domain prediction, RAN2 to discuss how to derive cell-level quality based on predicted beam qualities and measured beam qualities.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

FR1 to FR1 intra-frequency (temporal domain)

Observation 1: For measurement reduction Case B, L3 filtering Option 1 (i.e., L3 filtering is based on the L1 filtered result and the last L3 filtered cell result) may cause prediction error accumulation in sub-use case 2.

Observation 2a: For temporal domain prediction Case B, if the model input is L1 samples (without filtering), the prediction accuracy is quite close to the non-AI approach.

Observation 2b: For temporal domain prediction Case B, if the model input is L1 filtered RSRP, the prediction accuracy is better than non-AI approach.

Observation 3: For temporal domain prediction Case B, as the MRRT increases, prediction accuracy decreases.

Observation 4: For temporal domain prediction Case B, the performance of sub-use case 1 is better than that of sub-use case 2 and sub-use case 3.

Proposal 1: For measurement reduction Case B, sliding L1/L3 filtering is only applicable for sub-use case 1 when both input and output are L1 beam-level measurements without L1 filtering.

Proposal 2: For measurement reduction Case B, non-sliding L1/L3 filtering is applicable for all sub-use cases.

Proposal 3: The L1 sample (without filtering) is not expected to be the model input for RRM measurement prediction.

Proposal 4: For measurement reduction Case B, sub-use case 1 should be prioritized.

FR1 to FR1 inter-frequency (frequency domain)

Observation 5: For FR1 to FR1 inter-frequency prediction, the correlation coefficient of the RRM measurements on the two frequencies of co-located cells is quite good (0.915~0.926).

Observation 6: For FR1 to FR1 inter-frequency prediction, if part of the Inter-frequency RRM measurement results are available, non-AI approach can achieve good performance.

Proposal 5: For frequency domain prediction for measurement reduction, the input is a set of historical measurement instances on Frequency 1 and the output is the predicted value on Frequency 2 at the time of the last measurement instance on Frequency 1.

Proposal 6: For FR1 to FR1 inter-frequency prediction, non-colocated neighbouring cells should also be considered.

FR2 to FR2 intra-frequency (temporal domain)

Observation 7: Within the prediction window, the prediction accuracy of the later prediction instances is worse.

Proposal 7: Update the ratio between OW and PW to the value range {1/5, 2/5, 3/5, 4/5, 1, 2, 3, 4, 5}.

Beam-level RRM measurement prediction

Observation 8: For beam-level temporal domain prediction, the prediction accuracy of sub-use cases 4/5/6 is comparable.

Proposal 8: For beam-level temporal domain prediction, reuse the following KPIs of AI/ML-based beam management:

- Average RSRP difference of Top-1 predicted beam: The RSRP difference between the Top-1 predicted beam and the actual Top-1 beam.

- Beam prediction accuracy (%) with 1dB margin for Top-1 beam: the percentage of the Top-1 predicted beam whose prediction accuracy is within 1dB.

FR2 to FR2 intra-frequency (spatial domain)

Proposal 9: For spatial domain prediction, RAN2 to discuss how to derive cell-level quality based on predicted beam qualities and measured beam qualities.

- Reference

- R2-2409011	Text proposal on TR 38.744	OPPO