3GPP TSG-RAN WG2 Meeting #128	R2-2409668

Orlando, USA, Nov. 18th – 22nd, 2024

Source:	vivo

Title:	Discussion on generalization study for RRM prediction

Agenda Item:	8.3.5

Document for:	Discussion and Decision

- Introduction

At the RAN2#127bis meeting [1], RAN2 agreed to start the generalization study at RAN2#128 meeting:

Agreements:

- discuss on what (type of information)/how generalization study can take place in Nov. meeting

Agreements:

- discuss on what (type of information)/how generalization study can take place in Nov. meeting

In this contribution, we will present our analysis of which generalization factors may impact the RRM prediction accuracy performance, and also discuss how we should initiate research on generalization for RRM prediction.

- Discussion

  - Generalization impact factors

For the RRM prediction generalization study, we should first identify what generalization factors may impact the RRM prediction performance. In the following sections, we first discuss common generalization factors related to scenarios and configurations. Then, we consider RRM prediction specific generalization factors, i.e., RRC parameters related to L3 filtering.

    - Common generalization factors

In Rel-18, the following generalization factors are considered for generalization performance verification of AI/ML enhanced beam management [2]:

TR 38.843

Model generalization:

In the context of model generalization, scenarios may mean various deployment scenarios, various outdoor/indoor UE distributions, various UE mobility assumptions. Similarly, configurations may mean various UE parameters, various gNB settings, Various Set B of beam(pairs). The selected scenarios/configurations for generalization verification may consider the AI model inference node (e.g., @UE or @gNB) and use case (e.g., BM-Case1, or BM-Case2). Specifically, the following generalizations could be considered and clause 6.3.2 presents those which have been actually simulated by companies:

-	Scenarios:

-	Various deployment scenarios, e.g., UMa, UMi and others; e.g., 200m ISD or 500m ISD and others; e.g., same deployment, different cells with different configuration/assumption; e.g., gNB height and UE height;

-	Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others

-	Various UE mobility, e.g., 3km/h, 30km/h, 60km/h and others

-	Configurations (parameters and settings):

-	Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions)

-	Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions)

-	Various Set B of beam (pairs)

-	T1 for measurement /T2 for prediction for BM-Case2

-	Other scenarios/configurations(parameters and settings) are not precluded and can be reported

TR 38.843

Model generalization:

In the context of model generalization, scenarios may mean various deployment scenarios, various outdoor/indoor UE distributions, various UE mobility assumptions. Similarly, configurations may mean various UE parameters, various gNB settings, Various Set B of beam(pairs). The selected scenarios/configurations for generalization verification may consider the AI model inference node (e.g., @UE or @gNB) and use case (e.g., BM-Case1, or BM-Case2). Specifically, the following generalizations could be considered and clause 6.3.2 presents those which have been actually simulated by companies:

-	Scenarios:

-	Various deployment scenarios, e.g., UMa, UMi and others; e.g., 200m ISD or 500m ISD and others; e.g., same deployment, different cells with different configuration/assumption; e.g., gNB height and UE height;

-	Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others

-	Various UE mobility, e.g., 3km/h, 30km/h, 60km/h and others

-	Configurations (parameters and settings):

-	Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions)

-	Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions)

-	Various Set B of beam (pairs)

-	T1 for measurement /T2 for prediction for BM-Case2

-	Other scenarios/configurations(parameters and settings) are not precluded and can be reported

Observation 1: The following factors have been studied for the generalization of AI beam management:

- Scenario, including deployment scenarios, UE distributions, UE speeds, etc.

- Configurations, including UE parameters, gNB settings, Set B of beam (pairs), and the T1 for measurement/T2 for prediction.

Since the use cases for Rel-18 AI beam spatial domain and temporal domain prediction are similar to RRM measurement prediction in AI mobility, most of the factors identified for AI beam prediction above will also have an impact on RRM measurement prediction, with the exception of the following ones:

- Various outdoor/indoor UE distributions: Since a 100% outdoor scenario is used in RRM measurement prediction simulations, this factor is not required.

- Various Set B of beam (pairs): This factor will only be evaluated for spatial domain RRM measurement prediction because the assumption of set A = set B is used for temporal domain and frequency domain RRM measurement prediction.

In AI-based beam management, T1 represents the time duration to obtain the measurements for model input, and T2 is the time duration to predict the best beam. These correspond to the observation window (OW) and prediction window (PW), respectively, as defined in RRM prediction. Actually, the T1/T2 generalization study in RRM prediction implies that the number of input and output samples for RRM prediction remains constant. However, the sample period (for the sliding window method) or the measurement period (for the non-sliding window method) can vary. For instance, if the sample period is increased from 20ms to 40ms without changing the dimension of model’s input/output, both the OW length and PW length will double. Furthermore, if an AI model trained by a dataset in a sample period of 20ms is used to perform inferences with a dataset in a sample period of 40ms, its performance may decrease.

Proposal 1: The following factors are considered in the generalization study for RRM prediction:

- Scenarios, including deployment scenarios (e.g., UMa, UMi), ISD (e.g., 500m, 200m), UE speed (e.g., 30/90 km/h for goal 1 and 60/120 km/h for goal 2), gNB height (e.g., 25m, 10m), and UE height (e.g., 1m, 1.5m).

- Configurations (parameters and settings), including gNB settings (e.g., DL Tx beam codebook), UE parameters (e.g., Rx beam number), set B of beam (pairs) for spatial domain prediction, and sample period/measurement period.

    - Specific generalization factors for RRM measurement prediction

In addition to the common generalization factors shared with AI beam management, there may be other specific generalization factors that will impact the performance of RRM measurement prediction. For instance, if RRM use case 2 or use case 3 are employed for RRM measurement prediction, the L3 cell level measurement result will serve as the model input and/or output.

According to the measurement model in TS 38.300, L3 cell level measurement results are derived from the L1 beam level measurements results, through a process of beam selection/consolidation and L3 filtering. Therefore, different configurations for beam consolidation/selection and varying L3 filter coefficients can result in different model inputs/outputs, thereby affecting the model performance.

Figure 2.1-1: Measurement Model in TS 38.300

Considering that the network may configure different beam consolidation/selection parameters and varying L3 filter coefficients for different UEs in different cells, it is necessary to consider these factors to ensure the AI model works well when these factors change.

The following RRC parameters used for RRM measurement prediction simulation can be used as a baseline and additional value(s) can be considered for generalization evaluation:

Table 5.2.1-2 L3 filtering parameter; value. FR1 FilterCoefficient; 4. FR2 FilterCoefficient; 4. Table 5.2.1-3 Measurement period; value. FR1 to FR1 intra-frequency without gap; 200ms. FR1 to FR1 inter-frequency with gap; 200ms. FR2 to FR2 intra-frequency without gap; 400ms. Table 5.2.1-4 Consolidation parameter; value. nrofSS-BlocksToAverage for FR1; 1. nrofSS-BlocksToAverage for FR2; 3. absThreshSS-BlocksConsolidation for FR1; -110dbm. absThreshSS-BlocksConsolidation for FR2; -110dbm.

Proposal 2: For sub-use case 1 of RRM prediction, no need to consider RRC parameters related to L3 filtering in the generalization study.

Proposal 3: For sub-use cases 2 and 3 of RRM prediction, the RRC parameters related to L3 filtering are considered in the generalization study, including:

- L3 filter coefficient (e.g., 0 and 4).

- Beam consolidation threshold for FR2 (e.g., -110 dBm and -100 dBm).

- The number of measurements of SSB to average for FR2 (e.g., 1 and 3).

  - Generalization verification use case

In Rel-18 AI beam management, the following cases are agreed for generalization verification [2]:

TR 38.843

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

-	Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

-	Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

-	Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

-	Notes: Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two.

-	The following case for generalization verification, can be optionally considered by companies:

-	Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

-	Companies to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance.

TR 38.843

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

-	Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

-	Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

-	Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

-	Notes: Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two.

-	The following case for generalization verification, can be optionally considered by companies:

-	Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

-	Companies to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance.

Observation 2: Case 1/2/3 and optional case 2A defined in TR 38.843 are considered to verify the generalization performance of AI beam prediction.

In our view, Case 1 is what we have already done in a fixed scenario and configuration, and Case 2 involves training and inference in different scenarios and/or configurations to assess the impact of generalization factor(s). Finally, Case 3 or Case 2A is used for mixed training or fine-tuning across different scenarios and/or configurations to evaluate the potential gains compared with Case 2, as well as to determine whether there is any performance deterioration compared with Case 1. In fact, the generalization verification cases and procedures can be applied to all AI use cases which RRM measurement prediction can also adopt.

Proposal 4: Reuse the generalization verification cases (i.e., Case 1/2/3 and optional case 2A) for AI beam management from TR 38.843 for RRM prediction generalization verification.

  - The scope of generalization study

As we analyzed above, the generalization verification use cases in Section 2.2 and most of the generalization factors in Section 2.1 apply to all the scenarios for RRM measurement prediction (i.e., temporal domain, frequency domain, spatial domain). However, considering the significant simulation workload required for a generalization study, and the pending evaluation work on measurement event/RLF/system level simulations, we believe that temporal domain RRM measurement prediction should be prioritized over the frequency domain and spatial domain to reduce the workload.

Proposal 5: Generalization study for temporal domain RRM measurement prediction should be prioritized over those for frequency domain and spatial domain.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Observation 1: The following factors have been studied for the generalization of AI beam management:

- Scenario, including deployment scenarios, UE distributions, UE speeds, etc.

- Configurations, including UE parameters, gNB settings, Set B of beam (pairs), and the T1 for measurement/T2 for prediction.

Observation 2: Case 1/2/3 and optional case 2A defined in TR 38.843 are considered to verify the generalization performance of AI beam prediction.

Proposal 1: The following factors are considered in the generalization study for RRM prediction:

- Scenarios, including deployment scenarios (e.g., UMa, UMi), ISD (e.g., 500m, 200m), UE speed (e.g., 30/90 km/h for goal 1 and 60/120 km/h for goal 2), gNB height (e.g., 25m, 10m), and UE height (e.g., 1m, 1.5m).

- Configurations (parameters and settings), including gNB settings (e.g., DL Tx beam codebook), UE parameters (e.g., Rx beam number), set B of beam (pairs) for spatial domain prediction, and sample period/measurement period.

Proposal 2: For sub-use case 1 of RRM prediction, no need to consider RRC parameters related to L3 filtering in the generalization study.

Proposal 3: For sub-use cases 2 and 3 of RRM prediction, the RRC parameters related to L3 filtering are considered in the generalization study, including:

- L3 filter coefficient (e.g., 0 and 4).

- Beam consolidation threshold for FR2 (e.g., -110 dBm and -100 dBm).

- The number of measurements of SSB to average for FR2 (e.g., 1 and 3).

Proposal 4: Reuse the generalization verification cases (i.e., Case 1/2/3 and optional case 2A) for AI beam management from TR 38.843 for RRM prediction generalization verification.

Proposal 5: Generalization study for temporal domain RRM measurement prediction should be prioritized over those for frequency domain and spatial domain.

- Reference

- R2_127bis_ChairNotes

- TR 38.843	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)