3GPP TSG-RAN WG2 Meeting #128	R2-2409736

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	Functionality based LCM

Document for:	Discussion and Decision

1. Introduction

RAN2 has made following agreements and signalling procedure on applicable functionality reporting for beam management UE-sided model:

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

- (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

This contribution aims to address issued regarding applicable functionality reporting, data collection for UE sided training, and functionality continuity.

2. Discussion

Applicable Functionality Report

In the last RAN2 meeting, the content of the applicable functionality reporting and whether inference configuration can be signalled in step (3) were left as FFS

RAN2 #127bis - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. - FFS if inference configuration can be signalled in step3.

According to the RAN1 agreement, RAN1 is discussing applicable functionality reporting with three options. Regarding the contents of the applicable functionality report, it may vary depending on options as follows:

- For option 1, the UE may include CSI-ReportConfig related information

- For option 2, the UE may include a set of inference related parameters

- For option 3, the UE may include information/parameter and/or associated id

Also, regarding inference configuration, RAN2 can have common understanding as follows according to RAN1 agreement:

- The inference configuration includes CSI-ReportConfig.

- In Option 1, CSI-ReportConfig for inference operation is configured in step (3).

  - The functionality is not immediately activated in step (3) with CSI-ReportConfig. When functionality activation can occur remains FFS.

- In Options 2 and 3, it is configured in step (5).

  - The functionality can be activated in step (5) along with CSI-ReportConfig.

RAN1 #118bis agreement: RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration. In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

Since the RAN1 discussion about applicable functionality report is still going with three options, RAN2 need to wait for RAN1 input on the content of applicability reporting and whether inference configuration can be signalled in step (3).

Observation 1. The content of the applicable functionality reporting and whether inference configuration can be signalled in step (3) were left as FFS in the last RAN2 meeting. The FFSs may depend on RAN1’s progress.

Proposal 1. Wait for RAN1 input on the content of applicability reporting and whether inference configuration can be signaled in step (3).

It was also left as FFS whether the UE should explicitly report non-applicable functionality when applicable functionality becomes non-applicable. Implicit notification (simply sending only the applicable functionalities) can be beneficial for simplifying operation. With an implicit approach, it can be more straightforward for the network to interpret, allowing for easier and faster decision-making on the network side. Also, the UE can immediately report applicable functionalities without the extra step of determining and flagging non-applicable ones.

RAN2 #127bis UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality)

Proposal 2. To inform non-applicable functionality, UE reports updated applicable functionality, i.e., NW recognizes non-applicable functionality implicitly

The UE can decide the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. Especially since UE-side additional conditions are difficult for the network to recognize, immediate notification to the network is essential for appropriate actions. If the functionality has not yet been activated, the network should avoid activating it. This approach can also be applied to scenarios where the functionality is already active. In such cases, the network is responsible for deactivating the functionality to ensure precise model operation.

Proposal 3. UE is allowed to report applicable functionality when active functionality becomes non-applicable. NW is responsible for deactivating activated functionality when it recognizes that it is non-applicable

In considering the need for a prohibit timer to limit frequent reporting, a prohibit timer for notifying non-applicable functionality is unnecessary, as immediate notification is required for precise model operation. Likewise, for reports notifying applicable functionality, restricting reporting with a prohibit timer is unnecessary as AIML operation positively impacts the UE’s performance. Also, since the network controls the reports, it can de-configure the reporting configuration if frequent reports are generated. Therefore, no prohibit timer is need for either notifying non-applicable or applicable functionality, and the network should control report frequency if needed.

Proposal 4. No need prohibit timer for applicable functionality reporting via UAI, i.e., Deconfiguration of frequent reporting is under network control

Data collection for UE sided training

Regarding data collection for UE sided training, how the network determines whether data collection should be initiated remains FFS. To this end, RAN2 needs to discuss the configuration for data collection, the initiation and termination of data collection operation, and the data transfer.

RAN2 #127bis Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

RS configuration

In the UE-sided training case, there are four options for data collection: option 1a (OTT solution, 3GPP transparent), option 1b (OTT solution, 3GPP non-transparent), option 2 (CN solution), and option 3 (OAM solution). For the BM use case, the UE requires an RS configuration across all options. Since RS configuration depends on the UE's CSI-related capabilities—such as supported CSI reporting types, CSI-RS resources, antenna ports, and codebook types—these capabilities can be utilized for recognizing required RS configuration in the gNB.

Additionally, the applicable functionality reporting was originally intended to request inference configuration, but if such configuration is already configured, the network can utilize it for RS configuration for training. Note that RAN1 is currently discussing report procedure with three options, Option 1 (report based on CSI reporting configuration), Option 2(report based on inference related parameters), and Option 3(report inference related parameters or associated id). Option 1 and 2 are network configuration-based configuration requests, and Option 3 is a configuration request initiated by UE itself. No matter which option is chosen, the network can recognize the required RS configuration based on applicable functionality reporting.

Proposal 5: gNB can recognize required RS configuration for training based on UE capability/applicable functionality report

Initiation/Termination of Data Collection and Data Transfer

In data transfer options, since options 1a and 1b involve a UP solution, and option 2 involves NAS, decisions on when to initiate/terminate data collection or conduct data transfers may be considered outside RAN2's scope. However, option 3, as an OAM-related solution, could leverage the MDT approach being discussed for network-sided data collection. In network-sided data collection, discussions are ongoing regarding how to configure initiation/termination of data collection on the UE and set up reporting procedures.

For the BM case, if option 3 (OAM solution) is feasible for UE-sided data collection in SA5, this may impact RAN2 in initiation/termination of data collection and data transfer. However, RAN2 needs to discuss how MDT for UE-sided data collection differs from MDT for network-sided data collection. This discussion can help identify any additional considerations that may be necessary for the ongoing discussions regarding network-sided data collection.

Proposal 6. For BM use case, RAN2 understands that only option 3 (OAM solution) for initiation/termination of data collection and data transfer has an impact on RAN2. FFS on if there may be differences compared to network-sided data collection with immediate MDT

Functionality Continuity

For beam management, inference is performed on beams within a cell. In the case of inference, those configurations are significantly influenced by the current cell's CSI settings. Therefore, when a cell change occurs, the applicability exchange steps must be restarted based on the new cell's settings. To this end, the applicable functionality reporting procedure could apply during handover. In Step 3, the HO settings for the target cell could be included, and in Step 4, the applicability for the target cell could be exchanged, allowing the functionality to be quickly activated in the new cell. As a result, the functionality can continue operating after the HO without significant interruption. Therefore, to ensure proper operation during a handover, functionality should be deactivated during the HO process.

Proposal 7. As a baseline in R19, target cell can request applicability reporting during or after handover.

If functionality continuity needs to be maintained, the UE must receive applicable inference configurations from the target cell during handover. To support this, supported or applicable functionality information can be exchanged between gNBs through inter-node signaling, which may include the following information. Supported/Applicable (sub) use case information, CSI reporting configuration, or inference related parameter. RAN2 needs to discuss what information should be exchanged between gNBs to ensure functionality continuity, what inter-node signaling might be required, and whether any impact on RAN3 specifications is necessary.

Proposal 8. If functionality continuity should be supported, RAN2 needs to discuss what information should be exchanged between gNBs to ensure functionality continuity, what inter-node signalling might be required, and whether any impact on RAN3 specifications is necessary

Since the applicability for the target cell can be exchanged during the HO process, there is no need for the serving cell to receive applicable functionality reports for neighboring cells.

Proposal 9. Do not consider applicable functionality reporting for a neighbor cell

3. Conclusion

Observation 1. The content of the applicable functionality reporting and whether inference configuration can be signalled in step (3) were left as FFS in the last RAN2 meeting. The FFSs may depend on RAN1’s progress.

Proposal 1. Wait for RAN1 input on the content of applicability reporting and whether inference configuration can be signaled in step (3).

Proposal 2. To inform non-applicable functionality, UE reports updated applicable functionality, i.e., NW recognizes non-applicable functionality implicitly

Proposal 3. UE is allowed to report applicable functionality when active functionality becomes non-applicable. NW is responsible for deactivating activated functionality when it recognizes that it is non-applicable

Proposal 4. No need prohibit timer for applicable functionality reporting via UAI, i.e., Deconfiguration of frequent reporting is under network control

Proposal 5: gNB can recognize required RS configuration for training based on UE capability/applicable functionality report

Proposal 6. For BM use case, RAN2 understands that only option 3 (OAM solution) for initiation/termination of data collection and data transfer has an impact on RAN2. FFS on if there may be differences compared to network-sided data collection with immediate MDT

Proposal 7. As a baseline in R19, target cell can request applicability reporting during or after handover.

Proposal 8. If functionality continuity should be supported, RAN2 needs to discuss what information should be exchanged between gNBs to ensure functionality continuity, what inter-node signalling might be required, and whether any impact on RAN3 specifications is necessary

Proposal 9. Do not consider applicable functionality reporting for a neighbor cell