3GPP TSG-RAN WG2 Meeting #128		R2-2409765

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:		Discussion on LTM measurement event evaluation and reporting

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #127bis meeting, a bunch of agreements have been agreed for L1 event triggered MR.

However, during the discussions, some proposals are handled without consensus, e.g. TTT operation and measurement RS type alignment for the event evaluation. In this contribution, we would further discuss the following issues:

- The granularity for TTT for candidate cell(s) and serving cell;

- How to align the RS type for candidate cell(s) and serving cell in L1 event evaluation;

- What information needs to be included in the L1 measurement reporting MAC CE.

- Discussion

  - TTT operation

In RAN2 #127bis meeting, the following proposals were discussed[1]:

7. TTT operation per beam or per cell (for a candidate cell)? - Option1: Per beam (P1 in R2-2408260, MediaTek) - Option2: Per cell (P2 in R2-2408525, ZTE) - We will make decision next meeting. 8. TTT operation on current beam changing - Option1: TTT is not reset (P1 in R2-2408613, NEC) - Option2: TTT is reset (P4 in R2-2408948, Nokia) - We will make decision next meeting.

With respect to candidate cell, there are indeed three possible options to operate TTT based on the contributions and online discussions, which are:

- Per beam

- Per cell

- Per CSI-RS set

It is noticeable that in RAN2 #127 meeting we agreed[2]:

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

With this agreement, we understand the event should be evaluated separately for different beams of candidate cell, no matter the beam belongs to the same cell or not. In this sense, it is natural that the TTT should be independent, or in other words, a new TTT should be started to judge whether the beam measurement results satisfy the condition for enough time as long as the quality of a new beam different from current beam whose TTT is running satisfies the corresponding entry conditions. Therefore, if we make the TTT operation per cell, it is not aligned with our previous agreement.

Observation 1: TTT operating per candidate cell is not aligned with RAN2 previous agreement (i.e. Beam level measurement result is used for LTM event evaluation).

As a compromised way, some companies proposed that the TTT could be operated based on per CSI-RS set granularity, which means if an event and corresponding TTT configuration is associated with a CSI-RS resource set, once the candidate beam changes to another one which is inside the same CSI-RS resource set, the TTT is not restarted. However, this would have restriction on network configuration, e.g. only beams belong to a same candidate cell can be configured in a single CSI-RS resource set, which we do not have this kind of restriction on Rel-18 LTM. Also, this seems unnecessarily too flexible and not aligned with the idea of using beam level measurement for LTM event evaluation.

Observation 2: TTT operating per CSI-RS resource set will lead to restriction on network configuration e.g. only beams belong to a same candidate cell can be configured in a single CSI-RS resource set.

Therefore, to make the whole L1 event triggered measurement reporting scheme clean and simple, we prefer to stick to the idea that beam-level operation should be applied, and we propose:

Proposal 1: For candidate cell, the granularity for TTT is per candidate beam (i.e. an ongoing TTT is stopped when the candidate beam is not satisfying the enter/leave condition).

For TTT operation on serving cell, it is also simple that we just follow the beam level TTT. On the other hand, making TTT NOT restarted upon current beam change does have some benefits e.g. allowing the evaluation more in time if the new changed beam is still worse than the candidate beam and with a continued TTT the measurement report can be triggered more quickly.

Observation 3: If TTT is not restarted upon current beam changing, the measurement report can be triggered more quickly if the new changed beam is still worse than the candidate beam.

In this sense we think it is acceptable that we make the TTT not restart for serving cell, therefore we propose:

Proposal 2: For serving cell, an ongoing TTT keeps running when the current beam is changed and the candidate beam is still satisfying the enter/leave condition.

  - RS type alignment

The RS type alignment, i.e. to guarantee the same RS type is used for both serving and neighbouring cell for event LTM3 and event LTM5, was discussed in RAN2 #127bis meeting but no agreement was reached. The options then were as follows[1]:

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement. - Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement. - Option 3: NW provides the offset which can be applied one type of RS to evaluate the different type of RS. For example, If the current beam is SSB and the measured beam for the candidate cell is CSI-RS, the UE applies the configured the offset to the measured CSI-RS value and then compare it with the current beam quality i.e. SSB measurement. - Option 4: Rely on network configuration, i.e. same RS type is configured. - Option 5 (new option proposed by CATT): Which type to be used in measurement will be configured by NW. Then the UE only consider the configured RS type in the measurement. - Option6 (copied from P1, ZTE): The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam. - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). We will decide which option to go next meeting.

Although option 4 seems to be the simplest one, companies have concerns on it because it may introduce unnecessary restriction on network configuration.

It should be pointed out that the similar issue was actually discussed in RAN1 MIMO, as agreements listed below:

- RS determination scheme for the current beam (RAN1 #117) - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - When the QCL RS is CSI-RS for BM, the RS for current beam is the CSI-RS for BM; - When the QCL RS is only TRS, the RS for current beam is the TRS but the RS resource is also included in an NZP-CSI-RS-ResourceSet configured with repetition. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - RS determination scheme for the current beam (RAN1 #118) - The RS(s) for new beam(s) are explicitly configured by RRC. - Assumption of the same RS type is achieved implicitly based on the RS type of new beam(s) (RAN1 #118bis) If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

Observation 4: RAN1 agreed for MIMO that the measurement RS for the current beam is derived from the indicated TCI state of the serving cell, i.e., QCL RS or SSB QCLed with QCL RS, and the measurement RS for the candidate beam is configured by RRC.

Therefore, reusing the above solution for LTM is the simplest way. Thus, either SSB or CSI-RS for beam measurementBM can be configured as measurement RS of the candidate beam, which is included in the LTM-CSI-ResourceConfig and all potential cases are listed as below.

- Case-1: SSB->CSI-RS for BM (beam measurement)->indicated TCI state, SSB configured for candidate cell beam, serving cell beam is SSB; (I.e., in case the QCL RS of indicated TCI state is CSI-RS for BM, and there is a QLCed SSB with it, then if a beam of candidate cell is configured as SSB, then the compared RS in serving cell should be the QLCed SSB with the QCL RS in the indicated TCI state).

- Case-2: SSB->CSI-RS for BM->indicated TCI state, CSI-RS for BM configured for candidate cell beam, serving cell beam is CSI-RS for BM;

- Case-3: CSI-RS for BM->indicated TCI state, CSI-RS for BM configured for candidate cell beam, serving cell beam is CSI-RS for BM;

- Case-4: CSI-RS for BM ->indicated TCI state, SSB configured for candidate cell beam;

- Case-5: SSB->TRS ->indicated TCI state, SSB configured for candidate cell beam, serving cell beam is SSB;

- Case-6: SSB->TRS ->indicated TCI state, CSI-RS for BM configured for candidate cell beam;

- Case-7: TRS ->indicated TCI state, SSB configured for candidate cell beam;

- Case-8: TRS ->indicated TCI state, CSI-RS for BM configured for candidate cell beam, serving cell beam is CSI-RS for BM. In this case, the TRS should be included in an NZP-CSI-ResourceSet configured with trs-info.

The solution applies for most cases except for Case-4 (no QLCed SSB for current beam of CSI-RS), Case-6 (TRS is configured as QCL RS in indicated TCI state) and Case-7. However, these cases can be avoided by the network, e.g., reconfiguring the LTM-CSI-ResourceConfig associated with the LTM-ReportConfig to change the RS from CSI-RS for BM to SSB before TCI state switch for Case-6.

Observation 5: The case-4, 6 and 7 above can be avoided by proper network configuration.

Based on the analysis, we propose to go for option 6 as in RAN2 #127bis meeting discussion, thus:

Proposal 3: Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state.

Proposal 4: It is up to network implementation to ensure that:

- When candidate beam is configured as SSB, for current beam, there exists QLCed SSB with the QCL RS in the indicated TCI state.

- When candidate beam is configured as CSI-RS, for current beam, the QCL RS in the indicated TCI state should be CSI-RS as well.

  - Reporting MAC CE

In RAN2 #127bis meeting, the following agreement was reached:

Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

As for whether to include SSBRI/CRI or LTM configuration id + SSB/CSI-RS id, in our understanding, by using SSBRI/CRI:

- the MAC CE can achieve smaller size. Because if we use LTM configuration id + SSB/CSI-RS id, the indication of LTM configuration ID and corresponding number of reference signals per cell may cost two more octets;

- it seems more flexible as any number of beams in each candidate cell can be reported by this format (e.g. 5 beams for cell 1, 3 beams for cell 2). Otherwise we may only allow e.g. maximum 4 candidates with maximum 4 beams each.

Observation 6: Use SSBRI/CRI as the beam information can achieve a smaller MAC CE size and also more flexible.

Proposal 5: SSBRI/CRI of N beams is used as Beam information in MR MAC CE.

The next question is whether to report beams unsatisfying event condition. To us the motivation is not clear, the similar issue was also discussed in RAN1 when discussing whether UCI or MAC CE is used as the reporting signalling, and at that time as the UCI may be fixed size, it is beneficial to report unsatisfied beams otherwise the space may be wasted. Here as we agreed for MAC CE serving as reporting signalling, then upon UE is ready to perform measurement report, it could request suitable UL resource based on the number of beams satisfying the entry conditions, accordingly, the smart network will assign proper resources for this UE, thus it is questionable why unsatisfied beams should be reported as this information seems useless for network. Therefore,

Proposal 6: Only beams satisfying the event are included in MR MAC CE.

At last, as we agreed that MR MAC CE can include up to N beams and N is configured by network, it is not clear how UE reports/selects beams to be included in the MAC CE when the number of satisfied beams is bigger than N. In this case, as the beams are reported for mobility reason, we think the UE can simply report the best N beams.

Observation 7: It is not clear which beams to be reported when the number of satisfied beams is bigger than N (which is configured by NW).

Proposal 7: If the number of beams satisfying the event is bigger than N (upper bound of reported beam number), UE includes the best N beams in MR MAC CE.

- Conclusion

In this contribution, we discussed TTT operation, RS type alignment in event evaluation and the content of reporting MAC CE. We have the following Observations and Proposals:

Observation 1: TTT operating per candidate cell is not aligned with RAN2 previous agreement (i.e. Beam level measurement result is used for LTM event evaluation).

Observation 2: TTT operating per CSI-RS resource set will lead to restriction on network configuration e.g. only beams belong to a same candidate cell can be configured in a single CSI-RS resource set.

Observation 3: If TTT is not restarted upon current beam changing, the measurement report can be triggered more quickly if the new changed beam is still worse than the candidate beam.

Observation 4: RAN1 agreed for MIMO that the measurement RS for the current beam is derived from the indicated TCI state of the serving cell, i.e., QCL RS or SSB QCLed with QCL RS, and the measurement RS for the candidate beam is configured by RRC.

Observation 5: The case-4, 6 and 7 above can be avoided by proper network configuration.

Observation 6: Use SSBRI/CRI as the beam information can achieve a smaller MAC CE size and also more flexible.

Observation 7: It is not clear which beams to be reported when the number of satisfied beams is bigger than N (which is configured by NW).

Proposal 1: For candidate cell, the granularity for TTT is per candidate beam (i.e. an ongoing TTT is stopped when the candidate beam is not satisfying the enter/leave condition).

Proposal 2: For serving cell, an ongoing TTT keeps running when the current beam is changed and the candidate beam is still satisfying the enter/leave condition.

Proposal 3: Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state.

Proposal 4: It is up to network implementation to ensure that:

- When candidate beam is configured as SSB, for current beam, there exists QLCed SSB with the QCL RS in the indicated TCI state.

- When candidate beam is configured as CSI-RS, for current beam, the QCL RS in the indicated TCI state should be CSI-RS as well.

Proposal 5: SSBRI/CRI of N beams is used as Beam information in MR MAC CE.

Proposal 6: Only beams satisfying the event are included in MR MAC CE.

Proposal 7: If the number of beams satisfying the event is bigger than N (upper bound of reported beam number), UE includes the best N beams in MR MAC CE.

- References

- RAN2 #127bis meeting chair notes;

- RAN2 #127 meeting chair notes;